3GPP TSG-RAN WG3 Meeting #125bis                                                             R3-245255

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Impact to NG interface for 5G Femto

Agenda Item:	12.3

Document for:	Discussion and Decision

1 Introduction

In the last meeting, additional topological enhancements for NR are studied, and the conclusion and recommendation for 5G Femto is produced [1].

The objectives of the 5G Femto work are as follows:

The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

In this contribution, we will discuss the impact on NG interface for 5G Femto.

2 Discussion

In the TR [1], the architecture options for NG interface have been shown. Option 1 and option 2 are agreed in the last meeting. In option 1, the NR Femto node connects to the 5GC directly as a gNB by means of the NG interface. In option 2, the NG-RAN architecture deploys the NR Femto Gateway (NR Femto GW) to allow the NG interface between the NR Femto node and the 5GC.The only difference between option 1 and option 2 is the deployment of NR Femto GW. So, option 1 is a special case of option 2, which does not deploy NR Femto GW.

In architecture with the NR Femto GW for NG interface, the NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same regardless of whether the NR Femto node is connected to the 5GC via an NR Femto GW or not. It appears that the introduction of the 5G Femto architecture does not affect the signaling of the NG setup procedure between the NR Femto node and the NR Femto GW.

The NR Femto GW can connect to multiple AMFs. AMFs and Femto nodes transmit NG signal via the NR Femto GW which they connect with. The AMF needs the NR Femto GW to transfer NGAP signal of a specific UE to NR Femto, and the Femto nodes also need the GW to transfer NGAP signal to AMF. Therefore, how to find path is an issue that needs to be solved in 5G Femto.

In LTE, the HeNB is specified as in TS 36.300 [2]. The architecture of the 5G Femto node deployment is similar with the specified in TS 36.300 section 4.6.1 Architecture. GUMMEI IE indicates the globally unique MME identity. In TS 36.300, upon reception of the GUMMEI from a UE, the HeNB shall include it in the INITIAL UE MESSAGE message. HeNB Signals the GUMMEI of the Source MME to the HeNB GW in the S1 PATH SWITCH REQUEST message. The HeNB GW routes the S1 PATH SWITCH REQUEST message towards the MME based on the GUMMEI of the source MME received from the HeNB.

In TS 38.413 [3], 5G-S-TMSI IE has similar functionality. The IE includes 5G-TMSI, which is unique within the AMF that allocated it. And the IE also includes AMF Set ID IE and AMF Pointer IE, which can indicate a special AMF.

Therefore, 5G-S-TMSI IE can be reused by the Femto GW to select AMF for UE.

Proposal 1: 5G-S-TMSI IE sent from the Femto node in NGAP messages can be used to select AMF for UE.

In TS 36.413 [4], the MME UE S1AP ID 2 IE is used to indicate the MME UE S1AP ID assigned by the MME. The information element is included in the INITIAL CONTEXT SETUP REQUEST message, the HANDOVER REQUEST message, and the PATH SWITCH REQUEST ACKNOWLEDGE message. If HeNB architecture uses those messages, the IE will be mandatory. And it is used for subsequent X2 handovers

Since there is no existing IE with similar function in TS 38.413, a new IE needs to be added to the corresponding messages for subsequent Xn handover of UE from the Femto node to other NG-RAN nodes.

Observation: INITIAL CONTEXT SETUP REQUEST message, HANDOVER REQUEST message and PATH SWITCH REQUEST ACKNOWLEDGE message should introduce a new IE to indicate the AMF UE NGAP ID allocated by AMF for subsequent Xn handover.

Since the MME UE S1AP ID IE is similar with the AMF UE NGAP ID IE in TS 38.413, and the new IE indicates the AMF UE NGAP ID assigned by the AMF, the new IE in stage-3 spec can be named AMF UE NGAP ID 2 IE, which is added into the INITIAL CONTEXT SETUP REQUEST message, the HANDOVER REQUEST message, and the PATH SWITCH REQUEST ACKNOWLEDGE message for subsequent Xn handovers.

Proposal 2: The new IE AMF UE NGAP ID 2 is introduced to the INITIAL CONTEXT SETUP REQUEST message, HANDOVER REQUEST message and PATH SWITCH REQUEST ACKNOWLEDGE message to indicate the AMF UE NGAP ID assigned by the AMF.

3 Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1: 5G-S-TMSI IE sent from the Femto node in NGAP messages can be used to select AMF for UE.

Observation: INITIAL CONTEXT SETUP REQUEST message, HANDOVER REQUEST message and PATH SWITCH REQUEST ACKNOWLEDGE message should introduce a new IE to indicate the AMF UE NGAP ID allocated by AMF for subsequent Xn handover.

Proposal 2: The new IE AMF UE NGAP ID 2 is introduced to the INITIAL CONTEXT SETUP REQUEST message, HANDOVER REQUEST message and PATH SWITCH REQUEST ACKNOWLEDGE message to indicate the AMF UE NGAP ID assigned by the AMF.

4 Reference

- R3-244849, 3GPP TR 38.799,  Study on additional topological enhancements for NR, v1.2.0 (2024-08)

- 3GPP TS 36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 18), V18.2.0 (2024-06)

- 3GPP TS 38.413, NG-RAN, NG Application Protocol (NGAP), V18.2.0 (2024-06)

- 3GPP TS 36.413, Evolved Universal Terrestrial Radio Access Network(E-UTRAN); S1 Application Protocol (S1AP) (Release 18), V18.2.0 (2024-06)