3GPP TSG-RAN WG3 Meeting #125bis                                          R3-245256

Hefei, CN, Oct 14th – 18rd October, 2024

Source:	CATT

Title:	On support on-demand SSB SCell operation

Agenda Item:	17.2

Document for:	Discussion and Decision

Introduction

For NES on-demand SSB SCell operation topic, the WID[1] content is as follows:

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In previous RAN1 meetings, some agreements are achieved on various aspects related to on-demand SSB SCell, the agreements are as follows:

Agreement - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster Agreement Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4

In this contribution, we will continue to discuss the on-demand SSB SCell operation topic, our views will be given as well.

Discussion

2.1 On-demand SSB initiation and configuration

RAN1 has discussed different scenarios for Case#1 (no always-on SSB) and Case#2 (with always-on SSB periodically transmitted) for on-demand SSB. If the cell applies on-demand SSB, the SSB transmission opportunities are reduced and energy consumed by SSB transmission can be saved.

Considering on-demand SSB is designed for low traffic load, it’s not suitable to pre-configure on-demand SSB SCells, as the traffic load is variable. Thus, the cells to apply on-demand SSB should not depend on OAM configuration, it would be gNB-DU or gNB-CU to decide. Since SSB transmission is controlled and performed by gNB-DU, the energy used for SSB transmission is consumed by gNB-DU. Thus, the gNB-DU (instead of the gNB-CU) can be aware of energy consumed for SSB transmission. In addition, most of the energy used for base band processing are consumed by gNB-DU, so the energy efficiency of a cell can be obtained by gNB-DU. If the energy efficiency of the NES cell is low due to common signal transmission, e.g., when the user data rate is low or few UEs are connected to the cell, and the cell supports on-demand SSB operations, gNB-DU can apply on-demand SSB to the NES cell. When on-demand SSB is applied, SSB transmissions are stopped or decreased. The gNB-DU can also decide to not apply on-demand SSB due to increase of cell load. So, NES gNB-DU can initiate applying on-demand SSB.

In split architecture, gNB-DU is responsible for SCell configuration including configuration of SSB of the SCell to UE, so we assume the on-demand SSB SCell configuration for UE, including the configuration for on-demand SSB, should also be carried out by gNB-DU.

Besides the on-demand SSB SCell configuration, the cell applying on-demand SSB should support the on-demand SSB operations which are realized by MAC-CE or DCI signaling. The MAC CE or DCI is transmitted by gNB-DU. gNB-CU has no knowledge about whether a cell supports on-demand SSB SCell operations or not and thus cannot know whether to configure the on-demand SSB. So, on-demand SSB SCell should be configured by gNB-DU. There is no F1AP impact.

Observation 1: NES gNB-DU initiates on-demand SSB and takes the role of configuring on-demand SSB SCell.

2.2 Interaction over F1 interface

Whether NES gNB-CU needs to know the information of on-demand SSB

In the last RAN1 meeting, RAN1 have the following agreement about whether on-demand SSB operation applies to CD-SSB or NCD-SSB:

Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported. - On-demand SSB on CD-SSB or NCD-SSB the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB. FFS: Additional support of OD-SSB for CD-SSB located on sync-raster.

RAN1 has agreed that on-demand SSB on the cell is NCD-SSB and the additional support of on-demand SSB for CD-SSB located on sync-raster is FFS.

As CD-SSB is required for configuring PCell/PSCell and on-demand SSB is not supported by PCell/PSCell operation, if CD-SSB applies on-demand SSB on the cell, the cell is not suitable to be used as PCell/PSCell. The gNB-CU should know that information and prevent to configure such kind of cells as SpCell to UEs. Furthermore, UE will not be able to camp on the cell, and therefore the network coverage is changed due to the NES cell applying on-demand SSB to CD-SSB. So gNB-CU should know that a cell is applying on-demand SSB if on-demand SSB is used for CD-SSB, to decide the coverage of the cell.

On the other hand, if on-demand SSB on a cell is used by NCD-SSB only, coverage of the cell is not impacted for on-demand SSB, and the cell is still possible to be configured as UEs’ PCell/PSCell. NES gNB-CU does not need to know whether the cell is applying on-demand SSB.

Observation 2: If on-demand SSB is only used to NCD-SSB, NES gNB-CU would not need to know whether on-demand SSB is applied.

RAN1 has the conclusion that on-demand SSB can be used for L1 and/or L3 measurement. However, according to the WID[1], the on-demand SSB operations only target to SCell. Thus on-demand SSB based L1/L3 measurement should only be applied to SCell. That means on-demand SSB is not used for the neighbor cell measurements. In other words, gNB-CU should only configure on-demand SSB as the objective of SCell cell measurement.

Observation 3: NES gNB-CU should only configure on-demand SSB as the objective of SCell measurement, but not configure on-demand SSB as the objectives for neighbor cell measurements.

Therefore, the gNB-CU should be indicated by gNB-DU whether a SSB of gNB-DU’s cell is on-demand SSB or not. Note that the inter-node RRC message MeasurementTimingConfiguration defined in TS 38.331[3] is used for gNB-DU to convey the configurations of SSB/CSI-RS based measurement objectives to gNB-CU over F1 interface. The MeasurementTimingConfiguration RRC message can be used for gNB-DU indicating whether a configuration of SSB based measurement is for on-demand SSB.

Proposal 1: To make sure the gNB-CU not to configure on-demand SSB as the objectives for neighbor cell measurements, the gNB-DU needs to indicate the gNB-CU whether a SSB based measurement configuration is for on-demand SSB.

Whether gNB-CU needs to assist on decision of on-demand SSB activation

As discussed above, if CD-SSB applies on-demand SSB on the cell, the cell is not suitable to be used as PCell/PSCell and the cell coverage is impacted. The gNB-CU should know that information and needs to assist gNB-DU for deciding the cells to apply on-demand SSB. For example, gNB-CU can send the allowed cells list to gNB-DU for applying on-demand SSB to CD-SSB.

However, if on-demand SSB on a cell is NCD-SSB only, coverage of the cell is not impacted for on-demand SSB and the cell is still possible to be configured as UEs’ PCell/PSCell. In that case, gNB-DU can decide whether to apply on-demand SSB by itself, i.e. gNB-CU does not need to assist the gNB-DU to decide which cell to apply on-demand SSB. Nevertheless, as RAN1 has not conclude whether CD-SSB supports on-demand SSB, RAN3 should wait for RAN1’s conclusion on the issue of gNB-CU assisting gNB-DU on decision of on-demand SSB activation.

Observation 4: On the issue gNB-CU assisting gNB-DU to decide on-demand SSB activation, RAN3 wait for conclusion of RAN1 on whether CD-SSB supports on-demand SSB.

2.3 Interaction over Xn interface

As the cell coverage is determined by coverage of CD-SSB, if CD-SSB applies on-demand SSB, the cell coverage is not available since UE cannot camp on or handed over to the cell. There would be a need for NES gNB to inform the neighbour nodes on deactivation of the NES cell, that the neighbour cell can adjust the coverage to take the area of the NES cell for ensuring UE’s cell selection. But that can rely on existing Xn procedure for cell deactivation. On the other hand, when the CD-SSB transmission is resumed to normal SSB transmission, the NES gNB would need to inform the neighbour nodes about the activation and new coverage of the NES cell. That can rely on existing Xn procedures for exchanging coverage change between NG-RAN nodes. Therefore, the cell deactivation or coverage changes caused by applying on-demand SSB for CD-SSB can be informed to neighbour nodes via existing Xn procedure. So, there would be no impact to Xn procedure even if the CD-SSB applies on-demand SSB.

However, since the SSB applying on-demand SSB mode cannot be configured as objective of neighbor cell measurement, the neighbor nodes should not configure the on-demand SSB as a measurement objective.

Observation 5: The neighbor nodes of NES gNB should not configure an on-demand SSB based measurement.

To prevent the neighbor nodes configuring the on-demand SSB of NES cell as a measurement objective, one option is that the NES gNB does not convey the measurement configuration based on the on-demand SSB to the neighbor nodes over Xn, another option is that the NES gNB indicates to the neighbor nodes whether a SSB based measurement configuration is for on-demand SSB or not. Since the inter-node message MeasurementTimingConfiguration defined in TS 38.331 is used for exchanging the measurement configurations over Xn interface. If the latter solution is adopted, the MeasurementTimingConfiguration message can be used for the NES gNB indicating whether a configuration of SSB based measurement is for on-demand SSB.

Proposal 2: For neighbor nodes not to configure an on-demand SSB based measurement to NES cell, NES gNB does not convey measurement configuration based on on-demand SSB over Xn or it indicates the neighbour NG-RAN nodes whether a SSB based measurement configuration is for on-demand SSB.

Conclusion

According to the analysis in section 2, we have following observations and proposals:

Observation 1: NES gNB-DU initiates on-demand SSB and takes the role of configuring on-demand SSB SCell.

Observation 2: If on-demand SSB is only used to NCD-SSB, NES gNB-CU would not need to know whether on-demand SSB is applied.

Observation 3: NES gNB-CU should only configure on-demand SSB as the objective of SCell measurement, but not configure on-demand SSB as the objectives for neighbor cell measurements.

Proposal 1: To make sure the gNB-CU not to configure on-demand SSB as the objectives for neighbor cell measurements, the gNB-DU needs to indicate the gNB-CU whether a SSB based measurement configuration is for on-demand SSB.

Observation 4: On the issue gNB-CU assisting gNB-DU to decide on-demand SSB activation, RAN3 wait for conclusion of RAN1 on whether CD-SSB supports on-demand SSB.

Observation 5: The neighbor nodes of NES gNB should not configure an on-demand SSB based measurement.

Proposal 2: For neighbor nodes not to configure an on-demand SSB based measurement to NES cell, NES gNB does not convey measurement configuration based on on-demand SSB over Xn or it indicates the neighbour NG-RAN nodes whether a SSB based measurement configuration is for on-demand SSB.

Reference

- RP-234065, New WID: Enhancements of network energy savings for NR, RAN#103, Ericsson

- Chair notes RAN1#118 eom4

- 3GPP TS 38.331 V18.0.0