3GPP TSG-RAN3 Meeting #125bis	R3-245258

# Hefei, China, 14th-18th Oct, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1377 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on IAB TMM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2024-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - As specified in 38.401, the F1-terminating IAB-donor shall request the non-F1 IAB-donor to release all offloaded traffic and backhaul resources in case the IAB-node is “not authorized”, using IAB TMM Request message. However, according to present 423 spec, the behavior for release of BH resources is not specified in the IAB TMM procedure. - As the non-F1 donor needs to release both the offloaded traffic and BH resources in some cases, but only needs to release the offloaded traffic in other cases, it’s necessary to specify in which cases the non-F1 donor should release BH resources and in which cases not: In most cases, the non-F1 donor receives the authorization status from the 5GC. Thus, it can decide whether to release the backhaul resources or not according to the authorization status of IAB-node, upon reception of the all-offload-traffic release request. However, in case MT is NR dual-connected where MN is the F1-terminating donor and SN is non-F1-terminating donor, the non-F1-terminating donor does not receive the authorization status from 5GC. It cannot decide whether to release BH resources upon receiving the request of all offloaded traffic release based on the authorization status. Two feasible solutions are observed: Solution 1: All Traffic Indication IE in the Traffic to Be Released Information IE is used for indicating the non-F1 donor to release both offloaded traffic and BH resources. Solution 2: MN can indicate the authorization status of IAB-node received from 5GC to SN (i.e., non-F1 donor) via a S-NODE MODIFICATION REQUEST message, that the non-F1 donor can always be aware of the authorization status of IAB-node. Note that the corresponding correction to stage-2 spec would be necessary. - In present 38.423, the statement “the non-F1-terminating IAB-donor shall include the Traffic Released List IE in the IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message” is incorrect. Because maybe no offloaded traffic is to be released if the request of release is sent from the F1-terminating donor due to “non-authorized” status. The non-F1 IAB-donor should not include this IE if no offloaded traffic is released. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The behavior of non-F1 IAB-donor in IAB TMM procedure upon receiving the the Traffic To Be Released Information IE in IAB TMM Request message: - In case the All Traffic Indication IE is set to “true”, the non-F1-terminating IAB-donor should also release all backhaul resources if [the non-F1-terminating IAB-donor is SN of the IAB-node or] the authorization status of the IAB-node is “not authorized”. - The non-F1 IAB-donor shall include the Traffic Released List IE in the IAB TMM RESPONSE message only if there is offloaded traffic released. The description of Traffic To Be Released Information IE is changed to: “This IE is used to indicate the offloaded traffic to be released, or indicate to release all the offloaded traffic and backhaul resources.” Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, the impact can be considered isolated because it only impacts the IAB TMM procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure specified in 38.401 for IAB-node authorization cannot be supported. Or the description of the IAB TMM is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2.2, 9.2.2.84 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

-------------------------------------------Start of change-------------------------------------------

8.5.2.2	Successful Operation

Figure 8.5.2.2-1: IAB Transport Migration Management triggered by the F1-terminating IAB-donor, successful operation

The F1-terminating IAB-donor initiates the procedure by sending the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the non-F1-terminating IAB-donor.

The non-F1-terminating IAB-donor may respond with the IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message by indicating:

-	Traffic accepted for offloading, within the Traffic Added List IE;

-	Already offloaded traffic accepted for modification, within the Traffic Modified List IE;

-	Traffic not accepted for offloading, within the Traffic Not Added List IE;

-	Already offloaded traffic not accepted for modification within the Traffic Not Modified List IE.

If the Traffic To Be Released Information IE is contained in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message, the non-F1-terminating IAB-donor should release all offloaded traffic (if any) if the All Traffic Indication IE in the Traffic to Be Released Information IE is set to "true", or release only the offloaded traffic indicated by the Traffic to Be Released Item IE in the Traffic to Be Released Information IE. In case the All Traffic Indication IE is set to “true”, the non-F1-terminating IAB-donor should also release all backhaul resources if [the non-F1-terminating IAB-donor is SN of the IAB-node or] the received authorization status of the IAB-node is “not authorized” as specified in TS 38.401 [2].

If the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message contains the Traffic to Be Released Information IE, and there is offloaded traffic released, the non-F1-terminating IAB-donor shall include the Traffic Released List IE in the IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message.

If the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message contains the IAB IPv4 Addresses Requested IE or the IAB IPv6 Request Type IE in the IAB TNL Address Request IE, the non-F1-terminating IAB-donor shall, if supported, provide the allocated TNL address via the IAB TNL Address Response IE in the IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message. If the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message contains the IAB TNL Address To Remove List IE in the IAB TNL Address Request IE, the non-F1-terminating IAB-donor shall consider that the TNL address(es) are no longer used by the F1-terminating IAB-donor.

If the IAB TNL Address Exception IE is contained in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message, the non-F1-terminating IAB-donor shall, if supported, configure the related IAB-donor-DU to enable traffic re-routing over the inter-IAB-donor-DU tunnel.

If the IAB QoS Mapping Information IE is contained in the IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message, the F1-terminating IAB-donor, shall, if supported, use it to set DSCP and/or IPv6 flow label fields for the downlink IP packets of the offloaded traffic.

-------------------------------------------Next change-------------------------------------------

9.2.2.84	Traffic To Be Released Information

This IE is used to indicate the offloaded traffic to be released or used to request releasing all offloaded traffic and backhaul resources. This IE is only applicable to IAB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Traffic Release type | M |  |  |  |
| >Full Release |  |  |  |  |
| >>All Traffic Indication | M |  | ENUMERATED (true, ..) |  |
| >Partial Release |  |  |  |  |
| >>Traffic To Be Released List |  | 1 |  |  |
| >>>Traffic To Be Released Item IE |  | 1 .. <maxnoofTrafficIndexEntries> |  |  |
| >>>>Traffic Index | M |  | 9.2.2.80 |  |
| >>>>BH Info List | O |  | 9.2.2.99 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTrafficIndexEntries | Maximum no. of traffic offloaded to the non-F1-terminating IAB-donor. The value is 1024. |

-------------------------------------------End of change-------------------------------------------