3GPP RAN WG3 Meeting #125bis	R3-245266

# Hefei, China, 14th – 18th Oct 2024

Source:	CATT, Huawei, CMCC, NEC

Title:	Discussion on ENB Configuration Update procedure

Agenda Item:	9.1

Document for:	Discussion

- Introduction

During last RAN3 meeting, it was discussed how an eNB owning a dormant cell indicates to the neighbour node that the energy saving cell is re-activated. Additionally, companies discussed on the behaviour of sending node and receiving node during ENB Configuration Update procedure while no consensus was reached. In this contribution, we make some analysis on this issue and provide our proposals accordingly.

- Discussion

2.1 Background

During last RAN3 meeting, a joint CR[1] was submitted proposing to add description on how to indicate the reactivation of dormant cells. Unfortunately, no consensus was achieved. The objection raised was that the existing specifications already detail the receiving node overwriting serving cell information, rendering additional clarification unnecessary. However, another specification statement indicated that the receiving node updates serving cell information based on the information included in Served Cell Information IE, leading to apparent discrepancies. Consequently, companies agreed to conduct further internal reviews on this matter.

Upon reviewing the ENB Configuration Update message structure, we identified several IEs under the Cell to Modify IE, namely Deactivation Indication, Serving Cell Information, Neighbor Information, and NR Neighbor Cell Information. In the following paragraphs, we will analyze each of these IEs in detail.

2.2 Handling of Deactivation Indication IE

For Deactivation Indciation IE,as statd above,it is directly nested under the Cell to Modify IE, which operates at the same level as the Serving Cell Information IE. Therefore, the handling of the Deactivation Indication IE should adhere to the same principles as optional IEs within similar structures, such as other XXX to Modify IE. Essentially, unless explicitly stated otherwise, the absence of an optional IE indicates that its information remains unchanged.

Observation 1:Deactivation Indication IE is an Optional IE under Cell to Modify IE which is independent with the handling of Serving cell Information IE.

Upon examining the statement regarding the Deactivation Indication IE,,there is only description on the behaviour of eNB when this IE is present while no description on the absence of this IE, i.e. If the Deactivation Indication IE is contained in Served Cells To Modify IE, it indicates that the concerned cell was switched off to lower energy consumption. Given the necessity to indicate the reactivation of dormant cells and the fact that only one codepoint is defined for the Deactivation Indication IE, leveraging its absence to signify cell reactivation appears as a simple way. Furthermore, in the EN-DC scenario, the absence of the NR Deactivation Indication IE clearly indicates the reactivation of NR dormant cells. Therefore, adopting a similar mechanism for the ENB-ENB scenario seems reasonable. However, the current specifications lack clarity regarding the implications of the IE's absence, which could typically lead the receiving node to retain unchanged information.

Proposal 1: Add description in Deactivation Indication IE that If this IE is not included, indicates that the concerned cell is activated.

2.3 Handling of Serving cell Information and Neighbour cell Information

On the handling of Serving cell Information IE and Neighbour cell Information IE,it seems there are some statements in the spec which may not align with each other as below:

Upon reception of an ENB CONFIGURATION UPDATE message, eNB2 shall update the information for eNB1 as follows:

Update of Served Cell Information:

-	If Served Cells To Add IE is contained in the ENB CONFIGURATION UPDATE message, eNB2 shall add cell information according to the information in the Served Cell Information IE.

-	If Served Cells To Modify IE is contained in the ENB CONFIGURATION UPDATE message, eNB2 shall modify information of cell indicated by Old ECGI IE according to the information in the Served Cell Information IE.

When either served cell information or neighbour information of an existing served cell in eNB1 need to be updated, the whole list of neighbouring cells, if any, shall be contained in the Neighbour Information IE.

The eNB2 shall overwrite the served cell information and the whole list of neighbour cell information for the affected served cell.

-	If Served Cells To Delete IE is contained in the ENB CONFIGURATION UPDATE message, eNB2 shall delete information of cell indicated by Old ECGI IE.

According to the above highlighted yellow sentence, eNB2 shall only modify information which is included in the Served Cell Information IE whereas keep all other information unchanged. While in the subsequent green-marked sentences, it is stated that the receiving node shall overwrite the served cell information and the whole list of neighbour cell information for the affected served cell. It seems there is discrepancy between the statements.

To gain clarity on the situation, we delved deeper into the historical context of these statements. It was found that the yellow-highlighted sentence has been present since version 8.3.0 via CR R3-082164[2] whereas the green-highlighted sentences were newly introduced in V8.5.0 via CR R3-090521[3]. The initial intention of the CR R3-090521 is to avoid the mandatory inclusion of serving cell information and all neighbor cell list in ENB CONFIGURAITON UDPATE message in case there are any change in serving cell or neighbour cell information. And the original proposal of the CR is to change the presence of Served Cell Information IE from M to O and to replace the Neighbour Information IE with Neighbour Cells to Add IE, Neighbour Cells to Modify IE and Neighbour Cells to Delete IE as below.

| Served Cells To Modify |  | 0 to maxCellineNB |  |  | GLOBAL | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >Old ECGI | M |  | ECGI 9.2.14 | This is the old E-UTRAN Cell Global Identifier | - | - |
| >Served Cell Information | O |  | 9.2.8 |  | – | – |
| >Neighbour Information To Add |  | 0 to maxnnofNeighbours |  |  |  |  |
| >>ECGI | M |  | ECGI 9.2.14 | E-UTRAN Cell Global Identifier of the neighbour cell | – | – |
| >>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the neighbour cell | – | – |
| >>EARFCN | M |  | 9.2.26 | DL EARFCN for FDD and EARFCN for TDD | – | – |
| >Neighbour Information To Modify |  | 0 to maxnnofNeighbours |  |  |  |  |
| >>ECGI | M |  | ECGI 9.2.14 | E-UTRAN Cell Global Identifier of the neighbour cell | – | – |
| >>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the neighbour cell | – | – |
| >>EARFCN | M |  | 9.2.26 | DL EARFCN for FDD and EARFCN for TDD | – | – |
| >Neighbour Information To Delete |  | 0 to maxnnofNeighbours |  |  |  |  |

However,after online and offline dicussions, the final conclusion was contrary to the initial proposal i.e. the final version of the CR mandates that the sending node shall always include Neighbour Information IE for all the neighbour cells in case there is any update on either serving cell information or neighbour cell information and the receiving node should overwrite the previous information.

Based on the history of discussion, it becomes evident that the green-highlighted sentences and the yellow-highlighted sentence address distinct aspects.The former imposed constraints on the inclusion of Served Cell Information IE and Neighbour Information IE for all neighbour cell list whereasthe the latter specifically outlines the management of sub-IEs within Served Cell Information IE.

Observation 2:The green highlighted statement aims to clarify that,upon any update on either serving cell information or neighbour cell information ,the sending node shall always include Served Cell Information IE and Neighbour Information IE for all neighbour cell list.Nevertheless,it does not mandate the inclusionof all sub-IEs within Served Cell Information IE and Neighbour Information IE.

Observation 3: The yellow-highlighted statement clarifies whether/how the receiving node modify the information based on the sub-IEs present within the Served Cell Information IE.

Based on observation 1&2, one question arises regarding the behaviour of the receiving node if certain optional IEs within Served Cell Information IE and Neighbour Information IE are absent.

Looking at the optional sub-IEs within Served Cell Information IE,all of the IEs exhibit a broader range of potential values rather than being limited to  just two options. For instance, the optional Number of Antenna Ports IE specifies the number of cell specific antenna ports which can vary between one, two or four. When this IE is present in X2 SETUP procedure but omitted from the ENB CONFIGURAIOTN UPDATE procedure, the receiving node can not infer a specific value based solely on its absence, unlike category 1 IEs. Futhermore, according to current specification,i.e. eNB2 shall modify information of cell indicated by Old ECGI IE according to the information in the Served Cell Information IE., the receiving node should just keep the information unchanged.

Proposal 2: It is proposed for RAN3 to confirm that the receiving node keep the information unchanged in case the optional IE in Served Cell Information IE and Neighbour Information IE is absent.

Proposal 2a:The similar principle applied to both LTE and NR.

2.4 Handling of NR Neighbour Information IE

In Rel-15, with the introduction of NR, an NR cell can also serve as a neighbor cell of an NG-RAN node. Consequently, an NR Neighbor Cell IE has been introduced, which contains a list of NR neighbor cells for the cell indicated by the Old ECGI IE. However, there is ambiguity in the specification regarding whether the sending node should always include the whole list of NR neighbor cells. For instance, if there is no change in NR neighbor cell information, it is unclear whether the sending node should include the entire list or omit it. Similarly, the receiving node faces an issue: if the NR Neighbor Information IE is absent, should it consider the NR neighbor cell unchanged or assume no NR neighbor cell is available?

Proposal 3: It is proposed that RAN3 discuss the behavior of the receiving node when NR Neighbor Information IE is not available, as well as whether the sending node should always include the complete list of NR Neighbor Cell Information.The spec could be updated after consensus is reached.

Additionally, an NR cell has both NR neighbor cells and E-UTRAN neighbor cells. Currently, there is only a description stating that the NG-RAN node should provide the full list of NR neighbor cells to the peer node, and the peer node should overwrite the neighbor cell list instead of using delta configuration. However, there is no corresponding description for E-UTRAN neighbor cells. Therefore, it is equally necessary to address the handling of E-UTRAN neighbor cells for NR.

Proposal 3a: Similar to Proposal 3, it is proposed that RAN3 discuss the behavior of the receiving node when E-UTRAN Neighbor Information IE is not available, as well as whether the sending node should always include the complete list of E-UTRAN Neighbor Cell Information. The spec could be updated after consensus is reached

- Conclusion

According to the analysis above, the observations and proposals are provided as below:

Observation 1:Deactivation Indication IE is an Optional IE under Cell to Modify IE which is independent with the handling of Serving cell Information IE.

Proposal 1: Add description in Deactivation Indication IE that If this IE is not included, indicates that the concerned cell is activated.

Observation 2:The green highlighted statement aims to clarify that,upon any update on either serving cell information or neighbour cell information ,the sending node shall always include Served Cell Information IE and Neighbour Information IE for all neighbour cell list.Nevertheless,it does not mandate the inclusionof all sub-IEs within Served Cell Information IE and Neighbour Information IE.

Observation 3: The yellow-highlighted statement clarifies whether/how the receiving node modify the information based on the sub-IEs present within the Served Cell Information IE.

Proposal 2: It is proposed for RAN3 to confirm that the receiving node keep the information unchanged in case the optional IE in Served Cell Information IE and Neighbour Information IE is absent.

Proposal 2a:The similar principle applied to both LTE and NR.

Proposal 3: It is proposed that RAN3 discuss the behaviour of the receiving node when NR Neighbor Information IE is not available, as well as whether the sending node should always include the complete list of NR Neighbor Cell Information. The spec could be updated after consensus is reached.

Proposal 3a: Similar to Proposal 3, it is proposed that RAN3 discuss the behaviour of the receiving node when E-UTRAN Neighbor Information IE is not available, as well as whether the sending node should always include the complete list of E-UTRAN Neighbor Cell Information. The spec could be updated after consensus is reached

The corresponding CR is provided in [4]

- Reference

[1] R3-244507  Clarification on indication of dormant cell reactivation  CATT, Nokia, Huawei, CMCC

[2] R3-082164  GU Group Id List in eNB Configuration Update  Nokia Siemens Networks, Nokia

[3] R3-090521 Modification to ENB CONFIGURATION UPDATE message   CATT, NEC

[4]R3-245267 Clarification on indication of dormant cell reactivation CATT,Nokia,Huawei, CMCC,ZTE,NEC,Samsung