3GPP RAN WG3 Meeting #125bis	R3-245268

# Hefei, China, 14th – 18th Oct 2024

Source:	CATT, Deutsche Telekom, China Unicom

Title:	Discussion on definition of UE performance metrics

Agenda Item:	9.2

Document for:	Discussion and decision

- Introduction

This document discusses the definition of UE performance metrics introduced in the Rel-18 AI/ML WI

- Discussion

In RAN3#124 meeting, regarding how to derive UE level UE performance feedback, a common understanding is reached as below:

UE level performance is derived as the average value of per DRB level measurements of the UE

In RAN3#125 meeting, a joint CR was submitted to capture the above common understanding into spec. In the CR ,the following note was introduced:

Note：The value of each IE results from averaging the corresponding DRB level measurements over all configured DRBs in the UE

However, the CR was not agreed as there was concern that it may have potential impact on the finer granularity UE performance feedback which is to be discussed in REl-19 AI/ML WI.

To address the concern raised in RAN3#125 meeting, the CR was further updated to explicitly state that the note applies solely to UE-level UE performance feedback. During the pre-meeting offline discussion on the update of the CR, it was commented that 28.558 already supports UE-level UE performance feedback, and thus, there was no need for a dedicated note in the RAN3 specification outlining its derivation.

Digging deeper into the specifications, 37.320 clarifies that for UE throughput, both DRB-level and UE-level measurements are supported, whereas only DRB-level measurement is available for Packet Delay and Packet Loss.And the reference is 28.558 and 38.314

M5: Average UE throughput measurement separately for DL and UL, per DRB per UE and per UE for the DL, per DRB per UE and per UE for the UL, by gNB, see TS 28.558 [17].

⁻    M6: Packet Delay measurement separately for DL and UL, per DRB per UE, TS 28.558 [17] and TS 38.314 [18].

NOTE 0:   UL PDCP Excess Packet Delay measurement can be configured with a threshold as specified in TS 38.331 [15].

⁻    M7: Packet loss rate measurement separately for DL and UL, per DRB per UE, TS 28.558 [17] and TS 38.314 [18].

So one observation we could have is that per UE level packet delay and packet loss are not supported in SA5, how to derive the information is completely in RAN3 AI/ML scope.

Observation 1: Given per UE level UE packet delay and packet loss is not supported in 28.558, how to derive UE level measurement for these two metric is in RAN3 scope.

Regarding UE throughput, despite the mention of UE-level throughput being supported in 37.320/28.558, there remains ambiguity.

Firstly, there is a lack of a clear and concise definition outlining the process for calculating UE-level throughput. Within clause 6.3.1.4.1/2 of 28.558, a formula in field c is presented, which may potentially serve as the basis for deriving UE-level throughput.

If ThpTimeDl>0, ThpVolDlThpTimeDl×1000 [kbit/s]

If ThpTimeDl=0, 0 [kbit/s]

However, upon examination of field e within the same clause, it appears that only DRB-level, QoS flow-level, and SNSSAI-level UE throughput configurations are explicitly mentioned as being available as below. It seems there may be limitations in the current SA5 specifications regarding UE-level throughput,

e)  The measurement name has the form 
DRB.UEThpDl, or optionally 
DRB.UEThpDl.QoS, where QoS identifies the target quality of service class, and 
DRB.UEThpDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

Given the information provided, it remains unclear at which level of measurement the abovementioned formula is intended to operate.

Observation 2: It is not clear whether/how UE level UE throughput is supported in SA5.

Furthermore, even if we regard the formula  ThpVolUlThpTimeUl  in 28.558 as the way to determine the per-UE level throughput, there exists a notable discrepancy with the previously consensus reached in RAN3. Consider the following example: For a single UE, within a measurement period, two DRBs transmit data bursts with volumes of 100M bits and 1M bits, respectively, taking 2s and 1s to deliver. Applying the formula ThpVolUlThpTimeUl results in a UE-level throughput of (100+1)Mbit / (2+1) s = 33.3mbps. In contrast, adhering to the prior RAN3 conclusion, which calculates the average per-DRB throughput across all configured DRBs, yields (100Mbit/2s+1Mbit/1s)/2 = 25.5mbps.

Observation 3: Even if the formula list in 28.558 aimed at Per UE level measurement, it is not quite aligned with the previous agreement reached in RAN3.

Based on the above two observations, we propose to check with SA5 on whether/how to support UE level UE throughput from SA5 perspective and then discuss whether to align with SA5 if it is already supported.

Proposal 1: To send a LS to SA5 to ask whether/how to support UE level UE throughput from SA5 perspective and discuss whether to align with SA5 if it is already supported.

As outlined in Observation 1, it is evident that SA5 does not support UE-level packet loss and packet delay. Consequently, no conflict arises. Initially, the plan was to incorporate the agreement reached at the RAN3#124 meeting into the specification. However, concerns have been raised regarding the current conclusion on UE-level packet loss and packet delay.

- Within a measurement period, the number of packets delivered per DRB can vary significantly. Averaging packet delay or packet loss across DRBs may not accurately reflect UE performance. For instance, if DRB1 delivers 1000 packets with a delay of 10 ms, while another DRB delivers only 10 packets with a delay of 100 ms, averaging these values results in an unreasonable UE-level delay of 55 ms. Instead, weighted averaging should be considered

- Given the possibility that SA5 may provide feedback on a different mechanism for deriving UE-level throughput than RAN3, it is prudent for RAN3 to reconsider its conclusions on other measurements based on this feedback

In light of these concerns, we propose postponing the CR/discussion on UE-level packet loss and packet delay until feedback from SA5 is received.

Proposal 2: Postpone the CR/discussion on UE level Packet Loss and UE level packet delay until feedback from SA5 is received.

- Conclusion

Observation 1: Given per UE level UE packet delay and packet loss is not supported in 28.558, how to derive UE level measurement for these two metric is in RAN3 scope.

Observation 2: It is not clear whether/how UE level UE throughput is supported in SA5.

Observation 3: Even if the formula list in 28.558 aimed at Per UE level measurement, it is not quite aligned with the previous agreement reached in RAN3. RAN3 needs to decide whether to stick to the previous RAN3 agreement or follow the definition of SA5.

Proposal 1: To send a LS to SA5 to ask whether/how to support UE level UE throughput from SA5 perspective and discuss whether/how to introduce additional clarification in RAN3 spec after reply LS is received.

Proposal 2: Postpone the CR/discussion on UE level Packet Loss and UE level packet delay until feedback from SA5 is received.

The LS is provided in [1]

- Reference

[1] R3-24XXXX   LS on UE level measurement