3GPP RAN WG3 Meeting #125-bis	R3-245271

# Hefei, China, 14th – 18th October 2024

Source:	CATT

Title:	Open issues on the CCO use case

Agenda Item:	11.3

Document for:	Discussion

- Introduction

After the discussions in the SI phase, there are not many open issues left on the CCO use case. This document provides our opinion on the open issues and proposes change on specifications accordingly.

- Discussion

Whether to support a gNB-CU provides to a served gNB-DU the suggested future coverage state of a served cell

Some companies proposed supporting that a gNB-CU provides to a served gNB-DU the suggested future coverage state of a served cell (i.e. not a neighbour cell). We are generally negative on this proposal, as we prefer the gNB-DU to determine what the CCO problem is or will be and what action to take, as in legacy CCO mechanism.

Proposal 1: Do not support the gNB-CU to provide to the gNB-DU the suggested future coverage state.

Whether to use the new Data Collection messages or the legacy Configuration Update messages

We prefer using the legacy Configuration Update messages, considering that the impact on specification in this flavour is far less than the other one.

Proposal 2: Reusing the legacy Configuration Update messages to deliver the future CCO problem and coverage state.

Detail design of the timing information

The timing information is generally a component of prediction. Therefore we think a start point is aligning it with the “Requested Prediction Time” introduced in Rel-18:

| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Proposal 3: As a start point, the encoding of the timing information for CCO can be the same as the “Requested Prediction Time” introduced in Rel-18.

Node-level performance

Node-level performance information can be a type of feedback information, e.g. used as inputs in calculating the reward in the training phase or as monitor metrics. We consider it especially useful for the CCO use case as coverage adjustment may equally affects the UEs subject of handover and the UEs kept in the original cell. We propose discussing what Node-level performance report (e.g. what metric in TS 28.552) can be introduced for the CCO use case.

Proposal 4: To discuss what Node-level performance report (e.g. what metric in TS 28.552) can be introduced for the CCO use case.

- Conclusion

Proposal 1: Do not support the gNB-CU to provide to the gNB-DU the suggested future coverage state.

Proposal 2: Reusing the legacy Configuration Update messages to deliver the future CCO problem and coverage state.

Proposal 3: As a start point, the encoding of the timing information for CCO can be the same as the “Requested Prediction Time” introduced in Rel-18.

Proposal 4: To discuss what Node-level performance report (e.g. what metric in TS 28.552) can be introduced for the CCO use case.

Two TP are provided in the Annex.

- Annex A: Text proposal to TR 38.423

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 8.4.2	NG-RAN node Configuration Update

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

#### 8.4.2.2	Successful Operation

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

Update of Cell Coverage:

If the Coverage Modification List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information in the Cell Coverage State IE to identify the cell deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the cell(s) indicated by the Global NG-RAN Cell Identity IE, as described in TS 38.300 [9].

-	If the Cell Deployment Status Indicator IE is present in the Coverage Modification List IE, the NG-RAN node2 shall consider the cell deployment configuration of the cell to be modified as the next planned configuration and shall remove any planned configuration stored for this cell.

-	If the Time to Change IE is present in the Coverage Modification List IE, the NG-RAN node2 shall consider the cell deployment configuration of the cell to be modified after the indicated length of time and shall remove any planned configuration stored for this cell.

-	If the Cell Deployment Status Indicator IE or the Time to Change IE is present and the Cell Replacing Info IE contains non-empty cell list, the NG-RAN node2 may use this list to avoid connection or re-establishment failures during the reconfiguration, e.g. consider the cells in the list as possible alternative handover targets.

-	If neither the Cell Deployment Status Indicator IE nor the Time to Change IE is present, the NG-RAN node2 shall consider the cell deployment configuration of cell to be modified as activated and replace any previous configuration for the cells indicated in the Coverage Modification List IE.

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 9.1.3	Messages for Global Procedures

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| ///////////////////////////////////////////////////////////////////////skip unrelated/////////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Coverage Modification List |  | 0 .. 1 |  | List of cells with modified coverage. | GLOBAL | reject |
| >Coverage Modification Item |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of the cell to be modified. | – |  |
| >>Cell Coverage State | M |  | INTEGER (0..63, …) | Value ‘0’ indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| >>Cell Deployment Status Indicator | O |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. | – |  |
| >>Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  | – |  |
| >>>Replacing Cells |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>>>Global NG-RAN Cell Identity |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of a cell that may replace all or part of the coverage of the cell to be modified. | – |  |
| >>SSB Coverage Modification List |  | 0..1 |  | List of SSB beams with modified coverage. | – |  |
| >>>SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value ‘0’ indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam. | – |  |
| >>Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the reason for the coverage modification in NG-RAN node1. | YES | ignore |
| >>Time to Change | O |  | INTEGER (1..60, ...) | Indicates the Cell Coverage State is planned to be used after so many seconds. | YES | ignore |
| ///////////////////////////////////////////////////////////////////////skip unrelated/////////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 9.3.5	Information Element definitions

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

id-MobileIABCell,

id-TimeToChange,

maxEARFCN,

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

FROM XnAP-Constants

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

#### -- C

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

Coverage-Modification-List-Item ::= SEQUENCE {

globalNG-RANCell-ID				GlobalCell-ID,

cellCoverageState				INTEGER (0..63, ...),

cellDeploymentStatusIndicator	CellDeploymentStatusIndicator			OPTIONAL,

cellReplacingInfo				CellReplacingInfo						OPTIONAL,

-- Included in case the Cell Deployment Status Indicator IE is present

sSB-Coverage-Modification-List	SSB-Coverage-Modification-List,

iE-Extension			ProtocolExtensionContainer { { Coverage-Modification-List-Item-ExtIEs} } OPTIONAL,

...

}

Coverage-Modification-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-CoverageModificationCause		CRITICALITY ignore	EXTENSION CoverageModificationCause 		PRESENCE optional}|

{ ID id-TimeToChange					CRITICALITY ignore	EXTENSION TimeToChange						PRESENCE optional},

...

}

TimeToChange ::= INTEGER (1..60, ...)

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

### 9.3.7	Constant definitions

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

id-TimeToChange																						ProtocolIE-ID ::=

//////////////////////////////////////////////////////////////////end//////////////////////////////////////////////////////////////////

- Annex B: Text proposal to TR 38.473

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 9.3.1	Radio Network Layer Related IEs

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

#### 9.3.1.211	CCO Assistance Information

This IE provides assistance information for the Capacity and Coverage (CCO) actions for specific CCO issues detected, and for network energy saving.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CCO issue detection | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the type of CCO issue detected, or network energy saving cause. | - |  |
| Affected Cells and Beams | O |  | 9.3.1.212 |  | - |  |
| Time to Happen | O |  | INTEGER (1..60, ...) | Indicates the CCO issue is predicted to happed after so many seconds. | YES | ignore |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

#### 9.3.1.213	Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Coverage Modification List |  | 1 |  |  | - |  |
| >Coverage Modification Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | - |  |
| >>SSB Coverage Modification List |  | 0..1 |  |  | - |  |
| >>>SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block is inactive. Other values Indicates that the SS/PBCH block is active and also indicates the coverage configuration of the concerned SS/PBCH block. | - |  |
| >>Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …, network energy saving) |  | YES | ignore |
| >>Time to Change | O |  | INTEGER (1..60, ...) | Indicates the Cell Coverage State is planned to be used after so many seconds. | YES | ignore |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 9.4.5	Information Element Definitions

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

IMPORTS

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

id-MaxDataBurstVolume,

id-TimeToChange,

id-TimeToHappen,

maxNRARFCN,

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

FROM F1AP-Constants

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

#### -- C

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

cellCoverageState					CellCoverageState,

sSBCoverageModificationList			SSBCoverageModification-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Coverage-Modification-Cause	CRITICALITY ignore	EXTENSION CCO-issue-detection		PRESENCE optional }|

{ ID id-TimeToChange				CRITICALITY ignore	EXTENSION TimeToChange				PRESENCE optional },

...

}

CellCoverageState ::= INTEGER (0..63, ...)

TimeToChange ::= INTEGER (1..60, ...)

CCO-Assistance-Information ::= SEQUENCE {

cCO-issue-detection				CCO-issue-detection		OPTIONAL,

affectedCellsAndBeams-List		AffectedCellsAndBeams-List 		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CCO-Assistance-Information-ExtIEs} }	OPTIONAL,

...

}

CCO-Assistance-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

{ ID id-TimeToHappen				CRITICALITY ignore	EXTENSION TimeToHappen				PRESENCE optional },

...

}

CCO-issue-detection	::=	ENUMERATED {

coverage,

cell-edge-capacity,

...,

network-energy-saving}

TimeToHappen ::= INTEGER (1..60, ...)

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

### 9.4.7	Constant Definitions

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

id-TimeToChange										ProtocolIE-ID ::=

id-TimeToHappen										ProtocolIE-ID ::=

//////////////////////////////////////////////////////////////////end//////////////////////////////////////////////////////////////////