3GPP TSG-RAN WG3 Meeting #125bis	                                        R3-245278

Hefei, China, 14 Oct. – 18 Oct., 2024

Agenda Item:	8.1 LS In

Source:	NEC

Title:	Discussion on Draft Reply LS on multi-modality awareness at RAN (Response to S2-2409444)

Document for:	Discussion, Decision

Introduction

SA2 sent an LS reply on multi-modality awareness at RAN in S2-2409444 (copied below). The following two aspects SA2 is asking for confirmation/feedback (highlighted in yellow below) fall into RAN3 scope. We provide our views of them in this contribution.

1. Overall Description: SA2 thanks RAN2 LS on multi-modality awareness at RAN. SA2 would like to inform RAN2 the following: Question: In this meeting, as part of this study, RAN2 has agreed that multi-modality awareness at RAN is supported for both UL and DL. To continue the discussion during the study phase, RAN2 needs to understand whether/what information can be provided from SA2 (e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN). Answer: SA2 specified policy control enhancements to support multi-modal services in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF, please see TS 23.501 clause 5.37.2 for details. Forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts. Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work. Such feedback from RAN2 needs to also be assessed by RAN3 and SA4 to confirm the applicability and usage of this information. 2. Actions: To RAN WG2, SA WG4, RAN WG3. ACTION: SA2 asks RAN2, RAN3 to provide feedback if any, and SA4 and RAN3 to review RAN2 feedback from XR service perspective and NG-RAN protocols impacts perspective, respectively

Discussion

NG-RAN protocols impacts on supporting MMSID

According to TS23.501 clause 5.37.2, a Multi-modal Service ID (MMSID) can be provided for each data flow that belongs to the multi-modal service where

-	The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service.

Multi-modal Service ID (MMSID) is an explicit indication for each data flow belonging to the multi-modal service.

Each data flow from upper layer can be mapped to a QoS flow based on their individual QoS requirements. In the current NG application protocol (NGAP), the existing QoS Flow Level QoS Parameters IE, included in  SMF related IEs “PDU Session Resource Setup Request Transfer” and “PDU Session Resource Modify Request Transfer”, defines the QoS parameters to be applied to a QoS flow. Since MMSID is an explicit indication for each QoS flow belonging to the multi-modal service, we observe the simplest way to support MMSID over NGAP is to add it to the existing QoS Flow Level QoS Parameters IE.

The simplest way to support MMSID over NGAP is to add it to the existing QoS Flow Level QoS Parameters IE.

Based on Observation 2, we think NG-RAN protocols impacts on supporting MMSID is quite limited.

Reply to SA2 that NG-RAN protocols impacts on supporting MMSID is quite limited assuming to add MMSID to existing QoS Flow Level QoS Parameters IE.

NG-RAN behaviour for "coordinated handling of the flows at NG-RAN"

With the provided MMSID, potential NG-RAN behavior for "coordinated handling of the flows at NG-RAN" would be setup/release/modify multi-modality QoS flows with the same MMSID. For example, the NG-RAN node may establish, modify or release the DRB configuration and may change allocation of resources on NG or Uu accordingly by including QoS flows with the same MMSID into the QoS Flow Add or Modify Request List IE

Similar to the above, associated behavior enhancements can be supported by including QoS flows with the same MMSID into QoS Flow Accepted List, QoS Flow Failed to Setup List, QoS Flow Failed to Add or Modify List, QoS Flow Failed to Modify List and QoS Flow Failed to Resume List.

Reply to SA2 that NG-RAN may setup/release/modify multi-modality QoS flows together based on the provided MMSID (i.e., setup/release/modify multi-modality QoS flows with the same MMSID).

Summary

This contribution provided our views on NG-RAN protocols impacts on supporting MMSID and NG-RAN behaviour for “coordinated handling of the flows”.

NG-RAN protocols impacts on supporting MMSID

Proposal 1	Reply to SA2 that NG-RAN protocols impacts on supporting MMSID is quite limited assuming to add MMSID to existing QoS Flow Level QoS Parameters IE.

NG-RAN behaviour for "coordinated handling of the flows

Proposal 2	Reply to SA2 that NG-RAN may setup/release/modify multi-modality QoS flows together based on the provided MMSID (i.e., setup/release/modify multi-modality QoS flows with the same MMSID).

- References