3GPP TSG-RAN WG3 #125bis	R3-245284

Hefei, China, Oct 14th – 18th, 2024

Source:		NEC

Agenda item:		10.2. MRO Enhancements

Title:		(TP for SON BLCR of 38.473) Discussion on MRO for LTM

Document for:		Discussion and decision

- Introduction

During previous RAN3 meetings, MRO enhancements for LTM were discussed, and the following agreements were made:

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover. Take the existing detection mechanism descriptions as baseline. For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure. CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. Work on scenarios of near failure LTM. Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell. RAN3 to prioritize MCG LTM over SCG LTM. RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study. For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization. For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization. RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

In this contribution, we would like to provide our further consideration on MRO for LTM.

- Discussion

During last meeting, it was agreed that for failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure. And CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. The controversial issue is whether the CU indicates additional information related to the LTM cell switch failure other than RLF-report in the Access and Mobility Indication message to the DU.

One argument of not introducing additional information is the RLF-report is sufficient for the DU to analysis the LTM failure information.  In our understanding, the CU can detect LTM failure and perform initial analysis before/without reception of the RLF-report. For example, when LTE failure happens, the UE may initiate LTM recovery or RRC re-establishment procedure, and the CU can detect this and analysis whether the LTM failure is a too late, too early or LTM cell switch to a wrong cell. And the CU can indicate the failure information to the DU event without the RLF report. Therefore, we think additional information related to LTM failure information should be introduced in Access and Mobility Indication message.

For the content of the LTM failure information, the type of the LTM failure problem, the ID of the source cell, the target LTM cell, and re-established cell. As the Access and Mobility Indication message does not include UE ID information, the UE ID may be also included in the message.

Proposal 1: Include in Access and Mobility Indication message additional information regarding to LTM cell switch failure.

Proposal 2: The following additional information regarding to LTM cell switch failure can be considered to be included:

- type of the LTM failure

- ID of the source cell, target LTM cell, re-established cell

- ID of the UE

Additionally, for conventional mobility procedure, e.g., handover and PSCell change, the gNB-CU can perform MRO optimization and identifies ping-pong issue based on UE history information collected by the source gNB-CU or reported by the UE. However, for LTM cell switch, it is the gNB-DU makes decision on whether and when to trigger the cell switch. And if we only rely on existing mechanism, the gNB-DU will not have the UE history information, bad triggering time for LTM or ping-pong cell switch may happen.  Therefore, it is proposed that RAN3 considers F1 interface enhancements to transmit UE history information to the gNB-DU. And the possible F1AP message carrying UE history information may be CU-DU CELL SWITCH NOTIFICATION message.

Proposal 3: CU transmits, to the DU, F1AP message including UE history information.

To analysis near failure handover procedure, SHR and SPR were introduced with some triggering conditions, e.g., T310/T312/T304 triggers, to avoid storing and reporting information which are not necessary for the network. In legacy, the triggers are set by the gNB-CU. For LTM, one difference from the legacy handover procedure is that it is the source gNB-DU to triggers the execution of LTM cell switch, based on L1 measurement results from the UE. And should be the gNB-DU to perform the root cause analysis and optimization for the near failure of LTM cell switch. Therefore, we think the gNB-DU should also be able to determine the triggering condition of SHR and SPR, for the optimization of near LTM failure, e.g., to adjust associated mobility thresholds, or adjust T310/T312 timer values and etc. And if it is agreeable that the gNB-DU makes decision on the triggering condition of SHR and SPR, whether the existing triggering condition is reused or new triggering condition can be introduced needs further discussion.

Proposal 4: DU can configure the SHR/SPR triggering conditions for LTM cell switch.

Proposal 4a: Discuss whether to introduce new SHR/SPR triggering condition configured by DU.

# 3. Conclusion

In this paper, we provide our further consideration on MRO for LTM, and have the following proposals:

Proposal 1: Include in Access and Mobility Indication message additional information regarding to LTM cell switch failure.

Proposal 2: The following additional information regarding to LTM cell switch failure can be considered to be included:

- type of the LTM failure

- ID of the source cell, target LTM cell, re-established cell

- ID of the UE

Proposal 3: CU transmits, to the DU, F1AP message including UE history information.

Proposal 4: DU can configure SHR/SPR triggering conditions for LTM cell switch.

Proposal 4a: Discuss whether to introduce new SHR/SPR triggering condition configured by DU.

# Annex: TP to TS 38.473

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| LTM Failure information | O |  | x.x.x.x | Includes information of a LTM cell switch failure event. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| UE History Information | O |  | y.y.y.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

#### x.x.x.x	LTM failure Information

The LTM failure Information IE contains information about a LTM failure event.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| LTM failure type | M |  | ENUMERATED (LTM too early, LTM to wrong cell, LTM too late. …) |  |
| Source cell CGI | M |  | NR CGI 9.3.1.12 | NR CGI of source cell for LTM cell switch procedure |
| Target cell CGI | O |  | NR CGI 9.3.1.12 | NR CGI of target cell for LTM cell switch procedure |
| Re-establishment cell CGI | O |  | NR CGI 9.3.1.12 | NR CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the LTM failure |
| C-RNTI | M |  | 9.3.1.32 | C-RNTI of the UE allocated at the gNB-DU |

#### y.y.y.y	UE History Information

The UE History Information IE contains information about cells that a UE has been served by in active state prior to the target cell. The overall mechanism is described in TS 36.300 [20].

NOTE:	The definition of this IE is aligned with the definition of the UE History Information IE in TS 38.413 [3].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list |
| >Last Visited Cell Information | M |  | z.z.z.z |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum number of last visited cell information records that can be reported in the IE. Value is 16. |

#### z.z.z.z	Last Visited Cell Information

The Last Visited Cell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Last Visited Cell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Last Visited NG-RAN Cell Information | M |  | OCTET STRING | Defined in TS 38.413 [3]. |
| >E-UTRAN Cell |  |  |  |  |
| >>Last Visited E-UTRAN Cell Information | M |  | OCTET STRING | Defined in TS 36.413 [15]. |
| >UTRAN Cell |  |  |  |  |
| >>Last Visited UTRAN Cell Information | M |  | OCTET STRING | Defined in TS 25.413 [xx]. |
| >GERAN Cell |  |  |  |  |
| >>Last Visited GERAN Cell Information | M |  | OCTET STRING | Defined in TS 36.413 [15]. |