3GPP TSG RAN WG3 125-bis	R3-245289

Hefei, China, 14 - 18 October, 2024

Agenda item:		16.2

Title:		Discussion on architecture of Ambient IoT

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the last meeting, we made much progress and agreed on the TR R3-244837, where separate logic system architectures are adopted for Topology 1 and Topology 2. For topology 1, a protocol stack agreed to use the NG interface or a new interface over the SCTP-based transport layer. For the topology 2, the AIoT-enabled gNB and AIoT-enabled UE is defined, and the RRC based solution/NAS based solution/UP based solution is assumed to transfer the AIoT upper layer information.

System architecture figures agreed for Topology1 and Topology2. For Topology2, whether AIoT RAN needs to be revised as AIoT enabled gNB needs to be further checked in next meeting. Continue to discuss the architectures and protocol stack.

There are some open issues are left and this paper will discuss on them.

# 2. Discussion

2.1	If AIoT RAN needs to be revised as AIoT enabled gNB Topology 2

From our understanding, there are some terminologies are not clear enough in the given below:

AIoT RAN: hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

In the logic system architecture of Topology 1, the AIoT RAN includes the common reader function and AIoT RAN node function, and the AIoT RAN performs an AIoT RAN reader. We are not capturing them into the AIoT RAN definition, which may lead to confusion between the term AIoT-enabled gNB and the AIoT RAN. So, we need to update the AIoT RAN to the AIoT RAN reader, and the AIoT RAN reader supports the common reader function and AIoT RAN node function, which makes the definition more precise. Move this term to Section 6.3.1 because the AIoT RAN reader is defined only for Topology 1.

Proposal 1: We need to update the “AIoT RAN” to the “AIoT RAN reader”. The AIoT RAN reader supports the common reader function and AIoT RAN node function, which makes the definition clearer. The TP for TR 38.769 is provided in Annex A.

In addition, the definition of UE reader in Topology 2 is listed here:

AIoT-enabled UE: a UE supporting Common reader function, which is able to communicate with the AIoT device via the AIoT radio interface.

We think the term "AIoT-enabled UE" does not distinctly reflect the function of UE including the common reader function, and we propose changing it to "AIoT UE reader".

Proposal 2: Update the term "AIoT-enabled UE" to "AIoT UE reader". The TP for TR 38.769 is provided in Annex A.

Also, for the description of RRC based solution:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

The highlighted words "explicitly" seems hint that the content of AIoT upper layer information is visible for the XXAP interface and AIoT-enabled gNB. However, we do not have a conclusion on it. So, we suggest removing this word.

Proposal 3: Remove the word "explicitly" from the description of RRC based solution.

2.2	Protocol stacks of topology 2

The RRC based solution/NAS based solution/UP based solution for Topology 2 is agreed, and the corresponding protocol stacks are proposed by some companies but not captured in the TR 38.769. The protocol stacks are listed below:

- Protocol stack of RRC based solution:

Where the AIoT-enabled gNB receives the AIoT upper layer information (Inventory/Command) from AIoT CN via XX interface over the SCTP transport layer, and the AIoT-enabled gNB transfers the AIoT upper layer information to the AIoT UE reader by RRC signaling in a RRC container. Whether the content of Inventory/Command is visible to AIoT-enabled gNB and AIoT UE reader AS layer may be decided by RAN2. Also, the AIoT UE reader may report the collected D2R data in an RRC container by RRC signaling.

- Protocol stack of NAS based solution:

The AIoT upper layer information is transmitted from the AIoT-related CN to the AIoT UE reader via the NAS layer over the SCTP transport layer. The AIoT-enabled gNB and the AMF should be the transfer nodes for the AIoT upper layer information. So, the content of AIoT upper layer information is transparent to the AIoT-enabled gNB. Whether the content of AIoT upper layer information is transparent to AMF depends on SA2. The legacy Transport of NAS Messages Procedures can be reused.

- Protocol stack of UP based solution:

The AIoT upper layer information is transmitted from AIoT-related CN to AIoT UE reader via user plane PDUs over the GTP-U transport layer, where the AIoT-enabled gNB should be the transfer nodes for the AIoT upper layer information. So, the content of AIoT upper layer information is transparent to the AIoT-enabled gNB. The PDU session and DRB should be established to transfer the AIoT upper layer information. However, the type of QoS flow of the AIoT upper layer information should also be further discussed (e.g., GRB, Non-GBR).

Proposal 4: RAN3 is suggested to agree the protocol stacks list above.

# 3. Conclusion

Proposal 1: We need to update the AIoT RAN to the AIoT RAN reader. The AIoT RAN reader supports the common reader function and AIoT RAN node function, which makes the definition clearer. The TP for TR 38.769 is provided in Annex A.

Proposal 2: Update the term "AIoT-enabled UE" to "AIoT UE reader". The TP for TR 38.769 is provided in Annex A.

Proposal 3: Remove the word "explicitly" from the RRC based solution description.

Proposal 4: RAN3 is suggested to agree the protocol stacks list above.

# References

# Annex A: TP to TR 38.769 AIoT architecture aspects

## 6.2	Protocol stack and signalling procedures

Editor’s note: Corresponds to the RAN2 objective in the SID.

## 6.3	RAN architecture aspects

Editor’s note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

This chapter attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for AIoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

The logical system architecture for AIoT consists of the following architectural elements:

AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

AIoT radio: radio interface between AIoT device and AIoT RAN node in topology 1 and between AIoT device and AIoT-enabled UE in topology 2.

Editor’s Note 4: Further details on AIoT radio to be discussed by RAN1 and RAN2.

AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN.

NOTE: the details of AIoT CN are subject to SA2.

Editor’s Note 5: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface: interface between the AIoT RAN and the AIoT CN on which certain AIoT specific functions are performed.

Editor’s Note 6: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAN and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAN, or one instance for NG alone.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

Editor’s Note 7: Further details on Common reader function is to be discussed by RAN1 and RAN2.

AIoT RAN node function: a function that contains e.g. the control of the AIoT radio resources used towards the AIoT device.

Editor’s Note 8: further details are FFS. Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

### 6.3.1 Support of Topology 1

Figure 6.3.1-1 depicts a logical system architecture for topology 1, where the Common reader function and AIoT RAN node function are deployed within an AIoT RAN Reader.

The following definitions apply:

AIoT RAN Reader: A RAN node supporting AIoT RAN node function and common reader function that hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 1: Further details regarding AIoT functions hosted in the AIoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

Figure 6.3.1-1 Logical system architecture for topology 1

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.3.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

- Figure 6.3.1-2. Protocol Stack for Topology 1

    - Editor’s Note2: Figure 6.3.1-2 serves as a starting point for further discussions.

For topology 1, the XXAP is terminated at an AIoT RANreader.

Editor’s Note3: the signalling transport for XXAP is FFS.

Editor’s Note 4: The protocol stack does not detail how AIoT upper layer information is transported over XXAP, details are pending on SA2 agreements. And the AIoT CN may include AMF and AIoT related functions which is also up to SA2 decision.

Editor’s Note 5: aspects of interaction between upper layer information exchange and XXAP in order to trigger the AIoT RAN node functions are FFS.

### 6.3.2 Support of Topology 2

Figure 6.3.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an AIoT UE reader, and the AIoT RAN node function is located at an AIoT-enabled gNB.

The following definitions apply:

AIoT-enabled gNB: a gNB supporting AIoT RAN node function, which is able to communicate with the AIoT enabled UE via NR Uu interface.

AIoT UE reader: a UE supporting Common reader function, which is able to communicate with the AIoT device via the AIoT radio interface.

Figure 6.3.2-1 Logical system architecture for topology 2

Editor’s Note 1:	Figure 6.3.2-1 doesn’t illustrate the protocol between AIoT UE reader and AIoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The AIoT CN could include AMF and AIoT related functions. This is up to SA2 decision.

Editor’s Note 4:	The AIoT enabled gNB performs radio resource management for AIoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

#### 6.3.2.1 Solutions for Topology 2

To support Topology 2, the following solutions are to be studied for conveying AIoT upper layer information:

- RRC based solution. With this solution, AIoT CN applies AIoT upper layer information over XXAP signaling. AIoT upper layer information is then relayed explicitly to/from the AIoT UE reader via NR Uu RRC.

- NAS based solution. With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted over  AIoT UE reader 's NAS.

- UP based solution. With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted as  AIoT UE reader's user plane data.

Editor’s note 1: how to enable radio resource control, i.e. trigger AIoT RAN node functions for above solutions is FFS.

Editor’s note 2: depiction and further details of the options above are FFS.