3GPP TSG-RAN WG3 Meeting #125-bis	R3-245313

Hefei, China, Oct 14th - Oct 18th, 2024

Agenda item:	13.2

Source:	Qualcomm Incorporated, Vodafone, Bharti Airtel (India)

Title:	Signalling enhancements for inter-CU LTM HO

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #125 [1]), further progress was made in developing the procedures of inter-CU LTM considering the case when the gNB-CU is switched at LTM cell switch.

RAN3 #125 agreements:

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

In this contribution, we discuss the following aspects of inter-CU LTM for our proposed solution in which the source gNB serves as an anchor [2]; please see our companion contribution [2]. For each of the main procedures below, we discuss some of the signalling details of our source gNB anchor solution.

- LTM candidate cell configuration,

- Early TA RACH configuration,

- TA Information Transfer,

- LTM CSI configuration,

- LTM Cell Switch Notification,

- Handling of RRCReconfigurationComplete message transmitted by the UE upon LTM cell switch.

# 2 LTM candidate cell configuration

We first make some remarks regarding the LTM preparation procedure in our proposed solution in which the source gNB serves as an anchor.

Remarks regarding LTM preparation in our proposed solution in which the source gNB serves as an anchor

The signalling during the LTM preparation procedure proceeds along expected lines: upon receiving the RRC measurement report from the UE, the source CU determines the initial list of LTM candidate cells and requests its underlying DUs and the candidate CUs to prepare the LTM candidate cells using the F1 UE Context Setup Request and the Xn Handover Request messages, respectively. Each candidate CU then requests its underlying DUs to prepare LTM candidate cells using the F1 UE Context Setup Request messages.

There are some differences compared to the case when the RRC and PDCP anchor is moved when the UE moves to another CU, and these are discussed below. For example, since the UE remains connected to the source CU in our proposed solution, the source CU determines the upper layer configuration of a candidate cell, e.g., the radioBearerConfig containing, e.g., the PDCP and SDAP configuration, and the RRC configuration, of a candidate cell. During LTM preparation, the candidate CU forwards the CellGroupConfig of a prepared candidate cell provided by an underlying DU, to the source CU.

After UE performs LTM cell switch to a candidate cell of a candidate CU, the forwarding of Control Plane signalling between the UE and the source CU can reuse the Xn RRC Transfer procedure; see [2].

Observation 1. For Inter-CU LTM with Anchor PDCP/RRC solution, during the LTM preparation phase, source CU uses legacy Xn Handover signalling messages and F1-C messages to prepare candidate DU/Cell configuration. Candidate CU will retrieve CellGroupConfig from candidate DU and will provide to source CU via legacy Xn Handover signalling messages.

Observation 2. Once LTM Cell Switch procedure is done, Source CU and Candidate DU exchange RRC signalling via RRC Transfer Procedure and Candidate CU will do routing of control plane signalling messages between candidate DU and Source CU, i.e., UE specific RRC messages are transparent to candidate CU.

For the remainder of this section, we discuss the basic aspect of LTM candidate cell configuration during LTM preparation.

The source CU initiates LTM preparation based on L3 RRC measurement report received from the UE; see Figure 1 for the overall signalling flow for LTM candidate cell configuration.

Proposal 1. During initial preparation, the source gNB-CU determines the list of LTM candidate cells to be suggested for preparation and transmits Handover Request to a candidate gNB-CU including:

- LTM indication,

- Suggested candidate cell ID,

- UE measurement results.

The preparation of multiple LTM candidate cells belonging to a candidate gNB-CU can take place in parallel (similar as CHO).

We now discuss some of the important IEs of the Handover Request message and how such IEs are handled by the candidate gNB-CU; see Table 1.

Table 1: Handling of some important IEs of Handover Request message in the source gNB-CU anchor solution

| IE(s) in Handover Request | Presence | Handling of the IE in the source gNB-CU anchor solution |
| --- | --- | --- |
| UE Context Information > UE Security Capabilities/AS Security Information | M | These IEs are ignored by the candidate gNB-CU, upon reception of the message including Anchor based LTM Indication. This is because the UE remains connected to the source gNB-CU upon LTM execution. |
| UE Context Information > PDU Session Resources To Be Setup List | M | This IE is ignored by the candidate gNB-CU. Since the UE remains connected to the source gNB-CU after LTM cell switch, the source gNB-CU determines the mapping from QoS flows to DRBs for the candidate cell configuration and includes the DRB To Be Setup List IE (same as the existing IE in the UE Context Setup Request message) in Handover Request message. This IE is forwarded by the candidate gNB-CU to the candidate gNB-DU in the UE Context Setup Request message in the subsequent step of the procedure, for preparation of the candidate cell by the candidate gNB-DU. The DRB To Be Setup List IE is provided as a top-level IE in Handover Request. |
| UE Context Information > RRC Context | M | Information in this IE such as UE capabilities and source cell configuration need to be forwarded by the candidate gNB-CU to the candidate gNB-DU and are needed to determine the candidate cell lower layer configuration, e.g., the CellGroupConfig IE. |
| UE Context Information > Mobility Restriction List/5GC Mobility Restriction List Container | O | These IEs need not be included in the Handover Request message. This is because the source gNB-CU continues to enforce the restrictions on service areas and PLMNs for the UE as it is moving. |

Proposal 2. Regarding the handling of IEs in the Handover Request message (see Table 1) in the source gNB-CU anchor solution:

- An explicit indication is included in the message to indicate that the source gNB-CU will remain as the anchor upon LTM cell switch.

- The following mandatory IEs are ignored by the candidate gNB-CU upon reception of the message including an explicit indication of source keeping anchor-CU role.

  - UE Context Information > UE Security Capabilities/AS Security Information,

  - UE Context Information > PDU Session Resources To Be Setup List.

- The source gNB-CU determines and includes the mapping from QoS flows to DRBs for the candidate cell configuration and includes it in a DRB To Be Setup List IE. The DRB To Be Setup List IE (same as the existing IE in the UE Context Setup Request message) is included in the message, e.g., as a top-level IE.

- For the mandatory UE Context Information > RRC Context IE, information in this IE such as UE capabilities and source cell configuration need to be forwarded by the candidate gNB-CU to the candidate gNB-DU, which is legacy behavior.

Figure 1: Example signalling flow of LTM candidate cell configuration

Proposal 3. Similar as in Rel-18 intra-CU LTM, the source CU uses the F1 UE Context Setup procedure with its underlying candidate DUs for preparation of LTM candidate cells that belong to it.

Proposal 4. A candidate CU uses the F1 UE Context Setup procedure with its underlying candidate DUs for preparation of LTM candidate cells that belong to it. The candidate CU forwards the DRB To Be Setup List IE received in Xn Handover Request message to candidate DU in F1 UE Context Setup Request message.

Proposal 5. Similar as in Rel-18 intra-CU LTM, a candidate DU determines whether the suggested candidate cell can be prepared, and if it can be prepared, the candidate DU determines the lower layer configuration (CellGroupConfig) for the prepared candidate cell.

The candidate DU transmits the lower layer configuration (CellGroupConfig) of the prepared candidate cell along with the prepared candidate cell ID, to the candidate CU. The candidate CU forwards the lower layer configuration (CellGroupConfig) of the prepared candidate cell to the source CU along with the candidate cell ID over Xn in Handover Request Ack. The source CU generates the upper layer configuration (PDCP and SDAP configuration) for the candidate cell and adds it to the provided CellGroupConfig to form the RRC configuration of the candidate cell. Thus, the source CU generates all the LTM candidate configurations – for candidate cells belonging to it, as well as for candidate cells belonging to the other candidate CUs.

Proposal 6. An underlying candidate DU of a candidate CU transmits the lower layer configuration (CellGroupConfig) of the prepared candidate cell along with the candidate cell ID to the candidate CU. The candidate CU then forwards the CellGroupConfig of the prepared candidate cell to the source CU along with the candidate cell ID over Xn inside the inter-node RRC message Handover Command that is included in Handover Request Ack.

Proposal 7. The source CU generates the upper layer configuration, e.g., the radioBearerConfig containing, e.g., the PDCP and SDAP configuration, for the candidate cell, adds it to the provided CellGroupConfig to form the RRC configuration of the candidate cell.

Similar as in Rel-18 intra-CU LTM, an underlying candidate DU of the source CU transmits the lower layer configuration (CellGroupConfig) of a prepared candidate cell, along with the prepared candidate cell ID to the source CU.

Observation 3. As has been discussed above, the Handover Request/Handover Request Ack messages can be reused in the initial LTM preparation of the source gNB-CU anchor solution, with some Stage-3 enhancements that are discussed above and that involve low signaling complexity and overhead.

# 3 Early TA RACH configuration

In a previous RAN2 meeting (RAN2 #125-bis [3]), RAN2 agreed on the following:

- Early UL sync is supported for inter-CU LTM.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline.

The above agreements indicate that similar RACH configuration and TA estimation mechanism as in Rel-18 intra-CU LTM can be used for Rel-19 inter-CU LTM. In this and the following sections, we propose extensions to the Rel-18 backhaul signalling mechanism for RACH configuration for Early TA acquisition and the transfer of TA information from a candidate DU to the source DU. Please see Figure 2 for the overall signalling flow.

Proposal 8. During LTM preparation, in Xn Handover Request, the source gNB-CU may transmit a request to a candidate gNB-CU for obtaining RACH configuration and for allocation of CFRA resources, e.g., RAP IDs, for Early TA acquisition. The following information is included:

- LTM candidate cell ID,

- List of source gNB-DU ID and candidate gNB-DU IDs of source gNB-CU.

A set of CFRA resources should be allocated for an underlying DU of the source gNB-CU, and these are to be used by the UEs served by the DU to initiate RACH for Early TA acquisition.

Proposal 9. Upon receiving the request from the source gNB-CU, the candidate gNB-CU forwards the request to its underlying candidate gNB-DU in F1 UE Context Setup Request, including:

- LTM candidate cell ID,

- List of source gNB-DU ID and candidate gNB-DU IDs of source gNB-CU,

- The source gNB ID.

As per principles established in Rel-18 intra-CU LTM, upon receiving the request, the candidate gNB-DU determines the split of the available CFRA resources among the indicated gNB-DUs of the source gNB, and stores this information along with the source gNB ID.

Proposal 10. If the suggested LTM candidate cell can be prepared, in the F1 UE Context Setup Response message, the candidate DU provides to the candidate CU:

- LTM candidate cell ID,

- List of pairs of the form (gNB-DU ID of the source CU, CFRA resource set, i.e., RAP IDs),

- CFRA configuration for Early TA acquisition,

- The source gNB ID.

Proposal 11. The candidate gNB-CU forwards the provided information from its underlying candidate DU to the source gNB-CU in Xn Handover Request Ack:

- LTM candidate cell ID,

- List of pairs of the form (gNB-DU ID of the source CU, CFRA resource set, i.e., RAP IDs),

- CFRA configuration for Early TA acquisition.

The candidate gNB-CU also includes the candidate gNB-DU ID.

Figure 2: Example signalling flow of Early TA RACH configuration procedure

Proposal 12. Upon receiving the Xn Handover Request Ack from the candidate gNB-CU, the source gNB-CU forwards the received information to the corresponding underlying DU in F1 UE Context Modification Request:

- LTM candidate cell ID,

- CFRA resource set, i.e., RAP IDs,

- CFRA configuration for Early TA acquisition.

After the initial round of preparation of the candidate cells in a candidate gNB-CU, the source gNB-CU has information regarding the candidate gNB-DU IDs in a candidate gNB-CU.

In the “second round” of LTM preparation, the source gNB-CU uses the new UE-associated procedure, LTM Configuration Update, to obtain the RACH configuration and for allocation of CFRA resources for Early TA acquisition for the other candidate gNB-CUs.

Proposal 13. In Xn LTM Configuration Update message to a candidate gNB-CU, the source gNB-CU includes the following information:

- For each other candidate gNB-CU:

  - List of candidate gNB-DU IDs.

Proposal 14. Upon receiving the message from the source gNB-CU, the candidate gNB-CU forwards the request to its underlying candidate gNB-DUs in F1 UE Context Modification Request, including:

- For each other candidate gNB-CU:

  - List of candidate gNB-DU IDs,

  - The candidate gNB ID.

Upon receiving the request, the candidate gNB-DU determines the split of the available CFRA resources among the indicated candidate gNB-DUs of other candidate gNB-CUs and stores this information along with the candidate gNB IDs.

Proposal 15. In the F1 UE Context Modification Response message, the candidate DU provides to the candidate CU:

- For each prepared LTM candidate cell:

  - For each other candidate gNB-CU:

    - List of pairs of the form (candidate gNB-DU ID, CFRA resource set, i.e., RAP IDs),

    - CFRA configuration for Early TA acquisition,

    - The candidate gNB ID.

Proposal 16. In the Xn LTM Configuration Update Ack to the source gNB-CU, the candidate gNB-CU relays the information received from its underlying gNB-DU:

- For each prepared LTM candidate cell:

  - For each other candidate gNB-CU:

    - List of pairs of the form (candidate gNB-DU ID, CFRA resource set, i.e., RAP IDs),

    - CFRA configuration for Early TA acquisition,

    - The candidate gNB ID.

Proposal 17. The source gNB-CU reuses the Xn LTM Configuration Update procedure to forward the received Early TA configuration information from each candidate gNB-CU to the other candidate gNB-CUs.

# 4 TA Information Transfer

A candidate DU determines the TA value based on the received RACH preamble from the UE. However, the candidate DU cannot determine the UE ID from the received RACH preamble since the RAP ID is from the pool of assigned RAP IDs to the source DU, for the candidate cell.

Proposal 18. Upon successfully receiving the RACH preamble from the UE on a candidate cell, a candidate gNB-DU determines the TA value. The candidate gNB-DU transmits to the candidate gNB-CU the following TA information using F1 DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Source gNB ID,

- Source gNB-DU ID,

- LTM candidate cell ID.

Proposal 19. The candidate gNB-CU forwards the TA information to the source gNB-CU using TA Information Transfer message over Xn.

Proposal 20. The source gNB-CU forwards the TA information to the source gNB-DU using F1 CU-DU TA Information Transfer.

The source DU then identifies the UE based on the TA information associated with RAP ID and provides to the UE the TA value in the LTM Cell Switch MAC CE command during LTM execution.

Figure 3: TA information transfer from a candidate DU to source DU withsignalling over  Xn

During subsequent LTM operation, a similar set of proposals as the above hold.

Proposal 21. Upon successfully receiving the RACH preamble from the UE on a candidate cell, a candidate gNB-DU transmits to the candidate gNB-CU the following TA information using F1 DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Serving gNB ID,

- Serving gNB-DU ID,

- LTM candidate cell ID.

Proposal 22. The candidate gNB-CU forwards the TA information along with RAP ID to the source gNB-CU using TA Information Transfer over Xn.

Proposal 23. The source gNB-CU then forwards the TA information to the serving gNB-DU via the CU of the serving gNB-DU, using the TA Information Transfer and the CU-DU TA Information Transfer messages, respectively.

# 5 LTM CSI configuration

In order to perform L1 measurements on an LTM candidate cell, the UE should be provided with the configuration of the relevant RSs transmitted on that cell, and for L1 measurement reporting, the UE should be provided with the reporting configuration.

We make the following assumptions in order to determine the signaling for LTM CSI configuration:

- Similar as in Rel-18 intra-CU LTM, each CU receives from an underlying DU the following information during F1 Setup:

- List of cells managed by the DU,

- For each such cell, the cell RS configuration, which includes the list of transmitting SSBs of the cell.

- The source CU receives from a candidate CU similar information during Xn Setup:

- List of cells managed by the candidate CU,

- For each such cell, the cell RS configuration, which includes the list of transmitting SSBs of the cell.

The following principles for LTM CSI configuration established in Rel-18 intra-CU LTM should continue to hold for inter-CU LTM:

- The CSI resource configuration for measuring the LTM candidate cells is generated by the CU and is common for all the LTM candidate cells.

- The candidate DUs generate the CSI reporting configurations.

An example overall signalling flow of the CSI configuration procedure is shown in Figure 4.

Figure 4: Example signalling flow of the CSI configuration procedure

Proposal 24. During initial LTM preparation, source gNB-CU generates the LTM CSI resource configuration for the candidate cells, i.e., the list of SSBs to be measured by the UE for each candidate cell, based on:

- Information received from the candidate CUs and its own underlying DUs, in XN Setup and F1 Setup procedures, as mentioned in the assumptions above,

- RRC measurement report from the UE.

Source CU provides to its own underlying candidate DU:

- Suggested candidate cell ID,

- LTM CSI resource configuration.

The underlying candidate DU responds with:

- The prepared candidate cell ID, and its associated lower layer configuration, which includes the LTM CSI report configuration.

Proposal 25. During initial LTM preparation, the source gNB-CU provides to a candidate gNB-CU in Xn Handover Request:

- LTM CSI resource configuration.

This information is forwarded by the candidate CU to its underlying candidate DU using F1 UE Context Setup Request.

Proposal 26. A candidate DU provides to the candidate CU the following information in UE Context Setup Response, which is then forwarded by the candidate CU to the source CU in Xn Handover Request Ack:

- The prepared candidate cell ID and its associated lower layer configuration, which includes:

  - The LTM CSI report configuration,

  - TCI state configuration.

Proposal 27. The source CU may update the LTM CSI resource configuration based on the prepared cells of its underlying candidate DUs and those of the candidate CUs.

Proposal 28. The source gNB-CU includes in LTM Configuration Update message to a candidate gNB-CU:

- The updated LTM CSI resource configuration,

- TCI state configurations of prepared candidate cells of other candidate gNB-CUs.

This information is forwarded by the candidate CU to its underlying candidate DU using F1 UE Context Modification Request.

Proposal 29. A candidate DU provides to the candidate CU the following information in F1 UE Context Modification Response, which is then forwarded by the candidate CU to the source CU in Xn LTM Configuration Update Ack:

- For each prepared candidate cell, the associated lower layer configuration, which includes the updated LTM CSI report configuration.

# 6 LTM Cell Switch Notification

The following are the proposals for the LTM Cell Switch Notification, which reuse some of the related Rel-18 messages; see Figure 5 for an example signalling flow.

Proposal 30. Upon transmitting the LTM Cell Switch Command to the UE, the serving gNB-DU transmits the DU-CU Cell Switch Notification message to its gNB-CU (gNB-CU1), including the target cell ID and target TCI state ID(s) in the message.

Proposal 31. The gNB-CU1 forwards this information to the source gNB-CU, which then forwards this information to the target gNB-CU; in both cases, the Cell Switch Notification message is used.

Proposal 32. Upon receiving the Cell Switch Notification message, the target gNB-CU transmits the CU-DU Cell Switch Notification message to the target gNB-DU, including the target cell ID and target TCI state ID(s).

The above signalling enables the target DU to know the target cell and the target beam which the UE will access upon LTM cell switch so that, e.g., the network can prepare the target beam for the UE to use upon access of the target cell.

Figure 5: Example signalling flow of the Cell Switch Notification procedure

# 7 Handling of RRCReconfigurationComplete message transmitted by UE upon LTM cell switch

Upon completion of LTM cell switch, UE transmits an RRCReconfigurationComplete message to the target cell indicating that it has successfully applied the LTM target cell configuration. The RRCReconfigurationComplete message is forwarded to the source gNB-CU and is ciphered with the security key of the source gNB-CU. The RRCReconfigurationComplete message can be transmitted by the candidate gNB-CU to the source gNB-CU over Xn by using the RRC Transfer message; see Figure 6.

Proposal 33. The UE transmits a PDCP PDU (RLC SDU) containing RRCReconfigurationComplete message to the target gNB-CU. The PDCP PDU is ciphered with the key of the source gNB-CU.

The candidate gNB-CU forwards the PDCP PDU to the source gNB-CU as an octet string, using the RRC Transfer message.

Figure 6: Transmission of RRCReconfigurationComplete from the UE to the source gNB-CU using Xn RRC Transfer

# 8 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

LTM candidate cell configuration

Observation 1. For Inter-CU LTM with Anchor PDCP/RRC solution, during the LTM preparation phase, source CU uses legacy Xn Handover signalling messages and F1-C messages to prepare candidate DU/Cell configuration. Candidate CU will retrieve CellGroupConfig from candidate DU and will provide to source CU via legacy Xn Handover signalling messages.

Observation 2. Once LTM Cell Switch procedure is done, Source CU and Candidate DU exchange RRC signalling via RRC Transfer Procedure and Candidate CU will do routing of control plane signalling messages between candidate DU and Source CU, i.e., UE specific RRC messages are transparent to candidate CU.

Proposal 1. During initial preparation, the source gNB-CU determines the list of LTM candidate cells to be suggested for preparation and transmits Handover Request to a candidate gNB-CU including:

- LTM indication,

- Suggested candidate cell ID,

- UE measurement results.

The preparation of multiple LTM candidate cells belonging to a candidate gNB-CU can take place in parallel (similar as CHO).

Proposal 2. Regarding the handling of IEs in the Handover Request message (see Table 1) in the source gNB-CU anchor solution:

- An explicit indication is included in the message to indicate that the source gNB-CU will remain as the anchor upon LTM cell switch.

- The following mandatory IEs are ignored by the candidate gNB-CU upon reception of the message including an explicit indication of source keeping anchor-CU role.

  - UE Context Information > UE Security Capabilities/AS Security Information,

  - UE Context Information > PDU Session Resources To Be Setup List.

- The source gNB-CU determines and includes the mapping from QoS flows to DRBs for the candidate cell configuration and includes it in a DRB To Be Setup List IE. The DRB To Be Setup List IE (same as the existing IE in the UE Context Setup Request message) is included in the message, e.g., as a top-level IE.

- For the mandatory UE Context Information > RRC Context IE, information in this IE such as UE capabilities and source cell configuration need to be forwarded by the candidate gNB-CU to the candidate gNB-DU, which is legacy behavior.

Proposal 3. Similar as in Rel-18 intra-CU LTM, the source CU uses the F1 UE Context Setup procedure with its underlying candidate DUs for preparation of LTM candidate cells that belong to it.

Proposal 4. A candidate CU uses the F1 UE Context Setup procedure with its underlying candidate DUs for preparation of LTM candidate cells that belong to it. The candidate CU forwards the DRB To Be Setup List IE received in Xn Handover Request message to candidate DU in F1 UE Context Setup Request message.

Proposal 5. Similar as in Rel-18 intra-CU LTM, a candidate DU determines whether the suggested candidate cell can be prepared, and if it can be prepared, the candidate DU determines the lower layer configuration (CellGroupConfig) for the prepared candidate cell.

Proposal 6. An underlying candidate DU of a candidate CU transmits the lower layer configuration (CellGroupConfig) of the prepared candidate cell along with the candidate cell ID to the candidate CU. The candidate CU then forwards the CellGroupConfig of the prepared candidate cell to the source CU along with the candidate cell ID over Xn inside the inter-node RRC message Handover Command that is included in Handover Request Ack.

Proposal 7. The source CU generates the upper layer configuration, e.g., the radioBearerConfig containing, e.g., the PDCP and SDAP configuration, for the candidate cell, adds it to the provided CellGroupConfig to form the RRC configuration of the candidate cell.

Observation 3. As has been discussed above, the Handover Request/Handover Request Ack messages can be reused in the initial LTM preparation of the source gNB-CU anchor solution, with some Stage-3 enhancements that are discussed above and that involve low signaling complexity and overhead.

Early TA RACH configuration

Proposal 8. During LTM preparation, in Xn Handover Request, the source gNB-CU may transmit a request to a candidate gNB-CU for obtaining RACH configuration and for allocation of CFRA resources, e.g., RAP IDs, for Early TA acquisition. The following information is included:

- LTM candidate cell ID,

- List of source gNB-DU ID and candidate gNB-DU IDs of source gNB-CU.

Proposal 9. Upon receiving the request from the source gNB-CU, the candidate gNB-CU forwards the request to its underlying candidate gNB-DU in F1 UE Context Setup Request, including:

- LTM candidate cell ID,

- List of source gNB-DU ID and candidate gNB-DU IDs of source gNB-CU,

- The source gNB ID.

Proposal 10. If the suggested LTM candidate cell can be prepared, in the F1 UE Context Setup Response message, the candidate DU provides to the candidate CU:

- LTM candidate cell ID,

- List of pairs of the form (gNB-DU ID of the source CU, CFRA resource set, i.e., RAP IDs),

- CFRA configuration for Early TA acquisition,

- The source gNB ID.

Proposal 11. The candidate gNB-CU forwards the provided information from its underlying candidate DU to the source gNB-CU in Xn Handover Request Ack:

- LTM candidate cell ID,

- List of pairs of the form (gNB-DU ID of the source CU, CFRA resource set, i.e., RAP IDs),

- CFRA configuration for Early TA acquisition.

The candidate gNB-CU also includes the candidate gNB-DU ID.

Proposal 12. Upon receiving the Xn Handover Request Ack from the candidate gNB-CU, the source gNB-CU forwards the received information to the corresponding underlying DU in F1 UE Context Modification Request:

- LTM candidate cell ID,

- CFRA resource set, i.e., RAP IDs,

- CFRA configuration for Early TA acquisition.

Proposal 13. In Xn LTM Configuration Update message to a candidate gNB-CU, the source gNB-CU includes the following information:

- For each other candidate gNB-CU:

  - List of candidate gNB-DU IDs.

Proposal 14. Upon receiving the message from the source gNB-CU, the candidate gNB-CU forwards the request to its underlying candidate gNB-DUs in F1 UE Context Modification Request, including:

- For each other candidate gNB-CU:

  - List of candidate gNB-DU IDs,

  - The candidate gNB ID.

Proposal 15. In the F1 UE Context Modification Response message, the candidate DU provides to the candidate CU:

- For each prepared LTM candidate cell:

  - For each other candidate gNB-CU:

    - List of pairs of the form (candidate gNB-DU ID, CFRA resource set, i.e., RAP IDs),

    - CFRA configuration for Early TA acquisition,

    - The candidate gNB ID.

Proposal 16. In the Xn LTM Configuration Update Ack to the source gNB-CU, the candidate gNB-CU relays the information received from its underlying gNB-DU:

- For each prepared LTM candidate cell:

  - For each other candidate gNB-CU:

    - List of pairs of the form (candidate gNB-DU ID, CFRA resource set, i.e., RAP IDs),

    - CFRA configuration for Early TA acquisition,

    - The candidate gNB ID.

Proposal 17. The source gNB-CU reuses the Xn LTM Configuration Update procedure to forward the received Early TA configuration information from each candidate gNB-CU to the other candidate gNB-CUs.

TA Information Transfer

Proposal 18. Upon successfully receiving the RACH preamble from the UE on a candidate cell, a candidate gNB-DU determines the TA value. The candidate gNB-DU transmits to the candidate gNB-CU the following TA information using F1 DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Source gNB ID,

- Source gNB-DU ID,

- LTM candidate cell ID.

Proposal 19. The candidate gNB-CU forwards the TA information to the source gNB-CU using TA Information Transfer message over Xn.

Proposal 20. The source gNB-CU forwards the TA information to the source gNB-DU using F1 CU-DU TA Information Transfer.

Proposal 21. Upon successfully receiving the RACH preamble from the UE on a candidate cell, a candidate gNB-DU transmits to the candidate gNB-CU the following TA information using F1 DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Serving gNB ID,

- Serving gNB-DU ID,

- LTM candidate cell ID.

Proposal 22. The candidate gNB-CU forwards the TA information along with RAP ID to the source gNB-CU using TA Information Transfer over Xn.

Proposal 23. The source gNB-CU then forwards the TA information to the serving gNB-DU via the CU of the serving gNB-DU, using the TA Information Transfer and the CU-DU TA Information Transfer messages, respectively.

LTM CSI configuration

Proposal 24. During initial LTM preparation, source gNB-CU generates the LTM CSI resource configuration for the candidate cells, i.e., the list of SSBs to be measured by the UE for each candidate cell, based on:

- Information received from the candidate CUs and its own underlying DUs, in XN Setup and F1 Setup procedures, as mentioned in the assumptions above,

- RRC measurement report from the UE.

Proposal 25. During initial LTM preparation, the source gNB-CU provides to a candidate gNB-CU in Xn Handover Request:

- LTM CSI resource configuration.

This information is forwarded by the candidate CU to its underlying candidate DU using F1 UE Context Setup Request.

Proposal 26. A candidate DU provides to the candidate CU the following information in UE Context Setup Response, which is then forwarded by the candidate CU to the source CU in Xn Handover Request Ack:

- The prepared candidate cell ID and its associated lower layer configuration, which includes:

  - The LTM CSI report configuration,

  - TCI state configuration.

Proposal 27. The source CU may update the LTM CSI resource configuration based on the prepared cells of its underlying candidate DUs and those of the candidate CUs.

Proposal 28. The source gNB-CU includes in LTM Configuration Update message to a candidate gNB-CU:

- The updated LTM CSI resource configuration,

- TCI state configurations of prepared candidate cells of other candidate gNB-CUs.

This information is forwarded by the candidate CU to its underlying candidate DU using F1 UE Context Modification Request.

Proposal 29. A candidate DU provides to the candidate CU the following information in F1 UE Context Modification Response, which is then forwarded by the candidate CU to the source CU in Xn LTM Configuration Update Ack:

- For each prepared candidate cell, the associated lower layer configuration, which includes the updated LTM CSI report configuration.

LTM Cell Switch Notification

Proposal 30. Upon transmitting the LTM Cell Switch Command to the UE, the serving gNB-DU transmits the DU-CU Cell Switch Notification message to its gNB-CU (gNB-CU1), including the target cell ID and target TCI state ID(s) in the message.

Proposal 31. The gNB-CU1 forwards this information to the source gNB-CU, which then forwards this information to the target gNB-CU; in both cases, the Cell Switch Notification message is used.

Proposal 32. Upon receiving the Cell Switch Notification message, the target gNB-CU transmits the CU-DU Cell Switch Notification message to the target gNB-DU, including the target cell ID and target TCI state ID(s).

Handling of RRCReconfigurationComplete message transmitted by UE upon LTM cell switch

Proposal 33. The UE transmits a PDCP PDU (RLC SDU) containing RRCReconfigurationComplete message to the target gNB-CU. The PDCP PDU is ciphered with the key of the source gNB-CU.

The candidate gNB-CU forwards the PDCP PDU to the source gNB-CU as an octet string, using the RRC Transfer message.

# References

[1] Chair notes, RAN3 #125 meeting.

[2] R3-245314, Enabling subsequent inter-gNB LTM by decoupling of cell and anchor changes, RAN3 #125-bis meeting contribution, Qualcomm Incorporated, NTT DOCOMO, Vodafone, Bharti AirTel (India), Sony.

[3] Chair notes, RAN2 #125-bis meeting.