3GPP TSG-RAN WG3 #125-bis                                             	    R3-245315

Hefei, China, October 14th –18th, 2024

Agenda item:	   13.2

Source: 		Qualcomm Incorporated, Vodafone, NTT DOCOMO, Sony, Bharti Airtel (India)

Title: 	Impact analysis of coupling LTM cell switch and CU anchor changes on subsequent inter-gNB LTM

Document for:	Discussion

# 1	Introduction

In the last RAN3 meeting (RAN3 #125 [1]), further progress was made in developing the procedures of inter-CU LTM considering the case when the gNB-CU is switched at LTM cell switch.

RAN3 #125 agreements:

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

This paper discusses inter-gNB LTM with coupled cell/CU anchor changes.

# 2	Illustration of subsequent LTM with coupled cell switch and CU anchor changes

Figure 1 – subsequent inter-gNB LTM, with coupling of inter-gNB cell switch and RRC/PDCP anchor switch

Figure 1 depicts the subsequent inter-gNB LTM scenario, where each inter-gNB cell switch is accompanied by the transfer of the RRC/PDCP anchor to the target gNB.

# 3	Impact analysis

## 3.1	Compliance with WID

Subsequent inter-gNB LTM with coupled cell/anchor changes may necessitate RRC messages to include the security update for the next cell switch. As noted in [HW – RP-242080], this requirement is not aligned with existing WID objectives, which mandates to “avoid RRC configuration between cell switches.” A primary advantage of subsequent LTM is to enable cell switches without intermediate RRC signaling, a benefit that should be preserved from Rel-18 LTM design.

Observation 1a: A key advantage of subsequent inter-gNB LTM, as seen in Rel-18, is the avoidance of RRC configuration between cell switches. This benefit should be preserved in Rel-19 Inter-gNB LTM HO.

Observation 1b: To retain LTM benefits from Rel-18, and according to [HW – RP-242080], the Rel-19 LTM WID requires to avoid RRC configuration between cell switches.

Observation 1c: According to [HW - RP-242080], the WID requirement to avoid RRC configuration between cell switches may not be feasible for inter-gNB LTM with coupled cell/anchor changes.

Proposal 1: In compliance with the WID, RAN3 to only support solutions that avoid RRC signaling between subsequent inter-gNB cell switches.

## 3.2	Impact on UE

The WID states: “Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support.” Ideally, for genuine L1/L2 mobility, the UE should not need to differentiate between intra-gNB and inter-gNB candidate cells. The cell-to-gNB association is a RAN management issue that should not cause additional overhead or complexity for the UE.

Observation 2a: The WID states that Rel-18 LTM is baseline for inter-gNB LTM.

Observation 2b: In compliance with WID, the impact on the UE to support inter-gNB LTM on top of Rel-18 LTM should ideally be avoided or remain negligible.

If cell and anchor changes are coupled for subsequent inter-gNB LTM, the UE will be required to support new complex procedures that were not necessary in Rel-18 LTM. Specifically, the UE will need to:

- Support new schemes for PDCP reestablishment configuration

- Support new schemes for security updates

- Maintain awareness of intra-gNB versus inter-gNB mobility

The UE will also experience:

- Increased interruption due to PDCP reestablishment

- Increased interruption before UP traffic resumes due to additional signaling complexity

- Increased signaling overhead due to intermediate RRC configuration between inter-gNB cell switches

Observation 2c: Subsequent inter-gNB LTM with coupled cell/CU-anchor changes does have significant impact on the UE.

Observation 2d: Subsequent inter-gNB LTM with coupled cell/CU-anchor changes requires the UE to support complex procedures for PDCP entity handling and security update.

Observation 2e: Subsequent inter-gNB LTM with coupled cell/CU-anchor changes requires the UE to differentiate between various mobility types (e.g., intra-gNB vs. inter-gNB).

Observation 2f: Subsequent inter-gNB LTM with coupled cell/anchor changes degrades UE L1/L2 Mobility performance due to additional interruption and increased signaling overhead.

Given these impacts, it is evident that coupling cell and anchor changes for subsequent inter-gNB LTM does not adhere to the Rel-18 LTM baseline design, which is a WID requirement. The benefits of subsequent inter-gNB LTM with coupled cell/anchor are questionable compared to Rel-18 intra-gNB LTM or RACH-less L3 handover.

Proposal 2: In compliance with the WID statement that “Rel-18 intra-gNB LTM is the baseline for inter-gNB LTM”, RAN3 to strive to develop solutions that eliminate/minimize UE impact on top of Rel-18 LTM.

## 3.3	Network signaling overhead

One would expect that backhaul preparation of “lower”-layer mobility does not require more overhead compared to backhaul preparation of “upper”-layer mobility.

Observation 3a: The backhaul preparation/signaling overhead for Rel-19 LTM should be comparable to that of inter-gNB L3 handover.

In Rel-18 LTM, LTM preparation is executed once, which is then sufficient to support all subsequent cell switches as long as the UE is within the coverage of the LTM candidate cells. This is comparable to that of intra-gNB L3 handover.

Observation 3b: The backhaul preparation/signaling overhead for Rel-18 intra-gNB LTM is comparable to that of intra-gNB L3 handover, as a single LTM preparation phase is sufficient for the entire sequence of subsequent intra-gNB LTM cell switches.

However, subsequent inter-gNB LTM with coupled cell/anchor changes introduces significant complexity compared to inter-gNB L3 handover. Subsequent inter-gNB LTM with coupled cell/anchor change necessitates a security update by the UE Since any candidate cell could be the next target, all candidate gNBs must possess the necessary security keying information before LTM switch triggering. This requires the current serving gNB to pre-share the UE’s AS security information with the respective gNB for each inter-gNB LTM candidate cell. This process must then be repeated after each inter-gNB cell switch: since AS security information as part of the UE context is changed during the inter-gNB cell switch, all the prepared candidate gNBs need to be “re-prepared” with the updated UE security context after the inter-gNB cell switch. This contrasts with Rel-18 LTM, where a single LTM preparation phase is sufficient for the entire sequence of subsequent cell switches.

Observation 3c: The backhaul preparation/signaling overhead for subsequent inter-gNB LTM with coupled cell/CU anchor changes is significantly larger than that for inter-gNB L3 handover:

- UE context information must be shared during the initial LTM preparation phase with each candidate gNB.

- Updated UE context information, including new AS security information, must subsequently be shared with each candidate gNB before each inter-gNB cell switch.

For an LTM scenario involving (N) inter-gNB cells and (M) subsequent inter-gNB cell switches, the total number of LTM preparation procedures to share updated UE context information grows with (M(N-1)). These numbers become massive for the agreed maximum number of LTM candidate cells ((N=8)) and large number of subsequent inter-gNB LTM cell switches (M).

Observation 3d: For (N: max = 8) inter-gNB cells and (M: arbitrarily large) subsequent inter-gNB cell switches with coupled anchor changes, the number of LTM preparation/cleanup procedures to share or discard AS security information grows ‘quadratically’ with (N) and (M).

This is in stark contrast to Rel-18 LTM, where a single LTM preparation phase (involving no security update) was sufficient to cover all subsequent LTM cell switches among the LTM candidate cells. It is also significantly worse than non-subsequent L3 mobility, which requires only one HO preparation procedure per cell switch.

Proposal 3: Similar to Rel-18 subsequent intra-gNB LTM, where backhaul preparation overhead is comparable to that of intra-gNB L3 handover, RAN3 to discuss solutions for subsequent inter-gNB with overall backhaul signaling overhead comparable to that of L3 inter-gNB handover.

## 3.4	RRC anchor tracking

All candidate gNBs must always be aware of the current serving RRC anchor of the UE. This is necessary in case any candidate gNB needs to modify the UE’s RRC configuration, such as initiating cancellation or modification of the LTM candidate cells it serves.

Observation 4a: All candidate gNBs must always be aware of the current RRC anchor to support the cancellation or modification of LTM candidates.

This means that any subsequent inter-gNB LTM with coupled cell/anchor changes requires updating all candidate gNBs about the new UE’s RRC anchor after each inter-gNB cell switch. Consequently, all candidate gNBs, even those not involved in a given inter-gNB cell switch, will need to be updated.

Observation 4b: Each inter-gNB cell switch with an RRC anchor change affects all candidate gNBs, not just those involved in the switch, since all candidate gNBs must be notified about the new anchor.

This process introduces unnecessary signaling complexity and overhead. RAN3 should explore solutions to avoid such complications.

Proposal 4: RAN3 to discuss solutions that involve only the serving/target network entities during a cell switch instance, such as eliminating the need to track the current RRC anchor by other candidate gNBs as required by subsequent inter-gNB LTM with coupled cell/CU anchor changes.

## 3.5	Data forwarding

As explained in [ZTE – R3-244055], subsequent inter-gNB LTM with coupled cell/anchor changes requires each gNB to exchange data forwarding information per QoS flow and DRB mappings with every other gNB. This exchange could occur either at the very beginning or be spread out across the cell switches. In either case, a matrix of data forwarding tunnels among the candidate gNBs needs to be supported and maintained, resulting in quadratic complexity for tunnel information exchange, setup, maintenance and release. RAN3 should pursue alternative mechanisms to avoid this complexity, which counter-intuitively surpasses the complexity of L3 mobility.

Observation 5a: Complexity to handle [early] data forwarding for subsequent inter-gNB LTM should be comparable to that of inter-gNB L3 handover.

Observation 5b: As noted by [ZTE – R3-244055], subsequent inter-gNB LTM with coupled cell/anchor changes requires each gNB to exchange data forwarding information per QoS flow and DRB mappings with every other gNB.

Observation 5c: The complexity of handling [early] data forwarding for subsequent inter-gNB LTM with coupled cell/anchor changes is significantly higher compared to L3 mobility:

- A ‘matrix’ of data forwarding tunnels per QoS flow/DRB must be supported, including tunnel information exchange, setup, maintenance, and release.

Proposal 5: RAN3 has to discuss solutions that avoid complexities in handling early data forwarding that surpass those of L3 mobility, as would be introduced by subsequent inter-gNB LTM with coupled cell/CU anchor changes.

## 3.6	UE association among candidate gNBs

During the initial LTM preparation, the serving gNB establishes a UE association with each candidate gNB. At this point, it is not necessary for the candidate gNBs to have pairwise UE associations with each other. However, after the first inter-gNB cell switch, this target gNB – which was a prior candidate gNB - needs to be able to communicate with all other candidate gNBs on behalf of this UE. This is not possible unless the target gNB has pairwise UE associations with the other candidate gNBs. However, it is not clear how the target gNB establishes an UE association with another candidate gNB for this UE since these two gNBs have no common reference for this UE. Further, since subsequent handover can occur in any sequence, pairwise UE associations may be needed across all candidate gNBs.

Observation 6a: After each inter-gNB cell switch, the target gNB needs to have pairwise UE associations with all candidate gNBs.

Observation 6b: It is not clear how the target gNB establishes a UE association with a candidate gNB for this UE since these two gNBs have no common reference for this UE.

A similar issue arose in R18 mobile IAB with separate donors for mIAB-MT and mIAB-DU, where the BAP address of the mIAB-MT was used as an alternative means to create the UE association among the mIAB-node’s donors. For Rel-19 LTM, similar solutions may have to be considered.

Observation 6c: Rel-19 LTM could leverage solutions established for mobile IAB to create UE association between target gNB and candidate gNB.

This issue needs to be discussed by RAN3.

Proposal 6: To establish the necessary pairwise UE associations between target gNB and candidate gNBs after cell switch, RAN3 to consider reusing solutions introduced for mobile IAB.

## 3.7	Race conditions and RLF

One example of race conditions is brought up by [Nokia – R2-2407073]:

- UE performs a first inter-gNB cell switch from gNB1 to gNB2

- gNB2 initiates a path switch procedure towards the CN

- While the path switch procedure is ongoing, the UE’s serving link to gNB2 deteriorates

- A problem arises:

  - gNB2-DU cannot trigger a subsequent inter-gNB cell switch prior to finishing the NG path switch since the UE and the NW may become out of sync on the current active configuration, requiring the UE to reestablish its connection.

  - gNB2-DU cannot defer the subsequent inter-gNB cell switch since the UE may go into RLF

Another example of race condition:

- UE is connected to gNB1

- gNB1-CU is re-preparing AS security information with the candidate gNBs

- While the Xn re-preparation procedure is ongoing, the UE’s serving link to gNB1 deteriorates

- A problem arises:

  - gNB1-DU cannot trigger a subsequent inter-gNB cell switch prior to finishing the Xn re-preparation procedure since the UE and the NW may become out of sync on the current active configuration, requiring the UE to reestablish its connection

  - gNB1-DU cannot defer the subsequent inter-gNB cell switch since the UE may go into RLF

Observation 7a: Subsequent inter-gNB LTM with coupled cell/anchor changes suffers from race conditions:

- Between Xn LTM preparation before the cell switch and MAC-CE-based triggering of the cell switch

- Between MAC-CE-based triggering of the cell switch and NG path switch procedure after the cell switch, as noted by [Nokia – R2-2407073].

Ignoring these race conditions has serious consequences. For instance, the UE and the NW can become out of sync on the current active configuration, requiring the UE to reestablish its connection.

On the other hand, handling these race conditions is complex and can also have negative outcomes. For instance:

- Deferring the triggering of the subsequent inter-gNB cell switch may lead to RLF in case of wireless link degradation.

- Deferring the triggering of the subsequent inter-gNB cell switch until an Xn LTM preparation or NG path switch procedure concluded implies there will be a dependency between sending a MAC CE and performing an Xn or NG signaling handshake. This requires complex backhaul signaling to indicate to the gNB-DU when it should stop or resume LTM triggering based on ongoing Xn/NG procedures. The complex backhaul signaling may not fully resolve the issue because sending a MAC CE by the gNB-DU is much more dynamic compared to slower Xn or NG signalling procedures.

Observation 7b: Race conditions for subsequent inter-gNB LTM with coupled cell/anchor changes result in one of two outcomes:

- If ignored, the UE may need to reestablish its connection due to a configuration mismatch.

- If handled, the UE’s subsequent cell switch may be deferred, leading to RLF due to source link degradation.

Observation 7c: Handling race conditions between issuing a MAC CE by one network node and performing an Xn/NG procedure by another network node requires complex backhaul signaling and may not be fully resolvable due to the differing rates of dynamicity of the racing procedures.

This implies subsequent inter-gNB LTM with coupled cell/anchor changes fails to meet the robustness requirement for any UE mobility. It also introduces complex dependencies on actions carried out by separate NW entities and renders subsequent mobility infeasible.

Proposal 7a: RAN3 to only discuss inter-gNB LTM solutions that do not compromise robustness of UE mobility, e.g., due to race conditions.

Proposal 7b: RAN3 to discuss solutions that avoid race conditions as would be introduced by inter-gNB LTM with coupled cell/anchor changes.

## 4	Summary

Subsequent inter-gNB LTM with coupled cell/anchor changes has prohibitive complexity and fails to meet the Rel-19 LTM use case as per the WID.

None of the issues raised in the previous section arise if the anchor does not have to change with the cell as has been described in our companion paper [5]:

- No violation of WID: RRC signaling between cell switches is not needed.

- No impact on UE: Rel-18 procedures can be reused as is.

- No intermediate LTM preparation steps since there is no security update.

- No need to track the RRC anchor since it is fixed.

- No complex data forwarding since the anchor does not change.

- No missing UE association issue since all candidate gNBs communicate with the same anchor.

- No race conditions since the Xn preparation procedures prior to the cell switch and the NG path switch procedure after the cell switch are not needed.

Observation 8: If cell and anchor changes are decoupled for inter-gNB LTM, the complications of subsequent inter-gNB LTM with coupled cell/anchor changes vanish:

- No violation of WID, since no RRC signaling.

- Reuse of Rel-18 procedures for UE.

- No pre-cell-switch security preparations, since no security change after a switch.

- No RRC anchor tracking, since anchor is fixed after a cell switch.

- No complex data forwarding management among gNBs, since no NG path switch.

- No missing UE association problem, since association is not required among candidate gNBs.

- No race conditions.

RAN3 should pursue subsequent inter-gNB LTM with decoupled cell and anchor changes.

Proposal 8: RAN3 to support subsequent inter-gNB LTM with decoupled cell and anchor changes.

# Conclusion

This contribution discussed inter-gNB LTM with coupled cell/CU anchor changes. The following observations and proposals have been made:

Compliance with WID:

Observation 1a: A key advantage of subsequent inter-gNB LTM, as seen in Rel-18, is the avoidance of RRC configuration between cell switches. This benefit should be preserved in Rel-19 Inter-gNB LTM HO.

Observation 1b: To retain LTM benefits from Rel-18, and according to [HW – RP-242080], the Rel-19 LTM WID requires to avoid RRC configuration between cell switches.

Observation 1c: According to HW - RP-242080, the WID requirement to avoid RRC configuration between cell switches may not be feasible for inter-gNB LTM with coupled cell/anchor changes.

Proposal 1: In compliance with the WID, RAN3 to only support solutions that avoid RRC signaling between subsequent inter-gNB cell switches.

Impact on UE:

Observation 2a: The WID states that Rel-18 LTM is baseline for inter-gNB LTM.

Observation 2b: In compliance with WID, the impact on the UE to support inter-gNB LTM on top of Rel-18 LTM should ideally be avoided or remain negligible.

Observation 2c: Subsequent inter-gNB LTM with coupled cell/CU-anchor changes does have significant impact on the UE.

Observation 2d: Subsequent inter-gNB LTM with coupled cell/CU-anchor changes requires the UE to support complex procedures for PDCP entity handling and security update.

Observation 2e: Subsequent inter-gNB LTM with coupled cell/CU-anchor changes requires the UE to differentiate between various mobility types (e.g., intra-gNB vs. inter-gNB).

Observation 2f: Subsequent inter-gNB LTM with coupled cell/anchor changes degrades UE L1/L2 Mobility performance due to additional interruption and increased signaling overhead.

Proposal 2: In compliance with the WID statement that “Rel-18 intra-gNB LTM is the baseline for inter-gNB LTM”, RAN3 to strive to develop solutions that eliminate/minimize UE impact on top of Rel-18 LTM.

Network signaling overhead:

Observation 3a: The backhaul preparation/signaling overhead for Rel-19 LTM should be comparable to that of inter-gNB L3 handover.

Observation 3b: The backhaul preparation/signaling overhead for Rel-18 intra-gNB LTM is comparable to that of intra-gNB L3 handover, as a single LTM preparation phase is sufficient for the entire sequence of subsequent intra-gNB LTM cell switches.

Observation 3c: The backhaul preparation/signaling overhead for subsequent inter-gNB LTM with coupled cell/CU anchor changes is significantly larger than that for inter-gNB L3 handover:

- UE context information must be shared during the initial LTM preparation phase with each candidate gNB.

- Updated UE context information, including new AS security information, must subsequently be shared with each candidate gNB before each inter-gNB cell switch.

Observation 3d: For (N: max = 8) inter-gNB cells and (M: arbitrarily large) subsequent inter-gNB cell switches with coupled anchor changes, the number of LTM preparation/cleanup procedures to share or discard AS security information grows ‘quadratically’ with (N) and (M).

Proposal 3: Similar to Rel-18 subsequent intra-gNB LTM, where backhaul preparation overhead is comparable to that of intra-gNB L3 handover, RAN3 to discuss solutions for subsequent inter-gNB with overall backhaul signaling overhead comparable to that of L3 inter-gNB handover.

RRC anchor tracking:

Observation 4a: All candidate gNBs must always be aware of the current RRC anchor to support the cancellation or modification of LTM candidates.

Observation 4b: Each inter-gNB cell switch with an RRC anchor change affects all candidate gNBs, not just those involved in the switch, since all candidate gNBs must be notified about the new anchor.

Proposal 4: RAN3 to discuss solutions that involve only the serving/target network entities during a cell switch instance, such as eliminating the need to track the current RRC anchor by other candidate gNBs as required by subsequent inter-gNB LTM with coupled cell/CU anchor changes.

Data forwarding:

Observation 5a: Complexity to handle [early] data forwarding for subsequent inter-gNB LTM should be comparable to that of inter-gNB L3 handover.

Observation 5b: As noted by [ZTE – R3-244055], subsequent inter-gNB LTM with coupled cell/anchor changes requires each gNB to exchange data forwarding information per QoS flow and DRB mappings with every other gNB.

Observation 5c: The complexity of handling [early] data forwarding for subsequent inter-gNB LTM with coupled cell/anchor changes is significantly higher compared to L3 mobility:

- A ‘matrix’ of data forwarding tunnels per QoS flow/DRB must be supported, including tunnel information exchange, setup, maintenance, and release.

Proposal 5: RAN3 has to discuss solutions that avoid complexities in handling early data forwarding that surpass those of L3 mobility, as would be introduced by subsequent inter-gNB LTM with coupled cell/CU anchor changes.

UE association among candidate gNBs:

Observation 6a: After each inter-gNB cell switch, the target gNB needs to have pairwise UE associations with all candidate gNBs.

Observation 6b: It is not clear how the target gNB establishes a UE association with a candidate gNB for this UE since these two gNBs have no common reference for this UE.

Observation 6c: Rel-19 LTM could leverage solutions established for mobile IAB to create UE association between target gNB and candidate gNB.

Proposal 6: To establish the necessary pairwise UE associations between target gNB and candidate gNBs after cell switch, RAN3 to consider reusing solutions introduced for mobile IAB.

Race conditions and RLF:

Observation 7a: Subsequent inter-gNB LTM with coupled cell/anchor changes suffers from race conditions:

- Between Xn LTM preparation before the cell switch and MAC-CE-based triggering of the cell switch

- Between MAC-CE-based triggering of the cell switch and NG path switch procedure after the cell switch, as noted by [Nokia – R2-2407073].

Observation 7b: Race conditions for subsequent inter-gNB LTM with coupled cell/anchor changes result in one of two outcomes:

- If ignored, the UE may need to reestablish its connection due to a configuration mismatch.

- If handled, the UE’s subsequent cell switch may be deferred, leading to RLF due to source link degradation.

Observation 7c: Handling race conditions between issuing a MAC CE by one network node and performing an Xn/NG procedure by another network node requires complex backhaul signaling and may not be fully resolvable due to the differing rates of dynamicity of the racing procedures.

Proposal 7a: RAN3 to only discuss inter-gNB LTM solutions that do not compromise robustness of UE mobility, e.g., due to race conditions.

Proposal 7b: RAN3 to discuss solutions that avoid race conditions as would be introduced by inter-gNB LTM with coupled cell/anchor changes.

Observation 8: If cell and anchor changes are decoupled for inter-gNB LTM, the complications of subsequent inter-gNB LTM with coupled cell/anchor changes vanish:

- No violation of WID, since no RRC signaling.

- Reuse of Rel-18 procedures for UE.

- No pre-cell-switch security preparations, since no security change after a switch.

- No RRC anchor tracking, since anchor is fixed after a cell switch.

- No complex data forwarding management among gNBs, since no NG path switch.

- No missing UE association problem, since association is not required among candidate gNBs.

- No race conditions.

Proposal 8: RAN3 to support subsequent inter-gNB LTM with decoupled cell and anchor changes.

# References

- Chair notes, RAN3 #125.

- [HW – RP-242080], RP-242080, Discussion on Rel-19 mobility enhancements WID, Huawei, HiSilicon, RAN Plenary Meeting #105.

- [ZTE – R3-244055], R3-244055, Discussion on inter-CU LTM, ZTE Corporation, RAN3 #125.

- [Nokia – R2-2407073], R2-2407073, On inter-CU aspects for LTM, Nokia, RAN2 #127.

- R3-245314, RAN3 #125-bis contribution, Enabling subsequent inter-gNB LTM by decoupling of cell and anchor changes, Qualcomm Incorporated, Vodafone, NTT DOCOMO, Sony, Bharti Airtel (India).