3GPP TSG-RAN WG3 #125-bis																R3-245316

Hefei, China, 14th – 18th October 2024

Agenda Item:	13.2

Source:	Ericsson

Title:	Further discussion on XnAP signaling for inter-CU LTM

Document for:	Discussion, Agreement

# Introduction

In this contribution, we continue the discussion on XnAP signaling design for inter-CU LTM, considering the following agreements and open points captured at previous RAN3 meeting:

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

# Discussion

## General principles

Some general principles should be considered for Xn signaling design for inter-CU LTM:

- Independence from gNB internal architecture: from a logical perspective, a peer gNB is seen as a “black-box”. In general, the Xn interface has no notion of the logical nodes constituting a gNB (gNB-DU, gNB-CU), and as a consequence, transfer of information that relates to (or is associated with) such logical functions have no meaning over Xn. A direct implication of this is that an IE such as “LTM gNB-DU ID” defined in F1 should not be signalled over Xn.

- Minimize delay: since LTM aims to minimize mobility interruption times using some information (e.g., TA values) that can be short-lived, signaling this type of information over Xn should avoid unnecessary delays. For example, transferring a TA value calculated for a given UE should be done as quick as possible.

From the above, it is clear that Xn signaling design can consider choices made for F1 signaling design, but some differentiations are needed. More details are presented in the following sections.

- For inter-CU LTM, Xn has no notion of the logical nodes of a gNB (gNB-DU, gNB-CU).

- Keep the existing principle that Xn AP Identities and F1 AP Identities are independent.

- For inter-CU LTM, unnecessary delay should be avoided over Xn when designing the signalling (e.g., to transfer TA values).

## Initial request for LTM configuration

According to agreements taken at previous meetings, Handover Request and Handover Request Ack should be reused for Inter-CU LTM initial preparation, and the source gNB-CU initiates the handover preparation for inter-gNB LTM.

Some general aspects remain to be decided. In particular, we think the group should continue to discuss which of the two options below is to be used:

- a single Handover preparation that allows LTM preparation of multiple cells, or

- a single Handover preparation that allows LTM preparation of (only) one cell

If option A is selected, and the candidate gNB controls multiple LTM candidate cells, the source gNB can send a single HO Request message to a candidate gNB for LTM preparation of such cells.

If option B is selected, and the candidate gNB controls multiple LTM candidate cells, the source gNB sends multiple HO Requests to the candidate gNB, one for each candidate cell.

To select which one of the two options is more appropriate, we would like RAN3 to discuss the following aspects: 1) Handling of UE association(s), 2) Error Handling. A preliminary analysis in presented in the table below.

| Item | Option A (One HO Request for LTM preparation of multiple cells) | Option B (One HO Request for LTM preparation of (only) one cell) |
| --- | --- | --- |
| Handling of UE Association(s) | Only one UE association is established, regardless of the number of candidate cells. The target gNB, receiving the HO Request knows that all the candidate cells for which LTM is prepared are associated to a unique UE, the one referenced by the Source NG-RAN node UE XnAP ID reference IE. The handling of UE associations is simple. | In principle, a different UE association can be established for the same UE for each request to prepare a different candidate cell for that UE. For example, the target gNB receives for the same UE two HO Request, the first one for candidate cell 1 and containing a first Source NG-RAN node UE XnAP ID reference, and a second one for candidate cell 2 and containing a second Source NG-RAN node UE XnAP ID reference. How does the target gNB understand that the two Source NG-RAN node UE XnAP ID reference(s) point to the same UE? |
| Error handling | The error handling is simple, since it relates to a single procedure. For example, there can be only one value for non-LTM related IEs included in HO Request (e.g., GUAMI). | The error handling is complex, since it needs to consider that the same IE can be received, potentially with different values, across the multiple procedures. For examples, the non-LTM related IEs included in the multiple HO Requests may differ (e.g., different GUAMI). This requires more complex handling from implementation point of view. |

Additional discussions took place regarding the existing CHO mechanism, where the target cell ID is used to identify parallel transactions for the conditional handover, as outlined.

If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

As observed, parallel procedures currently are permitted exclusively for conditional handover. However, if Option B is selected, it remains unclear how the following two entities would agree on the state over the protocol for LTM cell preparation, as shown below. A single Source NG-RAN node UE XnAP ID and one Target NG-RAN node UE XnAP ID are assigned per UE. Signaling is handled over one LTM cell relation, while parallel signaling occurs across different LTM cell relations within the same UE association.

Figure 1. Parallel signalings for establishing LTM cell preparation

Based on the presented analysis, we think that one HO Request for LTM preparation of multiple candidate cells is preferrable as it provides simpler implementation.

- For the initial configuration of inter-CU LTM, it is preferable to use one Handover Preparation procedure to manage multiple LTM candidate cells.

## LTM configuration update

At the last RAN3 meeting, it was agreed to “Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.”. We propose to call the new procedure “LTM Configuration Update”.

- The name of the new Class-1 XnAP procedure to update LTM configurations is “LTM Configuration Update”.

Below, we discuss our view about when and for which purposes the new procedure can be used. In the discussion we also make use of the simple agreement taken at RAN3#125 to “Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.”. The CELL SWITCH NOTIFICATION XnAP message is mentioned below in the discussion, and captured in our TP for 38.423 in [1].

First, during LTM preparation, in the HANDOVER REQUEST ACKNOWLEDGE, the source gNB receives from each candidate target gNB, information (e.g., the TCI state configuration of candidate cells) needed to construct the consolidated LTM configuration. To support early sync in subsequent LTM in NG-RAN, each candidate target gNB needs to be aware of the other candidate gNBs’ LTM early sync configuration. To achieve this, we propose to reuse the logic of inter-gNB-DU LTM. As indicated in stage 2, the gNB-CU may send a UE CONTEXT MODIFICATION REQUEST to the candidate gNB-DU(s) with information for subsequent LTM or for updating the configurations of candidate cells.

7.  The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information for subsequent LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

A possible use of the new LTM Configuration Update procedure can therefore be to make the candidate gNBs aware of the consolidated LTM early sync configuration prepared by the source gNB. To achieve the same type of F1 support in Xn, the source gNB can initiate the new procedure after the HANDOVER REQUEST ACKNOWLEDGE and before the Cell Switch Command.

- To support subsequent LTM in the RAN, the source gNB sends to the candidate gNBs early sync configuration via the new XnAP procedure (e.g., LTM Configuration Update).

The newly agreed Xn procedure can be used to support subsequent LTM also in setting up the UE association over Xn (pair of XnAP UE IDs) after LTM execution. We note that, during LTM preparation, even if the source gNB (e.g., gNB1) can contact via Xn each target candidate gNBs (e.g., gNB2, gNB3, gNB4), i.e., even if Xn is available between the source gNB and each candidate gNBs, the source gNB cannot know whether Xn is available between each pair of candidates gNBs (e.g., gNB1 cannot know if Xn is available between gNB2 and gNB3). To support subsequent LTM at network side, setting up of the UE association between two nodes makes practical sense only when one of them serves the UE. For example, when gNB2 receives from gNB1 the CELL SWITCH NOTIFICATION, it knows that from now on it serves the UE, and can initiate the setup of the UE association for the UE between itself and gNB3 (assuming Xn is available between gNB2 and gNB3). In case (for any reason) no Xn connectivity is available between these two nodes, the UE association should not be established, and subsequent LTM is not supported over Xn between gNB2 and gNB3. As just described, setting up the UE association over Xn is crucial to support subsequent Xn-based LTM inter-CU, so we think the use the newly agreed procedure for this purpose is a natural choice. Using this procedure, the new source gNB chooses a UE XnAP ID and sends it to a candidate gNB in the initiating message, and the candidate gNB sends its assigned UE XnAP ID to the source gNB in the terminating message. When the procedure is executed successfully, the UE association is in place.

- To support subsequent LTM in the RAN, the new XnAP procedure (e.g., LTM Configuration Update) is used to allocate UE XnAP IDs between the new source gNB and the candidate gNBs.

A third important aspect to support subsequent LTM in the RAN is that the new source gNB needs to know which LTM candidate cells can be used and their configuration. In F1, this is achieved via the LTM Configuration ID Mapping List IE (in UE CONTEXT MODIFICATION REQUEST), and we can reuse the same approach in Xn. Note that to support a UE moving back and forth between two gNBs, the new source gNB can include the old source cell in the mapping (if configured as LTM candidate).

For sending the updated LTM Configuration ID Mapping List via Xn, we consider two options:

Option 1: after HANDOVER SUCCESS, the new source gNB sends the LTM Configuration ID Mapping List to all “new” candidates (including the original source if configured as candidate)

Option 2: the old source gNB sends the updated LTM Configuration ID Mapping List to the new source gNB in the CELL SWITCH NOTIFICATION

We prefer option 1, since the new source is the one deciding the candidate cells after LTM execution. The new procedure (e.g., LTM Configuration Update) can be used for the purpose.

- To support subsequent LTM in the RAN, the new source gNB sends to the candidate gNBs updated LTM Configuration ID Mapping List via the new XnAP procedure (e.g., LTM Configuration Update).

Another aspect in which the newly agreed procedure can support subsequent LTM in the RAN is for retrieving the Preamble Index allocated for a UE. To do so, the source gNB can request the candidate gNBs to provide Preamble Index, which is returned in the response message.

- To support subsequent LTM in the RAN, the source gNB can use the new XnAP procedure (e.g., LTM Configuration Update) to request Preamble Index for a UE to the candidate gNBs.

## TA information transfer

For proper design of the Xn message for transferring TA value (e.g., “TA information Transfer”) we think some clarifications are needed. First, it should be discussed whether the procedure for TA value transfer should be UE-associated (to transfer only one TA value), or non-UE associated procedure, to transfer more than one TA values in one go.

A certain TA value is always associated to only one UE, therefore the UE associated identities assigned by the source gNB and the candidate target gNB should be included in the message, to uniquely identify the UE to which the TA value corresponds to.

- Include the UE XnAP IDs of the source and target gNB in the message for TA information transfer.

A TA value can be regarded as short-lived, i.e., it may become invalid in a short time. For this reason, it is crucial to avoid unnecessary delay to transfer this information via Xn. Some companies proposed to use a non-UE associated message to transfer more than one TA values in one go. However if this approach is used, before the message is sent, we will introduce an unnecessary delay due to the different times at which the individual TA values to be included in the message becomes available. For example, if a non-UE associated message is used to transfer 8 TA values, there is an additional delay introduced, from the time when the 1st TA value becomes available, until the time when the 8th TA value becomes available. There is a high chance that when the TA values need to be used they are not valid anymore, and the advantage of the Early UL synch is completely lost. If we use a UE-associated procedure instead, there is no extra delay, at the expense of an increased number of messages to be sent over Xn. We think that the cost of sending more messages is justified, otherwise the whole Early UL synch step becomes useless. In summary, even if existing F1 messages (DU-CU/DU-CU TA Information Transfer) used to transfer TA values are non-UE associated, there is no need to use the same approach for Xn, and we prefer to use a UE-associated procedure.

- The Xn procedure to transfer a TA information is UE-associated.

Regarding whether the procedure should be a class 1 or a class 2, in this case we think the same approach used in F1 can be kept, i.e. use a class 2 procedure.

- The Xn procedure to transfer a TA information is a class 2 procedure.

Regarding the presence or not of a “Source DU ID” in Xn, we have already mentioned that a gNB has no visibility of whether a peer gNB is deployed in non-split architecture or in split architecture, therefore a Source DU ID cannot be included in the Xn message for TA value transfer. On the other hand, some kind of identifier is needed to realize in Xn the same type of support provided in F1 by the Source DU ID. We propose to use a new identifier, e.g., an “Early UL Sync Configuration Provider ID”.

- For inter-CU LTM, transfer via Xn an new identifier (e.g., an “Early UL Sync Configuration Provider ID”).

## LTM cancel

At RAN3#125-bis the following agreements and open points were minuted:

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

In our view, the cleanest approach is to introduce a new procedure, which also provides a future proof solution for LTM. We propose to call such procedure “LTM Handover Cancel”. A candidate gNB will send a LTM HANDOVER CANCEL to the source gNB to cancel one or more configured LTM candidate cells.

- Introduce a new class-2 procedure, LTM HANDOVER CANCEL for candidate gNB initiated LTM cancellation.

## Subsequent LTM and Release of UE Context at source gNB

At last meeting the following point is left FFS: “FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.”.

If no LTM candidate cell is left in the source gNB, it is reasonable to release the corresponding UE context. Therefore, we propose to remove the FFS and reuse the existing UE CONTEXT RELEASE message for this purpose.

- Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

# Conclusions and Proposals

In the previous sections we made the following observations and proposals:

Observation 1	For inter-CU LTM, Xn has no notion of the logical nodes of a gNB (gNB-DU, gNB-CU).

In this paper we propose:

Proposal 1	Keep the existing principle that Xn AP Identities and F1 AP Identities are independent.

Proposal 2	For inter-CU LTM, unnecessary delay should be avoided over Xn when designing the signalling (e.g., to transfer TA values).

Proposal 3	For the initial configuration of inter-CU LTM, it is preferable to use one Handover Preparation procedure to manage multiple LTM candidate cells.

Proposal 4	The name of the new Class-1 XnAP procedure to update LTM configurations is “LTM Configuration Update”.

Proposal 5	To support subsequent LTM in the RAN, the source gNB sends to the candidate gNBs early sync configuration via the new XnAP procedure (e.g., LTM Configuration Update).

Proposal 6	To support subsequent LTM in the RAN, the new XnAP procedure (e.g., LTM Configuration Update) is used to allocate UE XnAP IDs between the new source gNB and the candidate gNBs.

Proposal 7	To support subsequent LTM in the RAN, the new source gNB sends to the candidate gNBs updated LTM Configuration ID Mapping List via the new XnAP procedure (e.g., LTM Configuration Update).

Proposal 8	To support subsequent LTM in the RAN, the source gNB can use the new XnAP procedure (e.g., LTM Configuration Update) to request Preamble Index for a UE to the candidate gNBs.

Proposal 9	Include the UE XnAP IDs of the source and target gNB in the message for TA information transfer.

Proposal 10	The Xn procedure to transfer a TA information is UE-associated.

Proposal 11	The Xn procedure to transfer a TA information is a class 2 procedure.

Proposal 12	For inter-CU LTM, transfer via Xn an new identifier (e.g., an “Early UL Sync Configuration Provider ID”).

Proposal 13	Introduce a new class-2 procedure, LTM HANDOVER CANCEL for candidate gNB initiated LTM cancellation.

Proposal 14	Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

# References

[1] R3-245317 - (TP for TS 38.423) Xn support for inter-CU LTM, Ericsson

[2] R3-245318 - (TP for TS 38.300) Stage-2 signalling for inter-CU LTM, Ericsson