3GPP TSG-RAN WG3 Meeting #125bis	R3-245323

Hefei, China, 14 – 18 October 2024

Agenda item:	12.3

Source:	Nokia

Title:	Completion of Protocol and Functional aspects of NR Femto Architecture

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At the last RAN3#125 the study item of NR Femto was successfully completed and the outcome is visible in TR 38.799 which was agreed in R3-244849.

At the following RAN Plenary#105, the corresponding work item was agreed in [1] including the following objective:

The objectives of the 5G Femto work are as follows:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

This paper evaluates the corresponding functional and protocol impacts and proposes stage 2 CR in [2],[3] and stage 3 CR in [4] and LS in [5] to progress the work.

# 2	Discussion

The following conclusion was reached in the TR 38.799 for the NG interface:

### 6.2.1	Architecture for NG interface

Option 1, namely direct connection of an NR Femto node to the 5GC via the NG interface, is already possible.

In order to maintain the existing infrastructure for an operator who has deployed LTE HeNBs, Option 2 with an optional NR Femto GW is recommended for a normative phase.

It is clear from the above text that the agreement to have an optional NR Femto Gateway in the architecture was taken in consideration of smooth integration for an operator who has deployed HeNBs using HeNB GW. Therefore, it is assumed that the protocol and functional aspects of the NR Femto GW to be specified should be similar to the HeNB GW in 4g network.

The protocol and functional aspects of HeNB GW can be found in details in TS 36.300 section 4.6. We therefore assume to base the stage 2 CR for NR Femto Gateway adapting the section 4.6 of TS 36.300 to 5G.

Proposal 1: align the protocol and functional aspects of NR Femto GW with the equivalent protocol and functional aspects of HeNB GW.

Besides, in order to enable a smooth discussion, we propose to separate the discussion in two parts:

Part 1: the architecture and protocol aspects covered by the CR in [2].

Part 2: the functional aspects covered by the CR in [3].

While the Part 1 CR in [2] is straightforward, for the functional Part 2 in [3] the HeNB GW functions of TS 36.300 section 4.6 reveals several security verification checks. It FFS pending SA3 study whether the same verification checks would apply to the 5G NR Femto architecture agreed by RAN3 and this is why we propose to capture them for the time being in the stage 2 CR [3] as NOTEs and open issues to be further checked.

Proposal 2: agree the stage 2 CR in [2] for the architecture and protocol aspects.

Proposal 3: agree the stage 2 CR in [3] for the functional aspects including NOTEs for the security aspects which are still pending SA3.

Besides, the stage 3 impacts corresponding to the functional changes described in CR [3] are presented in the TS 38.413 CR in [4] and proposed for agreements.

Proposal 4: agree the stage 3 CR for TS 38.413 in [4].

Finally, it would be beneficial to emphasize explicitly to SA3 those security points which needs checking for parity with 4g and are pending in NOTEs the CR [3] so that SA3 can give a quick feedback.

Proposal 5: send the LS in [5] to SA3 to check whether the verification aspects which applied to HeNB GW architecture apply to NR Femto GW architecture.

# 3	Conclusion and Proposals

This paper has evaluated the objectives and corresponding work to complete the NR Femto work item agreed in [1] and made the following proposals:

Proposal 1: align the protocol and functional aspects of NR Femto GW with the equivalent protocol and functional aspects of HeNB GW.

Proposal 2: agree the stage 2 CR in [2] for the architecture and protocol aspects.

Proposal 3: agree the stage 2 CR in [3] for the functional aspects including NOTEs for the security aspects which are still pending SA3.

Proposal 4: agree the stage 3 CR for TS 38.413 in [4].

Proposal 5: send the LS in [5] to SA3 to check whether the verification aspects which applied to HeNB GW architecture apply to NR Femto GW architecture.

# 4	References

- RP-242395, Additional topological enhancements for NR, NTT Docomo, AT&T

- R3-245324, Introduction of NR Femto architecture and protocol aspects, TS 38.300

- R3-245325, Introduction of Functional aspects of NR Femto architecture, TS 38.300

- R3-245326, Introduction of NR Femto architecture with optional NR Femto GW, TS 38.413

- R3-245327, LS on security verifications related to NR Femtos