3GPP TSG-RAN3 Meeting #125bis	R3-245331

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	16.2

Source: 	Qualcomm Incorporated

Title: 	Architecture, interfaces and transport for Ambient IoT

Document for:	Discussion/Decision

# Background

In this paper, we continue to provide our views on the architecture, interface and transport aspects for Ambient IoT (A-IoT) based on the agreements and open issues identified in the previous RAN3 meetings.

# Discussion

## Terminologies

In RAN3#123bis, RAN3 started the discussion identifying the logical nodes for T1 and T2 e.g., “A-IoT RAN node”, “UE Reader”, “A-IoT-enabled gNB”, “A-IoT-aware CN node”. However, in RAN3#124, RAN3 went away from the discussion of logical nodes and started discussing functionalities and drawing logical boxes such as “A-IoT RAS”, “A-IoT CN” to identify the functional split between A-IoT RAN and A-IoT CN. But there was confusion on what A-IoT RAS refers to and whether it also applies to T2.

Subsequently in RAN3#125, A-IoT RAS was renamed to A-IoT RAN to refer to the BS Reader in T1, while keeping the terminology of A-IoT enabled gNB for T2. While this was a good clarification, there are still inconsistencies in the TR e.g., “AIoT RAN” and “AIoT RAN node” are used interchangeably.

In our view, RAN3 should continue its architecture discussion using logical nodes and not consider logical boxes e.g., A-IoT RAN and A-IoT CN for identifying the functional RAN-CN split. We therefore propose to rename “A-IoT RAN” as “A-IoT RAN node” which is the BS Reader in Topology 1.

Rename "A-IoT RAN" as "A-IoT RAN node". AIoT RAN node refers to the BS Reader in Topology 1.

Further, in the previous meetings, the details of the AIoT CN were left to SA2 as captured in the TR below and it was captured that the AIoT CN may include AMF and A-IoT related functions for both T1 and T2, pending SA2 decision as seen from the below text in the TR.

A-IoT CN:	Hosts certain functions for A-IoT as of the functional split between RAN and CN.

NOTE: the details of A-IoT CN are subject to SA2.

Editor’s Note 3: The protocol stack does not detail how A-IoT upper layer information is transported over XXAP, details are pending on SA2 agreements. And the A-IoT CN may include AMF and A-IoT related functions which is also up to SA2 decision.

Editor’s Note 3:	The A-IoT CN could include AMF and A-IoT related functions. This is up to SA2 decision.

In the last SA2 meeting, SA2 agreed on the following conclusion i.e., to support a new CN function to support AIoT as seen below from TR 23.700-13-050:

At least the following principles are agreed for the architecture to support topology 1:

-	A new core network function is introduced to support Ambient IoT.

Editor's note:	Whether the new core network function also applies to topology 2 is FFS.

To align with SA2 conclusion, we therefore propose to use a new terminology “A-IoT CN node” to refer to this new core network function.

SA2 concluded to support a new CN function to support A-IoT for topology 1. SA2 is still discussing whether this new CN function also applies to topology 2.

RAN3 should use the terminology “A-IoT CN node” to refer to the new CN function agreed by SA2 to support A-IoT, instead of referring to multiple core network functions as “A-IoT CN”.

Further the following FFS was captured in the Chair’s Notes.

For Topology 2, whether AIoT RAN needs to be revised as AIoT enabled gNB needs to be further checked in next meeting

In our view, it is still early to decide whether the A-IoT RAN node in T1 can be considered as an A-IoT enabled gNB (i.e., an NG-RAN node) because the legacy NG-RAN node as defined in 38.300 has a specific definition.

AS per TS 38.300, NG-RAN node is either a gNB (or ng-eNB), providing NR (or E-UTRA) user plane and control plane protocol terminations towards the UE, hosts host several functions (e.g., connection setup, paging, system information broadcast, header compression, ciphering, integrity protection), interconnected with each other via Xn interface, connects to AMF/UPF via NG-C/NG-U interface.

However, the A-IoT RAN node is an entity which interacts with an A-IoT device (not a UE) via a new A-IoT radio interface and several of the above functionalities e.g., header compression, ciphering, integrity protection, connection setup, “legacy” paging doesn’t apply to A-IoT RAN node.

A-IoT RAN node in Topology 1 interacts with a A-IoT device (not a UE) via a new A-IoT radio interface (not Uu) and doesn’t host several of the legacy NG-RAN node functionalities. It is also not clear whether there is a need for A-IoT RAN nodes to be interconnected via an Xn-like interface, whether A-IoT RAN node can connect to AMF etc.

We therefore propose the following:

Continue to use "A-IoT RAN node" for T1 and "A-IoT-enabled gNB" for T2 till there is clarity on differences/commonalities in T1 and T2 (e.g., on transport/functions). Decide in normative phase (or later in study phase) whether the A-IoT RAN node can be specified as an NG-RAN node or needs to be considered as a new entity.

## Topology 1

### XX interface and AMF involvement

For Topology 1, the interface between the A-IoT RAN node and A-IoT CN node (“XX interface”) is still not clear (e.g., whether it is a direct interface or are there any intermediate nodes (e.g., AMF) in between) as seen from the below text in the TR.

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between A-IoT RAN node and A-IoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between A-IoT CN and A-IoT RAN, or one instance for NG alone.

SA2 is still discussing whether existing 5GC entities (e.g., AMF) is involved in procedures related to A-IoT and which of the legacy 5GC functionalities (e.g., registration, connection management) apply to A-IoT.

In this section, we discuss two potential options for transporting A-IoT related communication (final decision up to SA2):

- T1-Option 1 (Indirect transport solution): A-IoT RAN node indirectly (via the AMF) communicates with an A-IoT CN node.

- T1-Option 2 (Direct transport solution): A-IoT RAN node directly communicates with an A-IoT CN node.

Table 1 further compares the two options by showing its protocol stack, discussing the interfaces involved and discussing its pros/cons.

Table 1: Evaluation of transport solutions for Topology 1

| T1-Option 1 (Indirect transport solution) | T1-Option 2 (Direct transport solution) |
| --- | --- |
| (Potentially) Two new protocols need to be defined. Protocol b/w A-IoT RAN node and AMF (i.e., YYAP), if NGAP is not reused Protocol b/w A-IoT RAN node and A-IoT CN node (i.e., XXAP) | Only 1 new protocol needs to be defined. Protocol between A-IoT RAN node and A-IoT CN node (i.e., XXAP). |
| (Potentially) Two new interfaces need to be defined. Interface between A-IoT RAN node and AMF (YY) if N2 is not reused Interface between AMF and A-IoT CN node (Nac) | Only 1 interface needs to be defined. Interface between the A-IoT RAN node and A-IoT CN node (i.e., XX) |
| Different protocol stack at A-IoT RAN node and A-IoT CN node SCTP-based stack between A-IoT RAN node and AMF. HTTP-based stack between AMF and A-IoT CN node. | Common protocol stack at A-IoT RAN node and A-IoT CN node Option 1: HTTP-based stack at A-IoT RAN node and A-IoT CN node. Option 2: SCTP-based stack at A-IoT RAN node and A-IoT CN node. |
| Need upgrades to legacy AMF. | No impact to legacy AMF. |
| Less developer/cloud friendly Uses point-to-point interface between A-IoT RAN node and AMF (YY interface) and service-based interface between AMF and A-IoT CN node. | More developer/cloud friendly If HTTP-based stack is used, this will use a Service Based Interface (SBI) between A-IoT RAN node and A-IoT CN node which is more developer/cloud friendly, RAT agnostic compared to a point-to-point interface. |
| More backhaul latency due to involvement of AMF. AMF is primarily used as a forwarding entity (“router”) for forwarding packets between A-IoT RAN node and A-IoT CN node. Potentially unnecessary involvement of AMF. | Less backhaul latency (no involvement of AMF) |

Based on the above analysis, we have the following proposals:

RAN3 to capture in TR that the following 2 options are possible for the AIoT RAN node to communicate with AIoT CN node in T1:

- T1-Option 1 (Indirect transport solution): AIoT RAN node indirectly (via the AMF) communicates with the AIoT CN node.

- T1-Option 2 (Direct transport solution): AIoT RAN node directly communicates with the AIoT CN node.

RAN3 to capture in TR “Table 1” which evaluates the two transport options for AIoT communication in Topology 1, lists their pros and cons along with the WG impacts

T1-Option 1 (Indirect transport solution) has the following drawbacks compared to T1-Option 2 (direct transport solution):

  - Need upgrades at legacy AMF to support two new interfaces (AIoT RAN node-AMF, AMF-AIoT CN node)

  - More backhaul latency (due to involvement of AMF)

  - Potentially unnecessary involvement of AMF (just as a “router”) for AIoT

  - Potentially additional protocol b/w AIoT RAN node and AMF (YYAP) needs to be defined to carry end-to-end packets between AIoT RAN node and AIoT CN node (XXAP)

RAN3 to capture in TP as a conclusion for T1 that “It is RAN3’s preference that the AIoT RAN node directly communicates with the AIoT CN node (without going via AMF) in the study of Topology 1 (referred to as T1-Option 2: Direct Transport Solution in this paper)”

In Topology 1, there is an end-to-end “XXAP” protocol between the AIoT RAN node and AIoT CN node over a direct XX interface.

### A-IoT upper layer information

Editor’s Note 3: The protocol stack does not detail how A-IoT upper layer information is transported over XXAP, details are pending on SA2 agreements. And the A-IoT CN may include AMF and A-IoT related functions which is also up to SA2 decision.

Assuming SA2/SA3 concludes that there is an “Upper layer” protocol terminated between the A-IoT device and A-IoT CN node e.g., for end-to-end security of A-IoT packets e.g., Command Request/Response, the protocol stack for T1 can be updated as follows:

SA2/SA3 is discussing several aspects on registration, device identification, security, authentication and there is no conclusion yet. Potentially, SA2 might conclude to support an “upper layer” terminating between the A-IoT device and the A-IoT CN node to support one or more of the above aspects.

Update the protocol stack of T1 to depict an “Upper layer” protocol terminating between the A-IoT device and A-IoT CN node. Details and need of this “Upper layer” protocol is up to SA2/SA3 conclusion.

Also, it is not yet clear what information is to be carried as “Upper layer information” over XXAP and what information is to be carried explicitly over XXAP. For example, if SA2/SA3 concludes that Device ID is to be end-to-end protected via “upper layer” security, Device ID should be carried over “upper layer” information over XXAP. Whereas some information e.g., contents of A-IoT Inventory Request such as Scope of Inventory can be carried explicitly over XXAP. Therefore, the following is proposed:

RAN3  assumes that some A-IoT related information (e.g., Device ID) might be carried as “upper layer information” over XXAP and some A-IoT related information (e.g., Scope of Inventory) might be carried explictly over XXAP. This is pending SA2/SA3 conclusion.

### Radio Resource management in T1

In the TR, the following editor’s notes are also captured:

Editor’s Note 4: aspects of interaction between upper layer information exchange and XXAP in order to trigger the A-IoT RAN node functions are FFS.

Consider an example scenario where Inventory Request is carried explicitly over XXAP whereas Command Request is carried as “Upper layer information” over XXAP. In such a case, A-IoT RAN node can determine A-IoT resource configuration based on inventory configuration received explictly over XXAP whereas it needs some A-IoT assistance information from the A-IoT CN node to determine the A-IoT resource configuration e.g.,

- Type of A-IoT service request (i.e., Command)

- Estimated number of devices for which the Command Request is sent

- Whether feedback is expected in D2R after sending Command (e.g., there might be no feedback for Write/Kill whereas there might be feedback for Read).

Inventory Request carried explicitly over XXAP

Command Request carried as “upper layer information” over XXAP

We therefore propose the following:

For Topology 1, A-IoT CN node can provide “A-IoT assistance information” (e.g., Type of A-IoT service request, estimated number of devices) explicitly in XXAP message along with the Upper layer information to assist in A-IoT radio resource management.

### Signaling transport for XXAP

In the TR, a protocol stack of T1 assuming a SCTP-based transport was captured, but there is an FFS on the signaling transport for XXAP as shown below:

Editor’s Note 2: the signalling transport for XXAP is FFS.

In addition to supporting an SCTP-based stack at A-IoT RAN node for carrying XXAP, we think an HTTP-based stack should also be considered to allow a Service Based Interface (SBI) for communication between the A-IoT RAN node and A-IoT CN node. A SBI is more developer/cloud friendly, RAT agnostic compared to a point-to-point interface and should also be considered as another viable option. We therefore propose the following:

In addition to the SCTP-based stack, AIoT RAN node should be able to support an HTTP based stack (XXAP/HTTP/TLS/TCP/L2/L1) to carry the XXAP PDUs and upper layer information. Capture another protocol stack for T1 showing the HTTP based stack.

## Topology 2

### Transport options for T2

#### A-IoT “upper layer” information

In the last meeting, 3 options were captured in the TR for communication between the A-IoT-enabled UE and A-IoT CN for conveying “A-IoT upper layer information”:

- RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

- NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

- UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Similar to the discussion in section 2.2.3, here we assume there is an “upper layer” between the A-IoT device and A-IoT CN node and depict the logical system architecture for T2 for the 3 transport options, showing the nodes and interfaces involved:

#### Protocol stack for T2 transport options

In this section, we depict the protocol stack for T2 for the 3 transport options.

RRC-based solution: A-IoT enabled UE uses its RRC connection with the A-IoT enabled gNB to send/receive A-IoT messages explicitly in RRC. A-IoT enabled gNB then forwards the A-IoT messages to the A-IoT CN node explicitly over XXAP (here direct interface between A-IoT enabled gNB and A-IoT CN node is assumed; whether this is sent indirectly via AMF depends on XX interface discussion in section 2.2.1). No end-to-end protocol (no R-AP) between the A-IoT enabled UE and the A-IoT CN node.

NAS-based solution: A-IoT enabled UE uses its NAS association with the AMF to send/receive A-IoT messages as NAS PDUs. AMF can then forward the A-IoT messages to the A-IoT CN node via SBI interfaces in the core. In this solution, the A-IoT enabled gNB is unaware of the contents of the A-IoT messages as it is received as a NAS container. This is similar to PDU Session Management messages carried in NAS (SMF is behind AMF and AMF does not process them).

User plane-based solution: A-IoT enabled UE uses user plane signalling (establishes a PDU session till UPF) to communicate with A-IoT CN node. UPF can then forward the A-IoT messages to the A-IoT CN node via a SBI interface (Nuc) in the core. In this solution, the A-IoT enabled gNB is also unaware of the contents of the A-IoT messages as it is sent over user plane.

RAN3 should capture three different protocol stacks for the three different transport options (RRC/NAS/user-plane based)

  - “upper layer” protocol terminating between the A-IoT device and A-IoT CN node (for all transport solutions)

  - R-AP protocol between the A-IoT enabled UE and the A-IoT CN node (in case of NAS/UP based solution).

### Radio Resource management in T2

In the TR, the following editor’s notes are captured i.e., how the A-IoT RAN node functions (e.g., A-IoT radio resource management) are triggered for the different transport options for T2.

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

Editor’s note 2: how and where to depict signaling suitable for triggering A-IoT RAN node functions for A-IoT radio resource management needs further discussions for direct communication between A-IoT CN and A-IoT-enabled UE.

For Radio resource management (e.g., to allocate T/F resources for A-IoT), AIoT-enabled gNB needs some knowledge of the upcoming/ongoing A-IoT Service Request e.g., whether the A-IoT CN node (or AF) is interested in performing inventory or command or both, approximate number of target devices (to allocate radio resources accordingly), how long the inventory/command would take etc.

In this section, we refer to such information as “A-IoT assistance information” i.e., that information which are useful to trigger AIoT RAN node functions (e.g., AIoT radio resource management).

In Topology 2, AIoT-enabled gNB needs some “assistance” information (can be implicitly or explicitly provided) to trigger AIoT RAN node functions (e.g., AIoT radio resource management). RAN3 should discuss what kind of “assistance” information is needed e.g.,

    - Service type of AIoT (e.g., inventory or command)

    - Approximate number of target devices (e.g., to assist in resource allocation)

#### RRC-based solution

In case of RRC-based solution, assistance information is not needed explicitly as the necessary information is provided by the AIoT CN node explicitly over XXAP as shown in the Figure below.

In case of RRC-based solution, there is no need for A-IoT CN node to explicitly provide A-IoT assistance information to the A-IoT enabled gNB as the A-IoT messages (e.g., Inventory/Command Request) is already provided explicitly over XXAP

#### NAS-based solution

In case of NAS based solution, A-IoT assistance information can be provided as follows:

Option 1: A-IoT assistance information can be provided to the A-IoT enabled gNB by A-IoT enabled UE via Uu after receiving the Inventory/Command Request NAS PDUs

Option 2: A-IoT assistance information can be provided to the A-IoT enabled gNB by the AMF as part of Downlink NAS Transport in NGAP along with the Inventory/Command NAS PDUs.

In case of NAS based solution, A-IoT assistance information can be provided to the A-IoT enabled gNB via any of the following options:

  - By the A-IoT enabled UE via Uu after receiving the Inventory/Command Request NAS PDUs

  - By the AMF as part of Downlink NAS Transport in NGAP along with the Inventory/Command NAS PDUs.

#### User-plane based solution

In case of User plane solution, three options are considered.

Option 1: A-IoT assistance information can be provided to the A-IoT enabled gNB by A-IoT enabled UE via Uu after receiving the Inventory/Command Request over user plane

Option 2: A-IoT assistance information can be provided to the A-IoT enabled gNB by UPF as part of the GTP-U header in DL PDU Session Information (38.415) carrying the Inventory/Command Request

Option 3: A-IoT assistance information can be provided to the A-IoT enabled gNB by the AMF via the NGAP (e.g., after receiving assistance information from the AIoT CN node)

In case of User plane solution, A-IoT assistance information can be provided to the A-IoT enabled gNB via any of the following options.

- Option 1: A-IoT enabled UE via Uu after receiving the Inventory/Command Request over user plane

- Option 2: UPF as part of the GTP-U header in DL PDU Session Information (38.415) carrying the Inventory/Command Request

- Option 3: AMF via the NGAP after receiving assistance information from the AIoT CN node

#### Assistance from A-IoT enabled UE for resource allocation

Irrespective of RRC/NAS/user-plane based solution for initial A-IoT radio resource allocation, the A-IoT enabled UE should be able to provide some A-IoT assistance information to the A-IoT enabled gNB e.g., requesting to release the previously provided A-IoT resources, requesting to provide more resources, to provide the A-IoT enabled UEs’ capabilities or interests (e.g., if it is no longer interested in performing A-IoT communication). We therefore propose the following:

In Topology 2, A-IoT enabled UE can assist the A-IoT enabled gNB in A-IoT radio resource allocation e.g., can request the A-IoT enabled gNB to provide resources or release the previously provided A-IoT resources.

### Reader selection

The AIoT CN to select suitable reader (AIoT enabled gNB or AIoT enabled UE reader) for both topology 1 and topology 2?

In topology 1, the A-IoT RAN node and the A-IoT CN node can discover each other via XXAP interface management procedures (if Point-to-point interfaces are used) or via discovery from NRF (e.g., if service-based interfaces and registration of AIoT RAN node into NRF is considered). Upon receiving an A-IoT service request from AF, A-IoT CN node can therefore discover/select the suitable A-IoT RAN nodes e.g, based on criteria provided by AF (e.g., a certain area of interest, certain targeted devices). Therefore, the A-IoT CN node should select the suitable A-IoT RAN node in T1.

In Topology 1, AIoT CN can select suitable AIoT RAN node(s) as BS Readers for performing A-IoT operations (e.g., Inventory/Command)

In Topology 2, we think that the A-IoT CN node can select suitable A-IoT enabled UE(s) as UE Readers for performing A-IoT operations (e.g., Inventory/Command) based on its implementation (e.g., knowledge of A-IoT enabled UE’s location, upon request from a person pushing a button on the UE Reader)

There were proposals to also enable A-IoT enabled gNB(s) to select A-IoT enabled UEs to act as UE Reader but that has several drawbacks e.g., A-IoT enabled gNB cannot select UE Reader(s) in RRC_IDLE (as UE context is released) whereas A-IoT CN node can select UE Readers in RRC_IDLE as well (as CN entities/AMF have UE context even in RRC_IDLE). Further, this would lead to different solutions for T1 and T2 since A-IoT CN node selects A-IoT RAN node in T1 as per Proposal 18.

Drawbacks of A-IoT enabled gNB selecting UE Reader(s) in T2

  - A-IoT enabled gNB cannot select UE Reader(s) in RRC_IDLE (as it doesn’t have the UE context)

  - Leads to different solutions for T1 and T2 (since A-IoT CN node selects A-IoT RAN node in T1)

  - In T2, UE Reader is mainly “selected” based on external request (e.g., from a worker pushing a button in the UE Reader) and not based on UE Reader selection as per a certain criteria

In Topology 2, AIoT CN can select suitable A-IoT enabled UE(s) as UE Readers for performing A-IoT operations (e.g., Inventory/Command) based on its implementation (e.g., knowledge of A-IoT enabled UE’s location, upon request from a person pushing a button on the UE Reader)

### Comparison of T2 transport options

In the Table 2 below, we compare the three options by discussing the protocol stack, interface impacts and their pros vs. cons.

| RRC-based solution | NAS-based solution | User plane based solution |
| --- | --- | --- |
| A-IoT enabled UE uses its RRC connection with the gNB to send/receive A-IoT messages explicitly in RRC. gNB then decodes the A-IoT RRC messages and forwards the A-IoT messages to the A-IoT CN node explicitly as XXAP PDUs. | A-IoT enabled UE uses its NAS association with the AMF to send/receive A-IoT messages as NAS PDUs. AMF can then forward the A-IoT messages to the A-IoT CN node via SBI interfaces in the core. | A-IoT enabled UE uses user plane signaling (establishes a PDU session till UPF) to communicate with A-IoT CN node. UPF can then forward the A-IoT messages to the A-IoT CN node via SBI interfaces in the core. |
| A-IoT enabled gNB is aware of the contents of the A-IoT messages as it is received explicitly over RRC. However, it adds unnecessary RRC ASN.1 complexity. | A-IoT enabled gNB is unaware of the contents of the A-IoT messages as it is received as a NAS container A-IoT enabled gNB can be made aware explicitly by the A-IoT CN node providing assistance information in N2. Assistance information can be provided as part of Downlink NAS Transport in NGAP | A-IoT enabled gNB is unaware of the contents of the A-IoT messages as it is sent over user plane A-IoT enabled gNB can be made aware explicitly by the A-IoT CN node providing assistance information in N2. Assistance information can be provided by UE Reader or UPF or AMF |
| No end-to-end protocol between the A-IoT enabled UE and the A-IoT CN node | There is an end-to-end protocol (R-AP) between the A-IoT enabled UE and the A-IoT CN node. | There is an end-to-end protocol (R-AP) between the A-IoT enabled UE and the A-IoT CN node |
| RRM like solution This is like how gNB can explicitly request RRM measurements to UE explicitly via RRC | LPP-like solution This is like how LPP is defined between UE and LMF and carried over NAS/NGAP/RRC | UPL like solution This is like how UPL (User Plane Location Protocol) is used to carry location between a UE and location server |
| A-IoT enabled UE selection can be performed by gNB or A-IoT CN node | A-IoT enabled UE selection performed by A-IoT CN node | A-IoT enabled UE selection performed by A-IoT CN node |
| Overload on AMF if there are lot of A-IoT devices | Overload on AMF if there are lot of A-IoT devices | UPF can handle IP traffic better. |
| RAN2 impact: High RAN2 would need to define new RRC signaling to support the reception/transmission of A-IoT related messages from gNB to A-IoT enabled UE RAN2 would need to define procedures when the A-IoT enabled UE’s serving gNB changes (e.g., due to mobility) or RRC state changes | RAN2 impact: No/minimal | RAN2 impacts: No/minimal |
| RAN3 impact: High RAN3 would need to define explicit signaling over N2 for the A-IoT messages In case the serving gNB of the A-IoT enabled UE changes (e.g., due to mobility) or the RRC state of the A-IoT enabled UE changes, Xn coordination is needed to provide A-IoT enabled UE’s context from source gNB to target gNB | RAN3 impact: No/minimal No new procedures needed when the A-IoT enabled UE’s serving gNB changes or the A-IoT enabled UE changes its RRC state | RAN3 impact: No/minimal No new procedures needed when the A-IoT enabled UE’s serving gNB changes or the A-IoT enabled UE changes its RRC state |
| SA2 impact: Medium SA2 needs to define the interface between AMF and A-IoT CN node (Nac interface) | SA2 impact: Medium SA2 needs to define the interface between AMF and A-IoT CN node (Nac interface) and support the end-to-end R-AP protocol | SA2 impact: Medium SA2 needs to define the interface between UPF and A-IoT CN node (Nuc interface) and support the end-to-end R-AP protocol |
| Non-homogeneous control plane transport RRC between A-IoT enabled UE and gNB N2 between gNB and AMF API between AMF and A-IoT CN node | Non-homogeneous control plane transport NAS between A-IoT enabled UE and AMF API between AMF and A-IoT CN node | Homogenous transport: End to-end IP IP connectivity (PDU session) between A-IoT enabled UE and UPF IP connectivity between UPF and A-IoT CN node |
| Needs support of RRC at A-IoT enabled UE (already supported by UEs today) | Needs support of NAS at A-IoT enabled UE (already supported by UEs today) | Needs support of HTTP client at A-IoT enabled UE (already supported by UEs today) |
| Fragmented protocol stacks for Reader in T1 and T2 A-IoT RAN node in T1 will have R-AP over IP A-IoT enabled UE in T2 will receive signaling over RRC | Fragmented protocol stacks for Reader in T1 and T2 A-IoT RAN node in T1 will have R-AP over IP A-IoT enabled UE in T2 will have R-AP over NAS | Can support common SBI protocol stack for Reader in T1 and T2 A-IoT RAN node in T1 will have R-AP over IP A-IoT enabled UE in T2 will have R-AP over IP |

RAN3 to capture in TP “Table 2” which compares the three options for transporting the AIoT upper layer information in Topology 2, lists their pros and cons along with their WG impacts

Considering there are high impacts to the A-IoT enabled UE, AIoT enabled gNB, as well as the specification efforts for RAN2/RAN3 with Option 1 (RRC-based), it is our view that RAN3 should not prefer this option.

Given the high impacts to RAN2/RAN3, RRC-based A-IoT data transport between the A-IoT enabled UE and the A-IoT enabled gNB is not considered in Rel-19.

T2-Option 2 (NAS based solution) has the following drawbacks compared to T2-Option 3 (PDU session-based solution)

- Overload on AMF (e.g., if there are a lot of AIoT devices)

- Result in different transport mechanisms for Reader in T1 and T2 (i.e., AIoT RAN node in T1 would use XXAP over SCTP/IP whereas NAS based solution would use R-AP over NAS)

- Non-homogeneous control plane transport (NAS between UE Reader and AMF and API between AMF and AIoT CN node)

RAN3 should capture in TP as a conclusion for T2 that

- “RAN3 recommends a PDU session-based solution (T2-Option 3 in this paper) for UE Reader to communicate with the AIoT CN node in Topology 2 i.e., the UE Reader establishes a PDU session to send AIoT data to UPF which forwards it to the AIoT CN node”.

# Summary

Based on the discussion above, we have following observations and proposals:

2.1 Terminologies

Proposal 1:	Rename "A-IoT RAN" as "A-IoT RAN node". AIoT RAN node refers to the BS Reader in Topology 1.

Observation 1:	SA2 concluded to support a new CN function to support A-IoT for topology 1. SA2 is still discussing whether this new CN function also applies to topology 2.

Proposal 2:	RAN3 should use the terminology “A-IoT CN node” to refer to the new CN function agreed by SA2 to support A-IoT, instead of referring to multiple core network functions as “A-IoT CN”.

Observation 2:	A-IoT RAN node in Topology 1 interacts with a A-IoT device (not a UE) via a new A-IoT radio interface (not Uu) and doesn’t host several of the legacy NG-RAN node functionalities. It is also not clear whether there is a need for A-IoT RAN nodes to be interconnected via an Xn-like interface, whether A-IoT RAN node can connect to AMF etc.

Proposal 3:	Continue to use "A-IoT RAN node" for T1 and "A-IoT-enabled gNB" for T2 till there is clarity on differences/commonalities in T1 and T2 (e.g., on transport/functions). Decide in normative phase (or later in study phase) whether the A-IoT RAN node can be specified as an NG-RAN node or needs to be considered as a new entity.

2.2 Topology 1

2.2.1	XX interface and AMF involvement

Proposal 4:	RAN3 to capture in TR that the following 2 options are possible for the AIoT RAN node to communicate with AIoT CN node in T1:

 T1-Option 1 (Indirect transport solution): AIoT RAN node indirectly (via the AMF) communicates with the AIoT CN node.

 T1-Option 2 (Direct transport solution): AIoT RAN node directly communicates with the AIoT CN node.

Proposal 5:	RAN3 to capture in TR “Table 1” which evaluates the two transport options for AIoT communication in Topology 1, lists their pros and cons along with the WG impacts

Observation 3:	T1-Option 1 (Indirect transport solution) has the following drawbacks compared to T1-Option 2 (direct transport solution):

- Need upgrades at legacy AMF to support two new interfaces (AIoT RAN node-AMF, AMF-AIoT CN node)

- More backhaul latency (due to involvement of AMF)

- Potentially unnecessary involvement of AMF (just as a “router”) for AIoT

- Potentially additional protocol b/w AIoT RAN node and AMF (YYAP) needs to be defined to carry end-to-end packets between AIoT RAN node and AIoT CN node (XXAP)

Proposal 6:	RAN3 to capture in TP as a conclusion for T1 that “It is RAN3’s preference that the AIoT RAN node directly communicates with the AIoT CN node (without going via AMF) in the study of Topology 1 (referred to as T1-Option 2: Direct Transport Solution in thi paper)”

Proposal 7:	In Topology 1, there is an end-to-end “XXAP” protocol between the AIoT RAN node and AIoT CN node over a direct XX interface.

2.2.2	A-IoT upper layer information

Observation 4:	SA2/SA3 is discussing several aspects on registration, device identification, security, authentication and there is no conclusion yet. Potentially, SA2 might conclude to support an “upper layer” terminating between the A-IoT device and the A-IoT CN node to support one or more of the above aspects.

Proposal 8:	Update the protocol stack of T1 to depict an “Upper layer” protocol terminating between the A-IoT device and A-IoT CN node. Details and need of this “Upper layer” protocol is up to SA2/SA3 conclusion.

Proposal 9:	RAN3  assumes that some A-IoT related information (e.g., Device ID) might be carried as “upper layer information” over XXAP and some A-IoT related information (e.g., Scope of Inventory) might be carried explictly over XXAP. This is pending SA2/SA3 conclusion.

2.2.3	Radio Resource management in T1

Proposal 10:	For Topology 1, A-IoT CN node can provide “A-IoT assistance information” (e.g., Type of A-IoT service request, estimated number of devices) explicitly in XXAP message along with the Upper layer information to assist in A-IoT radio resource management.

2.2.4	Signaling transport for XXAP

Proposal 11:	In addition to the SCTP-based stack, AIoT RAN node should be able to support an HTTP based stack (XXAP/HTTP/TLS/TCP/L2/L1) to carry the XXAP PDUs and upper layer information. Capture another protocol stack for T1 showing the HTTP based stack.

2.3 Topology 2

2.3.1	Transport options for T2

Proposal 12:	RAN3 should capture three different protocol stacks for the three different transport options (RRC/NAS/user-plane based)

- “upper layer” protocol terminating between the A-IoT device and A-IoT CN node (for all transport solutions)

- R-AP protocol between the A-IoT enabled UE and the A-IoT CN node (in case of NAS/UP based solution).

2.3.2	Radio Resource management in T2

Proposal 13:	In Topology 2, AIoT-enabled gNB needs some “assistance” information (can be implicitly or explicitly provided) to trigger AIoT RAN node functions (e.g., AIoT radio resource management). RAN3 should discuss what kind of “assistance” information is needed e.g.,

 Service type of AIoT (e.g., inventory or command)

 Approximate number of target devices (e.g., to assist in resource allocation)

Proposal 14:	In case of RRC-based solution, there is no need for A-IoT CN node to explicitly provide A-IoT assistance information to the A-IoT enabled gNB as the A-IoT messages (e.g., Inventory/Command Request) is already provided explicitly over XXAP

Proposal 15:	In case of NAS based solution, A-IoT assistance information can be provided to the A-IoT enabled gNB via any of the following options:

- Option 1: By the A-IoT enabled UE via Uu after receiving the Inventory/Command Request NAS PDUs

- Option 2: By the AMF as part of Downlink NAS Transport in NGAP along with the Inventory/Command NAS PDUs.

Proposal 16:	In case of User plane solution, A-IoT assistance information can be provided to the A-IoT enabled gNB via any of the following options.

 Option 1: A-IoT enabled UE via Uu after receiving the Inventory/Command Request over user plane

 Option 2: UPF as part of the GTP-U header in DL PDU Session Information (38.415) carrying the Inventory/Command Request

 Option 3: AMF via the NGAP after receiving assistance information from the AIoT CN node

Proposal 17:	In Topology 2, A-IoT enabled UE can assist the A-IoT enabled gNB in A-IoT radio resource allocation e.g., can request the A-IoT enabled gNB to provide resources or release the previously provided A-IoT resources.

2.3.3	Reader selection

Proposal 18:	In Topology 1, AIoT CN can select suitable AIoT RAN node(s) as BS Readers for performing A-IoT operations (e.g., Inventory/Command)

Observation 5:	Drawbacks of A-IoT enabled gNB selecting UE Reader(s) in T2

  - A-IoT enabled gNB cannot select UE Reader(s) in RRC_IDLE (as it doesn’t have the UE context)

  - Leads to different solutions for T1 and T2 (since A-IoT CN node selects A-IoT RAN node in T1)

  - In T2, UE Reader is mainly “selected” based on external request (e.g., from a worker pushing a button in the UE Reader) and not based on UE Reader selection as per a certain criteria

Proposal 19:	In Topology 2, AIoT CN can select suitable A-IoT enabled UE(s) as UE Readers for performing A-IoT operations (e.g., Inventory/Command) based on its implementation (e.g., knowledge of A-IoT enabled UE’s location, upon request from a person pushing a button on the UE Reader)

2.3.4	Comparison of T2 transport options

Proposal 20:	RAN3 to capture in TP “Table 2” which compares the three options for transporting the AIoT upper layer information in Topology 2, lists their pros and cons along with their WG impacts

Proposal 21:	Given the high impacts to RAN2/RAN3, RRC-based A-IoT data transport between the A-IoT enabled UE and the A-IoT enabled gNB is not considered in Rel-19.

Observation 6:	T2-Option 2 (NAS based solution) has the following drawbacks compared to T2-Option 3 (PDU session-based solution)

- Overload on AMF (e.g., if there are a lot of AIoT devices)

- Result in different transport mechanisms for Reader in T1 and T2 (i.e., AIoT RAN node in T1 would use XXAP over SCTP/IP whereas NAS based solution would use R-AP over NAS)

- Non-homogeneous control plane transport (NAS between UE Reader and AMF and API between AMF and AIoT CN node)

Proposal 22:	RAN3 should capture in TP as a conclusion for T2 that

“RAN3 recommends a PDU session-based solution (T2-Option 3 in this paper) for UE Reader to communicate with the AIoT CN node in Topology 2 i.e., the UE Reader establishes a PDU session to send AIoT data to UPF which forwards it to the AIoT CN node”.

# Annex 1 : TP to 38.769

## 6.4	RAN architecture aspects

Editor’s note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

This clause attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for A-IoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

The logical system architecture for A-IoT consists of the following architectural elements:

A-IoT device:	Equipment with characteristics outlined e.g. in TS 22.369 [10] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

A-IoT RAN node:	Hosts certain functions for A-IoT in topology 1, as part of the functional split between RAN and CN.

A-IoT enabled gNB:	Hosts certain functions for A-IoT in topology 2, as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding A-IoT functions hosted in the A-IoT RAN node in topology 1 and A-IoT-enabled gNB in topology 2 and the respective functional split to be decided by RAN2, RAN3 and SA2.

A-IoT radio:	Radio interface between A-IoT device and A-IoT RAN node in topology 1 and between A-IoT device and A-IoT-enabled UE in topology 2.

Editor’s Note 5: Further details on A-IoT radio to be discussed by RAN1 and RAN2.

A-IoT CN node:	Hosts certain functions for A-IoT as of the functional split between RAN and CN.

NOTE: the details of A-IoT CN node are subject to SA2.

Editor’s Note 6: Further details regarding A-IoT functions hosted in the A-IoT CN node and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface:	Interface between the A-IoT RAN node and the A-IoT CN node in topology 1 on which certain A-IoT specific functions are performed.

FFS whether XX interface is also needed between A-IoT enabled gNB and A-IoT CN node in topology 2.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between A-IoT RAN node and A-IoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between A-IoT CN and A-IoT RAN, or one instance for NG alone.

Upper layer: A protocol that is terminated between the A-IoT device and A-IoT CN node e.g., for carrying Command, Device IDs in a secure manner.

Editor’s Note: Details and need of this upper layer are pending SA2/SA3 conclusion

Common reader function:	A function that communicates with the A-IoT device by means of A-IoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

A-IoT RAN node function:	A function that contains e.g. the control of the A-IoT radio resources used towards the A-IoT device.

Editor’s Note 9: further details are FFS. Note that “control of A-IoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

### 6.4.1	Support of Topology 1

Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN node.

Figure 6.4.1-1 Logical system architecture for topology 1

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.4.1-2 shows the Protocol stack for Topology 1, with different signalling transport options:

Figure 6.4.1-2. Protocol Stack for Topology 1, with an SCTP-based transport

Figure 6.4.1-3. Protocol Stack for Topology 1, with an HTTP-based transport

For topology 1, the XXAP is terminated at an A-IoT RAN node.

Editor’s Note 4: aspects of interaction between upper layer information exchange and XXAP in order to trigger the A-IoT RAN node functions are FFS.

### 6.4.1.1	XX Interface

There are 2 options for AIoT RAN node to communicate with AIoT CN node in T1:

- T1-Option 1 (Indirect transport solution): AIoT RAN node indirectly (via the AMF) communicates with the AIoT CN node.

- T1-Option 2 (Direct transport solution): AIoT RAN node directly communicates with the AIoT CN node.

- Table 1: Evaluation of transport solutions for Topology 1

| T1-Option 1 (Indirect transport solution) | T1-Option 2 (Direct transport solution) |
| --- | --- |
| (Potentially) Two new protocols need to be defined. Protocol b/w A-IoT RAN node and AMF (i.e., YYAP), if NGAP is not reused Protocol b/w A-IoT RAN node and A-IoT CN node (i.e., XXAP) | Only 1 new protocol needs to be defined. Protocol between A-IoT RAN node and A-IoT CN node (i.e., XXAP). |
| (Potentially) Two new interfaces need to be defined. Interface between A-IoT RAN node and AMF (YY) if N2 is not reused Interface between AMF and A-IoT CN node (Nac) | Only 1 interface needs to be defined. Interface between the A-IoT RAN node and A-IoT CN node (i.e., XX) |
| Different protocol stack at A-IoT RAN node and A-IoT CN node SCTP-based stack between A-IoT RAN node and AMF. HTTP-based stack between AMF and A-IoT CN node. | Common protocol stack at A-IoT RAN node and A-IoT CN node Option 1: HTTP-based stack at A-IoT RAN node and A-IoT CN node. Option 2: SCTP-based stack at A-IoT RAN node and A-IoT CN node. |
| Need upgrades to legacy AMF. | No impact to legacy AMF. |
| Less developer/cloud friendly Uses point-to-point interface between A-IoT RAN node and AMF (YY interface) and service-based interface between AMF and A-IoT CN node. | More developer/cloud friendly If HTTP-based stack is used, this will use a Service Based Interface (SBI) between A-IoT RAN node and A-IoT CN node which is more developer/cloud friendly, RAT agnostic compared to a point-to-point interface. |
| More backhaul latency due to involvement of AMF. AMF is primarily used as a forwarding entity (“router”) for forwarding packets between A-IoT RAN node and A-IoT CN node. Potentially unnecessary involvement of AMF. | Less backhaul latency (no involvement of AMF) |

#### 6.4.1.2	Conclusion

- RAN3 recommends that a new radio network layer protocol XXAP is defined for A-IoT RAN node to directly communicate with the A-IoT CN node in Topology 1

### 6.4.2	Support of Topology 2

Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

The following definitions apply:

A-IoT-enabled gNB:	A gNB supporting A-IoT RAN node function, which is able to communicate with the A-IoT enabled UE via NR Uu interface.

A-IoT-enabled UE:	A UE supporting Common reader function, which is able to communicate with the A-IoT device via the A-IoT radio interface.

Figure 6.4.2-1 Logical system architecture for topology 2

In the above figure, 	RAN3 assumes that there is an “R-AP” protocol between A-IoT enabled UE and A-IoT CN node. This is pending SA2 confirmation.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The A-IoT CN could include AMF, UPF and A-IoT CN node. This is up to SA2 decision.

Editor’s Note 4:	The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

#### 6.4.2.1	Solutions for Topology 2

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

Editor’s note 2: Depiction and further details of the options above are FFS

#### 6.4.2.2	Protocol stack for Topology 2 transport options

In this section, we depict the protocol stack for T2 for the 3 transport options.

RRC-based solution: A-IoT enabled UE uses its RRC connection with the A-IoT enabled gNB to send/receive A-IoT messages explicitly in RRC. A-IoT enabled gNB then forwards the A-IoT messages to the A-IoT CN node explicitly over XXAP (here direct interface between A-IoT enabled gNB and A-IoT CN node is assumed; whether this is sent indirectly via AMF depends on XX interface discussion in section 2.2.1). No end-to-end protocol (no R-AP) between the A-IoT enabled UE and the A-IoT CN node.

NAS-based solution: A-IoT enabled UE uses its NAS association with the AMF to send/receive A-IoT messages as NAS PDUs. AMF can then forward the A-IoT messages to the A-IoT CN node via SBI interfaces in the core. In this solution, the A-IoT enabled gNB is unaware of the contents of the A-IoT messages as it is received as a NAS container. This is similar to PDU Session Management messages carried in NAS (SMF is behind AMF and AMF does not process them).

User plane-based solution: A-IoT enabled UE uses user plane signalling (establishes a PDU session till UPF) to communicate with A-IoT CN node. UPF can then forward the A-IoT messages to the A-IoT CN node via a SBI interface (Nuc) in the core. In this solution, the A-IoT enabled gNB is also unaware of the contents of the A-IoT messages as it is sent over user plane.

#### 6.4.2.3	Comparison of T2 transport options

| RRC-based solution | NAS-based solution | User plane based solution |
| --- | --- | --- |
| A-IoT enabled UE uses its RRC connection with the gNB to send/receive A-IoT messages explicitly in RRC. gNB then decodes the A-IoT RRC messages and forwards the A-IoT messages to the A-IoT CN node explicitly as XXAP PDUs. | A-IoT enabled UE uses its NAS association with the AMF to send/receive A-IoT messages as NAS PDUs. AMF can then forward the A-IoT messages to the A-IoT CN node via SBI interfaces in the core. | A-IoT enabled UE uses user plane signaling (establishes a PDU session till UPF) to communicate with A-IoT CN node. UPF can then forward the A-IoT messages to the A-IoT CN node via SBI interfaces in the core. |
| A-IoT enabled gNB is aware of the contents of the A-IoT messages as it is received explicitly over RRC. However, it adds unnecessary RRC ASN.1 complexity. | A-IoT enabled gNB is unaware of the contents of the A-IoT messages as it is received as a NAS container A-IoT enabled gNB can be made aware explicitly by the A-IoT CN node providing assistance information in N2. Assistance information can be provided as part of Downlink NAS Transport in NGAP | A-IoT enabled gNB is unaware of the contents of the A-IoT messages as it is sent over user plane A-IoT enabled gNB can be made aware explicitly by the A-IoT CN node providing assistance information in N2. Assistance information can be provided by UE Reader or UPF or AMF |
| No end-to-end protocol between the A-IoT enabled UE and the A-IoT CN node | There is an end-to-end protocol (R-AP) between the A-IoT enabled UE and the A-IoT CN node. | There is an end-to-end protocol (R-AP) between the A-IoT enabled UE and the A-IoT CN node |
| RRM like solution This is like how gNB can explicitly request RRM measurements to UE explicitly via RRC | LPP-like solution This is like how LPP is defined between UE and LMF and carried over NAS/NGAP/RRC | UPL like solution This is like how UPL (User Plane Location Protocol) is used to carry location between a UE and location server |
| A-IoT enabled UE selection can be performed by gNB or A-IoT CN node | A-IoT enabled UE selection performed by A-IoT CN node | A-IoT enabled UE selection performed by A-IoT CN node |
| Overload on AMF if there are lot of A-IoT devices | Overload on AMF if there are lot of A-IoT devices | UPF can handle IP traffic better. |
| RAN2 impact: High RAN2 would need to define new RRC signaling to support the reception/transmission of A-IoT related messages from gNB to A-IoT enabled UE RAN2 would need to define procedures when the A-IoT enabled UE’s serving gNB changes (e.g., due to mobility) or RRC state changes | RAN2 impact: No/minimal | RAN2 impacts: No/minimal |
| RAN3 impact: High RAN3 would need to define explicit signaling over N2 for the A-IoT messages In case the serving gNB of the A-IoT enabled UE changes (e.g., due to mobility) or the RRC state of the A-IoT enabled UE changes, Xn coordination is needed to provide A-IoT enabled UE’s context from source gNB to target gNB | RAN3 impact: No/minimal No new procedures needed when the A-IoT enabled UE’s serving gNB changes or the A-IoT enabled UE changes its RRC state | RAN3 impact: No/minimal No new procedures needed when the A-IoT enabled UE’s serving gNB changes or the A-IoT enabled UE changes its RRC state |
| SA2 impact: Medium SA2 needs to define the interface between AMF and A-IoT CN node (Nac interface) | SA2 impact: Medium SA2 needs to define the interface between AMF and A-IoT CN node (Nac interface) and support the end-to-end R-AP protocol | SA2 impact: Medium SA2 needs to define the interface between UPF and A-IoT CN node (Nuc interface) and support the end-to-end R-AP protocol |
| Non-homogeneous control plane transport RRC between A-IoT enabled UE and gNB N2 between gNB and AMF API between AMF and A-IoT CN node | Non-homogeneous control plane transport NAS between A-IoT enabled UE and AMF API between AMF and A-IoT CN node | Homogenous transport: End to-end IP IP connectivity (PDU session) between A-IoT enabled UE and UPF IP connectivity between UPF and A-IoT CN node |
| Needs support of RRC at A-IoT enabled UE (already supported by UEs today) | Needs support of NAS at A-IoT enabled UE (already supported by UEs today) | Needs support of HTTP client at A-IoT enabled UE (already supported by UEs today) |
| Fragmented protocol stacks for Reader in T1 and T2 A-IoT RAN node in T1 will have R-AP over IP A-IoT enabled UE in T2 will receive signaling over RRC | Fragmented protocol stacks for Reader in T1 and T2 A-IoT RAN node in T1 will have R-AP over IP A-IoT enabled UE in T2 will have R-AP over NAS | Can support common SBI protocol stack for Reader in T1 and T2 A-IoT RAN node in T1 will have R-AP over IP A-IoT enabled UE in T2 will have R-AP over IP |

#### 6.4.2.4	Conclusion

- Given the high impacts to RAN2/RAN3, RRC-based A-IoT data transport between the A-IoT enabled UE and the A-IoT enabled gNB is not considered in Rel-19.

- RAN3 recommends that a user-plane based solution is recommended for the A-IoT enabled UE to communicate with the A-IoT CN node in Topology 2 i.e., the A-IoT enabled UE establishes a PDU session to send A-IoT data to UPF which forwards it to the A-IoT CN node.