3GPP TSG-RAN3 Meeting #125bis	R3-245333

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	16.4

Source: 	Qualcomm Incorporated

Title: 	Methods for locating Ambient IoT devices

Document for:	Discussion/Decision

# Background

In this paper, we provide our views on locating an AIoT device based on the discussion and open issues identified in RAN3#123bis meeting.

# Discussion

## Encoding of AIoT RAN node ID

The following is captured in the TR:

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

For topology 1, A-IoT RAN node ID can be considered as a location of A-IoT device.

For topology 2, UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device.

Further, the following open issue is captured in the Chair’s agenda:

The encoding of AIoT RAN node ID needs to be further checked. Other granularities are FFS, e.g., TRP ID.

There were several proposals last meeting that the AIoT RAN node in T1 can consist of multiple readers (or multiple common reader functions) e.g., multiple TRPs.

In our view, this refers to a split architecture where there is an A-IoT CU which hosts the “A-IoT RAN node function” (e.g., radio resource allocation) and there are several distributed units A-IoT DU which hosts the common reader functions as illustrated in the below figure:

It is to be remembered that the RAN3 agreed to start with A-IoT RAN node being aggregated (i.e., deprioritize split architecture support) as shown by the following agreement:

For Topology 1, RAN3 starts with AIoT RAN node being aggregated.

We therefore propose to consider only the aggregated A-IoT RAN node in Rel-19 and therefore the A-IoT RAN node ID should be encoded without considering any sub-fields (e.g., TRP ID). The exact encoding of A-IoT RAN node ID can be discussed further in the normative phase e.g., whether it is fixed in length or have a flexible length, how many bits to use for encoding A-IoT RAN node ID etc.

RAN3 only considers the aggregated A-IoT RAN node in Rel-19 and will not consider any disaggregated units (e.g., DUs or TRPs) within an A-IoT RAN node.

There is no need to consider any sub-fields (e.g., TRP ID or DU ID) while encoding the A-IoT RAN node ID. The exact encoding of A-IoT RAN node ID can be discussed in the normative phase.

## Involvement of A-IoT CN node in locating A-IoT devices

In our view, the new core network entity agreed by SA2 (referred as the AIoT CN node), that is responsible for forwarding AIoT specific messages (e.g., inventory/commands) can also be involved in locating the AIoT device (e.g., determining A-IoT location configuration, collecting A-IoT location reports). The following is therefore proposed:

AIoT CN node is involved in locating an AIoT device (e.g., determining A-IoT location configuration, collecting A-IoT location reports), for both Topology 1 and 2.

## Signalling details for A-IoT location reporting

Editor’s Note 2:	Signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study.

In NR, to track UEs in RRC_CONNECTED, AMF can send a location report configuration to the gNB and the gNB sends the UE Location Information (ULI) by including the NR CGI (MCC + MNC + gNB ID + Cell Identity) and/or the Tracking Area Identity (TAI) to AMF.

In order track the AIoT devices at reader granularity, we think we can define a procedure similar to locating reporting procedure, but a procedure between the AIoT CN node and Reader is defined (instead of AMF and gNB as in NR).

The AIoT CN node can send a AIoT location report config (e.g., indicating the AIoT device(s) for which location is requested, location reporting conditions) to a Reader and the Reader can send the AIoT device’s “location” (e.g., the “Reader ID” to which the AIoT device was last associated with). An example procedure is shown below:

Step 1: Reader receives a message from the AIoT CN node requesting the location information of one or more AIoT device(s) via an AIoT location report config.

- The AIoT devices for which the location information is requested can be identified by a AIoT device ID. FFS on the format of AIoT device ID (pending SA2 progress).

- The AIoT location report config can include when/how to report location information e.g., location to be reported just “one-time” or periodically or upon some other event being met. FFS on the type of events.

- AIoT location report config can be sent as part of Inventory Request/Command Request or can be sent separately.

Step 2: Reader stores the AIoT location report config and determines whether the condition to report the location information is satisfied.

Step 3: Upon determining that the condition to report location is satisfied, Reader sends an AIoT Location Report to the AIoT CN node with the Reader ID(s) associated to the requested AIoT device(s).

In NR, AMF can send a location report configuration to the gNB to track UEs in RRC_CONNECTED and gNB sends the UE Location Information (ULI) by including the NR CGI (MCC + MNC + gNB ID + Cell Identity) and/or the Tracking Area Identity (TAI) to AMF.

The AIoT CN node can send a “AIoT Location Report Config” to a Reader, indicating the AIoT device(s) for which location is requested and the location reporting conditions.

A-IoT Location Report Config can be sent along with Inventory/Command Request or can be sent separately.

Upon determining that the condition to report location is satisfied, the Reader can send the AIoT device’s “location” (e.g., the “Reader ID” to which the AIoT device was last associated with) to the AIoT CN node.

## Reader location

Editor’s Note 1:	How to know the “reader” location is FFS.

In case of T1, the location of the AIoT RAN node can be known via knowledge of the deployment of the A-IoT RAN nodes or via OAM configuration.

In case of T2, the location of the A-IoT enabled UE (acting as UE Reader) can be known via existing positioning/LCS procedures if the A-IoT enabled UE supports positioning.

In case of T1, the location of the AIoT RAN node can be known via knowledge of the deployment of the A-IoT RAN nodes or via OAM configuration.

In case of T2, the location of the A-IoT enabled UE (acting as UE Reader) can be known via existing positioning/LCS procedures if the A-IoT enabled UE supports positioning.

## Multi-Reader based locating the A-IoT device

Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS.

- The A-IoT CN node can select multiple A-IoT RAN nodes (in case of T1) or multiple UE Readers (in case of T2) to “triangulate” A-IoT devices better. In case an A-IoT device is in the coverage of multiple selected Readers, both the Readers may send their Reader ID or Reader location or signal strength measurements at the Reader to the A-IoT CN node.

It is up to A-IoT CN node implementation whether to select one or more than readers for locating the A-IoT devices

## Finer granularities for locating AIoT device (besides Reader granularity)

The following Editor’s Notes are captured in the TR:

Editor’s Note 3:	Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4:	Analysing the gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

SA1 discussed several representative use cases for AIoT (Indoor/Outdoor, Inventory/Sensor/Positioning/Command) and this was summarized in Table 4.1.1-1 in TR 38.848. However, RAN plenary eventually agreed to focus on rUC1 (indoor inventory) and rUC4 (indoor command) use cases in Rel-19.

We revisited the rUC1 and rUC4 use cases in 22.840 and looked at the positioning requirements listed therein and is summarised in the below Observation.

| rUC1: Indoor inventory | 5.1 Automated warehousing 5.2 Medical instruments inventory management and positioning 5.4 Non-Public Network for logistics 5.5 Automobile manufacturing 5.7 Airport terminal / shipping port 5.15 Smart laundry 5.16 Automated supply chain distribution 5.18 Fresh food supply chain 5.27 End-to-end logistics 6.1 Flower auction 6.3 Electronic shelf label |
| --- | --- |
| rUC4: Indoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.29 Device Permanent Deactivation 6.3 Electronic shelf label |

In TR 22.840, the following positioning related requirements are listed for the following rUC1 (Indoor inventory) and rUC4 (Indoor command) use cases:

- 5.1 Automated warehousing: 2-3 m

- 5.2 Medical instruments inventory management and positioning: 3-5 m indoor

- 5.5 Automobile manufacturing: 3 m

- 5.7 Airport terminal / shipping port: Cell level (with 90% positioning service availability)

- 5.11 Online modification of medical instruments status: 3-5 m

- 5.16 Automated supply chain distribution: 3 m (Indoor, 90% confidence level and in horizontal), cell-level outdoor

Whether the Reader ID or Reader location is sufficient to locate the A-IoT device at a 3-5 m granularity indoors depends on the Reader coverage and the deployment of readers within an area of interest.

For example, if there is a single Reader in a whole factory which can inventory/command all the A-IoT devices in the factory e.g. for a 50 m coverage, then knowing just the reader ID or reader location won’t suffice to locate the A-IoT device at finer granularity e.g., within 3-5 m granularity.

In case there are multiple readers deployed in the factory and all these readers are used for locating the A-IoT devices, we can locate the A-IoT devices at a better granularity e.g., by knowing that the A-IoT device is in the intersection of the coverage of two readers.

Further, it is to be noted that RAN1 studied proximity determination and captured the following two solutions in the TR. In case of solution 2, measurements at reader side can also be utilized as shown below:

6.9.x	Proximity determination

Editor’s note: Proximity determination may be in a 6.9.x sub-clause, or another arrangement, depending on how the study proceeds.

Proximity determination is feasible with either of the two following solutions. Potential specification impact or not will not be determined in this study item.

Solution 1:	If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader.

Solution 2:	If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side.

Proximity determination based on device-side measurements is not considered.

Even for locating the A-IoT devices, reader side measurements can be utilized if such support is provided by RAN1.

RAN3 should study solutions to locate the A-IoT device at a granularity of 3-5 meters, which is the requirement for certain indoor inventory/command use cases e.g., automated warehousing, automobile manufacturing, medical instruments inventory management as per TR 22.840.

RAN3 should study whether reader-side measurements (also considered as part of solution 2 in proximity determination) can be used to achieve locating the A-IoT device at finer granularity than reader level (e.g., 3-5 m)

Agree the TP to 38.769 in Anex.

# Summary

2.1	Encoding of AIoT RAN node ID

Proposal 1:	RAN3 only considers the aggregated A-IoT RAN node in Rel-19 and will not consider any disaggregated units (e.g., DUs or TRPs) within an A-IoT RAN node.

Proposal 2:	There is no need to consider any sub-fields (e.g., TRP ID or DU ID) while encoding the A-IoT RAN node ID. The exact encoding of A-IoT RAN node ID can be discussed in the normative phase.

2.2	Involvement of A-IoT CN node in locating A-IoT devices

Proposal 3:	AIoT CN node is involved in locating an AIoT device (e.g., determining A-IoT location configuration, collecting A-IoT location reports), for both Topology 1 and 2.

2.3	Signalling details for A-IoT location reporting

Observation 1:	In NR, AMF can send a location report configuration to the gNB to track UEs in RRC_CONNECTED and gNB sends the UE Location Information (ULI) by including the NR CGI (MCC + MNC + gNB ID + Cell Identity) and/or the Tracking Area Identity (TAI) to AMF.

Proposal 4:	The AIoT CN node can send a “AIoT Location Report Config” to a Reader, indicating the AIoT device(s) for which location is requested and the location reporting conditions.

Proposal 5:	A-IoT Location Report Config can be sent along with Inventory/Command Request or can be sent separately.

Proposal 6:	Upon determining that the condition to report location is satisfied, the Reader can send the AIoT device’s “location” (e.g., the “Reader ID” to which the AIoT device was last associated with) to the AIoT CN node.

2.4	Reader location

Proposal 7:	In case of T1, the location of the AIoT RAN node can be known via knowledge of the deployment of the A-IoT RAN nodes or via OAM configuration.

Proposal 8:	In case of T2, the location of the A-IoT enabled UE (acting as UE Reader) can be known via existing positioning/LCS procedures if the A-IoT enabled UE supports positioning.

2.5	Multi-Reader based locating the A-IoT device

Proposal 9:	It is up to A-IoT CN node implementation whether to select one or more than readers for locating the A-IoT devices

2.6	Finer granularities for locating AIoT device (besides Reader granularity)

Observation 2:	In TR 22.840, the following positioning related requirements are listed for the following rUC1 (Indoor inventory) and rUC4 (Indoor command) use cases:

Proposal 10:	RAN3 should study solutions to locate the A-IoT device at a granularity of 3-5 meters, which is the requirement for certain indoor inventory/command use cases e.g., automated warehousing, automobile manufacturing, medical instruments inventory management as per TR 22.840.

Proposal 11:	RAN3 should study whether reader-side measurements (also considered as part of solution 2 in proximity determination) can be used to achieve locating the A-IoT device at finer granularity than reader level (e.g., 3-5 m)

Proposal 12:	Agree the TP to 38.769 in Anex.

# 4 Annex (TP to 38.769)

## 6.9	Locating ambient IoT devices

### 6.9.1	General

Editor’s note: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

A-IoT device location information may be used for the following purposes:

(a) improving the A-IoT operation itself, e.g. by A-IoT CN node sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the A-IoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

For topology 1, A-IoT RAN node ID can be considered as a location of A-IoT device. There is no need to consider any sub-fields (e.g., TRP ID or DU ID) while encoding the A-IoT RAN node ID. The exact encoding of A-IoT RAN node ID can be discussed in the normative phase.

For topology 2, UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device.

For topology 1, the location of the AIoT RAN node can be known via knowledge of the deployment of the A-IoT RAN nodes or via OAM configuration.

For topology 2, the location of the A-IoT enabled UE (acting as UE Reader) can be known via existing positioning/LCS procedures if the A-IoT enabled UE supports positioning.

It is up to A-IoT CN node implementation whether to select one or more than readers for locating the A-IoT devices

Editor’s Note 3:	Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4:	Analysing the gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

### 6.9.2 Signalling details for A-IoT location reporting

The study assumes that the A-IoT CN node can be involved in locating an AIoT device (e.g., determining A-IoT location configuration, collecting A-IoT location reports), for both Topology 1 and 2.

The signalling details are shown in the Figure below.

Step 1:

- Reader receives a message from the AIoT CN node requesting the location information of one or more AIoT device(s) via an AIoT location report config.

- The AIoT devices for which the location information is requested can be identified by a AIoT device ID. FFS on the format of the AIoT device ID (pending SA2 progress).

- The AIoT location report config can include when/how to report location information e.g., location to be reported just “one-time” or periodically or upon some other event being met. FFS on the type of events.

- The AIoT location report config is sent as part of Inventory Request/Command Request or can be sent separately.

Step 2: Reader stores the AIoT location report config and determines whether the condition to report the location information is satisfied.

Step 3: Upon determining that the condition to report location is satisfied, Reader sends an AIoT Location Report to the AIoT CN node with the Reader ID(s) associated to the requested AIoT device(s).