3GPP TSG-RAN3 Meeting #125bis	R3-245334

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	10.2

Source: 	Qualcomm Incorporated

Title: 	MRO enhancements for R18 mobility mechanisms

Document for:	Discussion/Decision

# Background

In this paper, we discuss potential MRO enhancements for R18 mobility mechanisms for the following objective in Rel-19 SON/MDT WI based on the open issues and agreements last meeting.

MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## MRO for LTM

### LTM connection failure definitions and detection mechanisms

For the LTM connection failure cases of Too late LTM, Too early LTM and LTM to wrong cell, it was agreed in the last meeting to reuse the existing definitions and add clarifications wherever applicable instead of defining new failure types for LTM to avoid duplication in stage-2 as much as possible. The following TP was agreed with some Editor’s Notes:

15.5.2.2.2            Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-     Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-     Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-     Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

Editor’s note 1: FFS if LTM recovery needs to be captured in the definitions.

Editor’s note 2: For the too late LTM cell switch, FFS if the re-establishment cell is a candidate cell, a cell not selected by CU, or both.

It is to be noted that the CHO recovery scenarios are also not captured in the definitions of Too late HO, Too Early HO, HO to wrong cell. Instead, it is captured as part of the detection mechanisms. We therefore propose to do the same for LTM.

CHO recovery scenario is not captured in the definitions of Too late HO, Too Early HO, HO to wrong cell. Instead, it is captured as part of the detection mechanisms

Similar to CHO Recovery, LTM recovery scenarios need not be captured in the definitions of Too Late HO, Too Early HO, HO to wrong cell and should be instead captured as part of the detection mechanisms. Remove Editor’s note 1.

For L3 HO, UE reports a timer (e.g., time elapsed since last HO initialization until connection failure or time elapsed since CHO execution until connection failure) which is compared against an OAM configured threshold Tstore_UE_cntxt to check if there has been an RLF shortly after a successful HO/CHO execution.

RAN2 already agreed to extend the use of timeConnFailure to LTM i.e., UE will report the Time from LTM execution to RLF as seen from the following RAN2 agreement:

Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC.

Regarding the detection mechanisms for LTM connection failures, we think we can just modify the detection mechanisms of Intra-system Too Late Handover, Intra-system Too Early Handover and Intra-system Handover to Wrong Cell with few clarifications as shown below (and also covering the LTM recovery scenarios).

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-     Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO is configured but the CHO execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if LTM is configured but the LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt)..

-     Intra-system Too Early Handover: there is a recent handover/LTM for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-     Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO execution until connection failure or the time elapsed since the LTM execution until connection failure.

We therefore propose the following:

To simplify stage-2, reuse the existing text on detection mechanisms of Intra-system Too Late Handover, Intra-system Too Early Handover and Intra-system Handover to Wrong Cell with necessary modifications for LTM e.g., clarify that the UE reported timer refers to the time elapsed since the LTM execution until connection failure in case of LTM and include the LTM recovery scenarios.

### F1 impacts

For failures due to inappropriate cell switch triggering (e.g., wrong cell selection at cell switch, wrong cell switch timing, …),

- Source DU performs optimization

- FFS whether CU or DU performs root cause analysis.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

In LTM, a gNB-DU might have selected the wrong target cell for LTM execution and therefore an RLF might have happened. Since gNB-DU determines the target cell to which LTM is executed, it should also be responsible to root cause why it selected a wrong target cell for LTM execution.

But Tstore_UE_cntxt is not known at gNB-DU (and is only configured at gNB-CU by OAM). Therefore, gNB-DU can’t categorize an LTM as Too early LTM or HO to wrong cell (e.g., in case of RLF shortly after a successful LTM execution).

Therefore, we think gNB-CU should identify the LTM failure type (Too early LTM/Too late LTM/LTM to wrong cell) and indicate to gNB-DU via F1AP along with the ACCESS and MOBILITY INDICATION, to assist the gNB-DU.

Tstore_UE_cntxt is not known at gNB-DU (and is only configured at gNB-CU by OAM). Therefore, gNB-DU can’t categorize an LTM as Too early LTM or HO to wrong cell, in case there is an RLF shortly after a successful LTM execution.

gNB-CU can identify the LTM failure type (Too early/Too late/wrong cell) and indicate the LTM failure type to gNB-DU via F1AP ACCESS and MOBILITY INDICATION.

### Recovery after RACH-less LTM failure

In the previous meeting, some companies proposed to discuss RACH-less LTM to RACH-based LTM “fall back” scenarios. In our understanding and as per TS 38.321, there is no “fall back” from RACH-less LTM to RACH-based LTM.

As per TS 38.321, UE considers RACH-less LTM as ongoing only if it receives a valid TA from LTM CSC or if the UE-based TA measurement is successful.

In other words, if the UE doesn’t receive a valid TA from LTM CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure.

If the UE doesn’t receive a valid TA from LTM CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure. In other words, there is no “fall back” from RACH-less LTM to RACH-based LTM.

However, there could be a scenario where the RACH-less LTM is initiated (because of UE having a valid TA either received from LTM CSC or via UE-based measurement) but the RACH-less LTM cell switch execution is not successful e.g., because UE couldn’t determine that the network has successfully received its first UL data. In such a case, T304 could expire.

RACH-less LTM cell switch execution may fail (i.e., T304 may expire) because UE couldn’t determine that the network has successfully received its first UL data after RACH-less LTM cell switch execution

Upon T304 expiry, UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after LTM execution failure, then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed. We propose RAN3 to consider these the following scenarios as part of MRO for LTM:

RAN3 to consider the following scenarios after RACH-less LTM cell switch execution failure:

  - RACH-less LTM failure followed by successful RACH-based LTM recovery

  - RACH-less LTM failure followed by RACH-based LTM recovery failure followed by reestablishment

  - RACH-less LTM failure directly followed by reestablishment (if LTM recovery is not configured)

### Successful LTM scenarios (SHR enhancements)

RAN2 Agreements:

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

RA-based access will not be a new triggering condition for the SHR report.

Based on the above RAN2 agreement, the following scenarios are covered:

- RACH-less LTM fails  Indication in RLF Report that this LTM was RACH-less

- RACH-based LTM fails  Indication in RLF Report that this LTM was RACH-based

We can include a similar “access type” (RACH-less or RACH-based) indicator in successful LTM scenarios as well (i.e., enhance SHR to include access type):

- Successful RACH-less LTM  Indication in SHR that this LTM was RACH-less

- Successful RACH-based LTM  Indication in SHR that this LTM was RACH-based

Include an indicator in SHR whether the last successful LTM execution was RACH-based or RACH-less

### Optimizing early TA acquisition procedure

In the previous meetings, there were several proposals to optimize aspects related to early TA acquisition procedure

There were few proposals for the UE to report whether the UE used UE-based TA measurement, or whether it used the network-provided TA value in LTM CSC.

During LTM preparation, source gNB provides the UE with Early UL Sync configuration (obtained from candidate cells) and whether UE-based TA measurement should be performed for each LTM candidate cell. Also, the source gNB-DU knows that it has sent a PDCCH order to trigger Early UL sync procedure. The network can store this Early UL sync configuration and whether it has provided a PDCCH order and there is no need for UE to report it in SON reports.

For each LTM candidate cell, source gNB knows whether it has provided the UE with Early UL Sync configuration, whether it has configured the UE to perform UE-based TA measurement and whether it has sent a PDCCH order to trigger Early UL sync procedure with that candidate cell.

Source gNB can store the Early UL sync config and UE-based TA configuration and can retrieve this information via the C-RNTI and timeConnFailure reported in RLF Report. There is no need for UE to report to the gNB whether the UE used UE-based TA measurement, or whether it used the network-provided TA value in LTM CSC

There were few proposals for the UE to report the TA value it measured via UE-based TA measurement to solve issues related to accuracy/validity of the UE computed value. In our view, this is up to UE implementation and there is no need to solve any issues related to accuracy/validity of the UE computed TA value.

There is no need to solve issues related to accuracy/validity of the UE computed TA value because this is up to UE implementation.

Further, there were few proposals for the candidate gNB-DU to optimize the TA value it sent to source gNB-DU during Early UL sync procedure. Even this would be up to candidate gNB-DU implementation, and we are not sure whether we need any mechanisms to optimize this.

Finally, there were also proposals to optimize the Early UL Sync configuration (i.e., the CFRA resources used to send Msg1). We think this can solved by existing RA Information included in RA/RLF/SHR reports as it already includes the BWP, SCS, frequency information.

UE already reports BWP, SCS and frequency information in RA-InformationCommon and this can be used to optimize the CFRA resources used for Early TA acquisition).

### Optimizing LTM candidate cells at gNB-CU

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

Aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

During LTM preparation, gNB-CU determines the LTM candidate cells (e.g., based on L3 measurements from UE). It was also agreed in RAN3 meeting that gNB-CU is responsible for optimizing LTM candidate cells. But it is not yet clear how the gNB-CU optimizes the LTM candidate cell list (e.g., based on UE assistance in RLF Report or based on a network-based solution).

In Rel-17, we similarly discussed how to optimize the CHO candidate cell list and there were 2 options discussed - a network-based solution and a UE based solution. After a contentious discussion, both solutions were agreed, albeit for different sub cases:

- UE based solution (for the case of CHO execution failure): UE reports the CHO candidate cell list in RLF Report, in case of CHO execution failure.

- Network based solution (for the case of RLF shortly after a successful CHO execution): Source gNB provides the CHO configuration (which includes CHO candidate cell list and CHO execution conditions) to target gNB in SN STATUS TRANSFER. Target gNB stores it and provides it back to source gNB in HANDOVER REPORT

In Rel-17, the UE based solution was agreed first, but then we realized that the UE releases the CHO configuration after a successful CHO execution and therefore UE based solution can’t work in the case that there is a RLF shortly after the successful CHO execution. Therefore, the network-based solution was also agreed.

To optimize CHO candidate cell list, Rel-17 specified two solutions - a UE based solution (UE reports the CHO candidate cell list in RLF Report) and a network-based solution (source gNB provides the CHO configuration to target gNB in SN STATUS TRANSFER which stores it and provides it back to source gNB in HANDOVER REPORT).

LTM is different from CHO in that we support subsequent LTM and therefore UE stores the LTM configuration even after a successful LTM execution. Also, Rel-18 only supports intra-CU LTM and therefore the gNB-CU can have the LTM candidate cells in its UE context even if there is inter-DU mobility within the gNB-CU.

In Rel-19 MRO for LTM, we should avoid having two solutions (i.e., both UE based and network-based solution) for optimizing LTM candidate cell list. Also existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF.

We propose to use a network-based solution as follows:

There is no need for UE to report the LTM candidate cell list (or an indication whether a cell was LTM candidate or not) in RLF Report. Existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF.

### UHI/MHR enhancements for LTM

A UE supporting LTM might undergo both LTM and normal L3 HO scenarios depending on the gNB implementation. A gNB might do a back-to-back L3 HO and LTM due to L3 execution conditions being met in gNB-CU and LTM execution condition met in gNB-DU.

Therefore, it useful for the gNB to know whether the UE’s visited cells were due to LTM or L3 HO. Without such knowledge, the gNB might unnecessarily be trying to optimize ping-pong handovers even though they happened due to different reasons (i.e., L3 HO and LTM)

We therefore propose RAN3 to discuss how to distinguish genuine ping-pong handovers vs. back-to-back L3 HO and LTM. One solution could be to optimize the UHI to include a HO-type (i.e., LTM or L3 HO) similar to how it has been agreed by RAN2 for RLF Report.

RAN3 should discuss how gNB can distinguish genuine ping-pong handovers vs. back-to-back L3 HO and LTM (e.g., via MHR/UHI enhancements)

Enhance UHI and/or MHR to include a “HO-type” (i.e., LTM or L3 HO).

### Optimizing beam information indicated in LTM CSC

From TS 38.300,

The gNB-DU decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state, or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available.

As mentioned above, the gNB-DU can indicate beam information (TCI state ID, UL TCI state ID) in LTM cell switch command MAC CE to the UE.

The UE can use this information during LTM execution e.g., considers the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell during RACH-less LTM cell switch.

It is possible that the gNB-DU might not have indicated the right TCI state ID and therefore the UE might have used the wrong one thereby resulting in LTM cell switch failure. We therefore propose the following:

RAN3 should discuss solutions on how to optimize the target beam(s) (indicated via TCI state(s) in LTM cell switch command MAC CE to UE) for LTM execution.

### LTM-CHO coexistence

Suppose CHO and LTM are both configured and there is a LTM failure (or a RLF shortly after a successful LTM). The following recovery mechanisms are possible, if configured by the network:

- UE will perform CHO recovery if the selected cell after RLF is a CHO candidate cell

- UE will perform LTM recovery if the selected cell after RLF is a LTM candidate cell

- It is up to UE on what recovery UE should perform if the selected cell after RLF is both a CHO candidate and LTM candidate cell, then

If no recovery is configured by the network, the UE simply performs cell selection and performs reestablishment on the selected cell

In such a scenario where a UE is configured with both CHO candidate cells and LTM candidate cells and UE encounters an RLF, RAN3 should discuss if any optimizations can be made for such coexistence scenarios.

Therefore, the following is proposed.

RAN3 should discuss failure scenarios when a UE is configured with both LTM and CHO and if any optimizations can be made for such coexistence scenarios.

## 2.2 MRO for CHO with candidate SCG

### 2.2.1 Failure Scenarios

Agreements and open issues so far:

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed

Do not distinguish between fast MCG recovery/no-fast MCG recovery (after the CHO with candidate SCG execution failure)

FFS whether there is any specification impact for case 4/5/6.

Case 1c, 2c, 3c, 7c, 8c are FFS

FFS whether to support concurrent failure cases (RLF + SCG failure). The definition of these cases needs to be further clarified.

Concurrent MCG and SCG failures can be deprioritized as this is not specific to CHO with candidate SCG. This was also considered in R17/R18 (e.g., MCG failure before UE sends SCGFailureInformation) but no enhancements were specified. The other scenario where fast MCG recovery via SCG could not be initiated because SCG fails immediately after MCG fails is already deprioritized as RAN3 agreed to not distinguish between fast MCG recovery/no-fast MCG recovery (after the CHO with candidate SCG execution failure). Therefore, the following is proposed:

There is no need to support concurrent failure cases (RLF + SCG failure) i.e., there is no need to support case 1c, 2c, 3c, 7c, 8c

### 2.2.1 Optimizing the association of PCell and PSCell

FFS whether to optimize the association of the PCell and PSCell.

FFS whether to enhance RLF Report with

- Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

Consider a scenario where the UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and non-associated PCells and PSCells met the execution condition

In the example below, Execution condition of candidate PCell A1 and candidate PSCell B3 is met but the CHO with candidate SCG execution didn’t happen because there is no (A1, B3) associated configuration.

It would be beneficial if the network can know what execution conditions were met (i.e., A1 and B3) so that the network can fine-tune the CHO with candidate SCG configuration eg., by including a (A1, B3) configuration or (A2, B3) confguration to avoid RLF or SCG failures in the future.

RAN3 considers “case 10: UE meets the execution conditions of non-associated candidate PCells and PSCells before encountering an RLF or SCG Failure” in the list of scenarios for MRO for CHO with candidate SCG

RAN3 agrees to optimize the association of PCell and PSCell in the CHO with candidate SCG configuration i.e., to avoid scenarios where UE meets the execution conditions of non-associated candidate PCells and PSCells before encountering an RLF or SCG Failure.

To optimize the association of PCell and PSCells in CHO with candidate SCG configuration, enhance RLF Report with:

- Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPC execution conditions before the RLF is encountered

### 2.2.1 Optimizing the CHO with candidate SCG execution due to “blocking” execution condition

UE includes following information in RLF report:

- Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS.

- We consider both the case when both CHO condition and associated CPC condition are fulfilled,  and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

-Measurement results of PCells and PSCells.

It would be useful for the gNB to know the time elapsed between the two fulfilled execution conditions (CHO and CPAC) or the time elapsed between the first fulfilled execution condition and RLF to optimize the CHO and CPAC execution conditions. Therefore, the following is proposed:

Enhance RLF Report to include the following time information:

  - Time elapsed between the first fulfilled execution condition and RLF

  - Time elapsed between the two fulfilled execution conditions (CHO and CPAC)

### 2.2.1 CHO Recovery after CHO with candidate SCG execution failure

When initial CHO execution attempt fails or HO fails, the UE performs cell selection, and if the selected cell is a CHO candidate and if network configured the UE to try CHO after handover/CHO failure, then the UE attempts CHO execution once, otherwise re-establishment is performed.

Suppose a UE receives a CHO with candidate SCG config, both the execution conditions are met but the CHO execution fails (e.g., RACH in target cell fails). UE performs cell selection, and the selected cell is also a CHO candidate cell.

As per the below text from 38.331, UE sets the choCellId (CHO recovery cell ID) in RLF Report only in case CHO recovery is attempted after a legacy CHO failure and not in case of CHO recovery is attempted after a CHO with candidate SCG execution failure.

1>  if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, except for an LTM cell switch procedure following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or mobility from NR failure, and

1>  if attemptCondReconfig is configured; and

1>  if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>  if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig:

2>  if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>  apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

NOTE 1:  It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

Currently, UE sets the choCellId (CHO recovery cell ID) in RLF Report only in case CHO recovery is attempted after a legacy CHO failure and not in case of CHO recovery is attempted after a CHO with candidate SCG execution failure.

We therefore have the following proposal:

RAN3 should consider scenarios where UE attempts CHO recovery after a CHO with candidate SCG execution failure.

UE should log the CHO recovery cell ID in RLF Report even when UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Consider Scenario 2 again where the UE encounters an RLF before the CHO with candidate SCG execution because candidate PSCell (B3 in the example below) satisfies the execution condition but the associated candidate PCell (i.e., A2) doesn’t satisfy the corresponding execution condition.

Suppose the UE attempts CHO recovery and observed that both PCell A2 and PCell A3 are suitable cells, the UE should be able to prioritize selecting PCell A2 for CHO recovery because its associated PSCell’s (B3) execution condition was met before rather than selecting PCell A3 for CHO recovery because its associated PCell’s (B4) execution condition was not met before.

We therefore have the following proposal:

In case the UE attempts CHO recovery after a CHO with candidate SCG execution failure and observes that there are 2 or more suitable cells, UE may prioritize selecting a cell for CHO recovery based on whether its associated PSCell’s execution condition was met before or not

## 2.3 MRO for Subsequent CPAC (S-CPAC)

RAN3 agreements:

Work on the scenarios of failure in S-CPAC.

The optimization of non-failure scenarios (e.g., near failure and ping-pong) is not excluded.

R19 SON/MDT solution discussion is based on R18 work.

Reuse SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.

FFS whether to capture in stage-2 the following text: “For the CPAC failures which occurred during an S-CPAC procedure, CPC Execution above refers to the initial CPC execution or the subsequent CPC execution”

FFS whether the differentiation between initial and following CPC is needed

A subsequent CPAC is defined as a conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, a PCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA.

The UE keeps the configured subsequent CPAC configuration (unless the network indicates to release it) and evaluates the execution conditions of candidate PSCells after completion of a PSCell addition, a PSCell change, a PCell change or an SCG release.

UE keeps the configured S-CPAC configuration unless the network indicates to release it

The S-CPAC configuration contains

- candidate SCG configuration(s) of candidate PSCell(s)

- execution conditions

- MCG configuration (to be applied when subsequent CPAC execution is triggered)

- reference configuration

- security update configuration

In case a UE configured with S-CPAC configuration encounters an SCG failure, it sends a SCGFailureInformation to MN (including the measurement results) and waits for MN’s subsequent action e.g., whether it wants UE to add a new PSCell or keep it in standalone mode

In case the MN wants to add a new PSCell for the UE and the new PSCell is an already prepared candidate PSCell whose configuration is provided to the UE as part of S-CPAC configuration, MN should be able to simply indicate the PSCell to add without providing the whole configuration. UE can simply activate the indicated PSCell’s configuration and start evaluating the S-CPAC configuration.

For example, UE encounters an SCG failure in source PSCell A0 and reports in SCGFailureInformation that B4 (an already prepared candidate, but not the immediate candidate) was the strongest cell at the time of SCG failure

We therefore have the following proposals:

RAN3 should discuss solutions how to perform an optimized PSCell addition post an SCG failure for a UE configured with S-CPAC configuration.

In case the MN wants to add a new PSCell for the UE and the new PSCell is an already prepared candidate PSCell whose configuration is provided to the UE as part of S-CPAC configuration, MN should be able to simply indicate the PSCell to add without providing the whole configuration.

Upon receiving such an indication to activate a PSCell, UE can simply activate the indicated PSCell’s configuration and start evaluating the S-CPAC configuration.

# Summary

2.1	MRO for LTM

Observation 1.	CHO recovery scenario is not captured in the definitions of Too late HO, Too Early HO, HO to wrong cell. Instead, it is captured as part of the detection mechanisms

Proposal 1:	Similar to CHO Recovery, LTM recovery scenarios need not be captured in the definitions of Too Late HO, Too Early HO, HO to wrong cell and should be instead captured as part of the detection mechanisms. Remove Editor’s note 1.

Proposal 2:	To simplify stage-2, reuse the existing text on detection mechanisms of Intra-system Too Late Handover, Intra-system Too Early Handover and Intra-system Handover to Wrong Cell with necessary modifications for LTM e.g., clarify that the UE reported timer refers to the time elapsed since the LTM execution until connection failure in case of LTM and include the LTM recovery scenarios.

Observation 2.	Tstore_UE_cntxt is not known at gNB-DU (and is only configured at gNB-CU by OAM). Therefore, gNB-DU can’t categorize an LTM as Too early LTM or HO to wrong cell, in case there is an RLF shortly after a successful LTM execution.

Proposal 3:	gNB-CU can identify the LTM failure type (Too early/Too late/wrong cell) and indicate the LTM failure type to gNB-DU via F1AP ACCESS and MOBILITY INDICATION.

Observation 3.	If the UE doesn’t receive a valid TA from LTM CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure. In other words, there is no “fall back”from RACH-less LTM to RACH-based LTM.

Observation 4.	RACH-less LTM cell switch execution may fail (i.e., T304 may expire) because UE couldn’t determine that the network has successfully received its first UL data after RACH-less LTM cell switch execution

Proposal 4:	RAN3 to consider the following scenarios after RACH-less LTM cell switch execution failure:

a. RACH-less LTM failure followed by successful RACH-based LTM recovery

b. RACH-less LTM failure followed by RACH-based LTM recovery failure

c. RACH-less LTM failure followed by reestablishment (if recovery is not configured)

Proposal 5:	Include an indicator in SHR whether the successful LTM execution was RACH-less or RACH-based

Observation 5.	For each LTM candidate cell, source gNB knows whether it has provided the UE with Early UL Sync configuration, whether it has configured the UE to perform UE-based TA measurement and whether it has sent a PDCCH order to trigger Early UL sync procedure with that candidate cell.

Proposal 6:	Source gNB can store the Early UL sync config and UE-based TA configuration and can retrieve this information via the C-RNTI and timeConnFailure reported in RLF Report. There is no need for UE to report to the gNB whether the UE used UE-based TA measurement, or whether it used the network-provided TA value in LTM CSC

Proposal 7:	There is no need to solve issues related to accuracy/validity of the UE computed TA value because this is up to UE implementation.

Proposal 8:	UE already reports BWP, SCS and frequency information in RA-InformationCommon and this can be used to optimize the CFRA resources used for Early TA acquisition).

Observation 6.	To optimize CHO candidate cell list, Rel-17 specified two solutions - a UE based solution (UE reports the CHO candidate cell list in RLF Report) and a network-based solution (source gNB provides the CHO configuration to target gNB in SN STATUS TRANSFER which stores it and provides it back to source gNB in HANDOVER REPORT).

Proposal 9:	There is no need for UE to report the LTM candidate cell list (or an indication whether a cell was LTM candidate or not) in RLF Report. Existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF.

Proposal 10:	RAN3 should discuss how gNB can distinguish genuine ping-pong handovers vs. back-to-back L3 HO and LTM (e.g., via MHR/UHI enhancements)

Proposal 11:	Enhance UHI and/or MHR to include a “HO-type” (i.e., LTM or L3 HO).

Proposal 12:	RAN3 should discuss solutions on how to optimize the target beam(s) (indicated via TCI state(s) in LTM cell switch command MAC CE to UE) for LTM execution.

Proposal 13:	RAN3 should discuss failure scenarios when a UE is configured with both LTM and CHO and if any optimizations can be made for such coexistence scenarios.

2.2 MRO for CHO with candidate SCG

Proposal 14:	There is no need to support concurrent failure cases (RLF + SCG failure) i.e., ther e is no need to support case 1c, 2c, 3c, 7c, 8c

Proposal 15:	RAN3 considers “case 10: UE meets the execution conditions of non-associated candidate PCells and PSCells before encountering an RLF or SCG Failure” in the list of scenarios for MRO for CHO with candidate SCG

Proposal 16:	RAN3 agrees to optimize the association of PCell and PSCell in the CHO with candidate SCG configuration i.e., to avoid scenarios where UE meets the execution conditions of non-associated candidate PCells and PSCells before encountering an RLF or SCG Failure.

Proposal 17:	To optimize the association of PCell and PSCells in CHO with candidate SCG configuration, enhance RLF Report with:

Proposal 18:	Enhance RLF Report to include the following time information:

a. Time elapsed between the first fulfilled execution condition and RLF

b. Time elapsed between the two fulfilled execution conditions (CHO and CPAC)

Observation 7.	Currently, UE sets the choCellId (CHO recovery cell ID) in RLF Report only in case CHO recovery is attempted after a legacy CHO failure and not in case of CHO recovery is attempted after a CHO with candidate SCG execution failure.

Proposal 19:	RAN3 should consider scenarios where UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Proposal 20:	UE should log the CHO recovery cell ID in RLF Report even when UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Proposal 21:	In case the UE attempts CHO recovery after a CHO with candidate SCG execution failure and observes that there are 2 or more suitable cells, UE may prioritize selecting a cell for CHO recovery based on whether its associated PSCell’s execution condition was met before or not

2.3 MRO for Subsequent CPAC (S-CPAC)

Observation 8.	UE keeps the configured S-CPAC configuration unless the network indicates to release it

Proposal 22:	RAN3 should discuss solutions how to perform an optimized PSCell addition post an SCG failure for a UE configured with S-CPAC configuration.

Proposal 23:	In case the MN wants to add a new PSCell for the UE and the new PSCell is an already prepared candidate PSCell whose configuration is provided to the UE as part of S-CPAC configuration, MN should be able to simply indicate the PSCell to add without proviing the whole configuration.

Proposal 24:	Upon receiving such an indication to activate a PSCell, UE can simply activate the indicated PSCell’s configuration and start evaluating the S-CPAC configuration.

# 4 Annex 1: TP to 38.300

15.5.2.2.2            Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-     Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-     Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-     Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

Editor’s note 1: FFS if LTM recovery needs to be captured in the definitions.

Editor’s note 2: For the too late LTM cell switch, FFS if the re-establishment cell is a candidate cell, a cell not selected by CU, or both.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-     Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO is configured but the CHO execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if LTM is configured but the LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt)..

-     Intra-system Too Early Handover: there is a recent handover/LTM for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-     Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO execution until connection failure or the time elapsed since the LTM execution until connection failure.

# 4 Annex 2: MRO for CHO with candidate SCG - Scenarios

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |