3GPP TSG-RAN3 Meeting #125bis	R3-245336

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	10.3.2

Source:	Qualcomm Incorporated

Title:	SON MDT for network slicing

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss aspects related to SON/MDT for network slicing based on agreements and open issues from previous meetings.

# Discussion

  - Immediate MDT collection in areas supporting a slice

RAN3 to support immediate MDT measurements collection on specific area per slice(s).

Slice ID is added as a new filter to the Area scope of MDT for the both management-based and signaling-based immediate MDT

In the last meeting, S-NSSAI was added as a new filter to the MDT Configuration-NR to support immediate MDT measurements collection only when the UE is camped on cells supporting a certain S-NSSAI and a TP to NGAP was agreed with the few procedural texts e.g.,

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the areas included in the Network Slice Area Scope of MDT IE.

Editor’s note: FFS if the procedural text above and corresponding in other sections, should be enhanced to describe in more details how the NG-RAN node derives the MDT area scope for MDT measurement collection

…

8.3.1.4	Abnormal Conditions

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE in the INITIAL CONTEXT SETUP REQUEST message, and the Area Scope of MDT IE is set to "Network Slice Based MDT", the NG-RAN node shall, if supported, use the Area Scope of MDT IE to derive the MDT area scope for MDT measurement collection, and ignore the Network Slice Area Scope of MDT IE.

..

Further in the TP, Network Slice Area Scope of MDT IE was added both inside and outside the Area Scope of MDT to support the following cases:

- Slice-specific immediate MDT collection

- Slice-specific immediate MDT collection + MDT collection as per legacy area scope of MDT

Consider an example scenario where:

- Cell 1 supports S-NSSAI 1 and 2

- Cell 2 supports S-NSSAI 1 and 3

- Cell 3 supports S-NSSAI 2 and 3

Case 1: Network Slice Area Scope of MDT IE is added only inside the Area Scope of MDT

E.g., if S-NSSAI = 1 inside the Area Scope of MDT is configured, then gNB should configure and collect immediate MDT only in cells supporting S-NSSAI = 1, which are cell 1 and cell 2

Case 2: Network Slice Area Scope of MDT IE is added outside the Area Scope of MDT and legacy Area Scope (e.g., list of cells level) is included

E.g., if S-NSSAI = 1 is configured the Area Scope of MDT and Area Scope of MDT = [cell 3], then gNB should configure and collect immediate MDT in cells supporting S-NSSAI = 1 (which are cells 1 and 2) and in cell 3. So immediate MDT will be collected in cells 1, 2 and 3.

Case 3: Network Slice Area Scope of MDT IE is added inside and outside the Area Scope of MDT

E.g., if S-NSSAI = 1 is configured inside the Area Scope of MDT and S-NSSAI = 2 is configured outside the Area Scope of MDT, then as per abnormal condition procedural text, the IE outside the Area Scope of MDT (i.e. S-NSSAI = 2) is ignored and only S-NSSAI = 1 is considered. In other words, gNB should configure and collect immediate MDT only in cells supporting S-NSSAI = 1 which are cell 1 and cell 2 (same as case 1).

Although adding S-NSSAI both inside and outside the Area Scope of MDT provides more flexibility, we need to consider more combination scenarios and see if all the scenarios make sense. If not, abnormal conditions need not be added.

Adding S-NSSAI both inside and outside the Area Scope of MDT requires the need to check other combination scenarios, might need abnormal handling and explicit semantics text to disable some combinations (e.g., slice-specific immediate MDT collection might not be needed for SNPN or PNI-NPNs). Following combinations are possible:

  - Network Slice Area Scope of MDT IE + Cell based

  - Network Slice Area Scope of MDT IE + TA based

  - Network Slice Area Scope of MDT IE + PLMN wide

  - Network Slice Area Scope of MDT IE + PNI-NPN Based MDT

  - Network Slice Area Scope of MDT IE + SNPN Cell Based MDT

  - Network Slice Area Scope of MDT IE + SNPN TAI Based MDT

  - Network Slice Area Scope of MDT IE + SNPN Based MDT

  - Network Slice Area Scope of MDT IE + Network Slice Based MDT (already considered in abnormal)

While adding S-NSSAI both inside and outside the Area Scope of MDT provides additional flexibilities in the MDT configuration and some overhead savings (compared to providing all the list of cells supporting the slice of interest), the complexities to handle the combination scenarios and abnormal conditions outweigh the benefits.

Remove Network Slice Area Scope of MDT IE from the MDT Configuration-NR (i.e., the IE outside the Area Scope of MDT) and only keep the Network Slice Based MDT IE inside the Area Scope of MDT

Also, in the TP agreed last meeting, PLMN Identity is also added in addition to the S-NSSAI in the Network Slice List for MDT IE. The reasoning provided was that this could be beneficial in RAN sharing scenarios, where an OAM can indicate to perform slice-specific MDT collection for a specific PLMN among the PLMNs sharing the RAN. We are not sure if this is needed, since OAM already indicates a MDT PLMN list to the RAN, which indicates the list of PLMNs (including equivalent PLMNs) where MDT collection is allowed. If needed, OAM can change this MDT PLMN list to allow slice-specific immediate MDT collection only in the PLMN of interest.

Remove PLMN Identity from the Network Slice List for MDT IE. Existing MDT PLMN List IE already indicates the list of PLMNs where MDT collection is allowed and there is no add this new IE to enable slice-specific immediate MDT collection only in certain PLMNs.

FFS on which immediate MDT measurements can be collected per slice.

An operator might be interested in collecting immediate MDT only in cells supporting a slice for M4/M5/M6/M7 to collect throughput/packet delay/packet loss in cells supporting a slice. We don’t see any reason why an operator might want to limit M1/M8/M9 measurements in cells supporting a slice. So, we think that the Slice-specific immediate MDT measurement collection only applies to M4, M5, M6 and M7.

Slice-specific immediate MDT measurement collection only applies to M4, M5, M6 and M7.

  - Logged MDT collection in areas supporting a slice

FFS on whether to support logged MDT on specific area per slice(s).

Similar to immediate MDT, an operator might be interested in collecting logged MDT only in cells where a certain slice is supported (e.g., it might be interested in developing coverage maps only in cells supporting a certain slice). This can be already achieved today by gNB identifying the cells where the slice of interest is supported and create an Area scope of logged MDT comprising of only those cells supporting the slice of interest.

Before we discuss solutions, we want RAN3 to discuss whether there is a benefit in supporting this use case. The following is therefore proposed:

RAN3 should discuss whether there is a need to support logged MDT collection only in cells supporting certain slice(s) of interest based on the Network Slice Area Scope of MDT indicated in the MDT Configuration.

  - Slice-usage based immediate MDT collection and UE selection

There was also discussion on whether to support immediate MDT measurements collection only when the UE is using a certain slice and whether to enhance UE selection based on slice usage.

A UE supporting multiple slices might dynamically use/switch to different slices/PDU sessions frequently based on the application(s) it is using.

Also, UE selection based on slice usage might not be efficient because a UE not using a certain slice at a given time might start using a certain slice at a later time. So, the gNB might miss out on selecting UEs if it is done based on slice usage.

We therefore propose the following:

RAN3 will not support any mechanisms to support slice-usage based immediate/logged MDT measurement collection or any enhancements to UE selection based on slice-usage.

  - Logged MDT enhancements for optimizing slice-based cell reselection

RAN3 to work on logged MDT enhancement for slice-based cell reselection.

In order to support slice-based cell reselection, a UE may be provided with slice-based cell reselection information via RRCRelease or SIB16 e.g., reselection priorities per NSAG per frequency and corresponding list(s) of cells where the slices of the NSAG are supported or not supported.

When a UE supports slice-based cell reselection, and when slice-based cell reselection information is provided to the UE, then the UE uses the slice-based cell reselection information.

When no slice-based cell reselection information is provided for any NSAG that was determined to be considered during cell reselection, then the UE uses the general cell reselection information, i.e., without considering the NSAG(s) and their priorities.

A UE provided with slice-based cell reselection information may reselect different cells compared to a UE not provided with slice-based cell reselection information parameters because of different reselection priorities and based on a bias introduced by whether the cell supports a certain slice or not. An operator trying to build coverage maps via logged MDT might not be able to build a true coverage map if it doesn’t know why UEs traversing a similar path see a different coverage (i.e., connect to different cells) even though the radio conditions are similar.

There are benefits in making the OAM aware that the logged MDT reports were collected by a UE using slice-based cell reselection information (and not general cell reselection information).

UE can indicate via a flag in logged MDT report that it collected logged MDT using the slice-based cell reselection parameters (and not general cell reselection parameters).

Further in the last meeting, there were proposals for UE to log the slice group(s) i.e., NSAGs or frequency priorities in the logged MDT report and report it to the gNB to optimize the mapping of S-NSSAI to NSAG or optimize the frequency priorities. We think adding these network configurations in the logged MDT report is not appropriate as this adds to burden for UE to store these configurations in RRC_IDLE and to Uu overhead in logged MDT report. gNB should be able to store this information if needed. And, it is be noted that the gNB can’t identify the UE across state transitions, so even if these network configurations are reported back, there can’t be any UE-specific optimizations.

Requiring the UE to log the NSAGs or frequency priorities in the logged MDT report adds burden for UE to store these configurations in RRC_IDLE and to Uu overhead in logged MDT report. gNB should be able to store this information if needed

There is no need for UE to log the NSAGs or frequency priorities in the logged MDT report and report it to the gNB to optimize the mapping of S-NSSAI to NSAG or optimize the slice specific cell reselection priorities

  - Slice-aware MRO

RAN3 agreements:

Work on the Slice aware MRO scenarios.

For successful handover, FFS if solutions are needed to optimize handovers in the case of slicing discontinuity, e.g., target cell not supporting slice.

The following is captured in TS 38.300

To make mobility slice-aware in case of Network Slicing, S-NSSAI is introduced as part of the PDU session information that is transferred during mobility signalling. This enables slice-aware admission and congestion control.

Both NG and Xn handovers are allowed regardless of the slice support of the target NG-RAN node i.e. even if the target NG-RAN node does not support the same slices as the source NG-RAN node.

Even though handovers are allowed regardless of the slice support of the target NG-RAN node, this leads to discontinuity in the slice service and interruption, if UE moves towards an area that does not support one of UE's ongoing slices (e.g., during inter-RA mobility).

To detect such scenarios and take correction action in the future, it was proposed by few companies last meeting to add slice-related information in SON reports (e.g., in UHI or SHR) e.g., include UE’s ongoing slice(s) in the source and target cell in UHI, collect SHR when a certain handover resulted in a slice service interruption.

Currently, a gNB already knows the slices supported by its served cells and the slices supported in the cells served by its neighbor gNBs via Slice Support List exchange in F1 and Xn. During handover preparation, the source gNB will know that a PDU session (which is associated to a slice) it is offloading to a certain target gNB might get interrupted due to non-support of that slice in the target gNB.

We therefore are not sure whether any enhancements to SON reports (e.g., collecting SHR in case of such handover to target cell supporting the ongoing slice) to do non-real optimizations are really needed when such things are known already at the gNB.

Also, a certain gNB via implementation can already take slice aware mobility decisions and mitigate slice service discontinuity. For example, a gNB can prioritize a target cell that supports UE’s ongoing slice in the source cell over another target cell that doesn’t support the UE’s ongoing slice (if radio conditions of the two target cells are comparable).

A gNB knows the slices supported by its served cells and the slices supported in the cells served by its neighbor gNBs via Slice Support List exchange in F1 and Xn.

Based on the knowledge of slice(s) supported at neighboring gNBs, a source gNB has knowledge whether a PDU session (which is associated to a slice) will get interrupted or not upon offloading to a target gNB.

There are no enhancements needed from UE in SON reports to assist the gNB to optimize handovers with slice service discontinuity (e.g., due to target cell not supporting UE’s ongoing slice). gNB already has sufficient knowledge to identify such scenarios.

# Conclusion

Observation 1.	Adding S-NSSAI both inside and outside the Area Scope of MDT requires the need to check other combination scenarios, might need abnormal handling and explicit semantics text to disable some combinations (e.g., slice-specific immediate MDT collection might not be needed for SNPN or PNI-NPNs). Following combinations are possible:

 Network Slice Area Scope of MDT IE + Cell based

 Network Slice Area Scope of MDT IE + TA based

 Network Slice Area Scope of MDT IE + PLMN wide

 Network Slice Area Scope of MDT IE + PNI-NPN Based MDT

 Network Slice Area Scope of MDT IE + SNPN Cell Based MDT

 Network Slice Area Scope of MDT IE + SNPN TAI Based MDT

Network Slice Area Scope of MDT IE + SNPN Based MDT

 Network Slice Area Scope of MDT IE + Network Slice Based MDT (already considered in abnormal)

Observation 2.	While adding S-NSSAI both inside and outside the Area Scope of MDT provides additional flexibilities in the MDT configuration and some overhead savings (compared to providing all the list of cells supporting the slice of interest), the complexities to handle the combination scenarios and abnormal conditions outweigh the benefits.

Proposal 1:	Remove Network Slice Area Scope of MDT IE from the MDT Configuration-NR (i.e., the IE outside the Area Scope of MDT) and only keep the Network Slice Based MDT IE inside the Area Scope of MDT

Proposal 2:	Remove PLMN Identity from the Network Slice List for MDT IE. Existing MDT PLMN List IE already indicates the list of PLMNs where MDT collection is allowed and there is no add this new IE to enable slice-specific immediate MDT collection only in certain PMNs.

Proposal 3:	Slice-specific immediate MDT measurement collection only applies to M4, M5, M6 and M7.

Proposal 4:	RAN3 should discuss whether there is a need to support logged MDT collection only in cells supporting certain slice(s) of interest based on the Network Slice Area Scope of MDT indicated in the MDT Configuration.

Proposal 5:	RAN3 will not support any mechanisms to support slice-usage based immediate/logged MDT measurement collection or any enhancements to UE selection based on slice-usage.

Observation 3.	There are benefits in making the OAM aware that the logged MDT reports were collected by a UE using slice-based cell reselection information (and not general cell reselection information).

Proposal 6:	UE can indicate via a flag in logged MDT report that it collected logged MDT using the slice-based cell reselection parameters (and not general cell reselection parameters).

Observation 4.	Requiring the UE to log the NSAGs or frequency priorities in the logged MDT report adds burden for UE to store these configurations in RRC_IDLE and to Uu overhead in logged MDT report. gNB should be able to store this information if needed

Proposal 7:	There is no need for UE to log the NSAGs or frequency priorities in the logged MDT report and report it to the gNB to optimize the mapping of S-NSSAI to NSAG or optimize the slice specific cell reselection priorities

Observation 5.	A gNB knows the slices supported by its served cells and the slices supported in the cells served by its neighbor gNBs via Slice Support List exchange in F1 and Xn.

Observation 6.	Based on the knowledge of slice(s) supported at neighboring gNBs, a source gNB has knowledge whether a PDU session (which is associated to a slice) will get interrupted or not upon offloading to a target gNB.

Proposal 8:	There are no enhancements needed from UE in SON reports to assist the gNB to optimize handovers with slice service discontinuity (e.g., due to target cell not supporting UE’s ongoing slice). gNB already has sufficient knowledge to identify such scenarios.

# 4. References