3GPP TSG-RAN3 Meeting #125bis	R3-245337

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	10.4

Source:	Qualcomm Incorporated

Title:	UHI enhancements for SCG (de) activation

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss aspects related to UHI enhancements for SCG (de)activation.

# Discussion

  - RAN2 incoming LS

RAN2 sent a LS in R2-245009 as shown below:

RAN2 agreed that it is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state:

- RAN2#127: It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state.

For how to collect the time spent information, there are following two options:

Option 1: Network based solution: The network collects the information about time spent in the PSCell in activated or deactivated state.

Option 2: UE based solution: The UE includes the information about time spent in the PSCell in activated vs deactivated state in MHI (Mobility History Information).

RAN2 kindly asks RAN3 to analyse whether Network based solution is sufficient enough for network optimization, or whether a UE based solution is needed.

RAN2 sent an LS to RAN3 seeking views on whether a NW-based solution is sufficient for SCG (de) activation related optimization, or whether a UE based solution is needed.

- Option 1: NW-based solution: The network collects the information about time spent in the PSCell in activated or deactivated state.

- Option 2: UE based solution: The UE includes the information about time spent in the PSCell in activated vs deactivated state in MHI (Mobility History Information).

  - Network based solution vs. UE based solution

The following is from 37.340:

The MN can configure the SCG as activated or deactivated upon e.g. PSCell addition, PSCell change, RRC Resume or handover.

SCG activation can be requested by the MN, by the SN and by the UE.

SCG deactivation can be requested by the MN and by the SN.

Based on the above text, in our view, MN has complete knowledge of when it activates or deactivates an SCG for the UE, can compute the time spent in activated or deactivated state and add it in the existing UHI.

MN always knows when an SCG is activated or deactivated for a UE, irrespective of whether the SCG (de)activation is MN or SN initiated.

Also, the NW based solution has the benefits that it can enable the collection of this SCG (de)activation information from legacy UEs as well without the need to have a Rel-19 UE supporting this feature. It is to be noted that SCG (de)activation feature was introduced in Rel-16 and it is useful to collect data from these Rel-16+ UEs as well and not just the Rel-19 UEs which might be very few.

SCG (de) activation feature was introduced in Rel-16 and there would be more Rel-16 UEs deployed than Rel-19 UEs supporting the UE-based solution

NW based solution enable the collection of this SCG (de)activation information from legacy UEs as well without the need to have a Rel-19 UE supporting this feature

Also, this NW based solution can enable a more dynamic optimization (e.g. MN can indicate these statistics to the SN before SN Release) rather than waiting this information to be retrieved from UE (which can take up to 48 hours).

NW based solution can enable a more dynamic optimization (e.g. MN can indicate these statistics to the SN before SN Release) rather than waiting this information to be retrieved from UE (which can take up to 48 hours).

Historically, MHR and UHI has a lot of duplicate information (e.g., time spent in PCell/PSCell is logged in both places) and the main difference for MHR is that it enables collection of UE History across RRC state transitions.

There were some companies in RAN2 proposing to have a UE-based solution to handle Connected  Idle  Connected transitions and argued that the NW-based solution (the UHI stored) is lost upon RRC state transitions. In our view, we don’t need to solve this because this is a problem for the legacy UHI and not specific to SCG (de)activation information. Also, once the UE goes to RRC_IDLE, there is no SCG and the SCG (de) activation information is not useful anymore. Rather, we should look to support the NW based solution which has more benefits as listed above.

Concerns of the loss of UHI upon RRC state transitions (compared to MHR which is persistent across RRC state transitions) is not specific to SCG (de)activation and is the inherent difference between UHI and MHR.

Send a Reply LS to RAN2 mentioning that NW-based solution is sufficient to collect information on time spent in SCG activated or deactivated state, since the NW-based solution has more benefits compared to the UE-based solution e.g.,

  - NW based solution enable the collection of this SCG (de)activation information from legacy UEs (even from Rel-16 UEs which support SCG (de) activation) without the need to have a Rel-19 UE supporting this feature

  - NW based solution can enable a more dynamic optimization (e.g. MN can indicate these statistics to the SN before SN Release) rather than waiting this information to be retrieved from UE (which can take up to 48 hours).

  - MN always knows when an SCG is activated or deactivated for a UE, irrespective of whether the SCG (de)activation is MN or SN initiated. There is no need to add burden to UE to collect this information, when MN can collect this.

# Conclusion

Observation 1.	RAN2 sent an LS to RAN3 seeking views on whether a NW-based solution is sufficient for SCG (de) activation related optimization, or whether a UE based solution is needed.

 Option 1: NW-based solution: The network collects the information about time spent in the PSCell in activated or deactivated state.

 Option 2: UE based solution: The UE includes the information about time spent in the PSCell in activated vs deactivated state in MHI (Mobility History Information).

Observation 2.	MN always knows when an SCG is activated or deactivated for a UE, irrespective of whether the SCG (de)activation is MN or SN initiated.

Observation 3.	SCG (de) activation feature was introduced in Rel-16 and there would be more Rel-16 UEs deployed than Rel-19 UEs supporting the UE-based solution

Observation 4.	NW based solution enable the collection of this SCG (de)activation information from legacy UEs as well without the need to have a Rel-19 UE supporting this feature

Observation 5.	NW based solution can enable a more dynamic optimization (e.g. MN can indicate these statistics to the SN before SN Release) rather than waiting this information to be retrieved from UE (which can take up to 48 hours).

Observation 6.	Concerns of the loss of UHI upon RRC state transitions (compared to MHR which is persistent across RRC state transitions) is not specific to SCG (de)activation and is the inherent difference between UHI and MHR.

Proposal 1:	Send a Reply LS to RAN2 mentioning that NW-based solution is sufficient to collect information on time spent in SCG activated or deactivated state, since the NW-based solution has more benefits compared to the UE-based solution e.g.,

a. NW based solution enable the collection of this SCG (de)activation information from legacy UEs (even from Rel-16 UEs which support SCG (de) activation) without the need to have a Rel-19 UE supporting this feature

b. NW based solution can enable a more dynamic optimization (e.g. MN can indicate these statistics to the SN before SN Release) rather than waiting this information to be retrieved from UE (which can take up to 48 hours).

c. MN always knows when an SCG is activated or deactivated for a UE, irrespective of whether the SCG (de)activation is MN or SN initiated. There is no need to add burden to UE to collect this information, when MN can collect this.

# 4. References