3GPP TSG-RAN WG3 Meeting #125-bis	R3-245346

Hefei, China, 14 - 18 Oct, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1384 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Xn Setup message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI15 |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The issue raised from real deployment. As defined in the specification, the Xn interface can be considered as operational after Xn Setup response message received by the candidate NG-RAN node. While the behavior is not specified if an identical Xn setup request message received on the same operational Xn interface in initial or candidate NG-RAN nodes. There are three options for NG-RAN node received the message: - Option 1: The receiving node considers the Xn interface as operational and send the Xn setup response message to the initial node. - Option 2: The receiving node considers the Xn interface as operational and ignores the Xn setup request message. - Option 3: The receiving node considers the Xn interface as not operational and response the Xn setup failure message. We think the Option 1 is a more rational solution since the Xn interface is already operational, it is benefit for service continuity to keep the interface operational. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add one abnormal case in Xn setup procedure that the receiving NG-RAN node shall consider the Xn interface as operational and may provide a new XN SETUP RESPOSNE message is identical to the content of the previously one. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it impacts the abnormal cases of Xn setup procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may lead to unexpected consequences when the receiving node makes the interface not operational. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

8.4.1.4	Abnormal Conditions

If the first message received for a specific TNL association is not an XN SETUP REQUEST, XN SETUP RESPONSE, or XN SETUP FAILURE message then this shall be treated as a logical error.

If the initiating NG-RAN node1 does not receive either XN SETUP RESPONSE message or XN SETUP FAILURE message, the NG-RAN node1 may reinitiate the Xn Setup procedure towards the same NG-RAN node, provided that the content of the new XN SETUP REQUEST message is identical to the content of the previously unacknowledged XN SETUP REQUEST message.

If the initiating NG-RAN node1 receives an XN SETUP REQUEST message from the peer entity on the same Xn interface:

-	In case the NG-RAN node1 answers with an XN SETUP RESPONSE message and receives a subsequent Xn SETUP FAILURE message, the NG-RAN node1 shall consider the Xn interface as non operational and the procedure as unsuccessfully terminated according to sub clause 8.4.1.3.

-	In case the NG-RAN node1 answers with an XN SETUP FAILURE message and receives a subsequent XN SETUP RESPONSE message, the NG-RAN node1 shall ignore the XN SETUP RESPONSE message and consider the Xn interface as non operational.

If the candidate NG-RAN node2 receives an XN SETUP REQUEST message from the NG-RAN node1 on the same operational Xn interface:

-	The NG-RAN node2 shall consider the Xn interface as operational and may provide a new XN SETUP RESPOSNE message is identical to the content of the previously one.

Change Ends