3GPP TSG-RAN WG3 Meeting #125-bis	R3-245361

Hefei, CN, 14 - 18 Oct, 2024

Agenda Item:	14.3

Source:	Huawei

Title:	(TP for BLCR for TS 38.300) Support of regenerative payload (including LS responses to SA2)

Document for:	discussion

# 1	Introduction

In the last RAN3-125 meeting, RAN3 made the following agreements for the regenerative payload [1]:

NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. Inactive UE support RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Feeder link switch issue RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

In this paper, we continue to discuss the NG impact and the inactive support in the regenerative payload.

# 2	Discussion

## 2.1 SA2 SI conclusion

SA2 completed and concluded the support of regenerative payload in TR 23.700-29 [2] as follows.

## 8.1 Conclusion for KI#1: Support of Regenerative-based satellite access The conclusion for supporting gNB/eNB on-board the satellite is based on the following principles: - When the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) any new procedures or updates to existing procedures to handle the N2/S1 connection will be defined by RAN WG3. SA WG2 will align its specifications once this has been determined by RAN WG3. - gNB /eNB IP address change due to feeder link switch can be supported using existing N2 and S1 procedures. There are no normative impacts to SA WG2 specifications due to this. SA WG2 will align its specifications if new solution is developed by RAN WG3. - RAN transitioning between different NTN GWs, TAIs and potentially between MMEs/AMFs can be supported by using existing procedures, e.g. the eNB/gNB releasing UEs to (E)CM_IDLE before changing NTN gateway, or by using existing load balancing, trigger handover when it realizes it is going to leave a tracking area due to its own movement, etc. - AMF/MME can treat the Mapped Cell IDs as for Rel-17. - Knowledge of when regenerative payload type could be used to assist with CN PDB determination and policy decisions. Introducing new RAT types is not required and it is assumed deployments can determine the usage of regenerative payload type if needed.

First it can be observed that there is no new architecture on the regenerative payload is introduced. Hence the existing RAN/CN architecture can be used. And the figure on the Overall illustration of an NTN with regenerative payload hosting a gNB in the BLCR for TS 38.300 in [4] is fine.

Second, SA2 does not have any requirement on existing procedures when the gNB leaves the service area of an AMF, and will aligned on RAN3, if any from RAN3.

Observation 1: SA2 SI concluded to not introduce new RAN/CN architecture nor requirements on the regenerative payload. SA2 will update their specification, if any enhancement from RAN3.

## 2.2 NG impact

The previous RAN3-125 meeting agreed to introduce the NG removal procedure, there is no strong need for introducing NG suspend/resume procedures. This is not only leading to the specification redundant to have two functions in place for the same purpose, but also is challengeable to satellite implementations. The only advantage for the NG suspend/resume procedure is a little signalling overhead reduction when establishing the NG connection with the same AMF again and not re-establishing the interface context (NG Configurations). However, the NG suspend operation requires the gNB stores those NG configurations related to multiple AMFs. Considering the earth rotation, it may take a long time for the gNB connects to the same AMF again, which causes much storage consumption on the gNB for retaining multiple AMFs’ NG configurations, particularly when the gNB/Satellite should use this time to power up the battery.

Observation 2: NG suspend/resume operation requires the gNB to store NG configurations relevant to multiple AMFs, which leads to much storage consumption on the regenerative payload/gNB.

- No need to introduce the NG suspend/resume towards the AMF.

## 2.3 Feeder link switch

Last RAN3-125 meeting also agreed to only consider the no AMF change during the feeder link switch. In this case, the gNB IP/TNL address may be changed, which is not mandatory but would be helpful for routing via the new GW.

The solution #35 in SA2 TR 23.700-29 already listed some existing methods as follows [2].

---Excerpt from TR 23.700-29--- Scenario 1 (no AMF/MME change): In case of deployment scenario 1, the TNL between gNB/eNB and AMF/MME would change while the gNB/eNB is serving a specific set of cells and Tracking Areas. To update the TNL association between gNB/eNB and AMF/MME different options are possible based on the existing standard: - Support for multiple TNL associations is available in 5GS (i.e. N2), as described in TS 23.501 [2] and TS 38.413 [12]. In this case the gNB can initiate a new TNL association via the new feederlink and remove the old TNL association that was active over the old feederlink. - Support for replacing an old N2/S1 interface with a new N2/S1 interface while preserving the active UE contexts is supported for both N2 and S1, as described in TS 38.413 [12] and TS 36.413 [14]. In this case the gNB/eNB establishes a new SCTP association via the new feederlink and sends a N2/S1 SETUP Request including the UE Retention Information IE. The AMF/MME accepts to retain the existing UE related contexts and signalling connections. The AMF/MME sends a N2/S1 SETUP Response and includes the UE Retention Information IE. - To update the N3/S1-U GTP-U tunnels with the new gNB/eNB TNL address, the gNB/eNB can provide the new tunnel endpoint information (F-TEIDs) to the core network by sending PDU SESSION RESOURCE MODIFY INDICATION / E-RAB MODIFICATION INDICATION. ---End of excerpt---

And as observed from section 2.1, SA2 has concluded that gNB /eNB IP address change due to feeder link switch can be supported using existing N2 and S1 procedures.

For the NG interface, as specified in TS 38.412, multiple TNL association can be bound to one NG interface and it is easily using a new TNL association to replace the old one. Another legacy solution is establishing a new NG interface to the same AMF pool with the UE Retention Information IE, so that all UE context can be kept and reused by the new NG interface, which can be also extended to the S1 case.

With regards to SA2 LS, all these solutions are applicable to eNB/MME, when existing.

- RAN3 to confirm that existing procedures can be supported by the gNB/eNB in feeder link switch.

## 2.4	INACTIVE supporting

The paging and UE context relocation for INACTIVE UEs are relying on the Xn interface. Due to the satellite mobility when there is no Xn interface between the UE’s last serving gNB and the UE’s current camped gNB, the RAN paging would fail.

To solve this problem, companies proposed that the last serving gNB can proactively push the UE context to the next satellite (we call it candidate gNB in our following analysis) [3]. But in our view there exists the following problem for pushing the UE context:

When the INACTIVE UE resuming connection, the UE’s verification is executed by the candidate gNB via comparing the shortMAC-I reported by the UE’s RRCResumeRequest and included in the RETRIEVE UE CONTEXT REQUEST, and the shortMAC-I computed by itself. Specifically, the candidate gNB computes the shortMAC-I using the Krrcint as the key, and the source C-RNTI, source PCI, target cell ID as the input parameters. The key Krrcint should be the key of the gNB releasing the UE to INACTIVE, i.e., the last serving gNB, which is different from the candidate gNB. But normally the key of a gNB cannot be transferred to another gNB. Therefore, to make the context pushing solution feasible, the last serving gNB should infer all the possible target cells where the UE may resume the connection, and generate the possible shortMAC-I values using the Krrcint of itself and the possible target cell IDs, and send these together with the UE context to the candidate gNB, which is hard to be implemented and with heavy signalling overhead.

As an alternative, we think the INACTIVE UE’s last serving gNB can simply initialize the RAN paging for the UE, when the Xn interface is to be disconnected between the last serving gNB and a member in the gNBs relevant to the UE's RNA. In this way, the UE context can be guaranteed to be relocated to its current camped gNB, which is the UE’s current serving gNB after the paging, and the key is also refreshed. Though this solution is at the cost of the RAN paging, the UE can be kept to be inactive state, and the whole benefits of the “inactive feature” are hold when the serving gNB is changed.  It is proposed:

- If the enhancement of inactive UE is needed, the simple solution (no stage 3 spec impact) is that the UE's last serving gNB can send paging messages to candidate gNB(s) relevant to the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and the candidate gNB relevant to the UE's RNA.

## 2.5	Update of Stage 2 BLCR

The BLCR was endorsed in [4], with good first phase introduction of regenerative payload.

First it can be observed that the detailed regenerative payload and non-regenerative payload are description are not necessarily consistent, we suggest then to clarify when it applies to only one payload and nothing when it apply to both.

Second, there is an editor’s note to check the basic termination descriptions of the regenerative payload. It should be updated to describe that the regenerative payload terminates the Uu interface towards the UE (via the service link), and terminates the NG interface toward the 5GC (via the feeder link).

The Annex provides our suggested changes to the stage 2 BLCR.

- RAN3 to agree the TP on the stage 2 changes in the Annex.

## 2.7	Reply LS to SA2

The RAN3-124 meeting received the LS from SA2 containing the following, and requesting feedback. And the response from RAN3 can be found in [5]

- Overall Description: RAN3 thanks SA2 for their candidate mechanisms and assumptions on the Rel-19 study on 5G System enhancements for satellite access. RAN3 has discussed the issues and provides the corresponding feedback as below. With respect to NG connection management: RAN3 has just begun to discuss the options, and there is no conclusion yet. RAN3 will continue discussing this issue in Rel-19 and inform SA2 on the conclusion. With respect to IP address changes due to soft feeder link switch: RAN3 confirms that the gNB IP address change due to soft feeder link switch is supported by the existing NGAP procedures. With respect to Mapped Cell ID handling: RAN3 also thinks that the AMF/MME can treat the Mapped Cell ID as in previous releases.

With the new agreements made in RAN3, it is ready for RAN3 to reply to SA2 with the following new replies, so that SA2 can continue during their normative phase.  Note that the MBS related replies should refer to discussion in agenda 14.2

-	The NG removal procedure can be initiated when the gNB on board leaves the service area of an AMF.

-	 RAN3 agreed that Mapped Cell ID(s) or the new geographic area (its encoding is referring to RAN2 specification) could be used to describe the intended MBS service area for NTN.

And it is expected for RAN3 to reply those issues related to the S1 interface during the IoT NTN discussion.

- RAN3 to reply LS to SA2 containing agreements related to NR regenerative payload (the LS draft in Annex).

# 3	Conclusion

This contribution discusses the regenerative payload for the NR NTN. It is proposed:

Observation 1: SA2 SI concluded to not introduce new RAN/CN architecture nor requirements on the regenerative payload. SA2 will update their specification, if any enhancement from RAN3.

Observation 2: NG suspend/resume operation requires the gNB to store NG configurations relevant to multiple AMFs, which leads to much storage consumption on the regenerative payload/gNB.

- No need to introduce the NG suspend/resume towards the AMF.

- RAN3 to confirm that existing procedures can be supported by the gNB/eNB in feeder link switch.

- If the enhancement of inactive UE is needed, the simple solution (no stage 3 spec impact) is that the UE's last serving gNB can send paging messages to candidate gNB(s) relevant to the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and the candidate gNB relevant to the UE's RNA.

- RAN3 to agree the TP on the stage 2 changes in the Annex.

- RAN3 to reply LS to SA2 containing agreements related to NR regenerative payload (the LS draft in Annex).

# 4	References

- RAN3#125 Chair notes.

- TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19), SA2.

- R3-243355, (TP for TS 38.300 and TS 38.413) Discussion on the support of Regenerative payload, Nokia, Nokia Shanghai Bell, RAN3 124.

- R3-244856 Support for Regenerative Payload in NR NTN, Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics

- R3-243954 Reply LS on Support of Regenerative-based Satellite Access, RAN3R3-245361TS 38.300

# 5	Annex - TP for BLCR for TS 38.300 on top of R3-244856

## 16.14	Non-Terrestrial Networks

### 16.14.1	Overview

Figures 16.14.1-1 and 16.14.1-X below illustrate examples of a Non-Terrestrial Network (NTN) providing non-terrestrial NR access to the UE by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload. In Figure 16.14.1-1 the payload is transparent; in Figure 16.14.1-X the payload is regenerative and hosts a gNB.

Figure 16.14.1-1: Overall illustration of an NTN with transparent payload

Figure 16.14.1-X: Overall illustration of an NTN with regenerative payload hosting a gNB

NOTE 1:	Figures 16.14.1-1 and 16.14.1-X illustrate an NTN; RAN4 aspects are out of scope.

The NTN transparent payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The regenerative payload terminates the Uu interface towards the UE (via the service link), and terminates the NG interface toward the 5GC (via the feeder link).

The following connectivity is supported by the NTN payload:

-	An NTN gateway may serve multiple NTN payloads;

-	A NTN payload may be served by multiple NTN gateways;

NOTE 2:	In this release, the NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

The following connectivity is supported by the NTN regenerative payload:

-	A regenerative NTN payload may terminate one or more inter-satellite links toward other regenerative payloads;

-	The feeder link is a transport link; it may transport the NG interface between the 5GC and the gNB hosted by the regenerative payload;

-	The inter-satellite link is a transport link; it may transport the Xn interface between the gNBs hosted by the two regenerative payloads.

NOTE 2:	In this release, the transparent NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

For NTN, the following applies in addition to Network Identities as described in clause 8.2:

-	A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

-	A Mapped Cell ID as specified in clause 16.14.5.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the gNB can provide either quasi-Earth-fixed service link or Earth-moving service link, while gNB operating with GSO satellite can provide Earth fixed service link or quasi-Earth-fixed service link.

In this release, the UE supporting NTN is GNSS-capable.

In NTN, the distance refers to Euclidean distance.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 16.14.4	Switchover

#### 16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported.

#### 16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

# 6 Annex – Reply LS to SA2

3GPP TSG-RAN WG3 Meeting #125-bis	R3-XXXXXX

Hefei, CN, 14 - 18 Oct, 2024

# Title:	[DRAFT] Reply LS on support of regenerative payload in NTN

# Response to:	LS S2-2405600/ R3-243017 on Support of Regenerative-based Satellite Access

# Release:	Rel-19

# Work Item:	NR_NTN_Ph3-Core

Source:	Huawei [to be: RAN3]

To:	SA2

Cc:	RAN2

Contact Person:

Name:	Philippe Reininger

Tel. Number:

E-mail Address:	philippe.reininger@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	 -

1. Overall Description:

RAN3 would like to thank SA2 for their LS on regenerative payload considerations. On top of RAN3 reply LS in R3-243954, RAN3 would like to provide the latest agreements as follows:

- The NG removal procedure can be initiated when the gNB on board leaves the service area of an AMF.

- RAN3 agreed that Mapped Cell ID(s) or the new geographic area (its encoding is referring to RAN2 specification) could be used to describe the intended MBS service area for NTN for MBS broadcast service.

RAN3 would like to reply those issues related to the S1 interface during the IoT NTN discussion.

2. Actions:

To SA2:

ACTION: 	RAN3 asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#126	2024-11-18 – 2024-11-22		Orlando, US

RAN3#127	2025-02-17 - 2025-02-21		Athens, GR