3GPP TSG-RAN WG3 Meeting #125bis	R3-245386

Hefei, China, 14– 18 October, 2024

Agenda item:	8.1

Source: 	Huawei

Title: 	Discussion on SA2’s LS (S2-2409444) on multi-modality awareness

Document for:	Discussion and Decision

# 1. Introduction

SA2 has sent an LS to RAN3 on multi-modality awareness, requesting RAN3 to confirm the protocol impacts for transferring MMSID from CN to RAN. Besides, SA2 also want RAN2 to check the usage of multi-modality information.

In this contribution, we will analyse the potential NG-RAN impacts for transferring MMSID and using it at RAN.

# 2. Discussion

## 2.1 NG-RAN impacts for forwarding MMSID

In S2-2409444 [1], SA2 replied that it is technically feasible to provide MMSID to RAN. But before deciding whether to provide it, SA2 would like RAN3 to confirm the NG-RAN impacts.

SA2 specified policy control enhancements to support multi-modal services in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF, please see TS 23.501 clause 5.37.2 for details. Forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts.

In our opinion, we think there is no difficulty to support transmitting MMSID from CN to RAN. A possible way is to include MMSID in the QoS flow associated information during the PDU session setup and modification procedures. For example, MMSID can be added in the PDU Session Resource Setup Request Transfer IE and the PDU Session Resource Modify Request Transfer IE. A potential design for NG-RAN protocol is shown as following.

TS 38.413 [2]: #### 9.3.4.1 PDU Session Resource Setup Request Transfer This IE is transparent to the AMF. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. ----- Unchanged skipped -----. QoS Flow Setup Request List; 1; YES; reject. >QoS Flow Setup Request Item; 1..<maxnoofQoSFlows>; -. >>QoS Flow Identifier; M; 9.3.1.51; -. >>QoS Flow Level QoS Parameters; M; 9.3.1.12; -. >>E-RAB ID; O; 9.3.2.3; -. >>TSC Traffic Characteristics; O; 9.3.1.130; Traffic pattern information associated with the QFI. Details in TS 23.501 [9].; YES; ignore. >>Redundant QoS Flow Indicator; O; 9.3.1.134; This IE indicates whether this QoS flow is requested for the redundant transmission.; YES; ignore. >>ECN Marking or Congestion Information Reporting Request; O; 9.3.1.266; YES; ignore. >>Multi-Modal Service ID; O; FFS; Indicates this QoS flow is related to a multi-modal service.; YES; ignore. ----- Unchanged skipped -----. #### 9.3.4.3 PDU Session Resource Modify Request Transfer This IE is transparent to the AMF. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. ----- Unchanged skipped -----. QoS Flow Add or Modify Request List; 0..1; YES; reject. >QoS Flow Add or Modify Request Item; 1..<maxnoofQoSFlows>; -. >>QoS Flow Identifier; M; 9.3.1.51; -. >>QoS Flow Level QoS Parameters; O; 9.3.1.12; -. >>E-RAB ID; O; 9.3.2.3; -. >>TSC Traffic Characteristics; O; 9.3.1.130; Traffic pattern information associated with the QFI. Details in TS 23.501 [9].; YES; ignore. >>Redundant QoS Flow Indicator; O; 9.3.1.134; This IE indicates whether this QoS flow is requested for the redundant transmission.; YES; ignore. >>UL NG-U UP TNL Information; O; UP Transport Layer Information 9.3.2.2; This IE indicates the UPF endpoint of NG-U transport bearer for the QoS flow.; YES; ignore. >>Downlink TL Container; O; OCTET STRING; Containing the Set Request message specified in TS 29.585 [56].; YES; ignore. >>ECN Marking or Congestion Information Reporting Request; O; 9.3.1.266; YES; ignore. >>Multi-Modal Service ID; O; FFS; Indicates this QoS flow is related to a multi-modal service.; YES; ignore. ----- Unchanged skipped -----.

It can be seen that the specification impact for RAN3 to support forwarding MMSID from CN to RAN is small. It is easy for RAN3 to support such update.

Proposal 1: RAN3 reply to SA2 that it is feasible to forward MMSID from CN to RAN from NG-RAN protocol perspective.

## 2.2 Usage of MMSID at RAN

In S2-2409444, SA2 also ask RAN2 to clarify the usage of MMSID and ask RAN3 to confirm it.

Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work. Such feedback from RAN2 needs to also be assessed by RAN3 and SA4 to confirm the applicability and usage of this information.

Actually, we noticed that RAN2 has already discussed a lot about the applicability and usage. Several agreements were concluded in RAN2#127 meeting, as excerpted below.

- RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

It can be seen that MMSID is considered useful for joint admission control and QoS flow to DRB mapping. From RAN3 perspective, joint admission control and DRB mapping are performed based on the network implementation. There is no additional RAN3 impacts in addition to transfer MMSID.

Proposal 2: RAN3 reply to SA2 that the usages of MMSID concluded by RAN2, i.e. joint admission control and QoS flow to DRB mapping, can be confirmed from RAN3 perspective.

Based on the above analysis and proposals, the draft reply LS is provided in [4].

# 3. Conclusion

In this contribution, we analyzed the question asked by SA2 in the incoming LSs. The following proposals were made:

Proposal 1: RAN3 reply to SA2 that it is feasible to forward MMSID from CN to RAN from NG-RAN protocol perspective.

Proposal 2: RAN3 reply to SA2 that the usages of MMSID concluded by RAN2, i.e. joint admission control and QoS flow to DRB mapping, can be confirmed from RAN3 perspective.

# 4. Reference

[1]. S2-2409444, TSG SA WG2, LS reply on multi-modality awareness at RAN.

[2]. 3GPP TS 38.413: "NG-RAN, NG Application Protocol (NGAP)".

[3]. Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands.

[4]. R3-245387, [draft]Reply LS on multi-modality awareness at RAN.