3GPP TSG-RAN WG3 Meeting #125bis	R3-245391

Hefei, China, 14th-18th, October, 2024

Agenda Item:	12.2

Source:	Huawei

Title:	(TP for TS 38.300/38.413) Architecture and Access control for WAB

Document for:	Discussion

1	Introduction

RAN3 has ended the study phase for the WAB, and made the conclusion in the TR 38.799 [1]. The new WID for WAB is approved in RP-242359 [2], which includes the following objectives for Rel-19 WAB:

The objectives of the Wireless Access Backhaul (WAB) work are as follows:

-	Specifications for the support of WAB including [RAN3]:

-	Support of a WAB-node including a WAB-gNB and a WAB-MT.

-	Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s).

-	Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs.

-	Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes.

-	Network integration procedures for WAB nodes.

-	Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes.

-	Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB.

-	UE’s ULI that reflect the WAB node’s location.

-	The handling of:

-	PCI collision avoidance.

-	Reconfiguration of TAC and RANAC on WAB-gNBs.

-	Mechanisms to avoid multi-hop WAB topology.

-	Radio-resource coordination between access and backhaul links.

-	NG connection management.

NOTE 1: For PCI collision avoidance and reconfiguration of TAC and RANAC on WAB-gNBs, follow the conclusion of mobile IAB.

NOTE 2: NG connection management should take the NTN conclusion into account, avoiding parallel discussions.

NOTE 3: No impact on the UE.

NOTE 4: Coordination with other WGs (e.g. SA2, RAN2) when needed.

NOTE 5: Backhaul link for WAB-MT can be TN or NTN.

NOTE 6: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures.

NOTE 7: The interface between the WAB-MT and the co-located WAB-gNB is out-of-scope for the normative phase.

NOTE 8: Split architecture of the WAB-gNB is out-of-scope for the normative phase.

NOTE 9: RAN2 impact should be identified as early as possible, and should be minimal.

In this paper, we further analyse the scenario, architecture, access control and the user location information report regarding WAB.

2	Discussion

2.1	Scenario and architecture of WAB

2.1.1	General architecture and protocol stack

The NG and Xn interface traffic of the WAB-gNB is transmitted via the backhaul PDU session(s). Specifically, the NG/Xn interface data is encapsulated in the WAB-MT's PDU session and sent through the BH-RAN node to the BH-UPF, and then the BH-UPF sends the packets via N6 interface, i.e., the IP routing, to its destination. The architecture and protocol stack shown in current TR 38.799 [1] should be also captured into TS 38.300 [3] during the normative phase.

Proposal 1: RAN3 to capture the architecture and protocol stack of NG & Xn support for WAB-gNB in TS 38.300.

2.1.2	Architecture: Management of Xn interface for WAB

The WID objective includes “Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs”. So, there are two issues to be discussed.

2.1.2.1 How to manage Xn interface efficiently for WAB-gNB

The mobility of the WAB-node results in dynamic changes to the Xn interface of the WAB-gNB. The WAB-gNB may need to establish new Xn interface to the newly appeared neighbour gNBs frequently. One issue here is how the WAB-gNB acquires the TNL address of the neighbour gNBs. In legacy scenario, the TNL address of the neighbour gNBs is either configured by OAM or obtained via the Xn-C TNL address discovery procedure. The frequent updates to the Xn interface results in large overhead for these interactions, and delay of such interactions will be another challenging factor especially considering all the transmission should be via MT’s PDU session.

So, we need to think about a more efficient way for the WAB-gNB to establish Xn interface towards neighboring gNBs (including BH-gNB). As an alternative, the BH-gNB can provide assistant information about its own neighboring gNBs (e.g., including its neighbour gNBs' TNL information) to the WAB-node.

Proposal 2: The BH-gNB can provides the TNL information of neighbour gNBs to the WAB node.

2.1.2.2 How to avoid setting up Xn between WAB-gNBs

Another issue is how to avoid establishing Xn between two WAB-nodes. The essential thing is the WAB-node should be aware of the node type of another WAB-gNB, for either to determine not initializing Xn setup towards a WAB-gNB, or to reject the Xn setup in case the Xn setup request is received from a WAB-gNB. It is proposed:

Proposal 3: To avoid establishing Xn between two WAB-nodes, the WAB-node should be aware of the node type of another WAB-gNB.

2.1.3	Scenario: in-band or out-band deployment

During SI phase, both in-band deployment and out-band deployment were considered for WAB.  For in-band scenario, to avoid interference between access link and backhaul link, the resources (e.g., frequency domain, time domain, spatial domain, etc.) to be used by access link and backhaul link must be coordinated. For example, the BH-RAN node should be upgraded to be aware of which WAB-gNB is co-located with a WAB-MT, it also needs to know the multiplexing capability of the WAB-MT and its co-located WAB-gNB, as well as the “Hard” resource configuration of the WAB-gNB. Such resource coordination needs to be performed very frequently between the WAB-node and the current BH-RAN node, considering the WAB-node is moving and the BH-RAN node is changing.  In addition, if the resource configuration of WAB-node changes, the resource coordination needs to be performed again.

For out-band scenario, the access link and backhaul link will use different frequency band, and there is no interference between the two links, so no coordination is needed between access and backhaul link, the operation will become very simple compared to the in-band mode.

Therefore, for the wide deployement of WAB node, out-band should be prioritized. And if the in-band mode occurs at a time, e.g., due to WAB mobility the WAB-MT switch to a serving cell with the same frequency as the collocated WAB-gNB, the BH-RAN node should be an upgraded gNB to be aware of the WAB and can support the WAB resource multiplexing. It is proposed:

- Proposal 4a: For WAB deployment, out-band should be prioritized.

- Proposal 4b: To support the in-band WAB deployment, the BH-RAN must be upgraded to aware of WAB and can support the resource multiplexing.

Based on the analysis above, the out-band mode and in-band mode bring different requirements for the BH-RAN, thus it is meaningful to discuss how to determine the operational mode of the WAB node. According to TR 38.799[3], after the WAB-MT setup, during the WAB-gNB initialization phase OAM provides the WAB-gNB the initial configuration including the operating frequency. On this stage the OAM may determine the in-band/out-band mode and configure the WAB-gNB frequency accordingly. Alternatively, the WAB node may determine the in-band/out-band mode based on the actual frequency band of the WAB-MT’s serving cell and the configured frequency band of WAB-gNB.

- Proposal 5: RAN3 to discuss how to determine the inband/outband mode of WAB-node.

2.1.4	Scenario: NTN backhaul

According to the NOTE 5 in the WID, NTN backhaul is considered between the WAB-MT and the BH-gNB. According to TS 23.501 [3] the UE’s 5QI is set to a fixed value (5QI=10) as long as the UE is served by NTN, with the PDB directly set to a really large value (1100ms).

Fig 1. Special QoS parameters for NTN UE in TS 23.501

In the legacy NTN scenario, according to the TS 38.413[4], the gNB will provide the RAT Information which indicates the NR satellite RAT Type to AMF during the NG Setup procedure or the RAN Configuration Update procedure as follows.

-------------Excerpted from TS 38.413-------------

9.3.1.125	RAT Information

This IE provides RAT related information associated with a TAC, used as described in TS 23.501 [5].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAT Information | M |  | ENUMERATED (unlicensed, nb-IoT, ..., nR-LEO, nR-MEO, nR-GEO, nR-OTHERSAT) |  |

In the WAB scenario, if the backhaul link uses NTN, the UE’s CN is naturally unaware of that according to the current RAT information report as shown above, because the WAB-gNB itself is not an NTN gNB. But if the WAB-MT adopts NTN backhaul, the served UE’s QoS parameters cannot be normally configured but should be configured considering the bottleneck of NTN backhaul. The UE’s CN needs to know the backhaul is via NTN and configures the NTN related QoS parameters for UE’s QoS flow. Therefore, it is proposed:

Proposal 6:	If the backhaul is NTN link, the UE’s CN should know the BH link type is NTN.

2.2	Access control of WAB

2.2.1	Initial access

1) Prevent multi-hop access

The Objective of the WID includes the handling of mechanisms to avoid multi-hop WAB topology. In addition, in the previous LS S2-2407345 from SA2, they also ask RAN3 to discuss the access control regarding how to prevent the multi-hop access and how to prevent the self-access for the WAB-MT. The WAB-MT should be able to acquire whether a cell is allowed for the WAB-MT to access. We can consider the following three options:

- Option 1: Broadcasting “WAB-barred”. The WAB-node consider the cell is barred if the SIB1 in this cell contains “WAB-barred” indication, otherwise, the WAB-node can consider this cell is accessable. With such option, the WAB-gNB should broadcast the “WAB-barred” indication to prevent other WAB-MT from accessing it. Option 1 allows the WAB-MT to access the legacy gNBs. It is worth noting that some companies also proposed a solution which requires the WAB-gNB to broadcast “WAB cell” indication in previous meeting. We think such solution can be classified as one special design of this option 1, i.e., the “WAB cell” indication works as a bar indication for another WAB-MT. But the “WAB-barred” indication is more general then the “WAB cell” indication, becasue not only the WAB-gNB but also the normal gNB (e.g., a gNB which do not allow WAB-node access) can broadcast the “WAB-barred” indication.

- Option 2: Broadcasting “WAB-support”. Similar as the access control for IAB-MT, the gNB which is capable for the WAB-MT accessing indicates a “WAB-support” IE in SIB1, and the WAB-gNB does not broadcast such indicator since a WAB-gNB cannot serve any WAB-MTs. The WAB-MT only accesses the cell broadcasting the “WAB-support” indicator. In option 2 the WAB-MT cannot access a legacy gNB but only access the upgraded BH-gNBs.

- Option 3: CN-based solution. WAB-gNB provides WAB-indication to AMF (proposed by SA2’s TR 23.700-06, section 6.2.3.6 [6]), and WAB-MT’s AMF reject a WAB-MT’s registration request if the WAB-MT accesses a cell served by WAB-gNB. Option 3 allows WAB-MT access legacy cells by default, but if it accesses a WAB-gNB cell, it takes long time for the situation awareness and reject. Moreover, after the registration failure, the WAB-MT may still choose another WAB-gNB to access because such solution can not avoid wrong choice from the WAB-MT.

It is observed:

Observation 1: The solution which requires the WAB-gNB to broadcast “WAB cell” indication is actually a one special design of the option 1 (i.e., the Broadcasting “WAB-barred” based solution).

Observation 2: Both Option 1 and option 2 can prevent the WAB-MT from accessing a WAB-gNB at early stage.

Observation 3: Option 1 allows the WAB-node to access a legacy gNB by default, while option 2 requires the BH-gNB to be upgraded to support WAB.

Observation 4: For in-band WAB deployment, only option 2 can ensure that the WAB-MT selects a proper BH-gNB.

Observation 5: Using option 3, the WAB-MT will access another WAB-node successfully but then the registration procedure will be rejected. Such solution works at very late stage and cannot prevent the WAB-MT to access another WAB-gNB again.

Based on the introduction and comparison, we can see that option 1 and option 2 are better than option 3, it is proposed:

Proposal 7: For multi-hop avoidance, RAN3 to discuss the following options:

- Option 1: Broadcasting “WAB-barred” solution. WAB-node is barred to a cell if the SIB1 in this cell contains “WAB-barred” indication. A WAB-gNB should broadcast “WAB-barred” in SIB1.

- Option 2: Broadcasting “WAB-support” solution. WAB-node only access the cell which broadcasts “WAB-support” in SIB1. A WAB-gNB will not broadcast “WAB-support”.

2) Prevent self-access

- The second case of accessing restriction listed in the SA2 LS [7] is to avoid WAB-MT access the co-located WAB-gNB.

- First, since the WAB-gNB and WAB-MT should be coordinated for the transmission and receiving operation (e.g., TDM, FDM, SDM, etc), it is not possible for the WAB-MT to consider the cell served by its co-located WAB-gNB as the suitable cell when performing cell selection even if full-duplex is supported since some isolation/interference cancellation needs to be conducted. Otherwise, the transmitting of WAB-gNB and WAB-MT will cause severe interference to each other.

- Second, even if the WAB-MT incidently detects the signal of its co-located WAB-gNB, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation, e.g., the implementation can set restriction that a WAB-MT never access the cell configured to this WAB-node by OAM. On the other hand, the RAN-based broadcasting “WAB-support/WAB-barred” can also avoid such issue.

- Observation 6: With proper multiplexing coordination/isolation, the WAB-MT will not detect signal of cell served by its co-located WAB-gNB. Even if the WAB-MT incidently detects the signalling, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation.

Proposal 8: RAN3 to agree that “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is not an issue to be solved, because this can be avoided by proper setting/implementation.

2.2.2	Access control during Handover

During WAB-MT handover, the WAB-MT should not switch to a target WAB cell. The following two options can be considered:

- Option 1: via explicit indication. The source BH-RAN includes the WAB-node indication in the HO request, then the target BH-RAN node can perform access control for this WAB-node. The WAB-node indication should be introduced in both Xn HO REQUEST and NG HO REQUEST. If the WAB can has some specific slice ID, it is also possible to use the S-NSSAI as an indication of WAB-node during handover.

- Option 2: via UE capability. If the target gNB is a WAB-gNB, the target WAB-gNB reject the WAB-node when receiving the UE capability information in the HO REQUEST for this WAB-MT, which contains information to indicate a UE is a WAB-MT. This solution is not feasible because the UE capability cannot be used for admission control during HO procedure. For example, a R18 UE can access R17 gNB even if some R18 UE capability cannot be supported by the legacy gNB.  In addition, this solution requires RAN2 to define WAB-specific UE capability, but it is unclear whether the WAB-MT will have some WAB-specific UE capability. Moreover, if a WAB-MT can work as a normal UE but not a WAB-node, it should be allowed to access another WAB-gNB, but such solution will result in that the “normal UE” being rejected by the target WAB-gNB.

Based on the above analysis, option 2 may bring some implementation requirements for reducing its advantage. It is proposed:

Proposal 9: In case of handover for a WAB-node, the WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node. RAN3 to discuss whether to introduce a new explicit indicator or use a special S-NSSAI value as the indicator.

2.3	User location information

The WID objective also includes “UE’s ULI that reflect the WAB node’s location”, regarding the issue 2 in SA2’s LS S2-2407345. The issue 2 was stated in the LS as “SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06”.

9.3.1.260	Mobile IAB-MT User Location Information

This IE contains the NR CGI and the TAI for the co-located mobile IAB-MT of the mobile IAB-DU which serves the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.7 | The NR CGI of the cell, which is the serving cell of the mobile IAB-MT co-located with the mobile IAB-DU that serves the UE. |
| TAI | M |  | 9.3.3.11 | The TAI supported by the cell, which is the serving cell of the mobile IAB-MT co-located with the mobile IAB-DU which serves the UE. |

The SA2’s conclusion is clear that the additional ULI to be provided to the UE’s AMF is the TAC/Cell ID of the cell serving the MWAB-UE, if the WAB-MT and WAB-gNB belongs to same PLMN, then it is same as the additional ULI for mobile IAB, which is excerpted from TS 38.413 as below.

--------------------Excerpted from TR 23700-60-j00------------------------

8.5	KI#5: Support of location services for UEs when MWAB(s) is involved Conclusion

The following aspects are taken as way forward during normative work:

-	Existing principles (e.g. the two options for LMF to retrieve the MWAB location either via NRPPa procedure or via GMLC) are re-used for UE location service when MWAB is involved as described in sol#11.

The MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is:

-	TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN.

-	If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN, which below option used for the Additional ULI will be determined in normative phase.

-	option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM.

-	option 2: geo-location of the MWAB.

-	option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN.

-	The AMF may provide the received Additional ULI to LMF in LCS request.

NOTE:	If option 2 is selected in normative phase, support of passing Additional ULI to LMF needs to be reviewed.

For the inter-PLMN case, i.e., the WAB-MT and WAB-gNB belongs to different PLMN, there are three options provided by SA2’s conclusion. The contents from SA2’s TR are copied as below.

- Among the three options, the signalling impact to NGAP looks same for option 1 and option 3. However, to support option 1, OAM should provide the mapping relationship between geo-location and the TAC/cell ID to the WAB-gNB, whether and how can OAM support such functionality needs to check with SA5. Moreover, in some location, there maybe no coverage for the UE’s PLMN, then it is unclear how can the “mapped TAC/cell ID” being provided.

- In option 2, the geo-location of the WAB will be transmitted in NG interface. This is technical feasible, but the stage-3 signaling design will be different from legacy location information used in NG interface (i.e., TAI and cell ID are normally used to indicate UE’s location information in NG interface), then the design for intra-PLMN and inter-PLMN scenario will be different.

- For option 3, the signalling design is same as the “Mobile IAB-MT User Location Information”, and how can the AMF use the TAC/cell ID of the WAB-MT’s serving cell should be up to CN’s implementation. Using such option, the impact to NG interface will be same as the same PLMN scenario.

- Observation 7: For additional ULI provision of inter-PLMN scenario of WAB:

- Option 1 requires additional functionality of OAM and this solution will be useless in the area without any coverage of WAB-gNB’s PLMN.

- Option 2 is technical feasible but the signalling design for location information is different from legacy and intra-PLMN case.

- Option 3 is same as the legacy design, then the intra-PLMN and inter-PLMN can have unified solution.

- Based on the above analysis, we think the option 3 is the best solution and it can ensure a unified solution for both inter-PLMN case and intra-PLMN case. It is proposed:

- Proposal 10: RAN3 to provide unified signalling design for additional ULI report in both intra-PLMN case and inter-PLMN case, then option 3 is recommended for the inter-PLMN case.

- After RAN3 has clear conclusion on the solutions, RAN3 should send reply LS to SA2 to provide the answer for the access control issue and ULI issue. A draft reply LS is provided in Annex C based on the analysis in this paper.

3	Conclusion

In this contribution we discuss the architecture and access control issues in the WAB scenario.

Scenario and architecture

Proposal 1：RAN3 to capture the architecture and protocol stack of NG & Xn support for WAB-gNB in TS 38.300.

Proposal 2: The BH-gNB can provides the TNL information of neighbour gNBs to the WAB node.

Proposal 3: To avoid establishing Xn between two WAB-nodes, the WAB-node should be aware of the node type of another WAB-gNB.

Proposal 4a: For WAB deployment, out-band should be prioritized.

Proposal 4b: To support the in-band WAB deployment, the BH-RAN must be upgraded to aware of WAB and can support the resource multiplexing.

Proposal 5: RAN3 to discuss how to determine the inband/outband mode of WAB-node.

Proposal 6: If the backhaul is NTN link, the UE’s CN should know the BH link type is NTN.

Access control

1) Prevent multi-hop access

Observation 1: The solution which requires the WAB-gNB to broadcast “WAB cell” indication is actually a one special design of the option 1 (i.e., the Broadcasting “WAB-barred” based solution).

Observation 2: Both Option 1 and option 2 can prevent the WAB-MT from accessing a WAB-gNB at early stage.

Observation 3: Option 1 allows the WAB-node to access a legacy gNB by default, while option 2 requires the BH-gNB to be upgraded to support WAB.

Observation 4: For in-band WAB deployment, only option 2 can ensure that the WAB-MT selects a proper BH-gNB.

Observation 5: Using option 3, the WAB-MT will access another WAB-node successfully but then the registration procedure will be rejected. Such solution works at very late stage and cannot prevent the WAB-MT to access another WAB-gNB again.

Proposal 7: For multi-hop avoidance, RAN3 to discuss the following options:

- Option 1: Broadcasting “WAB-barred” solution. WAB-node is barred to a cell if the SIB1 in this cell contains “WAB-barred” indication. A WAB-gNB should broadcast “WAB-barred” in SIB1.

- Option 2: Broadcasting “WAB-support” solution. WAB-node only access the cell which broadcasts “WAB-support” in SIB1. A WAB-gNB will not broadcast “WAB-support”.

2) prevent self-access

Observation 6: With proper multiplexing coordination/isolation, the WAB-MT will not detect signal of cell served by its co-located WAB-gNB. Even if the WAB-MT incidently detects the signalling, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation.

Proposal 8: RAN3 to agree that “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is not an issue to be solved, because this can be avoided by proper setting/implementation.

Proposal 9: In case of handover for a WAB-node, the WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node. RAN3 to discuss whether to introduce a new explicit indicator or use a special S-NSSAI value as the indicator.

User location information

- Observation 7: For additional ULI provision of inter-PLMN scenario of WAB:

- Option 1 requires additional functionality of OAM and this solution will be useless in the area without any coverage of WAB-gNB’s PLMN.

- Option 2 is technical feasible but the signalling design for location information is different from legacy and intra-PLMN case.

- Option 3 is same as the legacy design, then the intra-PLMN and inter-PLMN can have unified solution.

Proposal 10: RAN3 to provide unified signalling design for additional ULI report in both intra-PLMN case and inter-PLMN case, then option 3 is recommended for the inter-PLMN case.

4	References

- TR 38.799, Study on additional topological enhancements for NR, V19.0.0

- RP-242359, New WID on additional topological enhancements for NR

- TS 38.300, NR and NG-RAN Overall Description, Stage 2, V18.2.0

- TS 38.413, NG Application Protocol (NGAP), Stage2, V18.3.0

- TS 23.501, System architecture for the 5G System (5GS), Stage 2, V18.5.0

- TR 23.700-06, Study on architecture enhancements for vehicle-mounted relays - Phase 2, V1.0.0

- S2-2407345, LS on questions regarding FS_VMR_Ph2, SA2

Annex A:	TP for TS 38.300

Start of Change

4.X	Wireless Access and Backhaul

4.X.1	WAB Architecture

Figure 4.X.1-1 shows an example of WAB architecture for 5GS when the WAB-gNB’s NG traffic is transported via PDU session backhaul. The neighbour NG-RAN node can be a BH-RAN-node, or a surrounding NG-RAN node. The WAB-gNB’s OAM traffic can also be transferred over the BH PDU session(s).

Figure 4.X.1-1: The WAB architecture example for 5GS when the WAB-gNB traffic is transported via PDU session backhaul

4.X.2	Protocol Stacks

Figure 4.X.2-1 shows protocol stack examples of NG Control plane and User plane transport for a UE connected to the network via a WAB-node.

Figure 4.X.2-1: Protocol stack examples of NG Control plane and User plane transport for a UE connected via WAB-node

Figure 4.X.2-2 shows protocol stack examples of Xn Control plane and User plane transport for WAB-node.

Figure 4.X.2-2: Protocol stack examples of Xn Control plane and User plane transport

End of Change

Annex B:	TP for TS 38.413

Start of Change

9.3.1.16	User Location Information

This IE is used to provide location information of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  | - |  |
| >E-UTRA user location information |  |  |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >NR user location information |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 | This IE is ignored if the NR NTN TAI Information IE is present. | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >>NID | O |  | 9.3.3.42 |  | YES | reject |
| >>NR NTN TAI Information | O |  | 9.3.3.53 |  | YES | ignore |
| >>Mobile IAB-MT User Location Information | O |  | 9.3.1.260 | Indicates the user location information of a mobile IAB-MT, which is co-located with the mobile IAB-DU which serves the UE. | YES | ignore |
| >>WAB-MT User Location Information | O |  | 9.3.1.X | Indicates the user location information of a WAB-MT, which is co-located with the WAB-gNB which serves the UE. | YES | ignore |
| >N3IWF user location information |  |  |  |  |  |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TNGF user location information |  |  |  |  | YES | ignore |
| >>TNAP ID | M |  | OCTET STRING | TNAP Identifier used to identify the TNAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the TNGF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TWIF user location information |  |  |  |  | YES | ignore |
| >>TWAP ID | M |  | OCTET STRING | TWAP Identifier used to identify the TWAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | Non-5G-Capable over WLAN device's local IP address used to reach the TWIF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >W-AGF user location information |  |  |  | Indicates the location information via wireline access as specified in TS 23.316 [34]. | YES | ignore |
| >>W-AGF user location information | M |  | 9.3.1.164 |  | - |  |

Next Change

9.3.1.X	WAB-MT User Location Information

This IE contains the NR CGI and the TAI for the co-located WAB-MT of the WAB-gNB which serves the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.7 | The NR CGI of the cell, which is the serving cell of the WAB-MT co-located with the WAB-gNB that serves the UE. |
| TAI | M |  | 9.3.3.11 | The TAI supported by the cell, which is the serving cell of the WAB-MT co-located with the WAB-gNB which serves the UE. |

End of Change

Annex C:	Reply LS to SA2

3GPP TSG-RAN3 Meeting #125bis	R3-xxxxxx

Hefei, China, 14-18 October, 2024

Title:	[DRAFT] Reply LS on access restriction and additional ULI of WAB

Response to:	S2-2407345/ R3-244019

Release:	Rel-19

Study Item:	FS_VMR_Ph2

Source:	Huawei [to be: RAN3]

To:	SA2

Cc:	RAN2

Contact Person:

Name:	Yuanping Zhu

Tel. Number:

E-mail Address:	zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 -

1. Overall Description:

RAN3 would like to thank SA2 for their LS on the RAN impact considerations for WAB, and provide the following replies to SA2’s question:

- Issue 1:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode (documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

RAN3’s Answer:

RAN3 think broadcasting “WAB-support/WAB-barred” is beneficial for WAB-MT to select proper BH-gNB, and can prevent WAB-MT from accessing a WAB-gNB at early stage.

RAN3 do not think the “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is an issue to be solved, because this can be avoided by proper seting/implementation.

- Issue-2:

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

RAN3’s Answer:

RAN3 will provide unified NGAP signaling design which contains TAC/cell ID of the WAB-MT’s serving cell for the additional ULI report in both intra-PLMN case and inter-PLMN case.

2. Actions:

To SA2:

ACTION: 	RAN3 asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#126	2024-11-18 – 2024-11-22		Orlando, US

RAN3#127	2025-02-17 - 2025-02-21		Athens, GR