3GPP TSG-RAN WG3 Meeting #125bis	R3-245393

Hefei, China, 14 - 18 October, 2024

Agenda Item:	12.3

Source:	Huawei

Title:	(TP for TS 38.300) Discussion on the architecture for NR Femto

Document for:	Discussion

1	Introduction

RAN3 has ended the study phase for the NR Femto, and made the conclusion in the TR 38.799 [1]. The new WID for topology enhancement including the NR Femto is approved in RAN #105 [2].  The Objective about NR femto parts are shown as follows.

The objectives of the 5G Femto work are as follows:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

This contribution mainly focuses on the architecture of NR Femto and gives the corresponding TP for TS 38.300.

2	Discussion

2.1 Architecture of NR femto.

As included in the objective, the NR femto architecture with optional NR femto GW for NG interface will be specified. Therefore, the architecture for the NG interface which is the option 2 in TR 38.799 should be captured in the stage 2 specification during the normative phase.

Figure 1: Logical architecture for the NR femto with NR femto GW

While for the Xn interface, the NG Femto noed will maintain its Xn interface directly to neighboring WAB, same as the legacy gNB.

RAN3 to agree the TP for TS 38.300 to capture the architecture description for NR Femto.

2.2 gNB ID for NR femto node.

During the study phase, we proposed that one NR Femto serves only one NR cell [3], but that was not agreed. The NR Femto can serve one or multiple cells as the normal gNB, which is different from the LTE HeNB. In TS 36.300, it is captured “one HeNB serves only one cell.” And we also observed that in TS 36.413, the HeNB ID is represented as the ECGI of the cell served by the HeNB.

---Start of excerpt from TS 36.413---

#### 9.2.1.37	Global eNB ID

This information element is used to globally identify an eNB (see TS 36.401 [2]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.3.8 |  |
| CHOICE eNB ID | M |  |  |  |
| >Macro eNB ID |  |  |  |  |
| >>Macro eNB ID | M |  | BIT STRING (SIZE(20)) | Equal to the 20 leftmost bits of the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of each cell served by the eNB. |
| >Home eNB ID |  |  |  |  |
| >>Home eNB ID | M |  | BIT STRING (SIZE(28)) | Equal to the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of the cell served by the eNB. |
| >Short Macro eNB ID |  |  |  |  |
| >> Short Macro eNB ID | M |  | BIT STRING (SIZE(18)) | Equal to the 18 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB. |
| >Long Macro eNB ID |  |  |  |  |
| >> Long Macro eNB ID | M |  | BIT STRING (SIZE(21)) | Equal to the 21 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB. |

---End of excerpt from TS 36.413---

---Start of excerpt from TS 38.413---

9.3.1.6	Global gNB ID

This IE is used to globally identify a gNB (see TS 38.300 [8]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.3.5 |  |
| CHOICE gNB ID | M |  |  |  |
| >gNB ID |  |  |  |  |
| >>gNB ID | M |  | BIT STRING (SIZE(22..32)) | Equal to the leftmost bits of the NR Cell Identity IE contained in the NR CGI IE of each cell served by the gNB. |

---End of excerpt from TS 38.413---

- One HeNB serves only one cell, and the HeNB ID is the ECGI of the cell served by the HeNB.

Now, the NR Femto node may serve more than one cells. It is unreasonable to still use the NCGI of a served cell as the Femto ID. And for stage 3, we need to discuss how to identify the NR femto node. A straightforward solution is just to reuse the global gNB ID for NR Femto node, considering the length of the Global gNB ID is 22-32bits , there may be a risk/concern that the range cannot support the large number of NR femto nodes. Alternatively, if the scalability is a valid issue, dedicated NR Femto node ID may be also considered.

It is proposed:

RAN3 to investigate how to design the gNB ID for NR femto node, i.e., whether to introduce dedicated NR Femto Node ID, or reuse the Global gNB ID.

3	Conclusion

This paper focus on the architecture for the NR femto, and provide the following observation and proposals:

- One HeNB serves only one cell, and the HeNB ID is the ECGI of the cell served by the HeNB.

- RAN3 to agree the TP for TS 38.300 to capture the architecture description for NR Femto.

RAN3 to investigate how to design the ID for NR femto node, i.e., whether to introduce dedicated NR Femto Node ID, or reuse the Global gNB ID.

4. Reference

- 3GPP TR 38.799 V19.0.0, Study on additional topological enhancements for NR.

- RP-242395, New WID on additional topological enhancements for NR, RAN #105.

- R3-244529, Discussion on the architecture for NR Femto, Huawei, RAN3 #125.

Annex——TP for TS 38.300

Start of Change

4	Overall Architecture and Functional Split

-------------skip the unchanged parts-----------

4.X	Support of NR Femtos

4.X.1	Architecture

Figure 4.X.1-2 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto node to the 5GC.

Figure 4.X.1-2: Option 2 for NR Femto Architecture

The NG-RAN architecture may deploy an NR Femto Gateway (NR Femto GW) to allow the NG interface between the NR Femto node and the 5GC to support a large number of NR Femto nodes in a scalable manner. The NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface.

The NG interface is defined as the interface:

-	Between the NR Femto GW and the 5GC;

-	Between the NR Femto node and the NR Femto GW;

-	Between the NR Femto node and the 5GC;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same regardless of whether the NR Femto node is connected to the 5GC via an NR Femto GW or not.

- It is also possible that the NR Femto node connects to the 5GC directly as a gNB by means of the NG interface.

- The NR Femto Node also support Xn-connectivity to the other NR Femto Node(s)/gNB(s) directly.

End of Change