3GPP TSG-RAN WG3 Meeting #125-bis	R3-245396

# Hefei, China, 14th – 18rd October, 2024

Title: 	Discussion on SON enhancements for network slicing

Source: 	Ericsson, AT&T, Jio

Agenda item:	10.3.2

Document Type:	Discussion and decision

# 1	Introduction

The release 19 work item for Data Collection for SON/MDT in NR standalone and MR-DC focuses on enhancements for Rel-17/Rel-18 SON/MDT features such as MRO enhancement for LTM, CHO with candidate SCGs and subsequent CPAC, as well as on SON/MDT enhancements for Intra NTN mobility and Network Slicing [1].

In this paper, we discuss the topics of SON enhancements in the context of Network Slicing and slice-aware MRO. The focus in this discussion paper is on enhancing network slicing observability for SON/MDT functions/processes.

# 2	Discussion

During RAN3#123-bis, and RAN3#124 the following were the outcomes in the context of SON enhancements for Network Slicing.

- Work on the Slice aware MRO scenarios.

- For successful handover, FFS if solutions are needed to optimize handovers in the case of slicing discontinuity, e.g., target cell not supporting slice.

In RAN3#125 no progress was made in the context of SON enhancements for Network Slicing except for some offline discussions.

In our previous contribution to RAN3#125 (R3-244435 [2]) and to the prior Rel-19 discussions on SON enhancements for network slicing, we have presented discussions on SHR enhancements for network slicing, where SHR reports are enhanced with user plane interruption times.

Currently is no inclusion of slice performance related metrics in SON reports. This shortfall means that the RAN does not have timely visibility into issues arising due to slice performance during handover and is unable to react to issues that may be solvable if the RAN had indicators for such issues. As NR stand-alone deployment increases with more-and-more NR-SA features, such as AR, VR, XR, being deployed in the field, the offered performance on these services is critical for continued adoption and scaling of NR. Such services and the allocated slices may be governed by an SLA, and/or a contract, where certain targets may be specified. Enhancing observability in these situations is vital to first understand if an issue exists, or a violation of an SLA/contract is ongoing or has already occurred.

2.1	Proposals from other companies

Nokia in their contribution R3-244067 [3] to RAN3#125 propose to enhance SHR with a new trigger and a cause value (slice discontinuity after a handover event), with the intention to understand from the SHR if a slice discontinuity event was due to UE mobility or due to a coverage or slice deployment issue.

China Telecom in their contribution R3-244257 [4] propose a similar enhancement to SHR and RLF reports with network slice related information, which may then be used to identify issues leading to slice discontinuity or slice remapping events.

2.2	Our view in relation to the above proposals

We consider that both Nokia’s and China Telecom’s proposals are closely related to our proposals. Enhancements to the SHR report can certainly aid the RAN in identifying and reacting to slice-related issues both in terms of deploying a slice in the right cell, i.e., identifying issues due to wrong deployments, and in terms of understanding the performance offered to the UE on a slice during handovers. A legacy SHR report is triggered using thresholds placed on timers T304, T310, T312, or DAPS failure, and the intention with SHR is to capture near handover failure scenarios, analyse the root cause, and take remediating actions based on findings.

With regard to adding slice discontinuity after handover as a trigger for SHR reporting, this condition is not fully aligned with legacy SHR triggers, i.e., the intention with legacy SHR triggers is to identify near failure cases, and using slice discontinuity as a trigger does not fully abide by this principles. Furthermore, adding information about the slice support at the serving and target cells in the SHR report may be considered redundant, as this information may already be known/available and is largely static.

However, we agree that enhancements to SHR with slice related information can be valuable in understanding and debugging slice related issues during handover. The discussion in this topic, however, is two-fold, and are listed below:

- The trigger conditions for the enhanced SHR report, and

- The actual content of the enhanced SHR report

In the following section, we discuss these two points in further detail.

Proposal 1: RAN3 to discuss both the trigger conditions and the actual content included in the enhanced SHR report with network slicing related enhancements.

2.3	Further discussions and proposals

2.3.1	Content of enhanced SHR report

In order to perform slice-aware MRO, the RAN needs to know as a starting point:

- the status of slice deployment (both slice availability and resources) at the target, and

- the expected performance of a UE after a handover.

From the above list, the information about slice deployment at a target may be considered as static, while the expected UE performance after a handover and the resource availability of a slice at a target is dependent on other parameters such as the RRM policy, existing load, etc., at the target. Such information is largely time-varying, and a neighbour may only be able to learn it over a period of time. In the context of resource availability/utilization of a slice, there already exists Resource Status Reporting procedure to exchange such information.

On the other hand, RRM policies which allocate resources and assign QoS levels for the different slices accessed by the UE, may differ across neighbour cells and the resulting QoS treatment for UE traffic may even change depending on load on the cell and other local configuration for example. The nett performance delivered to a UE after a handover is the result of a combination of all the above factors. In the context of slicing especially, each cell may prioritize one or more slices over another, and this decision may be down to configuration, e.g., depending on the frequency band, available radio capabilities, ongoing energy savings, etc.

Perhaps the most important metric to obtain to understand the immediate performance delivered to a UE after a handover, is a measure of the interruption time experienced by the UE during the handover. Network slicing is an inherent component of 5G NR, and therefore measuring the interruption time experienced by the UE per slice allows for detailed examination of performance of these slices after a handover event. In addition to the above, such an enhancement can also enable better insight into the RRM policy in place at the target and also enable observability into slice service discontinuity at handover, i.e., a PDU session associated with a slice was dropped as the target did not support the specific slice.

Observation 1: User-plane interruption times can not only provide granular details on the delivered performance to a UE after a handover event, but also make it feasible to identify slice discontinuity events after a handover.

In context of the RRM policies a source node may learn, over a period of time, from SHR reports received from several handed-over UE’s to a specific target (Cell X) that a certain slice (Slice A) had consistently lower user plane interruption times, and analogously that a certain slice/s (Slice B) had consistently higher user plane interruption times. The underlying reason behind the different treatment given to the different slices arises from the RRM policy in use at the target, which may prioritize one slice over another. At a future point in time, when a handover is considered for a UE that is currently only using Slice A, the source may prefer handing over the UE to Cell X instead of Cell Y (which may have exhibited the opposite treatment to traffic on Slices A and B).

In the context of PDU sessions associated to a certain slice being dropped, the SHR report may log the user plane interruption times that exceed a threshold as a slice discontinuity event, such a threshold may either be given by configuration or by a default value. Provided that the threshold is set high-enough, user plane interruption times can serve to indicate slice discontinuity after a handover to the source (as proposed by other companies in prior meetings).

Observation 2: Some benefits of enhancing SHR reports with slice-based user plane interruption time include insights into RRM policies at a target, and observability into sessions dropped after a handover due to the slice not being supported at the target.

In the SHR report, the user plane interruption time may be measured as the difference in time between the last packet received on the slice on any DRB that is associated with a slice from the source cell until the first packet in the same slice in any associated DRB is received from the target node.

Proposal 2: Enhance SHR with slice-level user plane interruption times. The interruption time is measured as the time difference between the last packet reception on the slice in the source cell to the first packet reception on the same slice on the target cell.

2.3.2	Trigger conditions for enhanced SHR reports

The legacy triggers for SHR reporting are configured via successHO-Config IE which specifies a threshold (as a percentage) associated with timers T304, T310, and T312, with values ranging from 20 to 80 percentage depending on the timer. Whenever the timer value elapses over the specified fractional value, an SHR report is generated at the UE. An additional trigger called SourceDAPS-FailureReporting is also specified, which determines whether the UE shall generate the SHR upon successfully completing the DAPS handover to the target cell and if a radio link failure was experienced in the source PCell while executing the DAPS handover.

The timers used as triggers for SHR have the following definitions:

|  | Start condition | Stop condition |
| --- | --- | --- |
| T304 | Reception of RRC Connection Re-configuration message including the Mobility Control Info. message | As per criterion for successful completion of handover within 5GRA, handover to 5GRA |
| T310 | Upon detecting physical layer problems for PCell i.e. upon receiving N310 consecutive out-of-sync indications from lower layers | Upon receiving N311 consecutive in-sync indications from lower layers for PCell, upon triggering the handover procedure, and upon initiating the connection reestablishment procedure |
| T312 | Upon triggering a measurement report for measurement identity for which T312 has been configured while T310 timer is running | Upon receiving N311 consecutive in-sync indications from lower layers, upon triggering the handover procedure, upon initiating connection re-establishment procedure, and upon the expiry of T310 timer |

From the above definitions, it is clear that the thresholds are related to the handover procedure (T304) and to physical layer issues arising after a handover (T310/T312). However, the scope of conditions under which SHR enhancements for network slicing should be collected is broader than those specified by the above thresholds. Ideally, it would be valuable to have a trigger for SHR that is based on the observed performance over a slice. While Nokia’s proposal of logging an SHR at slice discontinuity is one such trigger, there exist other situations, e.g., large interruption times on PDU sessions over a slice, or SLA monitoring during handovers, or poor application-level QoE, etc., that require a trigger with a larger scope.

Observation 3: Both, the existing triggers for SHR and using slice discontinuity after handover as a trigger for SHR (Nokia’s proposal) is not fully sufficient to obtain observability under time varying conditions.

As mentioned previously, legacy SHR triggers are intended to identify near failure cases. Any proposed enhancement for SHR triggers shall also keep this in context such that the legacy SHR functionality is not affected. In the context of SON-based observability for slicing, handovers, and UE performance after handovers, as discussed in Sec. 2.3.1, user-plane interruption time experienced by the UE for services using different slices is one of the most important metrics characterising the handover event and the immediate performance following the handover. Therefore, in the same context, the trigger conditions for SHR reporting may be enhanced by including triggers based on user-plane interruption times. This enhances observability into near-failure events, where the failure is characterised by the user-plane interruption time experienced by the UE on services on specific slices during a handover event.

Proposal 3: RAN3 to discuss triggering conditions for enhanced SHR, where the triggering conditions are defined on the experienced user-plane interruption time measured on any of the slice in use at the UE.

Based on the above, a UE that is configured with an SHR report via the SuccessHO-Config message will receive an additional indication of a threshold corresponding to the measured user-plane interruption time for each slice in use at the UE. Whenever the measured interruption time for one of the slices is over the threshold, the SHR report will be logged by the UE. Below is an example ASN.1 realization of the above functionality.

SuccessHO-Config-r17 ::=                SEQUENCE {

thresholdPercentageT304-r17             ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT310-r17             ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT312-r17             ENUMERATED {p20, p40, p60, p80, spare4, spare3, spare2, spare1}         OPTIONAL, --Need R

sourceDAPS-FailureReporting-r17         ENUMERATED {true}                                                       OPTIONAL, --Need R

...

[[

sliceInterruptionTimeThresholdForSHRList-r19		SliceInterruptionTimeThresholdForSHRList-r19         OPTIONAL --Need R

]]

}

SliceInterruptionTimeThresholdForSHRList-r19 ::= SEQUENCE(SIZE (1..maxS-NSSAI)) OF SliceInterruptionTimeThresholdForSHR

SliceInterruptionTimeThresholdForSHR ::= SEQUENCE {

sliceID									S-NSSAI         OPTIONAL,

SliceInterruptionTimeThresholdForSHR	ENUMERATED {ms5, ms10, ms50, ms100, ms150, ms200, ms250, ms300, ms350, ms400, ms450, ms500, ms550, ms600, ms700, ms800, ms900, ms1000}         OPTIONAL,

}

A UE may have active sessions on more than one slice at a time. While some slices may only carry regular MBB traffic, other slices may carry traffic belonging to time critical services. The significant difference in the performance characteristics of different services implies that a slice-specific threshold is required to gain good trigger-based observability into the different slices’/services’ performance. However, it may be valuable to obtain the measured user-plane interruption times of all the slices in use at the UE when the trigger for one of the slices is satisfied.

Proposal 4: When the SliceInterruptionTimeThresholdForSHR is satisfied for one of the slices, the UE shall log the user-plane interruption times on all the slices on which the UE has active PDU sessions in the SHR report.

For the measurement quantity that triggers reporting, the user plane interruption time may be measured as the difference in time between the last packet received on the slice on any DRB that is associated with a slice from the source cell until the first packet in the same slice in any associated DRB is received from the target node.

In the ASN.1 example above, the specified thresholds for SliceInterruptionTimeThresholdForSHR range from 5ms to 1000ms. While the range provided above may be sufficient for most use-cases, the granularity of the interruption time to be used as a trigger for SHR needs further discussion.

Proposal 5: RAN3 to discuss both the range and granularity of the threshold used to trigger SHR based on user-plane interruption times.

It is also clear from the discussions in this paper that the proposed trigger for the SHR reporting is independent of the existing cause values associated to the SHR report (T304, T310, T312, and sourceDAPS-failure). The SHR-cause value list should include another cause value which is used to indicate that the SHR report was collected due to at least one UE slice experiencing a larger user-plane interruption time than the specified threshold associated to the slice.

Proposal 6: RAN3 to enhance SHR-cause value list with ‘slice user-plane interruption time’ as an additional cause value.

For the sake of completeness, below is an example realization of the SHR report which includes the measured user-plane interruption time for the slices in use at the UE.

SuccessHO-Report-r17 ::=                 SEQUENCE {

sourceCellInfo-r17                       SEQUENCE {

sourcePCellId-r17                        CGI-Info-Logging-r16,

sourceCellMeas-r17                       MeasResultSuccessHONR-r17             OPTIONAL,

rlf-InSourceDAPS-r17                     ENUMERATED {true}                     OPTIONAL

},

targetCellInfo-r17                       SEQUENCE {

targetPCellId-r17                        CGI-Info-Logging-r16,

targetCellMeas-r17                       MeasResultSuccessHONR-r17             OPTIONAL

},

measResultNeighCells-r17                 SEQUENCE {

measResultListNR-r17                     MeasResultList2NR-r16                 OPTIONAL,

measResultListEUTRA-r17                  MeasResultList2EUTRA-r16              OPTIONAL

}

locationInfo-r17                         LocationInfo-r16                          OPTIONAL,

timeSinceCHO-Reconfig-r17                TimeSinceCHO-Reconfig-r17                 OPTIONAL,

shr-Cause-r17                            SHR-Cause-r17                             OPTIONAL,

ra-InformationCommon-r17                 RA-InformationCommon-r16                  OPTIONAL,

upInterruptionTimeAtHO-r17               UPInterruptionTimeAtHO-r17                OPTIONAL,

c-RNTI-r17                               RNTI-Value                                OPTIONAL,

...,

[[

targetCell-PCI-ARFCN-r17                 PCI-ARFCN-NR-r16                          OPTIONAL

]],

[[

eutra-TargetCellInfo-r18                 SEQUENCE {

targetPCellId-r18                        CHOICE {

cellGlobalId-r18                         CGI-Info-Logging-r16,

pci-arfcn-r18                            PCI-ARFCN-EUTRA-r16

},

targetCellMeas-r18                       MeasQuantityResultsEUTRA              OPTIONAL

}                                                                                  OPTIONAL,

measResultServCellRSSI-r18                   RSSI-Range-r16                        OPTIONAL,

measResultNeighFreqListRSSI-r18              MeasResultNeighFreqListRSSI-r18       OPTIONAL,

eutra-C-RNTI-r18                             EUTRA-C-RNTI                          OPTIONAL,

timeSinceSHR-r18                             TimeSinceSHR-r18                      OPTIONAL

]],

[[

sliceInterruptionTimeList-r19				 SliceInterruptionTime-r19             		   OPTIONAL,

]]

}

SliceInterruptionTimeList-r19 ::= SEQUENCE(SIZE (1..maxS-NSSAI)) OF SliceInterruptionTime-r19

SliceInterruptionTime-r19                    SEQUENCE {

sliceID								     S-NSSAI,

InterruptionTime                         INTEGER (ms5..ms10000),

}

Proposal 7: RAN3 to discuss the proposed SHR enhancements (configuration, reporting, and thresholds). Depending on the agreements, send an LS to RAN2 describing the enhancements and RAN3 agreements.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals:

Proposal 1: RAN3 to discuss both the trigger conditions and the actual content included in the enhanced SHR report with network slicing related enhancements.

Proposal 2: Enhance SHR with slice-level user plane interruption times. The interruption time is measured as the time difference between the last packet reception on the slice in the source cell to the first packet reception on the same slice on the target cell.

Proposal 3: RAN3 to discuss triggering conditions for enhanced SHR, where the triggering conditions are defined on the experienced user-plane interruption time measured on any of the slice in use at the UE.

Proposal 4: When the SliceInterruptionTimeThresholdForSHR is satisfied for one of the slices, the UE shall log the user-plane interruption times on all the slices on which the UE has active PDU sessions in the SHR report.

Proposal 5: RAN3 to discuss both the range and granularity of the threshold used to trigger SHR based on user-plane interruption times.

Proposal 6: RAN3 to enhance SHR-cause value list with ‘slice user-plane interruption time’ as an additional cause value.

Proposal 7: RAN3 to discuss the proposed SHR enhancements (configuration, reporting, and thresholds). Depending on the agreements, send an LS to RAN2 describing the enhancements and RAN3 agreements.

# References

[1] RP-234038: WID: Data collection for SON/MDT in NR standalone and MR-DC

[2] R3-244435: On slice specific MDT measurements and SON enhancements, Ericsson

[3] R3-244067: MDT solution for slice support and slice-related mobility enhancements, Nokia

[4] R3-244257: Discussion on SON enhancements for network slicing, China Telecom