3GPP TSG-RAN WG3 #125-bis	R3-245397

# Hefei, China, 14th – 18rd October, 2024

Source:	Ericsson, AT&T, Jio

Title:	On MDT enhancements for Network Slicing

Agenda item:	10.3.2

Document for:	Discussion, Decision

# 1	Introduction

During TSG-RAN WG3 #125 meeting the following agreements and FFSs were made with respect to the future work on MDT/SON enhancements for Network Slicing:

- For the immediate MDT, it is not needed to add measurements related to the slice unavailability and the lack of resources. - FFS if new MDT measurements are needed for the rejected Slice. - The slice ID will be added as a filter to the area scope (for the both, management- and signalling-based immediate MDT related to slice). The intention is to limit the number of reports sent to the OAM, thus the UE selection will not be amended.

In this contribution, we discuss the FFSs and share our views and proposals.

# 2	Discussion

## 2.1	New MDT measurements for slice-coverage observability

An area where it is beneficial to have better observability is the area of slice coverage across the network. Indeed, configuring a perfect match between the coverage of the cells in which the slice is supported/available and the geographical areas where the users should have access to the services provided by the slice may be challenging in some cases and mismatches may occur.

For example, let us consider the case of an operator providing slice services for a customer so that the employees of the customer can access the slice services for working purposes.  The users may request access to slice services while working from the office premises of their employer but also while working remotely from home. While configuring a good overlap between the slice coverage and the customer’s office premises is rather easy, it may be comparatively more difficult to configure a good slice coverage overlapping with the remote working location of each employee. For the support of the later configuration task, it would be good to enhance MDT to log users’ access request to slice services. More particularly it would be very helpful to log the events when user’s request to a service provided by a slice is rejected. As a reminder the slice service request of a UE that is supposed to have access to the service may be rejected for one of the following reasons:

- the slice is not supported, or

- the slice is not available,

- or the radio resource allocated to the slice are fully used in the cell from which the UE is making the service request.

Observation 1: Ensuring a good overlap between the slice coverage and the areas in which the authorized UEs should have access to the services provided by the slice requires a good observability of the UEs’ slice service requests.

Observation 2: In order to identify holes in the coverage of a slice (cells in which UEs should have access to the services provided by the slice but in fact UEs’ service request are rejected in those cells) it is beneficial to extend the MDT measurements to enable logging of UE’s denied slice service requests.

Proposal 1: RAN3 to enhance network slice coverage observability by adding support of new MDT measurements to log the rejected slice service access requests of a UE due to slice not being supported in the cell from which UE is making the request.

This proposal may have impact on RAN2 and SA5 specifications. Thus, RAN3 needs to send an LS to SA5 and RAN2.

Proposal 2: If Proposal 1 is accepted, RAN3 to send an LS to RAN2 and SA5.

## 2.2	Slice observability under different load conditions

- In management-based immediate MDT, OAM configures each individual RAN nodes and RAN nodes perform UE selection and collection of MDT measurements. Afterwards, the measurements are sent back to OAM. The UEs are selected based on the configuration provided by the OAM in the area scope after taking considerations of UE capability. However, the MDT report collection starts upon selection of a suitable UE.

Observation 3: In management-based MDT, upon reception of the MDT configuration from the OAM, RAN nodes selects suitable UEs provided in the area scope taking consideration of UE capabilities and begins report collection.

- Hence, it may happen that the OAM collects MDT measurements from RAN when the slice utilization is low and the reported performance may be good, e.g., during non-peak utilization hours of a slice. It should however be noted that the peak utilization period of a slice may be different when compared to the peak utilization period of the overall RAN node. This is shown in the below table-

| - Time | - Slice A utilization | - Slice B utilization | - Overall RAN node utlization |
| --- | --- | --- | --- |
| - t1 | - 50% | - 70% | - 25% |
| - t2 | - 26% | - 65% | - 60% |
| - t3 | - 20% | - 10% | - 75% |
| - t4 | - 80% | - 90% | - 20% |

- The table shows possible different utilizations of different slices along with overall RAN node utilization. Different slices may have peak utilization at different times, and they can be uncorrelated with the overall RAN node utilization. Thus, if OAM collects MDT reports at t3, the slice-specific results would not reflect the actual load condition at t4.

Observation 4: If OAM collects MDT reports for a specific slice when its utilization is low, it would not be able to analyse any potential performance degradation of the slice.

- An alternative is to collect MDT measurements for a very long period; this, however, would lead to transmitting large amount of data to OAM of which very little would be of relevance.

- Thus, to increase slice specific observability and transmission of pertinent data, it is important that the OAM nodes collect MDT measurements based on some slice specific KPIs available in RAN. For example, if the RAN nodes are provided with a criterion to collect MDT measurements for a slice only when the slice specific load is very high or the overall RAN node utilization is very high, it would be able to observe if there is a degradation in the slice performance.

Observation 5: If OAM collects slice specific MDT reports during high slice utilization period, it would be able to analyse if there is degradation in slice performance under high load.

Proposal 3: To increase slice performance observability under different load conditions and reduce amount of data sent to OAM, RAN3 to discuss enhancement of immediate MDT configuration from OAM.

# Conclusion

In the previous sections we made the following observations:

Observation 1: Ensuring a good overlap between the slice coverage and the areas in which the authorized UEs should have access to the services provided by the slice requires a good observability of the UEs’ slice service requests.

Observation 2: In order to identify holes in the coverage of a slice (cells in which UEs should have access to the services provided by the slice but in fact UEs’ service request are rejected in those cells) it is beneficial to extend the MDT measurements to enable logging of UE’s denied slice service requests.

Observation 3: In management-based MDT, upon reception of the MDT configuration from the OAM, RAN nodes selects suitable UEs provided in the area scope taking consideration of UE capabilities and begins report collection.

Observation 4: If OAM collects MDT reports for a specific slice when its utilization is low, it would not be able to analyse any potential performance degradation of the slice.

Observation 5: If OAM collects slice specific MDT reports during high slice utilization period, it would be able to analyse if there is degradation in slice performance under high load.

Based on the discussion in the previous sections we propose the following:

Proposal 1: RAN3 to enhance network slice coverage observability by adding support of new MDT measurements to log the rejected slice service access requests of a UE due to slice not being supported in the cell from which UE is making the request.

Proposal 2: If Proposal 1 is accepted, RAN3 to send an LS to RAN2 and SA5.

Proposal 3: To increase slice performance observability under different load conditions and reduce amount of data sent to OAM, RAN3 to discuss enhancement of immediate MDT configuration from OAM.

# References

TS 37.320, Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2, 3GPP, V18.1.0, 2024-04-03

TS 28.552, Management and orchestration; 5G performance measurements, 3GPP, V18.6.0, 2024-04-05