3GPP TSG-RAN WG2 #125-bis	R3-245399

Hefei, China, Oct 14th – 18th 2024

Agenda Item:	10.4

Source:	Ericsson

Title:	(TP for BLCR for SON for 38.413) UHI for SCG activation and deactivation

Document for:	Discussion, Decision

# 1	Introduction

In the Rel-19 SON/MDT Work Item [3], MHI enhancement for SCG deactivation/activation is listed as one of the objectives for the Work Item. In RAN2#127, RAN2 agreed that it is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state. Two options for collecting the time with SCG state set to activated and deactivated was discussed, namely a network-based solution or a UE based solution. An LS [1] was written and agreed to be sent to RAN3, asking RAN3 to analyse whether a network-based solution is sufficient enough for network optimization, or whether a UE based solution is needed. This contribution will focus on the network-based solution.

# 2	Discussion

The history information on visited cells, logged by the UE in the Mobility History Information (MHI), and by the network in the UE History Information (UHI), helps the network to understand the movement of the UE and identify ping-pong or coverage issues in the network. From Rel-17, not only time spent in the PCell, but also time spent in the PSCell is logged in the history information. In Rel-17, SCG activation and deactivation was also introduced, which enables power saving while still allowing fast access to the SCG when needed.

Currently there is no support in the history information (UHI or MHI) for tracking how often or how long the SCG state has been set to activated or deactivated. Such information could be used by the network to perform mobility optimization as it tracks the actual usage of the SCG leg. It could also play an important role for traffic and pattern analysis, and the MN could track whether a particular SCG related PSCell is configured often or is seldom used. In addition to this, the information on SCG activation/deactivation could play an important role for mobility and traffic prediction in future AI/ML applications.

To increase the understanding of the secondary cell usage, information on the accumulated time with SCG state set to activated for each PSCell entry could be logged in the UE history information (UHI). Since the time spent with the PSCell configured is already logged in the history information, this new information will give an indication of how large part of the time spent in the PSCell was spent in activated or deactivated state.

Logging the accumulated time with SCG state set to activated during the stay in the PSCell in the history information will, together with the already logged time spent in the PSCell, give an indication of how large part of the time spent in the PSCell was spent in activated or deactivated state.

Based on the above reasoning, we propose the following.

For each PSCell entry of the UHI, log the accumulated time with SCG state set to activated during the stay in the PSCell.

A Text Proposal implementing Proposal 1 is provided in Appendix A below.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Logging the accumulated time with SCG state set to activated during the stay in the PSCell in the history information will, together with the already logged time spent in the PSCell, give an indication of how large part of the time spent in the PSCell was spent in activated or deactivated state.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For each PSCell entry of the UHI, log the accumulated time with SCG state set to activated during the stay in the PSCell.

# References

R3-245009, LS on MHI enhancement solution for SCG deactivation/activation, RAN2, 3GPP TSG RAN WG2#127, Maastricht, Netherlands, 19th - 23rd August 2024.

# Appendix A - Text Proposal to 38.413

#### 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. |
| Overall Activation Timer | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell while the SCG was in active state, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |

[Unchanged parts omitted]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

[Unchanged parts omitted]

id-OnboardingSupport,

id-OverallActivationTimer,

id-PagingAssisDataforCEcapabUE,

[Unchanged parts omitted]

LastVisitedPSCellList ::= SEQUENCE (SIZE(1..maxnoofPSCellsPerPrimaryCellinUEHistoryInfo)) OF LastVisitedPSCellInformation

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-OverallActivationTimer	CRITICALITY ignore	EXTENSION OverallActivationTimer	PRESENCE optional },

...

}

[Unchanged parts omitted]

OverallActivationTimer ::= INTEGER (0..40950)

[End of changes]