3GPP TSG-RAN WG3 Meeting #125bis	R3-245410

Hefei, China, October 14-28, 2024

Agenda Item:	14.3

Source:	THALES

Title:	Discussion on UE-Sat-UE communications

Document for:	Information

# 1	Introduction

As part of the integration of the satellite components in the 5G architecture phase 3 for the Rel-19 [1][2], the UE-sat-UE communication was studied [3] and an architecture is being specified. The use cases are described by SA1 in [4]-§5.14 and §5.15.

The objectives of the WI [2] are given as follow:

Using the consolidated potential requirements captured in TR 22.865 as baseline, the objective of this work item is to introduce normative service requirements related to the 5G system with satellite access for - Store and Forward Satellite operation - UE-Satellite-UE communication - GNSS independent operation - Positioning enhancements for satellite access - Other general enhancements aspects all related to the 5G system with satellite access including information collection via satellite connections.

The objective of this paper is to inform RAN3 on the UE-sat-UE communication architecture and the potential impacts.

# 2	Discussion

The UE-Sat-UE communications are justified in some scenario where the UEs need to communicate using satellite access without going to the ground network in order to avoid long delays and limited data rates as well as reducing the consumption of backhaul resources.

According to the study conclusions [3] #KI3, the architecture depicted on the figure below is considered.

Figure 1 - UE-Sat-UE communication architecture. In blue, the data flow; In green, the signaling flow

For this Release, only IMS voice/video are considered, e.g. no broadband connectivity between UEs. The IMS data flow remains in the constellation while the signaling needs a connectivity to the ground. The GSO is not considered for this use-case The access satellite needs to embark at least the gNB, a part of the UPF for uplink classifier (UL CL) and the PDU Session Anchor (PSA, terminating N6) and the ALG/AGW (Application Level Gateway / Access GateWay) for the IMS system. Note that the two UEs can be served by the same satellite. Note also that UE-Sat-UE communications can be possible only in the same HPLMN. The related procedures are:

- Association of relay entity between on-board UPF and P-CSCF (Proxy-Call Session Control Function)

- UE registration to the network as legacy with AMF on the ground

- IMS PDU establishment with on-board UPF

- IMS registration by the UE to the P-CSCF, the S-CSCF (Serving CSCF) and HSS (on-ground)

- SIP Voice call setup

- Mobility

The SIP voice call setup procedure is as follow:

- The UE1 is connected the NTN

- The UE1 starts a call by sending SIP Invite message

- The P-CSCF determine the possibility of UE-Sat-UE communication according to the subscription, the gNB ID and the access network information

- The route is established with the UPF and the AGW

- The SIP communication is established after SIP 183 Session established

The gNB implies that the regenerative payload supporting ISL is mandatory for such communications. Moreover, the UPF, or a part of the UPF (UL CL and PSA) needs to be embarked, that was not considered in the baseline architecture provided by the SA2 study [3] #KI1 and adopted in RAN2 and RAN3. It means a split of the NG interface with N2 for UE-Sat-UE and other data flow connected to the ground (through ISL and feeder link), N3 for UE-Sat-UE remaining in the satellite and N3 for the other data flow to the ground. For the UE-Sat-UE communications, the N6 interface terminates in the satellite.

The mobility management due to satellite movement includes a mobility at the RAN-level (legacy NR NTN handover procedures), at 5GC-level (UPF PSA change) and IMS (legacy AGW registration/mobility procedures)

Observation 1	UE-Sat-UE communications are supported by a LEO/MEO regenerative payload architecture where the RAN (i.e. full gNB) and a part of the UPF are embarked in the satellite payload

Observation 2	UPF on-board is out of the RAN scope

Observation 3	There is no difference with the regenerative payload baseline architecture adopted in RAN3 w.r.t. the NG/Xn interfaces mobility

Observation 4	Satellite movements implies to use RAN-level, 5GC-level and IMS mobility

The Voice over NR is associated to a QoS profile to ensure the usability of the service [5]-§5.7.4.1. Moreover, one 5QI is designed for Satellite services.

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 20 | 100 ms (NOTE 11, NOTE 13) | 10-2 | N/A | 2000 ms | Conversational Voice |
| 2 | (NOTE 1) | 40 | 150 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms | Conversational Video (Live Streaming) |
| 5 | Non-GBR | 10 | 100 ms (NOTE 10, NOTE 13) | 10-6 | N/A | N/A | IMS Signalling |
| 10 |  | 90 | 1100ms (NOTE 10,NOTE 13, NOTE 17) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access type with these characteristics |

NOTE 10: In RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 11: In RRC_IDLE and RRC_INACTIVE mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 13: A static value for the CN PDB of 20 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE 17: The worst case one way propagation delay for GEO satellite is expected to be ~270ms,~21 ms for LEO at 1200km, and 13 ms for LEO at 600km. The UL scheduling delay that needs to be added is also typically two way propagation delay e.g. ~540ms for GEO, ~42ms for LEO at 1200km, and ~26 ms for LEO at 600km. Based on that, the 5G-AN Packet delay budget is not applicable for 5QIs that require 5G-AN PDB lower than the sum of these values when the specific types of satellite access are used (see TS 38.300 [27]). 5QI-10 can accommodate the worst case PDB for GEO satellite type.

The 5QIs used for conversational voice and video streaming define a packet delay budget that should be achievable for a LEO/MEO satellite where a PDB of 80 ms (substracted from the static value for the CN PDB of 20 ms) corresponds to a one-way delay of 24000 km. The CN PDB for the UPF terminating N6 might less than 20 ms since the RAN and the UPF are embarked in the same satellite payload. However a clarification should be needed. The 5QI 10 defines: “any service that can be used over satellite access type with these characteristics”, that could include IMS if no well configured. The issue is that bitrate is not guaranteed, and the default priority is high, and the same than the other NTN data flows, possibly impacting the IMS Quality of Service. The QoS characteristics mapping table should precise that conversational voice (QFI 1), the conversational video (QFI 2) and IMS Signaling (QFI 5) can be supported by a satellite access.

Observation 5	The QoS profile to be used in UE-Sat-UE should be clarified in the standardized 5QI to QoS characteristics mapping table

The data transmission optimization for Voice over NR (i.e. IMS) such as pre-configured uplink grant or semi-persistent scheduling are not precluded for gNB in transparent NTN. For the regenerative, RAN2 assumes that procedures supported in transparent should be supported.

Observation 6	Data transmission optimizations for IMS (e.g. semi-persistent scheduling) can be re-used in a NTN regenerative architecture

Proposal 1		RAN3 assumes the impact of the UE-Sat-UE communications support on RAN3 is limited. FFS if any corrections or enhancements are needed, send a LS to SA2

# 3	Conclusion

In the previous sections we made the following observations and proposals:

Observation 1	UE-Sat-UE communications are supported by a LEO/MEO regenerative payload architecture where the RAN (i.e. full gNB) and a part of the UPF are embarked in the satellite payload

Observation 2	UPF on-board is out of the RAN scope

Observation 3	There is no difference with the regenerative payload baseline architecture adopted in RAN3 w.r.t. the NG/Xn interfaces mobility

Observation 4	Satellite movements implies to use RAN-level, 5GC-level and IMS mobility

Observation 5	The QoS profile to be used in UE-Sat-UE should be clarified in the standardized 5QI to QoS characteristics mapping table

Observation 6	Data transmission optimizations for IMS (e.g. semi-persistent scheduling) can be re-used in a NTN regenerative architecture

Proposal 1		RAN3 assumes the impact of the UE-Sat-UE communications support on RAN3 is limited. FFS if any corrections or enhancements are needed, send a LS to SA2

# References

SP-231199, SID “Study on Integration of satellite components in the 5G architecture Phase III”, Rel-19, 09-2023.

SP-230516, WID “Satellite access Phase 3”, Rel-19, 06-2024.

3GPP TR 23.700-29, “Study on integration of satellite components in the 5G architecture; Phase 3”, v19.0.0, 06-2024.

3GPP TR 22.865, “Study on satellite access Phase 3”, v19.0.0, 06-2024.

3GPP TS 23.501, “System architecture for the 5G System (5GS)”, Rel-19, 06-2024.