3GPP TSG-RAN WG3 Meeting #125bis	R3-245417

Hefei, China, 14th – 18th October, 2024

Title: 	(TP for AI/ML BLCR to TS 38.423) Discussion on the AI/ML-based Network Slicing

Source: 	Huawei

Agenda item:	11.2

Document Type:	Discussion and Decision

# 1. Introduction

In RAN#105 the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” was approved in [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (latest SID in [2]) whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing, as excerpted from [1] and highlighted in yellow below:

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743. The detailed objectives of the WI are listed as follows: - Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3] - Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states Note: Coordination with RAN2, SA5 when needed if any.

This contribution analyzes the technical enablers for the AI/ML-based Network Slicing use case to be introduced over the Xn interface, and provides proposals for discussion.

# 2. Discussion

This section starts with an analysis of the agreed AI/ML model’s input, output and feedback information as reported in the TR 38.743 [3] to assess which information are already supported by the standard based on what has been specified in Rel-18. Then, in Section 2.1, the discussion on the left issue from the study phase related to the granularity of UE performance feedback for the Network Slicing use case, as in the Note in clause 4.1.2.4. of TR 38.743 [3], is also discussed.

As stated before, RAN3 in Rel-19 will specify data collection enhancements and signaling support in NG-RAN for the AI/ML-based Network Slicing use case, among others. As clearly stated in the WID, the specification work in Rel-19 for AI/ML for NG-RAN is based on the conclusions captured in TR 38.743, which are excerpted from [3] and reported below:

# 6 Conclusion The new use cases and the Rel-18 leftover cases follow Rel-18 AI/ML framework. The following new use cases are recommended by RAN3 to be specified in the Rel-19 normative phase: - AI/ML-based Network Slicing - AI/ML-based Coverage and Capacity Optimization Recommended solutions and standard impacts for each use case are based on section 4.1.2 and section 4.2.2. For each use case above, it is recommended to take the corresponding section “Solutions and standard impacts” (section 4.1.2 and 4.2.2) as basis during the normative work. The following Rel-18 leftovers are recommended by RAN3 to be specified in Rel-19 normative phase: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states The corresponding descriptions and potential standard impacts for each Rel-18 leftovers above shall be taken as baseline during the normative phase.

For the AI/ML-based Network Slicing use case, the input, output and feedback data for the AI/ML model deployed in the gNB(-CU) have been identified and listed in TR 38.743 clauses 4.1.2.2, 4.1.2.3 and 4.1.2.4, respectively.

Concerning the input data, and focusing on the information from both the local node and the neighbour nodes, the following information have been agreed during the SI and reported in TR 38.743:

#### 4.1.2.2 Input data of AI/ML based Network Slicing To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing: From local node: - Measured/Predicted radio resource status per slice - Measured/Predicted slice available capacity - Legacy predicted UE trajectory From neighbouring gNBs: - Measured/Predicted radio resource status per slice - Measured/Predicted slice available capacity From the UE: #### - UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

From the local node perspective, the transfer of the measured radio resource status per slice as well as slice available capacity from neighbour nodes is already supported by the standard over the Xn interface, by means of the Radio Resource Status Reporting (Initiation) procedures. While for the legacy predicted UE trajectory, this input information is an internal input of the local node, hence there is no standard impact over the Xn as well.

Observation 1: For the AI/ML-based Network Slicing, there is already standard support for providing the measured radio resource status per slice and slice available capacity from neighbour nodes to the AI/ML model in the local node; for the legacy UE trajectory in the local node, this is an internal input in the local node itself, hence there is no standard impact over the Xn interface.

The predicted radio resource status per slice and predicted slice available capacity, instead, need to be introduced and exchanged it over the Xn interface in Rel-19. The most straightforward way to enable such transfer is to enhance the existing Data Collection Reporting (Initiation) procedures introduced in Rel-18.

Proposal 1: RAN3 to discuss and agree to enhance the Data Collection Reporting (Initiation) procedures to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface.

In the Annex of this paper we have a TP for the BLCR to TS 38.423 that is based on Proposal 1 above.

With regards to the output data, RAN3 agreed in the SI phase the following information are generated by the A/ML model, as reflected in TR 38.743:

#### 4.1.2.3 Output data of AI/ML based Network Slicing AI/ML-based network slicing model in a gNB can generate the following information as output: - Predicted radio resource status per slice - Predicted slice available capacity - Resource management decisions for resources within RRM policies (used by gNB internally) - Slice aware mobility decisions (used by gNB internally)

Other than the resource management decisions for resources within RRM policies and slice aware mobility decisions – with both remaining internal output in the local node – the predicted radio resource status per slice and predicted slice available capacity are also listed as output information of the AI/ML model. Therefore, the same Proposal 1 as above applies.

Observation 2: For the output information of the AI/ML model for Network Slicing that needs to be introduced and exchanged over the Xn interface, i.e., predicted radio resource status per slice and predicted slice available capacity, they are already addressed in Proposal 1; resource management decisions for resources within RRM policies and slice aware mobility decisions are internal output in the local node, hence there is no standard impact over Xn.

The above Proposal 1 reflects the indications reported in clause 4.1.2.5 of TR 38.743 concerning the potential standard impacts for enabling the AI/ML-based Network Slicing use case:

#### 4.1.2.5 Potential standard impacts Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18. Xn interface: - Enhanced existing procedure to collect predicted information between gNBs: ■ Predicted radio resource status per slice ■ Predicted slice available capacity

For the feedback information, instead, the following information have been agreed by RAN3 during the SI phase to be collected from (neighbour) gNBs, as reflected in TR 38.743:

#### 4.1.2.4 Feedback of AI/ML based Network Slicing To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs: - Measured Radio resource status per slice - Measured Slice available capacity - Legacy UE performance feedback for those UEs handed over from the source gNB - Finer granularity UE performance feedback for those UEs handed over from the source gNB to determine UE performance for a certain slice in use by a certain UE. NOTE: Exact level of finer granularity to be defined in normative phase.

Also in this case, the transfer over Xn of all the pieces of information listed above is already supported by the standard: measured radio resource status per slice and measured slice available capacity can be transferred over Xn via the Radio Resource Status Reporting (Initiation) procedures, while legacy (i.e., Rel-18 per UE) UE performance feedback can be exchanged over Xn via the Data Collection Reporting (Initiation) procedures.

Observation 3: For the AI/ML-based Network Slicing feedback information, the exchange over Xn of measured radio resource status per slice, measured slice available capacity and legacy (i.e., Rel-18 per UE) UE performance feedback is already supported by the standard.

Moreover, for the Network Slicing use case, another feedback information can be considered, i.e., the finer (with respect to UE level) granularity of UE performance feedback. As per the NOTE cited above and highlighted in yellow, the exact level of granularity needs to be discussed and agreed in RAN3 considering what has been discussed during the SI. Our views on this aspect is provided in the next section.

## 2.1 Finer granularity of UE performance feedback

In the current specification, the granularity of UE performance is per UE, which is requested by the source gNB to be reported by the target gNB to assess the UE performance after an AI/ML-driven mobility action (i.e., handover), as shown below.

#### From TS 38.423 v18.3.0: #### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4. Average UE Throughput UL; O; Bit Rate 9.2.3.4. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11.

Based on the discussions from the Rel-19 SI, we have the following candidate solutions on the table for the UE performance finer granularity [4], listed in ascending order in terms of granularity (i.e. from coarser to finer).

- slice level

- PDU session level

- QoS-flow-group level

- QoS-flow level

Considering that, for the AI/ML-based Network Slicing we considered slice-level resource allocation and slice-based load balancing/mobility actions, it would make sense to provide UE performance feedback on a per slice level as well to be reported by the target gNB so that the source gNB can assess whether the concerned slice can ensure proper UE performance after the handover. This feedback information sent from the target node to the source node can be used by the source node to optimize the AI/ML model so that better handover decisions can be determined. The slice level UE performance applies to all legacy UE performance metrics introduced in Rel-18, including average UE throughput in the DL/UL, average packet delay, and average packet Loss in the DL. Moreover, in our understanding, the slice-level granularity of UE performance metrics is already supported by the standard as per SA5’s TS 28.558 [4].

Observation 4: It makes sense to provide UE performance feedback on a per slice level, i.e., the same granularity as for the resource allocation and load balancing/mobility actions. Slice-level UE performance metrics are already supported by the standard, hence no additional standard impact is foreseen.

The left three granularities are finer than the slice level one, since one PDU session can be associated with one slice and different PDU sessions can share the same slice. One PDU session, in turn, would be mapped into multiple QoS flows.

Since the association between the PDU session and the slice is determined according to the URSP (UE Route Selection Policy) rules, the source node may benefit from the PDU session level UE performance from the target node. However, we should consider that NG-RAN UE level measurements as defined by SA5 in TS 28.558 only consider per DRB, per QoS-flow and per slice granularities – and hence there is already standard support for these granularities – while our understanding is that per PDU session level granularity may require additional standard effort from other working groups (e.g., SA5, RAN2) which seems unnecessary considering that other even finer granularities are already supported by the standard (i.e., per DRB, per QoS-flow).

Observation 5. The association between the PDU session and slice is determined according to the URSP rules, thus the source node may benefit from PDU session level UE performance feedback from the target node. But there are even finer granularities already supported by the standard, i.e., per DRB and per QoS-flow as for the NG-RAN UE level measurements in TS 28.558, which make per PDU session level granularity not motivated.

The motivation to introduce the per QoS-flow-group-level UE performance (QoS-flow level can be the corner case when there is only one QoS flow in the group) is that the average of UE performance metrics (throughput, packet loss and packet delay) over all configured DBRs as done in Rel-18 may not reflect the actual status of each QoS flow after the handover. It happens when the traffic of the QoS flows fluctuates dramatically, thus causing the average UE performance to change dramatically as well, as shown in the example in [5]. Based on the Rel-18 mechanism, the target node feeds back the UE performance collected within a very short time window after the handover, e.g., less than 1000ms. We think it can be kind of a corner case that the traffic associated to QoS flows of the UE fluctuates dramatically in this short time window immediately after handover. As a consequence, our view is that having per QoS-flow-group / per QoS-flow level UE performance is an unnecessary complication for UE performance collection and reporting considering the scenario during which this information is collected (i.e., after a very short time after the handover).

Observation 6: It can be kind of a corner case that the traffic associated to QoS flows of the UE fluctuates dramatically in a short time window immediately after the handover. This makes the per QoS-flow-group / per QoS-flow level UE performance an unnecessary complication for UE performance collection and reporting considering the scenario during which this information is collected (i.e., after a very short time after the handover).

Proposal 2: RAN3 to agree that for the AI/ML-based Network Slicing use case in Rel-19 the UE performance can be reported from target gNB to source gNB on a per slice granularity. Such granularity applies to all UE performance metrics already introduced in Rel-18, i.e., average UE throughput DL/UL, average packet delay and average packet loss DL.

The TP for the AI/ML BLCR to TS 38.423 in the Annex reflects the above proposals.

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

Observation 1: For the AI/ML-based Network Slicing, there is already standard support for providing the measured radio resource status per slice and slice available capacity from neighbour nodes to the AI/ML model in the local node; for the legacy UE trajectory in the local node, this is an internal input in the local node itself, hence there is no standard impact over the Xn interface.

Proposal 1: RAN3 to discuss and agree to enhance the Data Collection Reporting (Initiation) procedures to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface.

Observation 2: For the output information of the AI/ML model for Network Slicing that needs to be introduced and exchanged over the Xn interface, i.e., predicted radio resource status per slice and predicted slice available capacity, they are already addressed in Proposal 1; resource management decisions for resources within RRM policies and slice aware mobility decisions are internal output in the local node, hence there is no standard impact over Xn.

Observation 3: For the AI/ML-based Network Slicing feedback information, the exchange over Xn of measured radio resource status per slice, measured slice available capacity and legacy (i.e., Rel-18 per UE) UE performance feedback is already supported by the standard.

Observation 4: It makes sense to provide UE performance feedback on a per slice level, i.e., the same granularity as for the resource allocation and load balancing/mobility actions. Slice-level UE performance metrics are already supported by the standard, hence no additional standard impact is foreseen.

Observation 5. The association between the PDU session and slice is determined according to the URSP rules, thus the source node may benefit from PDU session level UE performance feedback from the target node. But there are even finer granularities already supported by the standard, i.e., per DRB and per QoS-flow as for the NG-RAN UE level measurements in TS 28.558, which make per PDU session level granularity not motivated.

Observation 6: It can be kind of a corner case that the traffic associated to QoS flows of the UE fluctuates dramatically in a short time window immediately after the handover. This makes the per QoS-flow-group / per QoS-flow level UE performance an unnecessary complication for UE performance collection and reporting considering the scenario during which this information is collected (i.e., after a very short time after the handover).

Proposal 2: RAN3 to agree that for the AI/ML-based Network Slicing use case in Rel-19 the UE performance can be reported from target gNB to source gNB on a per slice granularity. Such granularity applies to all UE performance metrics already introduced in Rel-18, i.e., average UE throughput DL/UL, average packet delay and average packet loss DL.

The TP for the AI/ML BLCR to TS 38.423 in the Annex reflects the above proposals.

# 4. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- TS 28.558, Management and orchestration; UE level measurements for 5G system, Rel-18,

- R3-243578, Discussion on definition of UE performance metrics, CATT, Deutsche Telekom, Nokia, China Telecom, China Unicom.

# 5. Annex – TP for AI/ML BLCR to TS 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Slice Radio Resource Status, Eleventh Bit = Predicted Slice Available Capacity, Twelfth Bit = Slice UE Performance. Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | – |  |
| >>>Slice To Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory, Seventh Bit = Slice UE Performance. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections, Fourth Bit = Predicted Slice Radio Resource Status, Fifth Bit = Predicted Slice Available Capacity. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Slice Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the Slice Radio Resource Status List IE. |  |  |
| >>Predicted Slice Available Capacity | O |  | Slice Available Capacity 9.2.2.55 |  |  |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| >>Slice UE Performance | O |  | 9.2.3.x |  |  |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x	Slice UE Performance

This IE indicates the per slice UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Slice UE Performance |  | 1..< maxnoofBPLMNs > |  |  |
| > PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| > Slice UE Performance List |  | 1 .. < maxnoofSliceItems> |  |  |
| >> Slice UE Performance Item |  |  |  |  |
| >>> S-NSSAI |  |  | 9.2.3.21 |  |
| >>> UE Performance |  |  | 9.2.3.179 |  |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>