3GPP TSG-RAN WG3 Meeting #125bis	R3-245420

Hefei, China, 14th – 18th October, 2024

Title: 	AI/ML-based Mobility Optimization in NR-DC

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and Decision

# 1. Introduction

In RAN#105 the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” was approved in [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (latest SID in [2]) whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to the mobility optimization for NR-DC, as excerpted from [1] and highlighted in yellow below:

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743. The detailed objectives of the WI are listed as follows: - Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3] - Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states Note: Coordination with RAN2, SA5 when needed if any.

This contribution analyzes the technical enablers for the mobility optimization for NR-DC and provides proposals for discussion.

# 2. Discussion

Rel-18 mainly considered the scenario where a UE is connected to a single NG-RAN node, and NR-DC scenario is not considered. The enablers for the AI/ML-based Mobility Optimization use case in Rel-18 over Xn are as follows:

- UE Trajectory Collection Configuration and UE Performance Collection Configuration IEs in the DATA COLLECTION REQUEST message;

- Cell Based UE Trajectory Prediction and Data Collection ID IEs in the HANDOVER REQUEST message;

- UE Performance and Measured UE Trajectory IEs in the DATA COLLECTION UPDATE message.

As clearly stated in the WID [1], the specification work in Rel-19 for AI/ML for NG-RAN – including mobility optimization for NR-DC – is based on the conclusions captured in TR 38.743, which are excerpted from [3] and reported below:

# 6 Conclusion The new use cases and the Rel-18 leftover cases follow Rel-18 AI/ML framework. The following new use cases are recommended by RAN3 to be specified in the Rel-19 normative phase: - AI/ML-based Network Slicing - AI/ML-based Coverage and Capacity Optimization Recommended solutions and standard impacts for each use case are based on section 4.1.2 and section 4.2.2. For each use case above, it is recommended to take the corresponding section “Solutions and standard impacts” (section 4.1.2 and 4.2.2) as basis during the normative work. The following Rel-18 leftovers are recommended by RAN3 to be specified in Rel-19 normative phase: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states The corresponding descriptions and potential standard impacts for each Rel-18 leftovers above shall be taken as baseline during the normative phase.

During the Rel-19 SI, for mobility optimization in NR-DC, the use case description and potential standard impacts have been investigated and reported in TR 38.743 clause 5.1 as follows:

## 5.1 Mobility optimization for NR-DC ### 5.1.1 Use case description Mobility in NR-DC can be optimized by means of AI/ML. Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only and Conditional Dual Connectivity procedures are out of scope. ### 5.1.2 Potential Standard impacts The Dual Connectivity procedures (e.g., SN Addition, MN-initiated SN Change) are enhanced to trigger the collection of measured UE performance.

In the next sections we provide our view on how the DC procedures could be enhanced to trigger the collection of the measured UE performance at the Secondary Node (SN) side and reported over the Xn interface back to the Master Node (MN).

## 2.1 UE performance collection in NR-DC

The UE performance metrics to be reported as part of the feedback information after an AI/ML-based action as well as the UE collection configuration have been specified in Rel-18 for XnAP as follows [4]:

#### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4. Average UE Throughput UL; O; Bit Rate 9.2.3.4. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11. **** Skip non-relevant text **** #### 9.2.3.186 UE Performance Collection Configuration This IE indicates the configuration for UE performance measurement collection. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Collection Time Duration for UE Performance; M; INTEGER(1..5000, ...); Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond. as

It is obvious that the UE performance metrics are only for the basic single connectivity case, where the collection and reporting is done at the target node side towards the source node. For the case of NR-DC, RAN3 needs to investigate how UE performance metrics are triggered to be collected by the SN after a NR-DC has been configured/modified/changed for the concerned UE. It should be recalled that the Rel-18 UE performance metrics are on a per-UE granularity. Let’s first provide an insight on how these UE performance metrics are specified:

- Average UE throughput DL/UL. As specified in TS 28.552 [5], This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. Since the data volume is counted on the RLC SDU level, it is the RLC layer which is in charge of calculating the per DRB level Average UE throughput DL/UL.

- Average packet delay. As specified in TS 38.314 [6], the packet delay includes RAN part of delay and CN part of delay, and the RAN part delay is separately defined for UL and DL as excerpted from [6], where also the granularity of such measurements is indicated (i.e., per DRB):

#### 4.2.1.2 Packet delay ##### 4.2.1.2.1 General Packet delay includes RAN part of delay and CN part of delay. The RAN part of DL packet delay measurement comprises: - D1 (DL delay in over-the-air interface), referring to Average delay DL air-interface in TS 28.552 [2] 5.1.1.1.1. - D2 (DL delay on gNB-DU), referring to Average delay in RLC sublayer of gNB-DU in TS 28.552 [2] 5.1.3.3.3. - D3 (DL delay on F1-U), referring to Average delay on F1-U in TS 28.552 [2] 5.1.3.3.2. - D4 (DL delay in CU-UP), referring to Average delay DL in CU-UP in TS 28.552 [2] 5.1.3.3.1. The DL packet delay measurements, i.e. D1 (the DL delay in over-the-air interface), D2 (the DL delay in gNB-DU), D3 (the DL delay on F1-U) and D4 (the DL delay in CU-UP), should be measured per DRB per UE. NOTE: The delay measurements D1, D2 and D4 are also applicable for EUTRA in case of EN-DC related DL delay measurements on the MN side. The RAN part (including UE) of UL packet delay measurement comprises: - D1 (UL PDCP packet average delay, as defined in clause 4.3.1.1). - D2.1 (average over-the-air interface packet delay, as defined in 4.2.1.2.2). - D2.2 (average RLC packet delay, as defined in 4.2.1.2.3). - D2.3 (average delay UL on F1-U, it is measured using the same metric as the average delay DL on F1-U defined in TS 28.552 [2] clause 5.1.3.3.2). - D2.4 (average PDCP re-ordering delay, as defined in 4.2.1.2.4). The UL packet delay measurements, i.e. D1(UL PDCP packet average delay), D2.1(average over-the-air interface packet delay), D2.2(average RLC packet delay), D2.3(average delay UL on F1-U) and D2.4(average PDCP re-ordering delay), should be measured per DRB per UE. The unit of D1, D2.1, D2.2, D2.3 and D2.4 is 0.1ms. **** Skip non-relevant text ****

- Average packet loss DL. As specified in TS 38.314 [6], this metric is to measure packets that are lost at Uu transmission. And each packet is corresponding to one RLC SDU. Thus, this metric is also calculated at RLC level, and it is per DRB.

Below we provide in Table 1 a summary indicating the measured UE performance information that need to be collected from SN and reported to MN, as a function of the bearer type.

Table 1 - UE performance components in the SN side as a function of the bearer type.

| Bearer types | Average UE throughput DL/UL | Average Packet Delay DL | Average Packet Delay UL | Average Packet Loss DL |
| --- | --- | --- | --- | --- |
| MN-terminated split bearer | RLC level THP for SCG part. | D1, D2, measured in SCG. | D2.1, D2.2, measured in SCG. And potentially D1(if UE reports to SN). | RLC level packet loss for SCG part. |
| SN-terminated split bearer | RLC level THP for SCG part. | D1, D2, D4 measured in SCG. | D2.4 measured in SN. And potentially D1(if UE reports to SN). | RLC level packet loss for SCG part. |
| MN-terminated SCG bearer | RLC level THP in SCG. | D1, D2, measured in SCG. | D2.1, D2.2, measured in SCG. And potentially D1(if UE reports to SN). | RLC level packet loss in SCG. |
| SN-terminated MCG bearer | No need to provide. | D4 measured in SN. | D2.4 measured in SN. And potentially D1(if UE reports to SN). | No need to provide. |
| SN-terminated SCG bearer | RLC level THP in SCG. | Average packet delay DL calculated by SN. | Average packet delay UL calculated by SN. | RLC level packet loss in SCG. |

We need to note that though DL/UL delay calculation per split-DRB per UE is already defined in TS 38.314 [7] clause 4.2.1.9, the above-mentioned information exchanging over Xn interface from SN to MN are to help the MN to obtain a full understanding of UE performance in SN side. Hence, we suggest using the above table as a start point for analysing the UE performance feedback enhancements in NR-DC.

Observation 1: The current Rel-18 UE performance metrics need further enhancement to suit the NR-DC scenario.

Proposal 1: RAN3 to take Table 1 as the start point for analysing UE performance metrics enhancements in NR-DC so that SN-relevant UE performance metrics components can be collected by the SN and reported to the MN.

At NR-DC establishment, within the S-NODE ADDITION REQUEST message, the Data Collection ID IE can be included so that the collection and reporting of SN-relevant UE performance metrics components can be configured at the SN by exploiting the Data Collection Reporting (Initiation) procedures. Moreover, considering the MN may continuously require the UE performance in SN for each SN-related event (e.g., MN-initiated SN Modification, MN-initiated SN Change), the S-NODE MODIFICATION REQUEST and S-NODE RELEASE REQUEST messages should carry the Data Collection ID IE as well.

Proposal 2: The S-NODE ADDITION REQUEST, S-NODE MODIFICATION REQUEST and S-NODE RELEASE REQUEST messages should carry the Data Collection ID IE indicating the Data Collection Reporting (Initiation) procedures used to configure the collection and reporting of SN-relevant UE performance metrics.

Another important issue to discuss is the granularity of the UE performance metrics that the SN needs to report. As mentioned above, basically all UE performance metrics are on a per DRB level granularity. However, the UE performance metrics in clause 9.2.3.179 of TS 38.423 [4] are intended to be reported on a per-UE granularity. Thus, we suggest RAN3 further discussing this issue and choose the granularity of the UE performance metrics to be reported in NR-DC scenarios.

Proposal 3: RAN3 to further discuss and choose the granularity of the UE performance metrics to be reported in NR-DC scenarios (e.g., per UE as in Rel-18 or a finer granularity).

## 2.2 UE trajectory feedback in NR-DC

In the Rel-18 mobility optimization use case, it is the source gNB to predict the cell-based UE trajectory and, based on that information, to choose the next hop target gNB. And as in the current Rel-18 TS 38.423 [4], the source gNB can configure the target gNB to collect and report the measured UE trajectory via the Data Collection Reporting (Initiation) procedures, to help the source gNB evaluating the AI/ML-driven mobility decision. The source gNB configures the collection time duration and number of visited cells as triggers to stop the collection of measured UE trajectory information. And the target gNB will feedback the measured UE trajectory, including the list of cells that the UE visited and the corresponding time spent in each cell.

#### 9.2.3.182 Measured UE Trajectory The Measured UE Trajectory IE contains information related to NG-RAN cells where the UE connected after being handed over to the target NG-RAN node. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Measured UE Trajectory; 1; List of cells where the UE connected, in chronological order. Most recent information is added to the top of this list.. >Measured UE Trajectory Item; 1..<maxnoofCellsTrajectory>. >>Measured Trajectory Cell Information; M; 9.2.3.183. Range bound; Explanation. maxnoofCellsTrajectory; Maximum number of cells that can be reported for UE trajectory. Value is 16.. - 9.2.3.183 Measured Trajectory Cell Information The Measured Trajectory Cell Information contains information related to the NG-RAN cells where a UE connected after being handed over to the target NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE Measured Trajectory Cell Information; M. >NG-RAN Cell. >>Global NG-RAN Cell Identity; M; 9.2.2.27; Indicates an NR Cell Identity.. >>Time UE Stayed in Cell; M; INTEGER (0..4095); The duration of time the UE stayed in the cell If the duration is more than 4095s, this IE is set to 4095.. **** Skip non-relevant text **** #### 9.2.3.185 UE Trajectory Collection Configuration This IE contains configurations for UE trajectory collection after successful handover. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Collection Time Duration for UE Trajectory; M; INTEGER (1..4096, …); Time duration starting at successful handover within which UE Trajectory Information is collected. Unit: second. Number of Visited Cells; O; INTEGER (1..16, …); Maximum number of intra-node visited cells.. as

In the NR-DC, the MN may predict the SN for DC, but the detailed PSCell handover strategies are decided by the SN. This means that the MN is not aware of possible PSCell changes. With the measured UE trajectory among PSCells, the MN may evaluate whether the previous decision on the SN selection via AI/ML was accurate or AI/ML model’s optimization is needed. Besides, if the MN wants UE performance information in specific PSCells to be reported by the SN, the MN should also in advance be aware of the list of PSCells visited by the UE.

Observation 2: The MN should be aware of the list of PSCells visited by the UE.

In our opinion, the current measured UE trajectory collection configuration related signalling can be the baseline for further enhancements aimed at including the configuration for reporting information about the measured UE trajectory in PSCells.

Proposal 4: The current measured UE trajectory collection configuration related signalling should be further enhanced to include the configuration for reporting information about UE trajectory in PSCells.

# 3. Conclusion

Based on the above discussion, we have the following observations and proposals:

Observation 1: The current Rel-18 UE performance metrics need further enhancement to suit the NR-DC scenario.

Proposal 1: RAN3 to take Table 1 as the start point for analysing UE performance metrics enhancements in NR-DC so that SN-relevant UE performance metrics components can be collected by the SN and reported to the MN.

Proposal 2: The S-NODE ADDITION REQUEST, S-NODE MODIFICATION REQUEST and S-NODE RELEASE REQUEST messages should carry the Data Collection ID IE indicating the Data Collection Reporting (Initiation) procedures used to configure the collection and reporting of SN-relevant UE performance metrics.

Proposal 3: RAN3 to further discuss and choose the granularity of the UE performance metrics to be reported in NR-DC scenarios (e.g., per UE as in Rel-18 or a finer granularity).

Observation 2: The MN should be aware of the list of PSCells visited by the UE.

Proposal 4: The current measured UE trajectory collection configuration related signalling should be further enhanced to include the configuration for reporting information about UE trajectory in PSCells.

# 4. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP TS 38.423, NG-RAN; Xn Application Protocol (XnAP), V18.3.0 (2024-09)

- 3GPP TS 28.552, Management and orchestration; 5G performance measurements, V19.1.0 (2024-09)

- 3GPP TS 38.314, NR; Layer 2 measurements, V18.0.0 (2024-03)