3GPP TSG-RAN WG3 Meeting #125bis	R3-245422

Hefei, China, 14th – 18th October, 2024

Title: 	On Continuous MDT for AI/ML

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In RAN#105 the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” was approved in [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (latest SID in [2]) whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to the Continuous MDT collection targeting the same UE across RRC states, as excerpted from [1] and highlighted in yellow below:

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743. The detailed objectives of the WI are listed as follows: - Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3] - Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states Note: Coordination with RAN2, SA5 when needed if any.

This contribution analyzes the technical enablers for the Continuous MDT feature to be specified and provides proposals for discussion.

# 2. Background

As stated before, RAN3 in Rel-19 will specify support for the Rel-18 leftover from the AI/ML for NG-RAN WI related to Continuous MDT, among others. As clearly stated in the WID, the specification work in Rel-19 for AI/ML for NG-RAN is based on the conclusions captured in TR 38.743, which are excerpted from [3] and reported below:

# 6 Conclusion The new use cases and the Rel-18 leftover cases follow Rel-18 AI/ML framework. The following new use cases are recommended by RAN3 to be specified in the Rel-19 normative phase: - AI/ML-based Network Slicing - AI/ML-based Coverage and Capacity Optimization Recommended solutions and standard impacts for each use case are based on section 4.1.2 and section 4.2.2. For each use case above, it is recommended to take the corresponding section “Solutions and standard impacts” (section 4.1.2 and 4.2.2) as basis during the normative work. The following Rel-18 leftovers are recommended by RAN3 to be specified in Rel-19 normative phase: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states The corresponding descriptions and potential standard impacts for each Rel-18 leftovers above shall be taken as baseline during the normative phase.

During the Rel-19 SI, for Continuous MDT, the use case description and problems to be solved have been acknowledged and reported in TR 38.743 clause 5.4 as follows:

## 5.4 Continuous MDT collection targeting the same UE across RRC states ### 5.4.1 Use Case Description The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection. The Data Collection continuity in this scenario can be split into two tasks as below: - Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states. - Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE. ### 5.4.2 Potential Standard impacts Potential solutions addressing the problems above can be discussed during normative phase.

The main motivation for specifying the Continuous MDT feature arose during the Rel-18 WI “AI/ML for NG-RAN” [4], in an effort to address the issue on how to collect measurements from UE(s) to be used as datasets for AI/ML models’ (re-)training. Back then it was argued that, for the AI/ML model (re-)training to be successful, the MDT data collected from the UE should be consecutive – i.e., data series not affected by time interruptions due to, e.g., stop of UE data collection and successive resuming – and a time series of information with a certain granularity. It was claimed that, when the UE enters to connected state from idle/inactive or when the UE is subject to connected mode mobility (handover), consecutive (without time gaps) information would not be collected, with the consequence that the data gathered from the UE would not constitute a valid training dataset for AI/ML models’ (re-)training. By means of the Continuous MDT, that is, an enhancement of the current MDT mechanism suited for AI/ML models’ (re-)training, the continuous collection of MDT data from the same UE during the same RRC state as well as across RRC states can be enabled.

The baseline framework for the data collection from the UE, the definition of Continuous MDT and its main application (i.e., AI/ML model training in OAM) were already agreed in RAN3 during the Rel-18 WI (RAN3#119bis-e meeting) [5]:

The existing MDT framework is used as baseline for data collection from the UE. Continuous collection of MDT traces is beneficial only for AI/ML training in OAM. Continuous MDT collection is to enable the continuous collection of MDT data from the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive).

The solution space for enabling the Continuous MDT feature was also investigated during the Rel-18 WI, by considering both management-based and signalling-based MDT, but only shortcomings of current MDT mechanisms were acknowledged in RAN3, which prevented RAN3 to agree on specific enhancements for Continuous MDT in Rel-18 (RAN3#121 meeting) [6]:

The following understanding is based on R18 use cases: Signalling-based MDT could provide continuous Data Collection from a certain UE, however, it has scalability issues when selecting large number of UEs and lacks cell-level granularity. Management-based MDT provides the advantage of being scalable when selecting large number of UEs. However, current management-based MDT mechanism cannot identify and group together MDT reports collected from the same UE across RRC states. UE selection is left to RAN implementation and it is not controllable by OAM. It is proposed that solutions to address the above shortcomings are not pursued in Rel.18.

In the next Section 3 we provide our views on how to properly address Problem A and Problem B as described in TR 38.743 clause 5.4.1 so to actually enable the Continuous MDT feature in Rel-19.

# 3. Discussion

## 3.1	Existing MDT framework for Continuous MDT

The MDT framework has been specified to mainly serve the purpose of collecting data from the UE over the Uu interface.

When MDT was specified in Rel-10, two types of MDT have been introduced: signalling-based MDT and management-based MDT. The former is used to collect measurements from a certain UE which is selected in OAM based on a permanent UE identity – Subscription Permanent Identifier (SUPI) or International Mobile Station Equipment Identity Software Version (IMEISV) – while the MDT configuration parameters are sent to the UDM in 5GC which takes the final decision to activate the MDT process for that UE based on whether the corresponding end user has consented to contribute to MDT. Then the UDM perform MDT activation towards the AMF by providing it with the MDT configuration parameters received from the OAM, and finally the gNB receives from the AMF MDT configuration parameters and starts a Trace session towards the UE according to the received parameters.

In management-based MDT, instead, measurements are collected from randomly chosen UEs or a group of UEs that enter a (certain) geographical area. Management-based MDT is typically activated in RAN nodes of a specific area (identified by a list of NG-RAN cells) and UE selection is performed by RAN nodes based on parameters received directly from the OAM, UE radio capability and other information such as user consent and roaming status.

Based on the above, it is clear that the main difference between signalling- and management-based MDT is the granularity of the UE selection process. By recalling the agreement from RAN3#119bis-e meeting that [5]:

The existing MDT framework is used as baseline for data collection from the UE.

and that data collection, in turn, serves the purpose of providing sufficient amount of data as input for AI/ML model (re-)training, we need to ensure that data is collected from as many UEs as possible, under different radio environments and RRC states. This is also motivated by the fact that the input data collection for AI/ML model (re-)training is not a time-critical task, it is a long-term and continuous task. In our understanding, fulfilling these requirements for proper data collection from UEs can mainly be achieved via management-based MDT. Moreover, TR 38.743 clause 5.4.1, explicitly refers to management-based MDT for the Continuous MDT problem statement as follows:

### 5.4.1 Use Case Description The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection. **** Skipped non-relevant text ****

Based on the above we propose:

Proposal 1: RAN3 to discuss and agree that the existing management-based MDT mechanism is used as baseline to ensure proper data collection for the purpose of Continuous MDT.

## 3.2	Trace correlation issue

The main limitation with management-based MDT to be used as the baseline approach for Continuous MDT, however, is related to the MDT data anonymization requirement, which impacts Problem B (trace correlation) as described in clause 5.4.1 of TR 38.743. TS 32.422, in fact, states that anonymization of MDT data gathered via management-based MDT is mandatory for both MDT types (the so-called Job Types), i.e., Immediate MDT and Logged MDT. In order to understand how this requirement could impact the possibility to actually implement the concept of Continuous MDT we can refer to the following Figure 1.

In Figure 1 it is assumed that the OAM has provided the NG-RAN with a management-based MDT configuration consisting of an Immediate MDT configuration and a Logged MDT configuration; each of these MDT configurations has been assigned with a separate Trace Reference (TR), i.e., TR#1 and TR#2 for Immediate MDT and Logged MDT, respectively. It is then the NG-RAN (gNB in Figure 1) that activates the MDT feature towards a certain number of UEs (for simplicity, only a single UE is represented in Figure 1). When the UE is in connected mode, it performs Immediate MDT measurements according to the Immediate MDT configuration it has received from the NG-RAN and reports those measurements to the NG-RAN itself which, in turn, stores the received MDT data into a Trace Record file and sends this file to the Trace Collection Entity (TCE). Then, when the UE goes to idle/inactive mode, it performs the Logged MDT measurements as per the Logged MDT measurement configuration provided by the NG-RAN. Once back in connected mode, and assuming for simplicity that the UE resumes under the same gNB as in Figure 1, the UE will report the MDT measurements performed while it was in idle/inactive mode to the gNB which, again, stores the received MDT data into another Trace Record file and sends this file to the TCE.

Figure 1 – Management-based MDT data acquisition from the same UE.

In order to actually implement the Continuous MDT concept and, more specifically, to create datasets that are represented as a time series of continuous information and that can be used for proper (re-)training of AI/ML models – i.e., data series not affected by time interruptions due to e.g. stop of UE data collection and successive resuming – it is needed that TCE identifies that the Immediate and Logged MDT measurement reports are from the same UE so that the TCE can correlate these MDT reports, hence creating a unique dataset. And since this unique dataset has been built by exploiting the MDT reports provided by the same UE irrespective of its RRC state, such dataset is a consistent representation of the network, not dependent on e.g. UE capabilities, UE implementation, etc.

However, as per the anonymization requirement of management-based MDT data, the MDT reports provided to the TCE are anonymized on purpose, hence it seems that currently there are no means for the TCE to recognize which MDT reports are provided by the same UE.

It is worth to highlight that there is no need for the TCE to identify the UE which provided the Immediate and Logged MDT reports, but what it is actually needed is that the TCE identifies which MDT reports are provided by the same UE so that the TCE can correlate these reports and hence create a unique dataset which can be used for AI/ML (re-)training purposes.

Observation 1: To enable Continuous MDT via management-based MDT, the TCE needs to be able to identify which management-based MDT reports (Logged and Immediate) are provided by the same UE and there is no need for the TCE to identify/recognize that UE.

In Rel-18 discussions there was an attempt to check whether existing identifiers, e.g. IMEI-TAC, NR CGI, TR/TRSR could be used for trace correlation at TCE level [7]. In our previous contribution in [8] we provided an in-depth analysis of such identifiers and concluded that they are not suited for being used in the trace correlation in TCE.

Observation 2: Neither IMEI-TAC, NR-CGI nor the use of TR/TRSR as per the current SA5 specifications can be used at TCE level to correlate Immediate MDT and Logged MDT reports from the same UE.

One possibility to address this issue could be to introduce a new Job Type in SA5 specifications which allows to configure a MDT session encompassing Immediate MDT and Logged MDT simultaneously, and assigning to this MDT session the same Trace Reference (TR). This should be possible because from SA5 TS 32.422 §5.9a it is already possible to configure a Job Type for two modes – refer to, e.g., Job Type value 3 which is Immediate MDT and Trace. Hence, if a new Job Type such as “Immediate MDT and Logged MDT” is introduced, then the same TR could be assigned to the MDT session corresponding to this new Job Type. TS 32.422 states that the OAM sends a Trace Session activation request to the gNB with configuration parameters including the Job Type and the TR. Then the gNB activates the MDT functionality for the selected UE and assigns a Trace Recording Session Reference (TRSR) to the selected UE.

Observation 3: The current specifications state that the OAM provides the TR while the TRSR is assigned by the gNB on a per-UE basis within the same MDT session identified by the specific TR.

Therefore, in management-based MDT, a single TR could be assigned to an MDT session with a Job Type equal to “Immediate MDT and Logged MDT”, and this configuration is valid for all UEs selected by the NG-RAN. In order to then correlate the Immediate and Logged MDT reports from the same UE (properly anonymized, e.g., via IMEI-TAC), the TCE could perform such correlation by “collating” those MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE, hence solving Problem B (trace correlation) in [3].

Since the above solution has impacts on SA5, it is proposed to liaise with SA5 asking for feedback on its feasibility.

Proposal 2: RAN3 to discuss and agree on the solution based on a new Job Type “Immediate MDT and Logged MDT” for solving Problem B (trace correlation). A LS to SA5 should be sent asking for feedback on the feasibility of such solution in Rel-19.

A draft LS to SA5 can be found in the Annex of this contribution.

## 3.3	Measurement continuity across UE’s RRC state changes and during the same RRC state

Aside Problem B (trace correlation), a proper Continuous MDT solution needs to address also Problem A (measurement continuity) in [3], that is, how to ensure that the same UE keeps on collecting MDT measurements both across different RRC states (i.e., after idle/inactive to connected state transition) as well as during the same RRC state (that is, after handover from source gNB to target gNB).

For the first case (i.e., after idle/inactive to connected state transition), the scenario involves a UE which has been configured with Immediate and Logged MDT and, after having performed Logged MDT measurements while in idle/inactive state, it re-establishes/resumes its RRC connection in a new cell and the UE reports the availability of Logged MDT measurements to the (potentially new) serving gNB. In order to avoid time interruptions in the measurement process, the UE should be re-configured to perform Immediate MDT immediately after it re-establishes/resumes the RRC connection, i.e., just after it moves to connected state.

Observation 4: For Problem A (measurement continuity) across different RRC states (i.e., after idle/inactive to connected state transition) the UE should be re-reselected by the (potentially new) serving gNB and re-configured to perform Immediate MDT immediately, i.e., just after it moves to connected state.

Considering that if the OAM provides a management-based MDT configuration with Job Type “Immediate MDT and Logged MDT” to gNBs in a certain area, this means that UEs that will be selected by the NG-RAN and configured according the OAM configuration are the only ones for which the Continuous MDT feature is requested to be enabled. To ensure measurement continuity across different RRC states it could be possible to exploit a network-based solution where the gNB serving a Continuous MDT UE sends parts of the MDT configuration to the AMF (e.g., within the NG: UE CONTEXT RELEASE REQUEST) which stores such information. These parts of the MDT configuration can be stored within the UE context in the 5GC/AMF by default only for those UEs selected for Continuous MDT, i.e., configured with a management-based MDT configuration with Job Type “Immediate MDT and Logged MDT”. In this manner, once the UE re-establishes/resumes its RRC connection, the (potentially new) serving gNB will be provided by the 5GC/AMF with UE context data including user consent information as well as the stored parts of the MDT configuration (e.g., within the NG: INITIAL UE CONTEXT SETUP REQUEST). These stored parts of the MDT configuration can be e.g. the Job Type “Immediate MDT and Logged MDT” and the TRSR previously allocated to the Continuous MDT UE by the (old) serving gNB, and these pieces of information can be used as an indication that the UE was previously selected for Continuous MDT, so that the (potentially new) serving gNB can re-select the same UE and re-configure the UE to continue performing the Immediate MDT measurements in connected mode. This removes the burden of the (potentially new) serving gNB to recognize which UE has been previously selected and configured with the Continuous MDT by the (old) serving gNB, because it will become aware of this by simply checking the presence of the parts of the MDT configuration in the UE context data provided by the 5GC/AMF. If present, this means that the UE was previously selected for Continuous MDT by the (old) serving gNB and needs to be re-selected and re-configured again.

Observation 5: Measurement continuity across different RRC states can be achieved by storing parts of the MDT configuration in 5GC/AMF (e.g., Job Type “Immediate MDT and Logged MDT” and the TRSR allocated to the UE by the serving gNB) at RRC state transition from connected to idle only for those UEs selected for Continuous MDT, i.e., configured with a management-based MDT configuration with Job Type “Immediate MDT and Logged MDT”.

Observation 6: After RRC state transition from idle/inactive to connected, the (potentially new) serving gNB will be provided by the 5GC/AMF with UE context data including user consent information as well as the stored parts of the MDT configuration (e.g., Job Type “Immediate MDT and Logged MDT” and the TRSR allocated to the UE by the (old) serving gNB), which can be used as an indication that the UE was previously selected for Continuous MDT, and hence the (potentially new) serving gNB can re-select the same UE and re-configure the UE to continue performing the Immediate MDT measurements in connected mode.

The above approach is summarized in the following Figure 2, where user consent of the UE to be selected for Continuous MDT is also handled based on current specifications (e.g., TS 32.422), and changes with respect to legacy management-based MDT to enable the Continuous MDT feature are highlighted in yellow.

Figure 2 - Network-based solution for management-based Continuous MDT: TCE correlation and measurement continuity across RRC state changes (from idle/inactive to connected transition).

- As part of the UE Context Setup procedure the AMF – which previously stored user consent information for UE1 from the UDM, not shown in Figure 2 – provides the gNB1 with user consent information if the user is within its home operator’s PLMNs and the user has given its consent, and user consent information of UE1 are stored in gNB1;

- OAM provides the gNB1 (and other gNBs in the Area scope, i.e., gNB2) with a management-based MDT configuration having Job Type “Immediate MDT and Logged MDT” and a corresponding TR “TR#1”;

- gNB1 performs user selection among consented users (or users for which consent is not required) and, since it has received user consent information for UE1 in Step 1., UE1 is selected for Continuous MDT

- gNB1 configures the UE1 to perform MDT measurements based on the management-based MDT configuration previously provided by the OAM and assigns a UE-specific TRSR “TRSR#1”;

- UE1 is in connected mode, hence it performs and reports to gNB1 the Immediate MDT measurements according to the management-based MDT configuration provided by the gNB1;

- gNB1 saves the received Immediate MDT measurements into a Trace Record file;

- The Trace Record file containing the Immediate MDT measurements is sent to the TCE including the couple TR/TRSR, i.e., “TR#1 / TRSR#1”;

- We now consider the case where UE1 moves to idle mode: in this RRC state UE1 performs (and logs) Logged MDT measurements according to the management-based MDT configuration provided by the gNB1 in Step 4.;

- As part of the UE Context Release procedure the gNB1 send at least the Job Type “Immediate MDT and Logged MDT” and “TRSR#1” allocated to UE1 to the AMF which stores such information;

- We now assume that UE1 triggers transition from idle to connected mode under a new serving gNB, i.e., gNB2; at this point gNB2 is not aware that UE1 was previously selected by gNB1 for Continuous MDT measurements;

- At the end of the RRC Setup procedure involving UE1 and gNB2, gNB2 receives the availability indication from UE1 related to Logged MDT measurements available in the UE1;

- As part of the UE Context Setup procedure the AMF prepares UE context data of UE1 (e.g., PDU session context, security Key, UE Radio Capability and UE Security Capabilities, etc.) and sends it to gNB2; AMF also checks if the user is within its home operator’s PLMNs and the user has given its consent: if so, the AMF provides gNB2 with user consent information of UE1 as well as Job Type “Immediate MDT and Logged MDT” and “TRSR#1” allocated to UE1 by gNB1 and stored in AMF in Step 9.;

- UE context information of UE1, including user consent, Job Type “Immediate MDT and Logged MDT” and “TRSR#1”, are stored in gNB2;

- UE1 is now in connected mode under the coverage of gNB2;

- gNB2 performs user selection among consented users (or users for which consent is not required) and, since it has received user consent information for UE1 in Step 12., UE1 can be a candidate. The presence of the Job Type “Immediate MDT and Logged MDT” and “TRSR#1” is an indication for gNB2 that UE1 was previously selected by gNB1 for Continuous MDT, implying that UE1 needs to be re-selected and re-configured by gNB2 to continue performing MDT measurements;

- As part of the UE RRC Reconfiguration towards UE1, gNB2 re-configures the UE1 to perform MDT measurements based on the management-based MDT configuration previously provided by the OAM in Step 2. and assigns a UE-specific TRSR: for simplicity, it is assumed that the same “TRSR#1” previously assigned to UE1 by gNB1 is also re-assigned by gNB2 – this is possible if “TRSR#1” was not already assigned to another UE served by gNB2, and how to handle TRSR assignment conflicts is discussed below as part of the measurement continuity during the same RRC state (i.e., at handover); UE1 can re-start Immediate MDT measurements;

- gNB2 retrieves the Logged MDT measurements from UE1;

- gNB2 saves the received Logged MDT measurements of UE1 into another Trace Record file;

- The Trace Record file containing the Logged MDT measurements is sent to the TCE including the couple TR/TRSR, i.e., “TR#1 / TRSR#1”;

- The TCE correlates the Trace Record files received from gNB1 in Step 7. (with Immediate MDT measurements of UE1) and from gNB2 in Step 19. (with Logged MDT measurements of UE1) as both Trace Record files are “marked with” (and hence identifiable by) the same couple TR/TRSR, i.e., “TR#1 / TRSR#1”.

Based on the above we propose the following:

Proposal 3: RAN3 to discuss and agree on a network-based solution for Continuous MDT for solving Problem A (measurement continuity) across RRC state changes (i.e., after idle/inactive to connected state transition) based on storing in AMF/5GC at least Job Type “Immediate MDT and Logged MDT” and TRSR assigned to the Continuous MDT UE. The stored information is then provided by the AMF/5GC (pending availability of the user consent) to the (potentially new) serving gNB so that the same UE can be re-selected and re-configured with management-based MDT immediately after it re-establishes/resumes its RRC connection.

Let’s now discuss the second scenario for Problem A (measurement continuity) in [3], that is, how to ensure that the same UE keeps on collecting MDT measurements during the same RRC state (that is, after handover from source gNB to target gNB). This scenario also addresses the TRSR allocation conflict issue between source and target gNB for the handed over UE, whose solution is also applicable for the scenario in Figure 2. The scenario is depicted in Figure 3 below where, same as in Figure 2, changes with respect to legacy management-based MDT to enable the Continuous MDT feature are highlighted in yellow:

Figure 3 - Network-based solution for management-based Continuous MDT: TCE correlation and measurement continuity during the same RRC state (connected mode mobility).

Step 1. to Step 6. are exactly as for Figure 2.

- UE1 is in connected mode under the coverage of gNB1 (source gNB), hence it performs and reports to gNB1 the Immediate MDT measurements according to the management-based MDT configuration provided by the gNB1. Due to radio conditions UE1 needs to be handed over to gNB2, therefore gNB1 initiates handover to gNB2;

- As per current TS 32.422 clause 4.10, UE1 stops the ongoing Immediate MDT measurements performed while under the coverage of gNB1;

- gNB1 saves the received Immediate MDT measurements into a Trace Record file which is sent to the TCE including the couple TR/TRSR, i.e., “TR#1 / TRSR#1”;

- As part of the Handover Preparation procedure, gNB1 sends to gNB2 the user consent information for UE1 and at least the Job Type “Immediate MDT and Logged MDT” and “TRSR#1” allocated to UE1 by gNB1;

- UE context information of UE1, including user consent, Job Type “Immediate MDT and Logged MDT” and “TRSR#1”, are stored in gNB2;

- The Handover procedure for UE1 from gNB1 to gNB2 takes place, at the completion of which UE1 accessed the target cell in gNB2;

- UE1 is now in connected mode under the coverage of gNB2;

- gNB2 performs user selection among consented users (or users for which consent is not required) and, since it has received user consent information for UE1 in Step 10., UE1 can be a candidate. The presence of the Job Type “Immediate MDT and Logged MDT” and “TRSR#1” is an indication for gNB2 that UE1 was previously selected by gNB1 for Continuous MDT, implying that UE1 needs to be re-selected and re-configured by gNB2 to continue performing MDT measurements;

- As part of the UE RRC Reconfiguration towards UE1, gNB2 re-configures the UE1 to perform MDT measurements based on the management-based MDT configuration previously provided by the OAM in Step 2. and assigns a UE-specific TRSR: it is assumed that the “TRSR#1” previously assigned to UE1 by gNB1 cannot be re-assigned by gNB2 to UE1 because “TRSR#1” was already assigned to another UE served by gNB2, therefore a new TRSR “TRSR#2” (different from “TRSR#1”) is allocated to UE1 by gNB2;

- Since UE1 has been allocated with a new TRSR (“TRSR#2”) it is important to keep the mapping relationship of TRSRs allocated to UE1 for the sake of trace correlation at TCE level – this mapping relationship can be kept by the target gNB (gNB2 in Figure 3);

- UE1 can re-start Immediate MDT measurements and report them to gNB2;

- gNB2 saves the received Immediate MDT measurements into another Trace Record file;

- The Trace Record file is sent by gNB2 to the TCE including the couple TR/TRSR, i.e., “TR#1 / TRSR#2”, as well as the mapping relationship of TRSRs allocated to UE1;

- The TCE correlates the Trace Record file received from gNB1 in Step 9. containing Immediate MDT measurements of UE1 performed under the coverage gNB1 and marked with the couple “TR#1 / TRSR#1” with the Trace Record file from gNB2 in Step 19. containing Immediate MDT measurements of UE1 performed under the coverage gNB2 and marked with the couple “TR#1 / TRSR#2”; this is possible because TCE also received the mapping relationship of TRSRs allocated to UE1 by the gNBs.

Based on the above we have the following proposals:

Proposal 4: RAN3 to discuss and agree on a network-based solution for Continuous MDT for solving Problem A (measurement continuity) during the same RRC state (i.e., connected mode mobility) by providing from source gNB to target gNB in the Handover Preparation at least Job Type “Immediate MDT and Logged MDT” and TRSR assigned by the source gNB to the Continuous MDT UE. The provided information, along with availability of the user consent, can be used by the target gNB in order for the same UE to be re-selected and re-configured with management-based MDT immediately after handover completion.

Proposal 5: RAN3 to discuss and agree on the target gNB to keep the mapping relationship of TRSRs allocated to the Continuous MDT UE in case of TRSR allocation conflicts between source gNB and target gNB. The target gNB provides the mapping relationship of TRSRs to the TCE for the sake of trace correlation.

# 4. Conclusion

Based on the discussion in this paper, we provide the following observations and proposals:

Proposal 1: RAN3 to discuss and agree that the existing management-based MDT mechanism is used as baseline to ensure proper data collection for the purpose of Continuous MDT.

Trace correlation issue

Observation 1: To enable Continuous MDT via management-based MDT, the TCE needs to be able to identify which management-based MDT reports (Logged and Immediate) are provided by the same UE and there is no need for the TCE to identify/recognize that UE.

Observation 2: Neither IMEI-TAC, NR-CGI nor the use of TR/TRSR as per the current SA5 specifications can be used at TCE level to correlate Immediate MDT and Logged MDT reports from the same UE.

Observation 3: The current specifications state that the OAM provides the TR while the TRSR is assigned by the gNB on a per-UE basis within the same MDT session identified by the specific TR.

Proposal 2: RAN3 to discuss and agree on the solution based on a new Job Type “Immediate MDT and Logged MDT” for solving Problem B (trace correlation). A LS to SA5 should be sent asking for feedback on the feasibility of such solution in Rel-19.

A draft LS to SA5 can be found in the Annex of this contribution.

Measurement continuity across UE’s RRC state changes and during the same RRC state

Observation 4: For Problem A (measurement continuity) across different RRC states (i.e., after idle/inactive to connected state transition) the UE should be re-reselected by the (potentially new) serving gNB and re-configured to perform Immediate MDT immediately, i.e., just after it moves to connected state.

Observation 5: Measurement continuity across different RRC states can be achieved by storing parts of the MDT configuration in 5GC/AMF (e.g., Job Type “Immediate MDT and Logged MDT” and the TRSR allocated to the UE by the serving gNB) at RRC state transition from connected to idle only for those UEs selected for Continuous MDT, i.e., configured with a management-based MDT configuration with Job Type “Immediate MDT and Logged MDT”.

Observation 6: After RRC state transition from idle/inactive to connected, the (potentially new) serving gNB will be provided by the 5GC/AMF with UE context data including user consent information as well as the stored parts of the MDT configuration (e.g., Job Type “Immediate MDT and Logged MDT” and the TRSR allocated to the UE by the (old) serving gNB), which can be used as an indication that the UE was previously selected for Continuous MDT, and hence the (potentially new) serving gNB can re-select the same UE and re-configure the UE to continue performing the Immediate MDT measurements in connected mode.

Proposal 3: RAN3 to discuss and agree on a network-based solution for Continuous MDT for solving Problem A (measurement continuity) across RRC state changes (i.e., after idle/inactive to connected state transition) based on storing in AMF/5GC at least Job Type “Immediate MDT and Logged MDT” and TRSR assigned to the Continuous MDT UE. The stored information is then provided by the AMF/5GC (pending availability of the user consent) to the (potentially new) serving gNB so that the same UE can be re-selected and re-configured with management-based MDT immediately after it re-establishes/resumes its RRC connection.

Proposal 4: RAN3 to discuss and agree on a network-based solution for Continuous MDT for solving Problem A (measurement continuity) during the same RRC state (i.e., connected mode mobility) by providing from source gNB to target gNB in the Handover Preparation at least Job Type “Immediate MDT and Logged MDT” and TRSR assigned by the source gNB to the Continuous MDT UE. The provided information, along with availability of the user consent, can be used by the target gNB in order for the same UE to be re-selected and re-configured with management-based MDT immediately after handover completion.

Proposal 5: RAN3 to discuss and agree on the target gNB to keep the mapping relationship of TRSRs allocated to the Continuous MDT UE in case of TRSR allocation conflicts between source gNB and target gNB. The target gNB provides the mapping relationship of TRSRs to the TCE for the sake of trace correlation.

# 5. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- RP-233441, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (CMCC, Ericsson), 3GPP RAN#102

- 3GPP RAN3#119bis-e Chairman’s notes (RAN3_119bis-e_agenda_20230426_EOM1.doc)

- 3GPP RAN3#121 Chairman’s notes (RAN3_121_agenda_20230825_EOM1.doc)

- 3GPP RAN3#120 Chairman’s notes (RAN3_120_agenda_20230530_EOM1.doc)

- R3-243471, On Continuous MDT for AI/ML, Huawei

# 6. Annex – Draft LS to SA5

3GPP TSG-RAN3 Meeting #125bis	R3-24abcd

Hefei, China, 14th – 18th October, 2024

Title:	[Draft] LS on Continuous MDT for AI/ML model training in OAM

Response to:

Release:	Release 19

Work Item:	NR_AIML_NGRAN_enh-core

Source:	Huawei [to be RAN3]

To:	SA5

Cc:	RAN2

Contact person:	Damiano Rapone

damiano.rapone@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

As part of the Rel-19 Work Item on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, RAN3 agreed to specify an enhancement of the MDT framework termed as Continuous MDT in order to continuously collect measurements from the same UE during the same RRC state (i.e., during connected mode mobility) and across RRC states (i.e., after idle/inactive to connected mode transition) that could be used for the AI/ML model (re-)training in OAM.

In order for the AI/ML model (re-)training to be successful the collected data from the UE should be consecutive – i.e., data series not affected by time interruptions due to, e.g., stop of UE data collection and successive resuming – and a time series of information with a certain granularity. RAN3 discussed solutions based on management-based MDT to tackle the following requirements as in TR 38.743:

- measurement continuity: how to ensure that the same UE keeps on collecting MDT measurements during the same RRC state and across different RRC states;

- trace correlation: how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

The above requirements could be fulfilled if a new Job Type is introduced in SA5 specifications, which allows the OAM to configure a MDT session encompassing Immediate MDT and Logged MDT simultaneously, and assigning to this MDT session the same Trace Reference (TR), while the Trace Recording Session Reference (TRSR) is assigned by the gNB on a per-UE basis within the same MDT session identified by the specific TR. In this manner, a single TR could be assigned to an MDT session with a Job Type equal to “Immediate MDT and Logged MDT”, and this configuration is valid for all UEs selected by the NG-RAN to perform management-based MDT. In order to then correlate the Immediate and Logged MDT reports from the same UE (properly anonymized, e.g., via IMEI-TAC), the TCE performs such correlation by “collating” the MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE, hence realizing the Continuous MDT for the UE.

# 2	Actions

To SA5

ACTION: 	RAN3 respectfully asks SA5 to take the above solution description into consideration and provide feedback to RAN3 on its feasibility in Rel-19.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#126				2024-11-18 – 2024-11-22				Orlando, US

RAN3#127				2025-02-17 – 2025-02-21				Athens, GR