3GPP TSG-RAN WG3 Meeting #125bis   							     		R3-245424

Hefei, China, October 14th - 18th, 2024

Agenda Item:	20.2

Source:	Huawei

Title:	Discussion on RAN3 impacts for AI/ML assisted positioning

Document for:	Discussion and decision

# Introduction

At the RAN#102 meeting, a new Rel-19 WI on AI for air interface [1]  was approved and the corresponding WID has been revised in RAN#105 meeting [2]. One objective which has RAN3 impacts is the specification support for positioning accuracy enhancements as follows:

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: **** Skip non-relevant text **** - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

This is the first meeting for RAN3 to discuss about the RAN3 impacts for supporting AI/ML-based positioning use cases, and in this contribution, we focus on AI/ML assisted positioning use cases and provide proposals for discussion accordingly. The contribution in [3], instead, focuses on RAN3 impacts for supporting Direct AI/ML positioning use cases.

# Discussion

The following sections will discuss the potential RAN3 impacts for AI/ML assisted positioning use cases, which are listed in the WID [2] – along with the corresponding prioritization – as excerpted below:

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: **** Skip non-relevant text **** - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: **** Skip non-relevant text **** - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning **** Skip non-relevant text **** - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

For AI/ML positioning, the preliminary Rel-18 study phase – whose outcome is summarized in TR 38.843 [4] – identified two possible approaches depending on the type of AI/ML model output as follows:

## 5.3 Positioning accuracy enhancements Finalization of representative sub-use cases: The following are selected as representative sub-use cases: - Direct AI/ML positioning: - AI/ML model output: UE location - e.g., fingerprinting based on channel observation as the input of AI/ML model - AI/ML assisted positioning: - AI/ML model output: new measurement and/or enhancement of existing measurement - e.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement **** Skip non-relevant text ****

Based on the definition in TR 38.843 [4], for AI/ML assisted positioning use cases the output of the AI/ML model is a new measurement and/or enhancement of existing measurement. The output of this AI/ML model will then be used to derive the UE location via existing positioning methods in the LMF.

Observation 1: For AI/ML assisted positioning use cases the output of the AI/ML model is a new measurement and/or enhancement of existing measurement.

Moreover, depending on the location of the AI/ML model used to generate the new measurement / enhancement of existing measurement (e.g. network-side or UE-side), there can be multiple cases which have been agreed to be specified according to a clear prioritization as indicated in the WID [2].

Observation 2: AI/ML assisted positioning use cases are distinguished based on the location of the AI/ML model used to generate measurements to then be used at LMF side to determine the UE location.

As indicated in the latest version of the WID in [2], a Note has been added stating that RAN1 will not discuss any proposals for AI/ML positioning cases which have been identified as 2nd priority cases in Q4 2024. Therefore, we start focusing our attention on the following 1st priority use case highlighted in yellow below:

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: **** Skip non-relevant text **** - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: **** Skip non-relevant text **** - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning **** Skip non-relevant text **** - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

In our understanding, RAN3 should focus on Case 3a above for AI/ML assisted positioning since Case 2a does not have any RAN3 impact. Case 2a, in fact, is an AI/ML assisted positioning case where the UE hosts an AI/ML model which is used to generate measurements (new or enhancement of existing ones) that will be reported to the LMF directly so that the LMF can determine the UE location via existing positioning methods.

Observation 3: Among the AI/ML assisted positioning cases, Case 2a does not have any RAN3 impact.

Based on the above Observations we provide the following:

Proposal 1: RAN3 to discuss the impacts of Case 3a for AI/ML assisted positioning.

In the following section 2.1 we discuss aspects related to model training, model inference and model monitoring for AI/ML assisted positioning Case 3a.

## 2.1 RAN3 impact for Case 3a

Case 3a is NG-RAN node-assisted positioning with a gNB-side model. A graphical representation of Case 3a is given in Figure 1.

Figure 1 - AI/ML assisted positioning Case 3a.

In order to assess the RAN3 impacts for supporting AI/ML assisted positioning Case 3a we will recall in below sections the relevant RAN1 agreements achieved so far concerning model training, model inference and model monitoring aspects.

### Model training considerations

In previous RAN1 meetings the following agreements were reached:

******** RAN1#116bis ******** Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: "Part A" and "Part B" terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. ******** RAN1#117 ******** Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed. Note: Part A and Part B may be generated by the same or different entities, depending on the cases.

For AI/ML assisted positioning Case 3a our understanding is that the AI/ML model training can be located either in the gNB – which also hosts the AI/ML model – or in the OAM. This is based on what has been reported in TR 38.843 clause 7.2.4, where the training data generation and termination points are identified for the gNB-side model as follows [4]:

### 7.2.4 Positioning accuracy enhancements **** Skip non-relevant text **** For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold when the data generation and termination entities differ. For instance, for: - Model Training: **** Skip non-relevant text **** - For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.  Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress. **** Skip non-relevant text ****

Both possibilities concerning the termination point for training data are aligned with the deployment scenarios also specified by RAN3 as part of the “AI/ML for NG-RAN” WI in Rel-18, as excerpted below from TS 38.300 clause 16.20.2 (for the non-split architecture; for split architecture, instead, similar text can be found in TS 38.401 clause 7.11) [5][6]:

### From TS 38.300 ### 16.20.2 Principles **** Skip non-relevant text **** For the deployment of AI/ML for NG-RAN the following scenarios may be supported: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node; - AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node. **** Skip non-relevant text **** From TS 38.401 ## 7.11 Support of AI/ML for NG-RAN The support of AI/ML for NG-RAN is specified in TS 38.300 [2]. In case of CU-DU split architecture, the following scenarios may be supported: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU.

Observation 4: For AI/ML assisted positioning Case 3a the AI/ML model training can be located either in the gNB or in the OAM.

According to previous RAN1 agreement cited above, it can be seen that a training data sample is composed of two parts, a “measurement” (Part A in RAN1 discussions) and a “ground-truth label” (Part B in RAN1 discussions). For the “measurement” RAN1 agreed that it is generated by the TRP/gNB, that is, NG-RAN, in line with what is indicated in TR 38.843 clause 7.2.4; while the “label” can be generated by the LMF. Therefore, it can be acknowledged that there is RAN3 impact in providing the “label” part of the training data sample from LMF to NG-RAN if the training is located in the gNB. For the “measurement”, instead, since it is generated by the TRP/gNB, there is no RAN3 impact if the AI/ML model training is located in the NG-RAN since this information is already available in the entity performing AI/ML model training, i.e., the NG-RAN.

Even though RAN1 didn’t agree on a specific indication concerning the user data privacy requirements for the provisioning of the “labels” from the LMF to the NG-RAN (discussed in this Section for AI/ML model training), we think that a similar note from RAN1 about the user data privacy as included in the description of Option A for the AI/ML model monitoring options should also be considered for the model training as well – note that the model monitoring considerations can be found in Section 2.1.3 of this contribution.

In addition, RAN1 agreed that the purpose for which a certain information is provided does not need to be indicated in the specifications. In our view, this is in line with the principle which drove the specification of “AI/ML for NG-RAN” since Rel-18, for which the following was specified in TS 38.300 clause 16.20.2 [5]:

### 16.20.2 Principles **** Skip non-relevant text **** Signalling procedures used for the exchange of information to support AI/ML for NG-RAN, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

Therefore, it is suggested that the same principle is also considered during the discussions in RAN3 about the impacts for supporting AI/ML assisted positioning Case 3a. Based on the above, we have the following proposal:

Proposal 2: There is RAN3 impact in providing the “label” part of the training data sample from LMF to NG-RAN if the AI/ML model training is located in the NG-RAN.

- RAN3 assumes that the provisioning of “labels” ensures and preserves the corresponding user data privacy requirements.

- Purpose, such as above “training data collection”, will not necessarily be specified in RAN3 specification.

Observation 5: The “measurement” part of the training data sample is already available at NG-RAN and does not need to be provided if the AI/ML model training is located in the NG-RAN. Hence, there is no RAN3 impact.

### Model inference considerations

To enable the AI/ML model in the gNB to generate measurements for successive determination at LMF side of the UE location, some input data needs to be provided to the AI/ML model itself which will then generate a proper output. In previous RAN1 meetings the following agreements were achieved concerning the AI/ML model input and output information:

******** RAN1#116 ******** Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. Note: The use of SRS for MIMO resource is transparent to UE. Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. ******** RAN1#118 ******** Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

Based on the above agreements, our understanding is that the AI/ML model in the gNB for AI/ML assisted positioning Case 3a uses SRS-based measurements as model input also for inference purposes (other than for other procedures such as training and monitoring). And, as per Proposal 2 above and based on RAN1 agreements, the purpose for which information are provided and used shall not be indicated in RAN3 specifications.  RAN1 made a conclusion in the last meeting that the existing procedures can be reused for the SRS configuration, even though the details of the existing parameters (e.g., values) can be further discussed.

Observation 6: The AI/ML model in the gNB for AI/ML assisted positioning Case 3a uses SRS-based measurements as model input also for inference purposes, by reusing the existing procedures for the SRS configuration.

With regards to the model output, i.e., information that could be reported from the NG-RAN to the LMF, RAN1 agreed that at least LOS/NLOS indicator and/or timing information are supported for reporting. For both pieces of information RAN1 also considered the existing methods already specified, that is, soft/hard indicator for the LOS/NLOS indicator, and UL RTOA (Uplink Relative Time of Arrival) or gNB Rx-Tx time difference for the timing information. However, the details of the reporting are pending further discussions in RAN1.

Proposal 3: RAN3 to wait for further RAN1 progress on the details of the reporting of the agreed information that needs to be provided to the LMF (i.e., at least LOS/NLOS indicator and/or timing information) for AI/ML assisted positioning Case 3a.

### Model monitoring considerations

Another important aspect for a proper AI/ML model’s handling is the model monitoring. In previous RAN1 meetings the following agreements were reached:

******** RAN1#116bis ******** Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation ******** RAN1#118 ******** Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

For model monitoring in Case 3a, RAN1 identified two options:

- Option A, where the NG-RAN (hosting the AI/ML model) performs the monitoring metric calculation

- Option B, where the LMF performs the monitoring metric calculation for the model hosted in NG-RAN.

The RAN1 agreements cited above are basically the same, apart from the fact that RAN1 agreed in RAN1#118 that both Options above are feasible from RAN1 perspective. However, when comparing the agreement from RAN1#118 with the agreement from RAN1#116, the following can be observed:

- RAN1 didn’t provide information on how to generate information on ground truth label for each option (as it was supposed to be done based on RAN1#116 agreement);

- RAN1 didn’t provide any recommendation about the option(s) to go for, hence leaving the final decision completely up to RAN3.

Before discussing the peculiarities of Option A and Option B, and then decide which option to specify, we think RAN3 should first agree on a general principle concerning the location of the model monitoring function with respect to the entity which is in charge of the AI/ML model inference or, in general, of taking a proper “monitoring decision” for the model based on inference results (e.g., (de-)activation, fallback to legacy positioning methods, etc.). In our view, it would be better that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation, which is also the entity which has knowledge of the model being used. This because in our understanding the “monitoring decision” should be based on the model-specific monitoring metric calculated by taking into account two main components, that is, the inference results (i.e., model output) and the “labels”.

Observation 7: The entity hosting the AI/ML model has knowledge of the model being used for inference. Based on inference results and “labels” such entity can perform the monitoring metric calculation which is model-specific without any further additional information provided by other network entities.

For the specific case of AI/ML assisted positioning Case 3a, the inference results are generated by the AI/ML model hosted in the gNB and sent to LMF for successive determination of the UE location, while “labels” are provided by the LMF and transferred to the gNB. It should be noted that the gNB could have the “labels” already available as they might have been previously provided by the LMF for training the model in the gNB, therefore in principle there could not be any further need to transfer “labels” for model monitoring purposes. If this is not the case, “labels” can anyway be made available at the gNB without any additional standard impact on top of the signalling to be specified by RAN3 to enable model’s training.

Observation 8: For AI/ML assisted positioning Case 3a, the gNB hosts the AI/ML model and might have both inference results and “labels” already available – “labels” can be provided/obtained without additional standard impact on top of the signalling to be specified by RAN3 to enable model’s training. Hence the gNB can calculate the monitoring metric and take a proper “monitoring decision” accordingly.

Based on the above Observations, in Option A, the NG-RAN node may calculate the monitoring metric based on the inference results and the “labels” already available in the gNB side, and take proper a “monitoring decision” accordingly.

For Option B, where the monitoring metric is calculated at the LMF, further specification impact would be required. For example, some model-specific information could be needed to be provided to the LMF for metric calculation as well as signalling back from LMF to gNB, e.g., to inform the gNB about the monitoring outcome would be needed to be specified.

Observation 9: Option B would induce further specification impact in addition to the transmission of inference results from the gNB to the LMF, e.g. model-specific information for monitoring metric calculation at LMF or reporting the monitoring result from LMF to gNB.

In Option A, instead, the monitoring metric calculation is performed in the gNB directly, which has knowledge of the AI/ML model used for inference and, in principle, the gNB already has the needed set of information (i.e., inference results, “labels” and knowledge of the model) to then calculate the monitoring metric and determine the most appropriate “monitoring decision” accordingly. Hence, in our understanding, Option A has less standardization impact with respect to Option B.

Observation 10: In Option A the monitoring metric calculation is performed in the gNB directly, which has knowledge of the AI/ML model used for inference and, in principle, the gNB already has the needed set of information (i.e., inference results, “labels” and knowledge of the model) to then calculate the monitoring metric and determine the most appropriate “monitoring decision” accordingly.

Based on the above Observations we have the following proposal:

Proposal 4: RAN3 to agree that the entity hosting the AI/ML model is also the entity in charge of the monitoring metric calculation which is model-specific, i.e., adopt Option A for the monitoring metric calculation of AI/ML assisted positioning Case 3a.

# Conclusions

General aspects of AI/ML assisted positioning

Observation 1: For AI/ML assisted positioning use cases the output of the AI/ML model is a new measurement and/or enhancement of existing measurement.

Observation 2: AI/ML assisted positioning use cases are distinguished based on the location of the AI/ML model used to generate measurements to then be used at LMF side to determine the UE location.

Observation 3: Among the AI/ML assisted positioning cases, Case 2a does not have any RAN3 impact.

Proposal 1: RAN3 to discuss the impacts of Case 3a for AI/ML assisted positioning.

Model training considerations

Observation 4: For AI/ML assisted positioning Case 3a the AI/ML model training can be located either in the gNB or in the OAM.

Proposal 2: There is RAN3 impact in providing the “label” part of the training data sample from LMF to NG-RAN if the AI/ML model training is located in the NG-RAN.

- RAN3 assumes that the provisioning of “labels” ensures and preserves the corresponding user data privacy requirements.

- Purpose, such as above “training data collection”, will not necessarily be specified in RAN3 specification.

Observation 5: The “measurement” part of the training data sample is already available at NG-RAN and does not need to be provided if the AI/ML model training is located in the NG-RAN. Hence, there is no RAN3 impact.

Model inference considerations

Observation 6: The AI/ML model in the gNB for AI/ML assisted positioning Case 3a uses SRS-based measurements as model input also for inference purposes, by reusing the existing procedures for the SRS configuration.

Proposal 3: RAN3 to wait for further RAN1 progress on the details of the reporting of the agreed information that needs to be provided to the LMF (i.e., at least LOS/NLOS indicator and/or timing information) for AI/ML assisted positioning Case 3a.

Model monitoring considerations

Observation 7: The entity hosting the AI/ML model has knowledge of the model being used for inference. Based on inference results and “labels” such entity can perform the monitoring metric calculation which is model-specific without any further additional information provided by other network entities.

Observation 8: For AI/ML assisted positioning Case 3a, the gNB hosts the AI/ML model and might have both inference results and “labels” already available – “labels” can be provided/obtained without additional standard impact on top of the signalling to be specified by RAN3 to enable model’s training. Hence the gNB can calculate the monitoring metric and take a proper “monitoring decision” accordingly.

Observation 9: Option B would induce further specification impact in addition to the transmission of inference results from the gNB to the LMF, e.g. model-specific information for monitoring metric calculation at LMF or reporting the monitoring result from LMF to gNB.

Observation 10: In Option A the monitoring metric calculation is performed in the gNB directly, which has knowledge of the AI/ML model used for inference and, in principle, the gNB already has the needed set of information (i.e., inference results, “labels” and knowledge of the model) to then calculate the monitoring metric and determine the most appropriate “monitoring decision” accordingly.

Proposal 4: RAN3 to agree that the entity hosting the AI/ML model is also the entity in charge of the monitoring metric calculation which is model-specific, i.e., adopt Option A for the monitoring metric calculation of AI/ML assisted positioning Case 3a.

# References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#105

- R3-24xxxx, Discussion on RAN3 impacts for Direct AI/ML positioning, Huawei

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (v18.0.0), Rel-18

- TS 38.300, NR and NG-RAN Overall Description; Stage 2, Rel-18

- TS 38.401, NG-RAN; Architecture description, Rel-18