3GPP TSG RAN WG3 125-bis	R3-245425

Hefei, China, 14 - 18 October, 2024

Title: 	Discussions on enhancements for AIML support for CCO

Source: 	Jio

Agenda item:	11.3

Document Type:	Discussion and decision

# 1. Introduction

During the last plenary RAN#105 meeting in Melbourne Australia, the SI AI/ML was concluded as for the 3GPP TR 38.743 V2.0.0 (2024-08) and a New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (RP-242385, Acronym: NR_AIML_NGRAN_enh) was announced with the aim of specifying new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743.

During last RAN3#125 meeting for the SI on AI/ML in RAN rel-19 it was summarized the agreements for the proactive approach in the TR 38.743 for AI/ML based CCO, where in the contrary to the legacy reactive approach, a more proactive approach is proposed to be used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

According to the available split and non-split architectures, the following agreements were reached:

For non-split architecture AI/ML based CCO,

-	AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference may be both located in the gNB.

For split architecture AI/ML based CCO,

-	AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the gNB-CU.

-	AI/ML Model Training and AI/ML Model Inference may be both located in the gNB-CU.

Following the 3GPP TR 38.743 V2.0.0 (2024-08) in subclause 4.2.2.3:

“4.2.2.3	Output data of AI/ML based CCO

AI/ML-based CCO model in a gNB can generate following information as output:

-	Predicted CCO issue (including predicted affected cells/beams)

-	Future CCO State

NOTE 1:	Future CCO State can also be derived by legacy means. Signalling future CCO state will not be described as prediction over Xn.

NOTE 2:	Future CCO state and predicted CCO issue will be provided together with timing information.

NOTE 3:	Future CCO state can be derived based on the predicted CCO issue and other information.”

Regarding the non-split architecture procedures to AI/ML support for CCO, the following agreements were reached:

Solution for non-split architecture:

Step 0: gNB predicts the CCO issue.

Step 1: gNB generates the future coverage status based on the predicted CCO issue and other information.

Step 2: gNB sends the predicted and future CCO issue (including predicted affected cells/beams) to neighbour gNBs together with timing information an associated coverage modification cause.

Furthermore regarding the split procedures to AI/ML support for CCO, as discussed in the Summary of offline Discussions (SoD) for AIRAN2_CCO (R3-244698) and the corresponding 3GPP TR 38.743 V2.0.0 (2024-08), the following agreements were also reached:

Solution for split-architecture:

Step 0: gNB-CU predicts the CCO issue

Step 1: gNB-CU sends the predicted CCO issue along with timing info to gNB-DU.

Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information.

Step 3: gNB-DU sends the future coverage status to gNB-CU.

Step 4: gNB-CU sends the predicted and future CCO issue (including predicted affected cells/beams) to neighbour gNBs together with timing information an associated coverage modification cause.

The following point has been captured as to be continued in the next meeting:

-	Whether the CU2 received the future CCO state from CU1 to forwards this future CCO state to the DU2?

In this Discussion contribution, we provide further discussion related the aforementioned open issue to AI/ML based Coverage and Capacity Optimization (CCO).

# 2. Discussion

## gNB CU Future CCO state Interpretation

Following 3GPP TS 38.300, sub-section 15.5.5.3:

“An NG-RAN node may autonomously adjust within and switch between coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with modified coverage included. The list contains the CGI of each modified cell with its coverage state indicator and optionally the SSB index of each modified SSB with its coverage state indicator.

The coverage state indicator may be used at the receiving NG-RAN node to adjust the functions of the Mobility Robustness Optimisation, e.g. by using the coverage state indicator to retrieve a previously stored Mobility Robustness Optimisation state. The coverage state indicator may also be used at the receiving NG-RAN node to adopt coverage configurations matching with neighbouring cells coverage configurations”

It is logically concluded that based on a reactively detected CCO issue or a proactively predicted CCO issue in NG-RAN node 1 (gNB1), NG-RAN node 1 adjusts its configuration as a first step and then in a second step it notifies to a neighbor NG-RAN node 2 the future CCO (i.e. a list of cells and SSB beams) with the respective CCO coverage state indicators. This facilitates the receiving NG-RAN node 2 to react on the future CCO and adopt matching coverage configurations.

Coming back to the first open question:

Whether the CU2 received the future CCO state from CU1 to further forward this future CCO state to the DU2?

That question refers back to the Summary of offline Discussions (SoD) for AIRAN2_CCO (R3-244698) where the following steps were discussed and left over for the next meeting to be decided:

Solution for split-architecture:

Step 0: gNB-CU predicts the CCO issue

Step 1: gNB-CU sends the predicted CCO issue to gNB-DU and timing information.

Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

Step 3: gNB-DU sends the future coverage status to gNB-CU.

Step 4: gNB-CU sends the future coverage status to neighbour gNBs, together with an associated coverage modification cause and timing information.

Step 5: neighbour gNB CU forwards the received future coverage status to the relevant DU over F1

Imagine the following scenario of three gNBs covering the same area with different SSB beams, as per figure 1. Between gNB1 and gNB2 there is a coverage hole detected which proactively needs some rectifications on the SSB beams. Hence gNB1 CU will predict the CCO issue, inform via F1 interface the gNB1 DU about the condition, thus forcing the gNB1 DU to generate the future CCO state to resolve the problem. As a consequence gNB1 DU will send the future CCO state to the gNB1 CU which, on its turn, will forward via Xn interface the predicted and future CCO issue to the gNB2 CU.

Now according to the agreed AI/ML CCO split architecture the model inference it is located in the gNB2-CU, thus it is for the gNB2 CU to decide on the next proposal via F1 interface to gNB2 DU to match the gNB1 CCO modifications. Hence since the intelligence is on the gNB2-CU there is no obvious reason for the gNB2 CU to send the received future CCO state from gNB1 CU towards the gNB2 DU. In the opposite way it would rather make the solution more complicated in general, increasing the processing load, degrading the overall RAN performance and increasing the F1 signaling overhead.

Observation 1: Since the legacy model inference resides in the gNB2 CU functionality, any received future CCO issue via Xn interface from a neighbour gNB1 CU is recommended or better to be kept in the gNB2 CU and not sent to the gNB2 DU, in order to avoid complications in the functionality and make the performance, processing load and F1 signalling overhead optimum

According to figure 1 and the aforementioned approach the gNB1 CU would forward the future CCO state, as concluded from gNB1 DU, to the gNB2 CU. So it is then more optimum for MRO and for the benefit of the functional feasibility for the gNB2 CU to:

- consider the received future CCO state from gNB1 CU,

- conclude on a gNB2 CU CCO modification, using all available inference based on new prediction and local info

- send the new predicted gNB2 CU CCO via F1 interface to gNB2 DU for further actions, without sending the prior gNB1 CU received future CCO.

Proposal 1: The gNB2 CU considers the received gNB1 CU future CCO state, predicts with the help of local inference proactively a new CCO state based on received future CCO and any other local info, and finally sends to the gNB2-DU the new predicted CCO issue to generate and execute a new future CCO to match the gNB1 CCO issue, thus optimizing as well the MRO functionality.

Figure 1: Example of two coverage holes in a cluster of neighbor cells, that could be resolved with mutual and joint cell shaping and CCO configuration changes

Referring back to figure 1, the existence of one or more CCO coverage holes would imply a major problem and further complications in case of multiple neighbor relations in a group (cluster) of neighbor cells via X2 interface. Continue our previous example, for the detected coverage hole gNB1 CU would forward the future CCO state, as concluded from gNB1 DU, to the gNB2 CU. In the same time and in a similar way between gNB2 and gNB3 there is another coverage hole detected which proactively needs some rectifications on the SSB beams. Hence gNB3 CU will predict the CCO issue, inform via F1 interface the gNB3 DU about the condition, thus forcing the gNB3 DU to generate the future CCO state to resolve the problem. As a consequence gNB3 DU will send the future CCO state to the gNB3 CU which, on its turn, will forward via Xn interface the predicted and future CCO issue to the gNB2 CU. Hence gNB2 CU receives two future CCO issue proposals via Xn interface, from gNB1 CU and gNB3 CU.

Now if we allow the gNB2 CU to send those two simultaneous future CCO issues to gNB2 DU it might cause severe problems in case of contradicting DU actions to solve simultaneously the coverage CCO holes from both proposals.

Observation 2: When more than one future CCO issues are received in one gNB CU, it is not recommended to send them to gNB DU since they might be totally contradicting and the gNB DU does not have the intelligence and the necessary AI/ML algorithms to handle.

Moreover it is not obvious following 3GPP TS 38.300 or 3GPP TR 38.743 how gNB2 DU would be able or capable to understand the simultaneous needs from all the reporting neighbor gNB CU future CCOs, or to jointly interpret them, or prioritize by any means and algorithms the multiple future forwarded CCO issues since it hold no intelligence or AI/ML algorithms for inference. In this case it makes no sense for the gNB2 CU to forward the two received future CCOs via F1 interface to the gNB2 DU.

Observation 3: If any number of received future CCO issues via Xn interface from a number of neighbour cells (i.e. gNB1 CU and gNB3 CU in figure 1) would be forwarded from gNB2 CU to gNB2 DU, it would eventually increase complexity leading into a non-optimal solution for the gNB2 DU CCO problem resolution.

Since the legacy model inference is located in the gNB2-CU, it is highly proposed to use the model inference intelligence in the CU functionality to review both received CCO issues from gNB1 and gNB3 CUs and decide on a joint optimum solution to the problem by predicting the most preferable CCO issue and sending it via F1 interface to the gNB2 DU.

Observation 4: It is more efficient and beneficial for the network performance and F1 signalling overhead to use the gNB CU intelligence and inference capabilities to handle and analyse multiple received via Xn future CCOs and not to send them as individual future CCOs to gNB DU.

Proposal 2: Any received future CCO issue (including as well the single or multiple CCO issues from a cluster of multiple neighbour cells) should be kept on the receiving gNB CU entity and never forwarded to the gNB DU.

According to figure 1 gNB2 CU would:

- consider both the received future CCO state from gNB1 and gNB3 CUs,

- conclude on a jointly optimum gNB2 CU CCO modification, using all available inference based on new prediction and local info

- send the new predicted gNB2 CU CCO via F1 interface to gNB2 DU for further actions, without sending the prior gNB1 CU received future CCO.

We then propose and support the scenario of using the gNB CU AI/ML inference to (jointly in case of multiple) interpret the received future CCOs.

Proposal 3: The gNB CU should keep the received future CCO for AI/ML intelligent interpretation.

We further propose and support that gNB CU using AI/ML inference on the received neighbor future CCOs should conclude on a (jointly in case of multiple) predicted CCO to forward it via F1 interface to the gNB DU

Proposal 4: The gNB CU, receiving neighbour future CCOs, should conclude on a new predicted CCO to forward it via F1 interface to the gNB DU.

## gNB CU Future CCO state per Content

Following figure 2, gNB2 CU will receive a future CCO issue from gNB1 CU for the two different coverage holes which most probably will be related to different SSB beams. If the contents of the future CCO issue include only the related cells, gNB2 CU would have limited information on which SSB beams are affected from the future CCO issue. , It might be really difficult then for the gNB2 CU to interpret the reported issue and conclude on the concerned SSB beams, forcing it to go through exhaustive brute force algorithms to figure out, if possible, the corresponding SSB beams.

Figure 2: Example of two coverage holes in one neighbor cell CCO issue scenario

Observation 5: If the received future CCO issue, signalled via Xn interface to a neighbour gNB CU, does not include the SSB beams, the information is incomplete and vague forcing the gNB CU to undergo into complicated and processing consuming algorithms to estimate the related SSB beams.

In the same way coming back to figure 1, gNB2 CU will receive two different future CCO issues from gNB1 and gNB3 CUs. If the contents of the future CCO issues include only the related cells, gNB2 CU would have limited information on which SSB beams are affected from the future CCO issues. Furthermore since the two reported CCO issues might have different or contradicting cells on the issue detected, it might be really difficult and vague to the gNB2 CU on how to interpret the reported issue and how to figure out the joint interpretation, forcing it to go through exhaustive brute force algorithms to figure out, if possible, the corresponding SSB beams.

Observation 6: The content of the future CCO issues should include among others the related cells and the corresponding SSB beams in order to provide a concise and accurate information about the predicted CCO issue to be resolved. We believe that this is mandatory since any more generic information would lead into a generic problem and the eventual solution-action from gNB2 CU would be sub-optimal.

Proposal 5: Any future CCO issue signalled via Xn interface to a single or group of neighbour gNB CUs should include among others the related cells and the corresponding SSB beams.

Following the legacy CCO, an AI/ML predicted CCO issue is always associated with a list of concerned neighbour cells and related group of SSB beams, which typically the gNB-CU communicates via F1 interface to the gNB-DU. It is then the most logical conclusion to extend this functionality to the aforementioned discussed scenario of receiving a future CCO from a neighbour cell. This is because would the gNB CU send the received future CCO via F1 interface to the gNB DU with only the involved cells but without the corresponding SSB beams, would be again very difficult and somehow unclear on the SSB beams, forcing the DU to undergo through heavy and complicated brute force algorithms to conclude on the related SSB beams since it lacks the AI/ML intelligence.

Observation 7: If the gNB CU after receiving a future CCO issue, signalled via Xn interface from a neighbour gNB CU, concludes on a predicted CCO issue and forwards it to the gNB DU without including the SSB beams, the information would be incomplete and unclear forcing the gNB DU (due to lack of AI/ML inference and intelligence) to undergo into complicated and processing consuming algorithms to conclude on the related SSB beams.

Proposal 7: Any predicted CCO issue, concluded after receiving a future CCO issue signalled via Xn interface from a neighbour gNB CU, should include among others the related cells and the corresponding SSB beams when signalled via F1 interface to the gNB DU.

## CCO issues and future coverage state signalling aspects

Following the previous clause 2.2, the signalling of a CCO issue via F1 interface from gNB-CU to gNB-DU and among a group of neighbour gNBs and cells via Xn interface should be concise, clear and capable to lead to a fast and accurate decision from the gNB CU AI/ML inference. It is then proposed to add to the F1AP and XnAP messages the proper IEs to indicate the involved cells and beams in the gNB CU predicted issue after receiving a single future issue or in the gNB CU jointly decided predicted CCO issue after receiving a group of future CCOs from a group of neighbour gNB CUs.

Proposal 8: Enhance the legacy F1AP and XnAP messages with new information elements (IEs) to indicate a predicted CCO issue and corresponding received future CCO issue, the corresponding involved cells and beams.

We are further in flavour of using the already specified in NG-RAN Configuration Update in Xn and gNB-DU/gNB-CU Configuration Update in F1 signalling messages, in order to avoid any further complexity and to make the overall AI/ML CCO solution backwards compatible to the legacy procedures.

Proposal 9: Use the same NG-RAN Configuration Update in Xn Interface and F1 interface to support the AI/ML based CCO functionality.

# 3. Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Observation 1: Since the legacy model inference resides in the gNB2 CU functionality, any received future CCO issue via Xn interface from a neighbour gNB1 CU is recommended or better to be kept in the gNB2 CU and not sent to the gNB2 DU, in order to avoid complications in the functionality and make the performance, processing load and F1 signalling overhead optimum

Proposal 1: The gNB2 CU considers the received gNB1 CU future CCO state, predicts with the help of local inference proactively a new CCO state based on received future CCO and any other local info, and finally sends to the gNB2-DU the new predicted CCO issue to generate and execute a new future CCO to match the gNB1 CCO issue, thus optimizing as well the MRO functionality.

Observation 2: When more than one future CCO issues are received in one gNB CU, it is not recommended to send them to gNB DU since they might be totally contradicting and the gNB DU does not have the intelligence and the necessary AI/ML algorithms to handle.

Observation 3: If any number of received future CCO issues via Xn interface from a number of neighbour cells (i.e. gNB1 CU and gNB3 CU in figure 1) would be forwarded from gNB2 CU to gNB2 DU, it would eventually increase complexity leading into a non-optimal solution for the gNB2 DU CCO problem resolution.

Observation 4: It is more efficient and beneficial for the network performance and F1 signalling overhead to use the gNB CU intelligence and inference capabilities to handle and analyse multiple received via Xn future CCOs and not to send them as individual future CCOs to gNB DU.

Proposal 2: Any received future CCO issue (including as well the single or multiple CCO issues from a cluster of multiple neighbour cells) should be kept on the receiving gNB CU entity and never forwarded to the gNB DU.

Proposal 3: The gNB CU should keep the received future CCO for AI/ML intelligent interpretation.

Proposal 4: The gNB CU, receiving neighbour future CCOs, should conclude on a new predicted CCO to forward it via F1 interface to the gNB DU.

Observation 5: If the received future CCO issue, signalled via Xn interface to a neighbour gNB CU, does not include the SSB beams, the information is incomplete and vague forcing the gNB CU to undergo into complicated and processing consuming algorithms to estimate the related SSB beams.

Observation 6: The content of the future CCO issues should include among others the related cells and the corresponding SSB beams in order to provide a concise and accurate information about the predicted CCO issue to be resolved. We believe that this is mandatory since any more generic information would lead into a generic problem and the eventual solution-action from gNB2 CU would be sub-optimal.

Proposal 5: Any future CCO issue signalled via Xn interface to a single or group of neighbour gNB CUs should include among others the related cells and the corresponding SSB beams.

Observation 7: If the gNB CU after receiving a future CCO issue, signalled via Xn interface from a neighbour gNB CU, concludes on a predicted CCO issue and forwards it to the gNB DU without including the SSB beams, the information would be incomplete and unclear forcing the gNB DU (due to lack of AI/ML inference and intelligence) to undergo into complicated and processing consuming algorithms to conclude on the related SSB beams.

Proposal 7: Any predicted CCO issue, concluded after receiving a future CCO issue signalled via Xn interface from a neighbour gNB CU, should include among others the related cells and the corresponding SSB beams when signalled via F1 interface to the gNB DU.

Proposal 8: Enhance the legacy F1AP and XnAP messages with new information elements (IEs) to indicate a predicted CCO issue and corresponding received future CCO issue, the corresponding involved cells and beams.

Proposal 9: Use the same NG-RAN Configuration Update in Xn Interface and F1 interface to support the AI/ML based CCO functionality.

# References

[1] 3GPP TR 38.743 V2.0.0 (2024-08)

[2] R3-244698 Summary of offline Discussion (SoD) for AIRAN2_CCO