3GPP TSG RAN WG3 125-bis	R3-245426

Hefei, China, 14 - 18 October, 2024

Title: 	Further discussions on enhancements for AIML support for CCO

Source: 	Jio, Deutsche Telekom

Agenda item:	11.3

Document Type:	Discussion and decision

# 1. Introduction

During the last RAN3#125 meeting the TR 38.743 for AI/ML based CCO summarized the agreements on the AI/ML in RAN rel-19. Regarding the split procedures to AI/ML support for CCO, as discussed in the Summary of offline Discussions (SoD) for AIRAN2_CCO (R3-244698) and the corresponding 3GPP TR 38.743 V2.0.0 (2024-08), the following agreements were also reached:

Solution for split-architecture:

Step 0: gNB-CU predicts the CCO issue

Step 1: gNB-CU sends the predicted CCO issue to gNB-DU, together with suggested future CCO state .

Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

Step 3: gNB-DU sends the future coverage status to gNB-CU.

Step 4: gNB-CU sends the future coverage status to neighbour gNBs.

The following open issue point has been captured as to be continued in the this next meeting:

-	Whether the CU1 generates the suggested future CCO state to DU1 as assistance information?

In this Discussion contribution, we provide further discussion related the aforementioned open issue to AI/ML based Coverage and Capacity Optimization (CCO).

# 2. Discussion

## Future CCO state as assistance information

Following legacy 3GPP standards, in 3GPP TS 38.300, sub-section 15.5.5.3:

“An NG-RAN node may autonomously adjust within and switch between coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with modified coverage included. The list contains the CGI of each modified cell with its coverage state indicator and optionally the SSB index of each modified SSB with its coverage state indicator”

And in sub-section 16.20.2

“For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105.

AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105.”

Moreover following the 3GPP TS 38.401 in sub-clause 7.9.3:

“In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU signals to the gNB-DU the CCO issue and the affected cells and beams. The gNB-DU resolves the CCO issue concerning own served cell by local action within the OAM configured limits. The gNB-DU may also take into account information received for other cells when adopting the CCO configuration. The gNB-DU informs the gNB-CU of the new coverage states adopted.”

and the sub-clause 7.11

“In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.”

From the above legacy 3GPP standards statements we have the following observations:

Observation 1: The AI/ML inference model, in case of NG-RAN gNB split architecture, resides in the gNB CU.

Observation 2: In case of NG-RAN gNB split architecture the gNB CU detects existing CCO issues and predicts future CCO issues. In a next step gNB CU signals via F1 interface to the gNB DU the CCO issue and the affected cells and beams.

Observation 3:  In case of NG-RAN gNB split architecture the gNB DU resolves the owned cell CCO issue (creating a future CCO action) by local actions and/or received future CCO issues from neighbour cells (via Xn and gNB CU).

It is logically concluded that based on a reactively detected CCO issue or a proactively predicted CCO issue in NG-RAN node gNB CU signals the CCO issue to the gNB DU which is authorized and capable to adjusts its configuration for Mobility Robust Optimization (MRO) or matching the new cell status.

Coming back to the open question:

Whether the CU1 generates the suggested future CCO state to DU1 as assistance information?

that question implies that although in the legacy 3GPP architecture for AI/ML-based CCO, the role of the gNB CU in generating proactive insights into network issues, (i.e. coverage holes or interference etc) is critical, enhancing this by allowing the gNB CU to not only detect or predict CCO issues but also create and send future CCO suggestions as part of an assistance functionality to gNB Du could be of some benefit in the overall functionality and architecture. If we refer back to the Summary of offline Discussions (SoD) for AIRAN2_CCO (R3-244698) the following steps were discussed and left over for the next meeting to be decided:

Solution for split-architecture:

Step 0: gNB-CU predicts the CCO issue

Step 1: gNB-CU sends the predicted CCO issue to gNB-DU, together with suggested future CCO state.

Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

Step 3: gNB-DU sends the future coverage status to gNB-CU.

Step 4: gNB-CU sends the future coverage status to neighbour gNBs.

There are indeed few important benefits for such gNB CU enhancement.

Such an enhancement in step 1 with the suggested future CCO would enable the gNB DU to quickly implement corrective actions without needing to spend additional time generating its own solution. This would reduce latency in response to network issues, leading to more immediate improvements in user experience and network performance.

Observation 4:  In case of NG-RAN gNB CU suggested future CCO signalling via F1 interface to gNB DU enables fast corrective actions reducing latency for many time critical services.

Moreover the CU’s broader view of network conditions and access to AI/ML inference models would allow it to generate more informed future CCO suggestions, where these suggestions, based on predictive insights and historical trends, can be more accurate, reducing the likelihood of suboptimal CCO decisions at the DU level.

Observation 5:  In case of NG-RAN gNB CU support for suggested future CCO, signalled via F1 interface to gNB DU, enables the usage of historical data and real-time network statistics to predict future network conditions and propose actions accordingly.

Furthermore regarding the AI/ML training and inference, sending future CCO suggestions from the CU to the DU fosters a feedback loop, where the CU can learn from the DU’s actions. In a long run and over time, this data can be used to further refine AI/ML models, improving prediction accuracy and decision-making.

Observation 6:  In case of NG-RAN gNB CU support for suggested future CCO, signalled via F1 interface to gNB DU, enables the fine tuning of the AI/ML inference models by the feedback link becoming more accurate, leading to better CCO predictions and suggestions by continuously improving network performance.

It is also worth mentioning that whatever might be the suggested future CCO from gNB CU to DU, the gNB Du is not obliged or mandated to follow the suggestion. The DU can receive the CU’s future CCO suggestion and either implement it as is or adapt it based on real-time network conditions. This flexibility ensures that the CCO solution is adaptable to dynamic network environments while still benefiting from the CU’s analytical predictions.

Nevertheless the discussion on enabling the gNB CU to generate future CCO suggestions, alongside predicting CCO issues, presents challenges. The core challenge lies in the violation of the legacy NG-RAN AI/ML functionality regarding the division of responsibilities between the gNB CU and DU.

Observation 7:  The potential NG-RAN gNB CU support for suggested future CCO to the gNB DU, violates the legacy 3GPP split architecture functionality.

Especially considering that the gNB CU does not have direct access to certain key parameters managed by the gNB DU (i.e. physical layer cell configuration, SSB beam coverage, and real-time resource allocation etc) it is unclear and obscure how the gNB CU would be able to conclude on an accurate suggested future CCO issue action.

Observation 8: The gNB CU for AI/ML supported CCO is not designed to support resource allocation functionality.

Observation 9:  It is totally unclear and questionable how does the gNB CU will acquire the necessary knowledge and information to suggest the future CCO state.

There are some more minor challenges and drawbacks related to the gNB CU generated future CCO suggestion. The gNB CU would need additional computational capacity and memory to not only predict issues but also generate future CCO suggestions. These suggestions would require real-time or near-real-time analysis of traffic patterns, interference, and user mobility—operations that are computationally expensive. This could overload the CU, which already handles complex resource management, control-plane functions, and AI/ML inference.

Observation 10:  The potential NG-RAN gNB CU support for suggested future CCO to the gNB DU increases the demand on computational overhead to the CU module.

The gNB CU has an abstracted and less granular view of the radio environment compared to the DU, which has direct access to real-time RAN data (e.g., link quality, interference levels). As a result, the CU's future CCO suggestion might be less informed or optimal compared to what the DU could achieve using its local, precise, and more immediate information. That would result into an implementation with frequent signaling messages via F1 interface between CU and DU. It is important to keep in mind that the AI/ML gNB CU inference algorithms and the gNB DU functionality is not standardized by 3GPP but it is leftover totally to the vendors. The current F1AP signaling protocol, which governs the interaction between the CU and DU, may not be designed to handle the additional information burden that future CCO suggestions would place on it.

Observation 11: To overcome the limited local context at gNB CU, the potential NG-RAN gNB CU support for suggested future CCO to the gNB DU would require multiple feedbacks via the F1 interface between distant located gNB CU and gNB DU, increasing the F1 interface capacity and load/overhead.

Extending the F1AP protocol to support gNB CU suggested future CCO signaling would require significant standardization changes, which might also lead to compatibility and interoperability issues across different vendor equipment.

Observation 12: The F1AP protocol limitations and F1 interface capacity capabilities would introduce severe burden and incompatibilities among different vendor equipment.

In summary, the gNB CU capability to generate future CCO suggestions may not be feasible mainly due to major restrictions in the 3GPP legacy standards and the unclear and questionable capability of gNB CU to conclude and generate the future suggested CCO state. There might be also some other reasons, as due to latency concerns, computational overload, distributed decision-making complexity, lack of local context, training burdens, limited real-time adaptability, F1AP protocol constraints, multi-DU coordination challenges, security vulnerabilities, and network overhead.

These technical issues collectively challenge the efficiency and practicality of delegating future CCO recommendations to the CU. So concluding on the open question:

Whether the CU1 generates the suggested future CCO state to DU1 as assistance information?

we are not in flavor of such implementation.

Proposal 1: The gNB CU should produce the predicted CCO issue (assisted by the AI/ML inference) and signal it via F1 interface to the gNB DU without any further suggested future CCO stare included in the message.

Proposal 2.  The gNB CU should not produce any suggested future CCO state for gNB DU.

# 3. Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Observation 1: The AI/ML inference model, in case of NG-RAN gNB split architecture, resides in the gNB CU.

Observation 2: In case of NG-RAN gNB split architecture the gNB CU detects existing CCO issues and predicts future CCO issues. In a next step gNB CU signals via F1 interface to the gNB DU the CCO issue and the affected cells and beams.

Observation 3:  In case of NG-RAN gNB split architecture the gNB DU resolves the owned cell CCO issue (creating a future CCO action) by local actions and/or received future CCO issues from neighbour cells (via Xn and gNB CU).

Observation 4:  In case of NG-RAN gNB CU suggested future CCO signalling via F1 interface to gNB DU enables fast corrective actions reducing latency for many time critical services.

Observation 5:  In case of NG-RAN gNB CU support for suggested future CCO, signalled via F1 interface to gNB DU, enables the usage of historical data and real-time network statistics to predict future network conditions and propose actions accordingly.

Observation 6:  In case of NG-RAN gNB CU support for suggested future CCO, signalled via F1 interface to gNB DU, enables the fine tuning of the AI/ML inference models by the feedback link becoming more accurate, leading to better CCO predictions and suggestions by continuously improving network performance.

Observation 7:  The potential NG-RAN gNB CU support for suggested future CCO to the gNB DU, violates the legacy 3GPP split architecture functionality.

Observation 8: The gNB CU for AI/ML supported CCO is not designed to support resource allocation functionality.

Observation 9:  It is totally unclear and questionable how does the gNB CU will acquire the necessary knowledge and information to suggest the future CCO state.

Observation 10:  The potential NG-RAN gNB CU support for suggested future CCO to the gNB DU increases the demand on computational overhead to the CU module.

Observation 11: To overcome the limited local context at gNB CU, the potential NG-RAN gNB CU support for suggested future CCO to the gNB DU would require multiple feedbacks via the F1 interface between distant located gNB CU and gNB DU, increasing the F1 interface capacity and load/overhead.

Observation 12: The F1AP protocol limitations and F1 interface capacity capabilities would introduce severe burden and incompatibilities among different vendor equipment.

Proposal 1: The gNB CU should produce the predicted CCO issue (assisted by the AI/ML inference) and signal it via F1 interface to the gNB DU without any further suggested future CCO stare included in the message.

Proposal 2.  The gNB CU should not produce any suggested future CCO state for gNB DU.

# References

[1] 3GPP TR 38.743 V2.0.0 (2024-08)

[2] R3-244698 Summary of offline Discussion (SoD) for AIRAN2_CCO