3GPP TSG-RAN3 Meeting #125bis	R3-245429

Heifei, China, Oct 14-18, 2024

Agenda Item:	9.1

Source: 	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell

Title: 	(TP for TS 36.300) Discussion on UE Location for NTN NB-IoT UE

Document for:	Discussion/Decision

# Background

In RAN3#123-bis meeting, an LS from SA2 was received R3-241523 on UE location for NB-IoT UEs for NTN. The LS response could not be concluded in RAN3#123-bis meeting and it was decided to wait for RAN2 response on the LS. RAN2 responded back to the SA2 LS in R3-243009 which was treated in RAN3#124 meeting. Though RAN2 mentioned that UE location may be useful in the LS response, companies couldn’t converge on usefulness of UE location in RAN3. NTN operators voiced their view during the discussion in RAN3#124 that they need UE location in RAN for RRM management within RAN. In RAN3#125 based on discussion with NTN operators, common understanding was reached that UE location is needed for load balancing purposes for NB IoT UEs in NTN.

RAN3#125 Address the issue of load balancing, i.e. intra-beam carrier allocation / intra-beam load balancing / inter-beam load balancing. Possible solutions, e.g. signaling coarse UE location from MME to eNB only once at UE connection may be discussed at the next meeting. To be continued…

In this paper we discuss our views on the solution for NB IoT UE location from MME to eNB.

# Discussion

NB-IoT UEs provide their location directly to MME via NAS. NB-IoT UE location is not provided to RAN due to AS security issues. Hence eNB in unaware of the NB-IoT UE location for NTN.

NB-IoT UEs sends coarse location to MME via NAS due to AS security issues. Hence eNB is unaware of the UE location.

In most NTN cases RAN gets the UE location and passes it to Core Network as User Location Information including Mapped Cell ID and TAC derived based on the UE location. The mapping between the TAC and a fixed geographical area is configured in the RAN. This mapping of TAC to geographical area is only configured in RAN, but not configured in CN.  So only the eNB can generate the UE location derived TAC, which is sent in ULI to MME.

36.300 defines A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

The NB-IoT UE location is useful also in eNB for populating the information used in Paging optimization that uses the “Information On Recommended Cells And eNodeBs”, where eNB can send the last visited cells to MME using the “Information On Recommended Cells And eNodeBs” IE in S1 release and MME can store this IE provide it back including the Recommended cells in the Paging message. While how to populate the list of Recommended Cells in this IE is out scope of 3GPP, without “coarse location info” provided by MME, the eNB has no choice but provide nothing or only the cell-id that corresponds to the entire beam. Alternatively if coarse location info is provided from MME what will be contained in the list of Recommended Cells can be optimised

It is also useful for eNB to initiate UE Context Release when the satellite moves out the UE coverage area.

eNB can also use the UE location to decide the redirection target for Release with redirection due to satellite movement. Other than the above RAN3 use cases, RAN2 in their LS response also mentioned that UE location may be useful in eNB for RAN2 pov, for example, configuration of UE specific Koffset to UE.

NTN operators had a strong objection in RAN3#124 meeting to mention in LS response to SA2 that UE location is not useful in eNB. They mentioned that the UE location when available can be used to map the UEs to certain frequencies in a particular location and use it for RRM management like load balancing. In RAN3#125, based on discussion with NTN operators, common understanding was reached that UE location is needed for load balancing across different frequencies in NTN cells.

Based on discussion with NTN operators a common understanding was reached that NB IoT UE location is needed in NTN cells for load balancing purposes and a compromised solution to transfer UE location from MME to eNB with minimal S1 signalling impacts can be discussed further.

As per TS 23.401 and TS 24.301, MME requests UE location from UE during Attach, TAU or Service Request procedures. Hence understanding is that MME does not frequently retrieve the location from UE.

TS 23.401 In the case of NB-IoT, the UE and the MME may support reporting of Coarse Location Information from the UE to MME in the NAS Security Mode Command procedure. As described for the Attach procedure (clause 5.3.2.1), TAU procedures (clauses 5.3.3.1 and 5.3.3.2) and Service Request procedure (clause 5.3.4.1), in the case of NB-IoT, the MME may, if supported by the UE and if the MME has obtained user consent based on proprietary mechanisms depending on local regulations, request the UE, in a NAS Security Mode Command message, to report its Coarse Location Information. TS 24.301 If: - the NAS security mode control procedure is initiated during an ongoing attach procedure, tracking area updating procedure, or service request procedure in NB-S1 mode via satellite E-UTRAN access; - the network supports coarse location information reporting; and - the UE has set the RCLIN bit to "reporting coarse location information via NAS supported" in the UE network capability IE of the ATTACH REQUEST message or TRACKING AREA UPDATE REQUEST message: then the MME may request the UE to include its coarse location information in the SECURITY MODE COMPLETE message.

As per TS 23.401 and TS 24.301 MME can retrieve coarse location from UE during Attach, TAU or Service Request procedures. This indicates MME does not frequently retrieve coarse location from UE in NTN.

To transfer the UE location from MME to eNB different options are possible as listed below. Other options are not excluded.

Option 1 – Coarse UE location provided in Location Update procedure

Option 2 – Coarse UE location provided in UE Context Modification procedure

Option 3 – Coarse UE location provided in Initial Context Setup procedure

Option 4 - Coarse UE location provided in new S1AP procedure

Coarse UE location can be sent from MME to eNB in the following options. Other options are not excluded.

- Option 1 - Coarse UE location provided in Location Update procedure

- Option 2 – Coarse UE location provided in UE Context Modification procedure

- Option 3 – Coarse UE location provided in Initial Context Setup procedure

- Option 4 - Coarse UE location provided in new S1AP procedure

Option 1 is currently used for location services. Hence, we think option 1 is more suitable to send UE location from MME to eNB and has less standards impact. The frequency of transfer of UE location can be restricted in stage 2. However, we are open to discuss other options too.

RAN3 to discuss on different options in Proposal 1 and agree on the solution to transfer coarse UE location from MME to eNB. TP for Stage 2 for Option 1 to restrict one time UE location transfer is provided in Clause 4 of this document.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	NB-IoT UEs sends coarse location to MME via NAS due to AS security issues. Hence eNB is unaware of the UE location.

Observation 2.	Based on discussion with NTN operators a common understanding was reached that NB IoT UE location is needed in NTN cells for load balancing purposes and a compromised solution to transfer UE location from MME to eNB with minimal S1 signalling impacts can be discussed further.

Observation 3.	As per TS 23.401 and TS 24.301 MME can retrieve coarse location from UE during Attach, TAU or Service Request procedures. This indicates MME does not frequently retrieve coarse location from UE in NTN.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Coarse UE location can be sent from MME to eNB in the following options. Other options are not excluded.

	Option 1 - Coarse UE location provided in Location Update procedure

	Option 2 – Coarse UE location provided in UE Context Modification procedure

	Option 3 – Coarse UE location provided in Initial Context Setup procedure

	Option 4 - Coarse UE location provided in new S1AP procedure

Proposal 2:	RAN3 to discuss on different options in Proposal 1 and agree on the solution to transfer coarse UE location from MME to eNB. TP for Stage 2 for Option 1 to restrict one time UE location transfer is provided in Clause 4 of this document.

# TP To TS 36.300

### 23.21.6	Signalling

The Cell Identity, as defined in TS 36.413 [25] and TS 36.423 [42], corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported in the following cases:

-	The Cell Identity indicated by the eNB to the Core Network as part of the User Location Information, or as E-UTRAN CGI in the related S1AP messages;

-	The Cell Identity used for Paging Optimization in S1 interface;

-	The Cell Identity used for PWS.

For a BL UE or a UE in enhanced coverage, the Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the S1 and X2 handover messages, X2 Setup and X2 eNB Configuration Update procedures is expected to be Uu Cell ID.

The mapping between Mapped Cell ID(s) and geographical area(s) is configured in the RAN and Core Network.

NOTE 1:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate different geographical areas (e.g. overlapping and/or with different dimensions).

The eNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE or based on the coarse location information received from the MME upon OAM configuration, if available. The mapping may be pre-configured (e.g., depending on operator's policy) or up to implementation.

NOTE 2:	As described in TS 23.401 [17], the User Location Information may enable the MME to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The eNB reports the broadcasted TAC(s) of the selected PLMN to the MME. In case the eNB knows the UE's location information, the eNB may determine the TAI the UE is currently located in and provide that TAI to the MME.