3GPP TSG-RAN3 Meeting #125bis	R3-245433

Heifei, China, Oct 14-18, 2024

Agenda Item:	14.2

Source: 	Qualcomm Incorporated

Title: 	(TP for TS38.300) Discussion on Support for NR NTN MBS Broadcast Service

Document for:	Discussion/Decision

# Background

In the last RAN3#125 meeting it was agreed to turn the WA into agreement to use Mapped Cell ID for MBS Service area. However in TS38.300, the agreement was captured as “Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.” With Editor’s Note that it will be further clarified.

RAN3#125 Turn WA into agreement: Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.

In this paper we discuss our views on geo fencing for MBS Service area and how gNB un-joins and re-joins the MBS broadcast session due to the satellite movement.

# Discussion

## MBS Broadcast Service Area

RAN3 agreed to turn WA into agreement to use Mapped Cell ID for geo fencing for MBS broadcast. However in the TS 38.300 the agreement was not clearly captured.

  - The Cell Identity used for MBS broadcast service may be a Mapped Cell ID.

  - Editor’s Note: the wording for above texts of using Mapped Cell ID for MBS may be further clarified.

With the above text we see the following issues:

- Since single IE “NR Cell ID” is used to indicate both Uu Cell ID and Mapped Cell ID, with the above statement, it causes IoT issues between AMF and gNB. Both the nodes cannot understand if the NRCell ID IE is indicating Uu Cell ID or Mapped Cell ID.

- Based on the discussion with NTN operators, they prefer not to use Uu Cell ID for MBS service area, as they mentioned Uu cell ID covers a large area and is constantly changing. Also GEO satellites are not much in use for cellular service.

Due to the above issues, we propose to reword the text in TS38.300 as “The Cell Identity used for MBS broadcast service may be a is Mapped Cell ID.

Reword the text in TS 38.300 as “The Cell Identity used for MBS broadcast service may be a is Mapped Cell ID”. Corresponding TP to TS 38.300 is provided in the Annex.

If RAN2 agrees to have a different format for MBS broadcast area in the SIB1, then RAN3 can introduce a new IE to define MBS broadcast area to align with RAN2 agreement.

Depending on the RAN2 agreement for MBS Broadcast area, if needed RAN3 can introduce a new IE to define MBS Broadcast area to align with RAN2 agreement.

## MBS Broadcast Trigger for a moving gNB

### Option 1 - AMF Triggered

When the gNB on satellite leaves a geographical area which is intended for MBS Broadcast, AMF can trigger MBS Broadcast Session Release. For the incoming gNB to the geographical area intended for MBS Broadcast, AMF can trigger MBS Session setup.

AMF can trigger Broadcast Session Release for the outgoing gNB and Broadcast Session Setup for the incoming gNB. Frequent Broadcast Setup and Release due to the movement of the satellite will impact NG signaling.

### Option 2 - AMF Preconfigured

AMF can send MBS Broadcast Session setup to the possible gNBs covering the intended geographical area as the path of the gNB is known. When the gNB starts providing coverage to the intended broadcast geographical area, gNB can join the NG-U tunnel for MBS session based on the transport details provided to gNB. When gNB leaves the intended broadcast geographical area, gNB can release the NG-U tunnel for MBS session.

AMF can send MBS Broadcast Session setup to the satellite gNBs covering the intended geographical area. gNBs can join and leave the NG-U tunnel for an already setup Broadcast MBS session based on its coverage to the broadcast service area as per the transport details provided.

AMF sending MBS Broadcast session setup message to gNB is a legacy behaviour to invite gNB to join the broadcast session. Due to the movement of the satellite though Option 1 has additional NG signalling impacts, option 1 looks simpler and follows legacy behaviour. Hence we prefer to reuse legacy behaviour for gNB joining and leaving the MBS broadcast session.

AMF shall send MBS broadcast session setup message to the incoming gNB in a MBS broadcast area and MBS Session setup release for a gNB leaving the MBS broadcast area (option 1) based on the knowledge of satellite movement configured in AMF for MBS broadcast for NTN regenerative payload. Agree to TP for TS 38.300 in the Annex.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	AMF can trigger Broadcast Session Release for the outgoing gNB and Broadcast Session Setup for the incoming gNB. Frequent Broadcast Setup and Release due to the movement of the satellite will impact NG signaling.

Observation 2.	AMF can send MBS Broadcast Session setup to the satellite gNBs covering the intended geographical area. gNBs can join and leave the NG-U tunnel for an already setup Broadcast MBS session based on its coverage to the broadcast service area as per the tranport details provided.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Reword the text in TS 38.300 as “The Cell Identity used for MBS broadcast service may be a is Mapped Cell ID”. Corresponding TP to TS 38.300 is provided in the Annex.

Proposal 2:	Depending on the RAN2 agreement for MBS Broadcast area, if needed RAN3 can introduce a new IE to define MBS Broadcast area to align with RAN2 agreement.

Proposal 3:	AMF shall send MBS broadcast session setup message to the incoming gNB in a MBS broadcast area and MBS Session setup release for a gNB leaving the MBS broadcast area (option 1) based on the knowledge of satellite movement configured in AMF for MBS broadcast for NTN regenerative payload. Agree to TP for TS 38.300 in the Annex.

# Annex

TP to TS 38.300

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

The Cell Identity used for MBS broadcast service is Mapped Cell ID.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

AMF triggers MBS Broadcast Session Setup and Release procedure to the gNB when gNB on satellite enters and leaves the MBS Broadcast area respectively based on the knowledge of ephemeris of the satellite.