3GPP TSG-RAN WG3 Meeting #125bis	R3-245436

Hefei, China, 10 - 14 October 2024

Agenda item:	 10.2

Source:		Lenovo

Title:		(TP for SON BLCR for 38.300) Discussion on MRO for LTM

Document for:		Discussion and Decision

1	Introduction

RAN3#123bis meeting agreed that:

Work on scenarios of near failure LTM

Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell

RAN3#124 meeting agreed that:

RAN3 to prioritize MCG LTM over SCG LTM.

RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study.

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

RAN3#125 meeting agreed that:

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

FFS to enhance failure scenario.

Take the existing detection mechanism descriptions as baseline.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

In this paper, we would further discuss the details of MRO for LTM.

2	Discussion

In RAN2#125bis meeting, it has agreed that:

RAN2 considers SHR, RA report and RLF for MCG LTM SON.

RAN2 will start work on MCG LTM.

In RAN2#126 meeting, it has agreed that:

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

In RAN2#127 meeting, it has agreed that:

Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC.

introduce a new field in RLF report to indicate the LTM recovery cell id.

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

RA-based access will not be a new triggering condition for the SHR report.

HO interruption time will not be a new triggering condition for the SHR report

Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

We will not log interruption time in SON reports (e.g. SHR) for LTM.

We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

Based on agreements in RAN2 and RAN3, in this paper, we would mainly discuss RLF report for MCG LTM. For SCG LTM, it is introduced in R18 with same priority as MCG LTM to reduce interruption time during PSCell change procedure. SCG LTM failure case and even near failure successful SCG LTM case should be avoided to improve network performance, we would further discuss MRO for SCG LTM in later meetings when MRO for MCG LTM is stable.

Last RAN3 meeting agreed to reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. There are two FFSs as captured in the BL CR to TS 38.300 [1]:

- FFS if LTM recovery needs to be captured in the definitions.

- For the too late LTM cell switch, FFS if the re-establishment cell is a candidate cell, a cell not selected by CU, or both.

In legcay CHO, the UE may perform CHO recovery if configured after CHO execution failure, the curent wording “the UE attempts to re-establish the radio link connection” covers that case that the UE re-connects to a CHO candidate cell or a non CHO candidate cell. Simialily, for LTM failure, “the UE attempts to re-establish the radio link connection” covers that case that the UE re-connects to a LTM candidate cell or a non LTM candidate cell, there is no need to capture additional description concerning LTM recovery in the stage 2 failure type definition.

For too late LTM cell switch, “the UE attempts to re-establish the radio link connection in a different cell” means the UE re-connects to a cell other than the source cell, the re-connected cell may be a LTM candidate cell or a non LTM candidate cell (i.e. a cell not selected by CU during LTM preparation phase), it is not necessary to add description reagrding “for the too late LTM cell switch, the re-establishment cell is a candidate cell, a cell not selected by CU, or both” in stage 2 specification.

Last RAN3 meeting also agreed to take the existing detection mechanism descriptions as baseline, updates as shown in Annex are needed.

Proposal 1: Agree the TP for TS38.300 in Annex.

In LTM preparation phase, the CU provides a list of LTM candidate cell to each candidate target DU or target DU, the candidate target DU or target DU may accept or reject cell(s) in the list of LTM candidate cell provided by the CU. RAN3#124 meeting agreed that for failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization, e.g. based on the RLF report, the CU makes root cause analysis to decide that the failure is caused due to wrong selection of LTM candidate cell for the case that the cell for re-establish is not one of LTM candidate cells provided by CU to candidate target DU or target DU, if so, the CU may modify the list of LTM candidate cell.

In LTM execution phase, the source DU selects the target cell for LTM cell switch execution. When a connection failure happens in a LTM procedure, it may be inappropriate cell switch triggering at source DU (e.g. wrong target cell selection, or wrong cell switch timing). Based on RAN2’s agreements, we can find that the RLF report includes necessary information e.g. L1 measurements, cell id of an LTM candidate cell in which performing LTM failure recovery, reconnect cell id and mobility type indicating the LTM cell switch. RAN3#125 meeting agreed that for failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure. Since CU receives the RLF report earlier than DU no matter what is the failure cause, and CU has overview information than DU, similar as the failure case due to wrong selection of candidate LTM cell, the CU is also the better node to perform root cause analysis for the failure case due to inappropriate cell switch triggering at source DU (e.g. wrong target cell selection, or wrong cell switch timing).

We agreed that CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. It is FFS for additional information needed.

For the case that the cell for re-establishment is one of the LTM candidate cells provided by CU to candidate target DU or target DU, but not the target cell included in the LTM cell switch command, the CU may detect that the connection failure is due to wrong target cell selection at source DU, then CU may send RLF report and the detected failure cause (e.g. LTM cell switch to Wrong Cell) to source DU in F1 interface e.g. via the existing Access and Mobility Indication message, in such a way, the source DU may optimize the target cell for LTM execution.

On the other hand, if the failed PCell and previous PCell indicated in the RLF report are the source PCell, the CU may detect that the connection failure is due to late cell switch timing at source DU, CU may send RLF report and the detected failure cause (e.g. Too Late LTM cell switch) to source DU in F1 interface e.g. via the existing Access and Mobility Indication message.

Additionally, if the cell for re-establishment is the source PCell which triggers the LTM cell switch, the CU may detect that the connection failure is due to early LTM cell switch timing at source DU, CU may send RLF report and the detected failure cause (e.g. Too Early LTM cell switch) to source DU in F1 interface e.g. via the existing Access and Mobility Indication message.

Proposal 2: CU performs root cause analysis and sends the failure type (e.g. Too Late LTM cell switch, Too Early LTM cell switch, or LTM cell switch to Wrong Cell) in F1 interface e.g. via the Access and Mobility Indication message.

3	Conclusion

In this paper, MRO for LTM are further discussed. We have the following proposals:

Proposal 1: Agree the TP for TS38.300 in Annex.

Proposal 2: CU performs root cause analysis and sends the failure type (e.g. Too Late LTM cell switch, Too Early LTM cell switch, or LTM cell switch to Wrong Cell) in F1 interface e.g. via the Access and Mobility Indication message.

Reference

[1] R3-244853, (BL CR to 38.300) MRO enhancement for LTM, China Unicom, ZTE, Samsung, Nokia, Lenovo

Annex: TP for TS38.300

##### 15.5.2.2.2	 Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell is the cell that served the UE at the last handover initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.