3GPP TSG-RAN WG3 Meeting #125bis	R3-245438

Hefei, China, 10 - 14 October 2024

Agenda item:	 10.3.1

Source:		Lenovo

Title:		Discussion on MRO for intra-NTN mobility

Document for:		Discussion and Decision

1	Introduction

It was agreed in RAN3#123bis meeting that [1]:

MRO mechanisms for intra-NTN should be addressed based on failure scenarios first, work on near failure scenarios later.

RAN3#124 meeting agreed that [2]:

Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility.

MRO for intra-NTN mobility shall consider the following trigger conditions:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

Do not consider MRO for feeder link switch.

MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover.

RAN3#125 meeting further agreed that [3]:

MRO for CHO with a time-based trigger condition:

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

- Distinguish if the RLF happens before or after T1

Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

- FFS on whether it matters if RLF happens within the duration or after the duration period.

The RLF report includes for single time-based trigger condition:

-      Absolute time (measured UTC time) when RLF occurred

-      Measured UTC time when RLF CHO is executed

Whether to consider the fallback issue from RACH-less HO to RACH-based HO for NTN?

Whether to introduce any enhancement for the Area Scope of logged MDT for NTN over NG interface?

In this paper, we would discuss details of MRO for intra-NTN mobility.

2	Discussion

An NTN provides non-terrestrial NR access to the UE by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

Currently, intra-NTN mobility is supported. The UE may support mobility between gNBs operating with NTN payloads in different orbits (e.g., GSO, NGSO at different altitudes). Intra-NTN mobility may be an ordinary handover, or a CHO. Furthermore, for CHO in NTN, location based CHO and time based CHO are supported.

For normal mobility in NTN, Event D1 is introduced that measurement report is triggered when distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 becomes shorter than configured threshold distanceThreshFromReference2.

In time based intra-NTN CHO, CondEvent T1 is introduced that the time measured at UE becomes more than configured threshold t1-Threshold but is less than t1-Threshold + duration. Time-based CHO can be performed via RACH-less.

In location based intra-NTN CHO, CondEvent D1 is introduced for Quasi-Earth-fixed case, location based trigger condition is satisfied when distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2. On the other hand, CondEvent D2 is introduced for Earth-moving case, location based trigger condition is satisfied when distance between UE and a moving reference location determined based on referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a moving reference location determined based on referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2.

A time-based or a location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5). It is up to UE implementation how the UE evaluates the time-or location-based trigger condition together with the RRM measurement-based event.

RAN3#124 meeting agreed that MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover, and the following trigger conditions for MRO for intra-NTN mobility shall be considered:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

Last RAN3#125 meeting agreed to consider Case 1 Too Late CHO Execution for CHO with a time-based trigger condition: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell. Since network can derive the time when the RLF failure happens based on the existing time information IE (e.g. timeSinceCHO-Reconfig IE, timeConnFailure IE, or timeSinceFailure IE) in RLF report, and also know the absolute time T1 e.g. based on UE context, network can distinguish whether the RLF happens before or after T1, there is no need to report additional information concerning whether the RLF happens before or after T1 (e.g. an explicit indication) in the RLF report.

Proposal 1: In case of Too Late CHO Execution for CHO with a time-based trigger condition, there is no need to report additional information concerning whether the RLF happens before or after T1 in the RLF report.

Last RAN3#125 meeting also agreed to consider Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell for CHO with a time-based trigger condition: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell, it is FFS on whether it matters if RLF happens within the duration or after the duration period. For Case 2, CHO execution is performed, the HOF or RLF may be caused due to CHO execution conditions are not proper or RACH issue, whether HOF or RLF happens within or after the duration period is not the key issue.

Proposal 2: In case of Too Early CHO Execution/CHO Execution to Wrong Cell for CHO with a time-based trigger condition, it is not needed to distinguish whether the HOF or RLF happens within or after the duration period.

For CHO configured with a time-based trigger condition, or CHO configured with time-based and measurement-based combined trigger condition, a connection failure may happen due to configuring inappropriate parameter(s), besides the time when the connection failure happens, the time when CHO is executed may also be derived based on the existing time information IE (e.g. timeSinceCHO-Reconfig IE, timeConnFailure IE, or timeSinceFailure IE) in RLF report. It is unnecessary to include the absolute time (measured UTC time) when connection failure occurred and the absolute time (measured UTC time) when CHO is executed in the RLF report.

Proposal 3: It is unnecessary to include the absolute time (measured UTC time) when connection failure occurred and the absolute time (measured UTC time) when CHO is executed in the RLF report.

For CHO configured with time-based and measurement-based combined trigger condition, there may be the case that the absolute time value t1-Threshold + duration arrives, but measurement based CHO execution condition is not fulfilled, to enable network know that measurement based CHO execution condition is setting strictly, measurement results of source cell, target cell and candidate cell(s) at the absolute time value t1-Threshold or t1-Threshold + duration, may be stored or reported by the UE in RLF report.

Proposal 4: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) at the absolute time value t1-Threshold or t1-Threshold + duration.

For CHO configured with a location-based trigger condition, or CHO configured with location-based and measurement-based combined trigger condition, a connection failure may happen due to inappropriate execution condition(s) (e.g. RRM measurement based CHO execution condition or distance based CHO execution condition) setting, besides measurement results of source cell, target cell and candidate cell(s) when the connection failure happens, it is beneficial to report the distance between UE and a reference location/moving reference location of source cell when the connection failure happens, and the distance between UE and a reference location/moving reference location of candidate or target cell when the connection failure happens.

Proposal 5: RLF report can be enhanced to include the distance between UE and a reference location/moving reference location of source cell when the connection failure happens, and, the distance between UE and a reference location/moving reference location of candidate or target cell when the connection failure happens.

For CHO configured with location-based and measurement-based combined trigger condition, there may be the case that location based CHO execution condition is fulfilled but measurement based CHO execution condition is not fulfilled when RLF  happens, to enable network know that measurement based CHO execution condition is setting strictly, measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled, may be stored or reported by the UE in RLF report. Additionally, there may be the case that measurement based CHO execution condition is fulfilled but location based CHO execution condition is not fulfilled when RLF  happens, to enable network know that location based CHO execution condition is setting strictly, the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled, may be stored or reported by the UE in RLF report.

Proposal 6: RLF report can be enhanced to include the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled.

Fallback from RACH-less HO to RACH-based HO is supported in NTN. If RA-InformationCommon is included in RLF report, it means that fallback from RACH-less HO to RACH-based HO is performed, then network may optimize RACH-less HO related configurations. So, an explicit indication regarding fallback from RACH-less HO to RACH-based HO is not needed in the RLF report.

Proposal 7: An explicit indication regarding fallback from RACH-less HO to RACH-based HO is not needed in the RLF report.

3	Conclusion

In this paper, MRO enhancements for intra-NTN mobility are discussed. We have the following proposals:

Proposal 1: In case of Too Late CHO Execution for CHO with a time-based trigger condition, there is no need to report additional information concerning whether the RLF happens before or after T1 in the RLF report.

Proposal 2: In case of Too Early CHO Execution/CHO Execution to Wrong Cell for CHO with a time-based trigger condition, it is not needed to distinguish whether the HOF or RLF happens within or after the duration period.

Proposal 3: It is unnecessary to include the absolute time (measured UTC time) when connection failure occurred and the absolute time (measured UTC time) when CHO is executed in the RLF report.

Proposal 4: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) at the absolute time value t1-Threshold or t1-Threshold + duration.

Proposal 5: RLF report can be enhanced to include the distance between UE and a reference location/moving reference location of source cell when the connection failure happens, and, the distance between UE and a reference location/moving reference location of candidate or target cell when the connection failure happens.

Proposal 6: RLF report can be enhanced to include the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled.

Proposal 7: An explicit indication regarding fallback from RACH-less HO to RACH-based HO is not needed in the RLF report.

Reference

[1] Chairman notes for RAN3#123bis meeting

[2] Chairman notes for RAN3#124 meeting

[3] Chairman notes for RAN3#125 meeting