3GPP TSG-RAN WG3 Meeting #125bis	R3-245439

Hefei, China, 10 - 14 October 2024

Agenda item:	 10.4

Source:		Lenovo

Title:		Discussion on MRO for R18 leftovers

Document for:		Discussion and Decision

1	Introduction

The following agreements have been achieved until now:

MRO for MR-DC SCG failure:

Keep the following R18 agreement in R19:

Support MRO for SCG failure in EN-DC, NGEN-DC and NE-DC scenarios.

Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for EN-DC and NGEN-DC SCG failure, and necessary updates can be added on top of it if needed.

Introduce new SCG FAILURE INFORMATION REPORT message and SCG FAILURE TRANSFER message over X2 interface.

RACH optimization for SDT:

Work on scenario of RACH optimization for SDT first.

There is no need for UE to report SDT configurations, e.g., RSRP threshold, data volume threshold, T319a, and other configurations.

The information that stored and reported in the RA report for SDT:

c. the time from the start of RA-SDT to the reporting of RA report

In this paper, we would further discuss the details of MRO for R18 leftovers.

2	Discussion

  - 2.1 MRO for MR-DC SCG failure

Last RAN2#127 meeting agreed:

To support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure.

RAN2#126 meeting agreed that RAN2 understands that whether also supporting (NG)EN-DC has no additional RAN2 impact hence RAN3 can decide. From RAN3 point of view, we have already agreed to support MRO for SCG failure in EN-DC, NGEN-DC and NE-DC scenarios in RAN3#123bis meeting. Therefore, similar as enhancements for EN-DC SCG failure, it is beneficial to enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId and timeSCGFailure for NGEN-DC.

Proposal 1: Enhance SCGFailureInformationNR message for NGEN-DC SCG failure to include previousPSCellId, failedPSCellId and timeSCGFailure.

Last RAN3#125 meeting agreed to introduce new SCG FAILURE INFORMATION REPORT message and SCG FAILURE TRANSFER message over X2 interface, and explicit IEs of SCG failure information (e.g. Source PSCell CGI, Failed PSCell CGI and Time SCG Failure) were included in SCG FAILURE INFORMATION REPORT message for SCG failure reason detection and optimisation. Currently, previousPSCellId and failedPSCellId are included in SCG FAILURE INFORMATION REPORT message in Xn interface, similar as enhancements for EN-DC SCG failure in X2 interface, the timeSCGFailure can also be included in SCG FAILURE INFORMATION REPORT message in Xn interface.

Proposal 2: Enhance SCG FAILURE INFORMATION REPORT message in Xn interface for NGEN-DC SCG failure to include timeSCGFailure.

  - 2.2 RACH optimization for SDT

SDT is a procedure allowing data transmission while remaining in RRC_INACTIVE without transitioning to RRC_CONNECTED state. SDT is enabled on a radio bearer basis and is initiated by the UE only if less than a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, measured RSRP in the cell is above a configured threshold, and a valid resource for SDT transmission is available.

Until now RAN2 have agreed that:

For the purpose of SON enhancements for SDT, include RSRP/data volume related information

Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

Do not add logging of sdt-RSRP-Threshold, since already agreed by RAN3 to not support it.

UE logs and reports the failure cause for SDT to the network. FFS the details, e.g. if we down select some of the failure causes.

A SDT procedure may fail due to a low threshold of radio link quality or due to a high threshold of data volume, for example, the UE triggers SDT but data transmission fails because of bad radio link quality or heavy data volume. When SDT fails, the UE may re-trigger SDT transmission or enter RRC connected for data transmission.

To improve SDT performance, it is beneficial to store and report the RSRP when SDT fails, then network can analyse the failure issue e.g. whether the RSRP threshold used to decide whether to trigger SDT is set properly, for example, in the case that the RSRP threshold and RSRP when SDT fails are close, it means that the RSRP threshold is set too low, network may modify with a higher RSRP threshold. Similarly, it is beneficial to store and report the data volume when SDT fails, network can analyse whether the data volume threshold used to decide whether to trigger SDT is set properly, e.g., in the case that the data volume threshold and data volume when SDT fails are close, it means that the data volume threshold is set too high, network may modify with a lower data volume threshold. The detailed informtion to be stored or reported by the UE in the RA report can be further checked with RAN2, and an LS to RAN2 may be needed if RAN3 achieves some conclusions.

Proposal 3: RSRP when SDT fails and data volume when SDT fails can be stored and reported in the RA report for SDT optimization.

3	Conclusion

In this paper, MRO for R18 leftovers are discussed. We have the following proposals:

Proposal 1: Enhance SCGFailureInformationNR message for NGEN-DC SCG failure to include previousPSCellId, failedPSCellId and timeSCGFailure.

Proposal 2: Enhance SCG FAILURE INFORMATION REPORT message in Xn interface for NGEN-DC SCG failure to include timeSCGFailure.

Proposal 3: RSRP when SDT fails and data volume when SDT fails can be stored and reported in the RA report for SDT optimization.