3GPP TSG-RAN WG3 #125bis	R3-245440

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	11.2

Source:	Lenovo

Title: 		(TP to BLCR 38.423) Discussion on AIML based network slicing

Document for:	Discussion and Approval

- Introduction

In this paper, we discuss issues related to AIML based network slicing.

- Discussion

2.1	Transferring Prediction Result over Xn interface

4.1.2.5 Potential standard impacts Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18. Xn interface: - Enhanced existing procedure to collect predicted information between gNBs: ■ Predicted radio resource status per slice ■ Predicted slice available capacity

During the study phase discussion, RAN3 concluded that for network slicing use case, a NG-RAN node could predict its 1. Radio resource status per slice, and 2. Slice available capacity, and transfer to its neighbour NG-RAN node over Xn interface.

From standard impact point of view, we believe it can be achieved by simply extend the legacy Data Collection Reporting Initiation and Data Collection Reporting procedure. An example is given as below:

- NG-RAN node 1 sends a DATA COLLECTION REQUEST Xn message to NG-RAN node 2 requesting predicted radio resource status per slice and predicted slice available capability from NG-RAN node 2, which includes configuration of the periodicity for the subsequent transfer of prediction result over Xn interface.

- NG-RAN node 2 responds with a DATA COLLECTION RESPONSE message acknowledging the request.

- Later on, according to the request and periodicity configured in step 1, NG-RAN node 2 will generate the prediction result and transfer to NG-RAN node 1 over Xn interface via DATA COLLECTION UPDATE message.

RAN3 enhances the legacy Data Collection Reporting Initiation and Data Collection Reporting procedures to support the predicted radio resource status per slice and predicted slice available capacity request and report over Xn interface

In particular, a new bit should be used in the Report Characteristics for Data Collection IE within the DATA COLLECTION REQUEST message to indicate the request of predicted slice available capacity. The NG-RAN node 2 should respond and provide the prediction result accordingly. Accordingly, the DATA COLLECTION UPDATE message should include a Predicted Slice Available Capacity IE.

A new bit (i.e., tenth bit) of the Report Characteristics for Data Collection IE within the DATA COLLECTION REQUEST message is used to indicate the request of predicted slice available capacity.

DATA COLLECTION UPDATE message includes a new Predicted Slice Available Capacity IE.

To request the per slice level radio resource status prediction, the existing first bit “Predicted Radio Resource Status” can be reused together with a Slice To Report List newly added under Cell To Report Item for Data Collection IE. Accordingly, the Predicted Radio Resource Status IE within the DATA COLLECTION UPDATE can include Slice Radio Resource Status List IE if requested.

RAN3 adds a Slice To Report List IE under Cell To Report Item for Data Collection, which can be used to request per slice predicted radio resource status for a concerned cell when the first bit “Predicted Radio Resource Status” is set to “1” in the DATA COLLECTION REQUEST message.

The Predicted Radio Resource Status IE within the DATA COLLECTION UPDATE can include Slice Radio Resource Status List IE, if requested.

2.2	Transferring UE performance feedback over Xn interface

4.1.2.4 Feedback of AI/ML based Network Slicing To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs: - Measured Radio resource status per slice - Measured Slice available capacity - Legacy UE performance feedback for those UEs handed over from the source gNB - Finer granularity UE performance feedback for those UEs handed over from the source gNB to determine UE performance for a certain slice in use by a certain UE. NOTE: Exact level of finer granularity to be defined in normative phase.

For slice-based handover, it is agreed that finer granularity UE performance feedback should be supported, while it was under debate if finer granularity means

- Per slice level UE performance,

- Per QoS flow level UE performance, or

- Per DRB level UE performance.

In our view, per slice level UE performance would be enough to serve the purpose, and both source NG-RAN node and target NG-RAN node share the same understanding of the network slice. In contrast, during the handover, the target NG-RAN node may modify the QoS flow and DRB configuration, which may cause misaligned understanding between source NG-RAN node and target NG-RAN node about the implication behind.

During handover, the target NG-RAN node may modify the QoS flow and DRB related configuration.

For AIML based network slicing, RAN3 supports per slice level UE performance feedback transferred over Xn interface from target NG-RAN node to source NG-RAN node.

From standard impact point of view, we believe it can be achieved by simply extend the legacy Data Collection Reporting Initiation, and Data Collection Reporting procedure. An example is given as below:

- NG-RAN node 1 sends a DATA COLLECTION REQUEST Xn message to NG-RAN node 2 requesting per slice level UE performance feedback after relevant handover procedure, which includes configuration of the periodicity for the subsequent transfer of prediction result over Xn interface.

- NG-RAN node 2 responds with a DATA COLLECTION RESPONSE message acknowledging the request.

- NG-RAN node 1 initiates a handover procedure, in the HANDOVER REQEUST message, it includes the relevant Data Collection ID IE associated to the DATA COLLECTION REQUEST in step 1.

- Later on, according to the request and periodicity configured in step 1, NG-RAN node 2 will measure the per slice level UE performance feedback and transfer to NG-RAN node 1 over Xn interface via DATA COLLECTION UPDATE message.

It’s worth notice that the HANDOVER REQUEST message may remain unchanged in Rel19, since it only carries a Data Collection ID IE that refers to an enhanced Data Collection Reporting Initiation procedure.

RAN3 enhances the legacy Data Collection Reporting Initiation and Data Collection Reporting procedures to support the transfer of per slice level UE performance feedback from target NG-RAN node to source NG-RAN node. Legacy Handover Preparation procedure can be reused.

In particular, we believe whether target NG-RAN node should provide UE performance at per UE level or per slice level can be explicitly configured in the UE Performance Collection Configuration IE within the DATA COLLECTION REQEUST message, then in the UE Performance IE within the DATA COLLECTION UPDATE message, the target NG-RAN node should provide either per UE level or per slice level UE performance accordingly.

UE Performance Collection Configuration IE within the DATA COLLECTION REQUEST message is enhanced to indicate if UE performance shall be measured at per slice level.

UE Performance IE within the DATA COLLECTION UPDATE message is enhanced to support per slice level UE performance report.

2.3	E1/F1 impacts

For the E1/F1 impacts that are relevant to AIML based network slicing, the only aspect we observe is to support per slice level UE performance measurement and exchange via E1/F1 interface, which is related to the split architecture discussion in agenda 11.4. For the sake of simplify the discussion in agenda 11.2 and having a unified solution transferring UE performance over E1 and F1 interface, RAN3 is kindly suggested to discuss the per slice UE performance transfer after the conclusion of UE performance transfer for Rel18 use cases in agenda 11.4.

RAN3 supports the per slice level UE performance transfer over E1/F1 interface. Stage 3 details can be discussed following the conclusion of split architecture for Rel18 use cases.

- Conclusion

Based on the discussion above, we observe:

Observation 1	During handover, the target NG-RAN node may modify the QoS flow and DRB related configuration.

Based on the discussion above, we propose:

Proposal 1	RAN3 enhances the legacy Data Collection Reporting Initiation and Data Collection Reporting procedures to support the predicted radio resource status per slice and predicted slice available capacity request and report over Xn interface

Proposal 2	A new bit (i.e., tenth bit) of the Report Characteristics for Data Collection IE within the DATA COLLECTION REQUEST message is used to indicate the request of predicted slice available capacity.

Proposal 3	DATA COLLECTION UPDATE message includes a new Predicted Slice Available Capacity IE.

Proposal 4	RAN3 adds a Slice To Report List IE under Cell To Report Item for Data Collection, which can be used to request per slice predicted radio resource status for a concerned cell when the first bit “Predicted Radio Resource Status” is set to “1” in the DATA COLLECTION REQUEST message.

Proposal 5	The Predicted Radio Resource Status IE within the DATA COLLECTION UPDATE can include Slice Radio Resource Status List IE, if requested.

Proposal 6	For AIML based network slicing, RAN3 supports per slice level UE performance feedback transferred over Xn interface from target NG-RAN node to source NG-RAN node.

Proposal 7	RAN3 enhances the legacy Data Collection Reporting Initiation and Data Collection Reporting procedures to support the transfer of per slice level UE performance feedback from target NG-RAN node to source NG-RAN node. Legacy Handover Preparation procedure can be reused.

Proposal 8	UE Performance Collection Configuration IE within the DATA COLLECTION REQUEST message is enhanced to indicate if UE performance shall be measured at per slice level.

Proposal 9	UE Performance IE within the DATA COLLECTION UPDATE message is enhanced to support per slice level UE performance report.

Proposal 10	RAN3 supports the per slice level UE performance transfer over E1/F1 interface. Stage 3 details can be discussed following the conclusion of split architecture for Rel18 use cases.

4	TP to 38.423

***************************************** START OF CHANGE ****************************************************

8.4.13	Data Collection Reporting Initiation

8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the duration and granularity configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput DL IE, if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Measured UE Trajectory IE, if the ninth bit, "Measured UE Trajectory" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Slice Radio Resource Status List IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", when the Slice To Report List IE is included for the requested cell, and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Slice Available Capacity IE, if the tenth bit, "Predicted Slice Available Capacity“ of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

8.4.13.3	Unsuccessful Operation

Figure 8.4.13.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, NG-RAN node2 shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

8.4.13.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the NG-RAN node1 may reinitiate the Data Collection Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message.

If the NG-RAN node2 receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the NG-RAN node2 Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the NG-RAN node2 shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then NG-RAN node2 shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the NG-RAN node1 Measurement ID IE corresponding to an existing on-going Data Collection reporting, then NG-RAN node2 shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

8.4.14	Data Collection Reporting

8.4.14.1	General

This procedure is initiated by an NG-RAN node to report information accepted by the NG-RAN node following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

8.4.14.2	Successful Operation

Figure 8.4.14.2-1: Data Collection Reporting, successful operation

NG-RAN node2 shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

8.4.14.3	Unsuccessful Operation

Not applicable.

8.4.14.4	Abnormal Conditions

Void.

***************************************** NEXT CHANGE ****************************************************

9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Slice Available Capacity Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | – |  |
| >>>Slice To Report Item |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per cell. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

9.1.3.28	DATA COLLECTION FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for all of the requested objects the reporting cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and optionally the Slice Radio Resource Status List IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

***************************************** NEXT CHANGE ****************************************************

9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |
| UE Slice Performance List |  | 0..1 |  |  |
| > UE Slice Performance Item |  |  |  |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  |
| >>Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| >>Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| >>Average Packet Delay | O |  | 9.2.3.187 |  |
| >>Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

9.2.3.186	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond |
| UE Performance Per Slice | O |  | ENUMERATED (true, …) |  |

***************************************** END OF CHANGE ****************************************************