3GPP TSG-RAN WG3 Meeting #125bis	R3-245441

Hefei, China, 10 - 14 October 2024

Agenda item:	 11.3

Source:		Lenovo

Title:		(TP to BLCR 38.473) Discussion on AI/ML based CCO

Document for:		Discussion and Decision

1	Introduction

In this contribution, we would discuss issues related to AI/ML based CCO.

2	Discussion

  - 2.1 AI/ML based CCO over Xn interface

In non-split architecture, it has agreed that:

- Step 0: gNB predicts the CCO issue.

- Step 1: gNB generates the future coverage status based on the predicted CCO issue and other information.

- Step 2: gNB sends the future coverage status to neighbour gNBs.

One issue left to normative phase is how to transmit future CCO State together with an associated coverage modification cause and Timing Information over Xn. The potential solutions are:

- Alt 1: NG-RAN node 1 may send the future CCO State together with an associated coverage modification cause and Timing Information to NG-RAN node 2, and adjust the CCO state based on feedback from NG-RAN node 2.

- Alt 2: NG-RAN node 1 is fully responsible for prediction and only sends the future CCO State together with an associated coverage modification cause and Timing Information to NG-RAN node 2.

In Alt 1, RAN node 1 transfers future CCO state to RAN node 2 in order to get RAN node 2’s confirmation before executing the future scheduled CCO state. In Alt 2, RAN node 1 will simply inform RAN node 2 about future CCO state to be executed by the RAN node 1 and RAN node 2’s confirmation is not needed.

In our understanding, the second alternative is the most straightforward way. In the legacy, when a RAN node decides to modify the CCO state, it does not require any confirmation/handshake with neighbour RAN nodes. The same principle can be applied to the execution of a future CCO state as well. Besides, if RAN node 1 has multiple neighbour RAN nodes, it will be complicated/problematic if only some of the neighbour RAN nodes confirms while other neighbour RAN nodes reject.

Proposal 1: NG-RAN node can predict and execute a future CCO state without handshaking with neighbour NG-RAN nodes.

With respect to the procedure to collect information between gNBs, since the predicted CCO issue or generation of future CCO state is rather an event-triggered strategy than a periodically change, RAN3 can consider either

- Option 1: Enhance the Rel18 Data Collection Report procedure to support transmission of future CCO State together with an associated coverage modification cause and Timing Information, or

- Option 2: Enhance the conventional RAN Node Configuration Update procedure to support transmission of future CCO State together with an associated coverage modification cause and Timing Information.

Option 2 is slightly preferred due to simpliy, the legacy Coverage Modification List IE in RAN Node Configuration Update procedure can be reused for AI/ML based CCO, and the spec impact is to add Timing Information. However, for Option 1, RAN3 need further discuss whether future CCO State together with an associated coverage modification cause and Timing Information is transmitted based on request from neighbour NG-RAN node(s) or subscription mechanism.

Proposal 2: NG-RAN node can inform a neighbour NG-RAN node about future CCO State together with an associated coverage modification cause and Timing Information over Xn interface via either:

- Enhance the Rel18 Data Collection Report procedure, or

- Enhance the conventional RAN Node Configuration Update procedure.

Since the future CCO state is a prediction or based on a predicted CCO issue, the prediction might be updated or might not be correct. For example, NG-RAN node may predict a future CCO state or CCO issue for 10 seconds later and transfer the related information to a neighbour NG-RAN node, such that the neighbour NG-RAN node would expect a CCO state change after 10 seconds. However, the NG-RAN node may later detects/predicts the CCO issue to happen at 5 seconds sooner than previously predicted. In such case, it is important for the neighbour NG-RAN node to understand the newly detected or predicted CCO issue with corresponding CCO state and the neighbour NG-RAN node should consider the previously received future CCO state and/or predicted CCO issue to be no longer valid.

An example is depicted below to illustrate the point, wherein the NG-RAN node 1 first informs NG-RAN node 2 about a predicted CCO state/issue after time T, but then trigger a CCO state modification before time T due to detected CCO issue. The NG-RAN node 2 should consider the previously received CCO state/issue prediction no longer valid.

Proposal 3: NG-RAN node can update a previously transferred future CCO state and/or CCO issue (e.g., due to newly detected/predicted CCO issue).

  - 2.2 AI/ML based CCO over F1 interface

In split-architecture, it has agreed that:

- Step 0: gNB-CU predicts the CCO issue.

- Step 1: gNB-CU sends the predicted CCO issue to gNB-DU.

- Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

- Step 3: gNB-DU sends the future coverage status to gNB-CU.

- Step 4: gNB-CU sends the future coverage status to neighbour gNBs.

Regarding the predicted CCO issue (including predicted affected cells/beams) from CU to DU, the following two scenarios are agreed:

- CU1 predicts the CCO issue for DU1 and sends to DU1

- Based on the received future CCO state (including an associated coverage modification cause) from CU1, CU2 will deduce for DU2 the predicted CCO issue and affected cells/beams, and send it to DU2.

In legacy, GNB-CU CONFIGURATION UPDATE procedure is used to transfer detected CCO issue and affected cells/beams, and, GNB-DU CONFIGURATION UPDATE procedure is used to transfer a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU to solve the CCO issue. Considering the legacy IEs concerning legacy CCO in GNB-CU CONFIGURATION UPDATE procedure (e.g. Affected Cells and Beams) can be reused for AI/ML based CCO, it is preferred to enhance GNB-CU CONFIGURATION UPDATE procedure to send predicted CCO issue with time information from gNB-CU to gNB-DU. Moreover, the legacy IEs concerning legacy CCO in GNB-DU CONFIGURATION UPDATE procedure (e.g. NR CGI, Cell Coverage State and SSB Coverage Modification List) can be reused for AI/ML based CCO, and only Timing Information needs to be added (e.g. only when Timing Information is included in the GNB-DU CONFIGURATION UPDATE, the cell and/or SS/PBCH block indexes with the corresponding coverage state is generated by the gNB-DU based on the received predicted CCO issue using legacy or AI/ML function), it is preferred to enhance GNB-DU CONFIGURATION UPDATE procedure to send future CCO state with time information from gNB-DU to gNB-CU.

Proposal 4: Enhance GNB-CU CONFIGURATION UPDATE procedure to send predicted CCO issue (including predicted affected cells/beams) with time information from gNB-CU to gNB-DU.

Proposal 5: Enhance GNB-DU CONFIGURATION UPDATE procedure to send future CCO state with time information from gNB-DU to gNB-CU.

Whether gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information is also left to normative discussion. Similar as legacy CCO mechanism, the gNB-DU may resolve the predicted CCO issue concerning own served cell by local action within the OAM configured limits, and the gNB-DU may also take into account information received for other cells when generating future CCO state. Therefore, it is unnecessary for the gNB-CU to generate suggested future CCO state and send it to gNB-DU as assistance information.

Proposal 6: It is unnecessary for the gNB-CU to generate suggested future CCO state and send it to gNB-DU as assistance information.

3	Conclusion

In this contribution, AI/ML based CCO are discussed. We have the following proposals:

Proposal 1: NG-RAN node can predict and execute a future CCO state without handshaking with neighbour NG-RAN nodes.

Proposal 2: NG-RAN node can inform a neighbour NG-RAN node about future CCO State together with an associated coverage modification cause and Timing Information over Xn interface via either:

- Enhance the Rel18 Data Collection Report procedure, or

- Enhance the conventional RAN Node Configuration Update procedure.

Proposal 3: NG-RAN node can update a previously transferred future CCO state and/or CCO issue (e.g., due to newly detected/predicted CCO issue).

Proposal 4: Enhance GNB-CU CONFIGURATION UPDATE procedure to send predicted CCO issue (including predicted affected cells/beams) with time information from gNB-CU to gNB-DU.

Proposal 5: Enhance GNB-DU CONFIGURATION UPDATE procedure to send future CCO state with time information from gNB-DU to gNB-CU.

Proposal 6: It is unnecessary for the gNB-CU to generate suggested future CCO state and send it to gNB-DU as assistance information.

4	TP to TS38.473

<<<<<<<<<<<<<<<<<<<<<<<< START OF CHANGE >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.211	CCO Assistance Information

This IE provides assistance information for the Capacity and Coverage (CCO) actions for specific CCO issues detected or predicted, and for network energy saving.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CCO issue detection | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the type of CCO issue detected, or network energy saving cause. |
| Affected Cells and Beams | O |  | 9.3.1.212 |  |
| CCO issue prediction | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of CCO issue predicted. |
| Timing Information | O |  | INTEGER (1..60, ...) | Indicates the valid time of CCO issue prediction, from reception of the GNB-CU CONFIGURATION UPDATE message (unit: second) |

<<<<<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.213	Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Coverage Modification List |  | 1 |  |  | - |  |
| >Coverage Modification Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | - |  |
| >>SSB Coverage Modification List |  | 0..1 |  |  | - |  |
| >>>SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block is inactive. Other values Indicates that the SS/PBCH block is active and also indicates the coverage configuration of the concerned SS/PBCH block. | - |  |
| >>Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …, network energy saving) |  | YES | ignore |
| >>Timing Information | O |  | INTEGER (1..60, ...) | Indicates the valid time of cell or SSB Coverage Modification, from reception of the GNB-DU CONFIGURATION UPDATE message (unit: second) | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |