3GPP TSG-RAN WG3 Meeting #125bis	R3-245447

Hefei, China, 14th – 18th Octorber, 2024

Agenda Item:	12.2

Source: 	Lenovo

Title: 	Integration and migration for WAB node

Document for:	Discussion and decision

- Introduction

A new WID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for Wireless Access Backhaul (WAB) focus on RAN3.

The objectives of the Wireless Access Backhaul (WAB) work are as follows: - Specifications for the support of WAB including [RAN3]: - Support of a WAB-node including a WAB-gNB and a WAB-MT. - Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s). - Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs. - Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes. - Network integration procedures for WAB nodes. - Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes. - Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. - UE’s ULI that reflect the WAB node’s location. - The handling of: - PCI collision avoidance. - Reconfiguration of TAC and RANAC on WAB-gNBs. - Mechanisms to avoid multi-hop WAB topology. - Radio-resource coordination between access and backhaul links. - NG connection management.

In this contribution, the procedures for WAB-node integration and migration is discussed, including WAB-MT and WAB-gNB migration.

- Discussion

WAB node integration

For the WAB-node integration procedure, it has been already discussed in the study item phase, and also has been well captured in the TR.399 [2] as follow.

Figure 4.3.1-1 WAB-node integration procedure Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling. Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases: Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information of AMF(s) to serve the UE) and service-authorized by the SeGW or by the OAM. Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step may follow legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s). Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB may establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s).

Proposal 1: Fully follow the WAB-node integration procedure defined in the TR 38.799.

WAB node migration

In RAN3 #124 meeting [3], following agreements had been achieved for scenarios of WAB-node migration.

Support the following scenarios for intra-PLMN WAB-node mobility: - UE´s AMF/UPF remains unchanged as WAB-gNB moves inside a PLMN. - UE´s AMF/UPF changes as WAB-gNB moves inside a PLMN. - BH UPF/AMF remains unchanged as WAB-MT moves inside a PLMN. - BH UPF/AMF changes as WAB-MT moves inside a PLMN.

Case 1: BH UPF/AMF remains unchanged as WAB-MT moves inside a PLMN

In case of WAB-MT migration without BH UPF/AMF changing, the WAB-MT performs handover from a source BH RAN node to a target BH RAN node, which both connect to the same BH AMF, and the WAB-MT anchors at the same BH UPF in the source and target path. A brief flow chart and migration procedure are as follow.

Figure 1: An example of WAB-MT migration procedure with BH 5GC unchanged

1.	The WAB-MT sends a MeasurementReport message to the source BH RAN node. This report is based on a Measurement Configuration the WAB-MT received from the source BH RAN node before.

2.  The source BH RAN node sends a HANDOVER REQUEST message to the target BH RAN node over the Xn interface.

3.	The target BH RAN node performs admission control and provides the new RRC configuration as part of the HANDOVER REQUEST ACKNOWLEDGE message to the source BH RAN node.

4.	The source BH RAN node forwards the received RRCReconfiguration message to the WAB-MT.

5.	A random access procedure is performed at the target BH RAN node.

6.	The WAB-MT responds to the target BH RAN node with an RRCReconfigurationComplete message.

7.	The target BH RAN node triggers the path switch procedure for migration of the WAB-MT, then all the PDU sessions of the WAB-MT are switched to the target path, and all NG and Xn traffic are transported via the target path from/to the BH UPF over the PDU sessions.

8.	The target BH RAN node sends UE CONTEXT RELEASE message to the source BH RAN node after WAB-node migration.

Proposal 2: Agree the procedure for WAB-MT migration with BH UPF/AMF unchanged.

Case 2: BH UPF/AMF changes as WAB-MT moves inside a PLMN

For this scenario, it may be further divided into follow sub-scenarios:

- BH AMF remains unchanged and BH UPF changes as WAB-MT moves inside a PLMN

- Both BH AMF and BH UPF change as WAB-MT move inside a PLMN

Because all the NG/Xn traffic of the WAB-gNB and UP traffic of served UEs need to be backhauled via the BH PDU session, when the BH UPF changing during WAB-MT migration, the WAB-MT needs to setup new PDU session with the target BH UPF and release the old PDU session with the old BH UPF. The detail procedure has main impact on the core network and can be captured in SA2.

As defined in the scope, WAB-gNB cannot serve WAB-MT, which means that a WAB-MT cannot be handed over to a target WAB-node. The WAB-MT migration scenario is the same to the mIAB-MT migration, where the mIAB-MT cannot connect to another mIAB-node during migration. In order to avoid handover failure of mIAB-MT, mobile IAB cell information is exchanged via XnAP. In the same way, WAB cell information can be also exchanged between source BH RAN node and the target BH RAN node to avoid WAB-MT migrating to a WAB-gNB.

Proposal 3: WAB cell information may be shared between source BH RAN node and target BH RAN node to avoid WAB-MT migrating to a WAB-gNB.

Case 3: UE´s AMF/UPF remains unchanged as WAB-gNB moves inside a PLMN

As the UE’s AMF remains unchanged, the gNB ID for the served UEs of the WAB-node also remains unchanged. While for the cell ID and TAC, they can be also remains unchanged and no additional impacts on UE are introduced.

In some other case, the cell ID, PCI and TAC may be changed without the change of the UE’s AMF(s). For example, if the TAC changes, the WAB-gNB needs to inform its AMF with the new TAC, and the served UE may need to perform a Mobility Registration Update procedure. Then the legacy procedure can be reused for the case of WAB-gNB migration without UE’s AMF/UPF changed.

Proposal 4: The legacy procedures for change of cell ID, PCI and TAC can be reused for the case of WAB-gNB migration without UE’s AMF/UPF changed.

Case 4: UE´s AMF/UPF changes as WAB-gNB moves inside a PLMN

This scenario is similar to mobile IAB-DU migration, where the mobile IAB-node supports two logical mobile IAB-DUs having F1 connections with the source F1-terminating IAB-donor-CU and target F1-terminating IAB-donor-CU, respectively. To realize WAB-gNB inter-AMF migration, the WAB-node needs to support two logical WAB-gNB concurrently during the migration. If the WAB-node only has one WAB-gNB, the NG connection will suffer interruption during the migration which have impacts on service continuity of the served UEs.

Proposal 5: During inter-AMF WAB-gNB migration, the WAB-node support two logical WAB-gNB concurrently, which have NG connections with the source AMF and target AMF, respectively.

For the UE(s) served by the WAB-node, they are handed over from the cells of the source logical WAB-gNB to the cells of the target logical WAB-gNB. And after all UEs are handed over, the NG connection between source logical WAB-gNB and source AMF may be removed. A brief flow chart and migration procedure are as follow.

Figure 2: An example of WAB-gNB migration procedure with UE´s AMF/UPF change.

1.	The target logical WAB-gNB sends a NG SETUP REQUEST message to the target UE’s AMF in the UE’s 5GC.

2.  The target UE’s AMF responses to the target logical WAB-gNB with a NG SETUP RESPONSE message. After the NG is set up, the target logical WAB-gNB can start serving UE(s).

3.	The source logical WAB-gNB performs NG-based UE handover from from the cells of the source logical WAB-gNB to the cells of the target logical WAB-gNB.

4.	After all the UEs are handed over, the NG connection between the source logical WAB-gNB and the source UE’s AMF may be removed.

Proposal 6: Agree the procedure for WAB-gNB migration with UE´s AMF/UPF change.

As for the single logical WAB-gNB solutions, they can be discussed after further inputs from SA2 (and RAN2).

Proposal 7: For the single logical WAB-gNB solutions for inter-AMF WAB-gNB migration, they can be discussed after further inputs from SA2 (and RAN2).

# Conclusion

This contribution aims to discuss WAB-node integration and migration procedures. And following observations and proposals are concluded.

Proposal 1: Fully follow the WAB-node integration procedure defined in the TR 38.799.

Proposal 2: Agree the procedure for WAB-MT migration with BH UPF/AMF unchanged.

Proposal 3: WAB cell information may be shared between source BH RAN node and target BH RAN node to avoid WAB-MT migrating to a WAB-gNB.

Proposal 4: The legacy procedures for change of cell ID, PCI and TAC can be reused for the case of WAB-gNB migration without UE’s AMF/UPF changed.

Proposal 5: During inter-AMF WAB-gNB migration, the WAB-node support two logical WAB-gNB concurrently, which have NG connections with the source AMF and target AMF, respectively.

Proposal 6: Agree the procedure for WAB-gNB migration with UE´s AMF/UPF change.

Proposal 7: For the single logical WAB-gNB solutions for inter-AMF WAB-gNB migration, they can be discussed after further inputs from SA2 (and RAN2).

# Reference

- RP-242395. New WID on additional topological enhancements for NR.

- 3GPP TR 38.799.

- Chairman notes of RAN3 #124.