3GPP TSG-RAN WG3 Meeting #125bis	R3-245448

Hefei, China, 14th – 18th Octorber, 2024

Agenda Item:	12.3

Source: 	Lenovo

Title: 	Architecture and access control for NR Femto

Document for:	Discussion and decision

- Introduction

A new WID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for 5G Femto fucus on RAN3.

The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

In this contribution, the architecture for NR Femto with GW for NG interface, access control and handover mechanisms for NR Femto are discussed.

- Discussion

5G Femto, with the same motivation as 4G Home eNB, can provide NR access to the UEs at home or at enterprise premises with higher frequency spectrum, to provide better experience for those UEs. In the study item phase, four options below are proposed for the architecture of NG interface:

- Option 1: NR Femto node connects to 5GC directly as a gNB

- Option 2: Support an NR Femto GW between NR Femto node and 5GC

- Option 3: An SCTP concentrator acts as an IP proxy between an NR Femto node and the AMF

- Option 4: NR Femto node is a gNB-DU and connects to gNB-CU via F1 interface

After evaluations among 4 options, only Option 1 and Option 2 are agreed to be continued. In addition, Option 1 is already supported by the legacy NG interface. Then we only need to further investigate Option 2 in the normative phase.

In the study item phase, following details have been agreed for Option 2 and captured in the TR 38.799. Then the normative work for NR Femto on NG interface should be based on Option 2 below.

Proposal 1: The normative work for NR Femto on NG interface should be based on Option 2 captured in TR 38.799.

5.2.1.2 Option 2 Figure 5.2.1.2-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto node to the 5GC. Figure 5.2.1.2-1: Option 2 for NR Femto Architecture NOTE: The SeGW and the NR Femto Management System are out of RAN3 scope. The NG-RAN architecture may deploy an NR Femto Gateway (NR Femto GW) to allow the NG interface between the NR Femto node and the 5GC to support a large number of NR Femto nodes in a scalable manner. The NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface. The NG interface is defined as the interface: - Between the NR Femto GW and the 5GC; - Between the NR Femto node and the NR Femto GW; - Between the NR Femto node and the 5GC; The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same regardless of whether the NR Femto node is connected to the 5GC via an NR Femto GW or not.

In addition, 5G Femto is analogous to LTE HeNB, which deployed at e.g. home or at enterprise premises with limited coverage, then only one cell for NR Femto node may be enough.

Proposal 2: One NR Femto node serves only one cell.

The NR Femto GW appears to the NR Femto node as an AMF, and in legacy, NG-RAN node supports connection to multiple AMFs in the 5GS architecture. However, to simplify the complexity of the NR Femto node and reuse LTE HeNB as much as possible, a NR Femto node shall only connect to a single NR Femto GW at one time, and will not simultaneously connect to another NR Femto GW or AMF, and Xn interface shall not be established between the NR Femto GW and other nodes.

Proposal 3: A NR Femto node shall only connect to a single NR Femto GW at one time, and will not simultaneously connect to another NR Femto GW or AMF.

Proposal 4: Not support Xn interface between NR Femto GW and other nodes.

As specified in TS 38.410, a NAS node selection function is located in the NG-RAN node to determine the AMF association of the UE. However, as for LTE HeNB, selection of an MME at UE attachment is hosted by the HeNB GW instead of the HeNB. In case of NR Femto node, because NR Femto node is a simplified node and may only connect to a single NR Femto GW at one time, then the NNSF function can be located at the NR Femto GW.

Proposal 5: NNSF function is located at the NR Femto GW.

With the architecture above, the NR Femto GW appears to the 5GC as an gNB, and the NR Femto GW appears to the NR Femto node as an 5GC. The NG-C and NG-U protocol stacks between NR Femto and 5GC(AMF/UPF) without GW are same as normal gNB. And the NG-C and NG-U protocol stacks between NR Femto and 5GC(AMF/UPF) are relayed via the NR Femto GW in case with deployment of gateway, as shown in Figures 1 and 2.

Figure 1: NG-U interface for 5G Femto with GW

Figure 2: NG-C Interface for NR Femto with GW

Proposal 6: Agree the protocol stacks for NG-C/NG-U of NR Femto via the NR Femto GW.

As concluded in TR 38.799, NR Femto node can support operating in open, hybrid and closed modes reusing existing CAG functionality.

For the access control, the open, hybrid and closed access mode can be supported as specified in the TR 38.799.

- With the existing CAG mechanism, the open, hybrid and closed access mode, can be supported as follows: - To support the open access mode: The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG. - To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - To support the closed access mode: The NR Femto node activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

While for mobility of NR Femto, following Xn-based handover scenarios can be supported as LTE HeNB.

-  gNB or any NR Femto node -> open access NR Femto node

-  gNB or any NR Femto node -> hybrid access NR Femto node

-  Hybrid access NR Femto node or closed access NR Femto node -> closed access NR Femto node

-  Any NR Femto node -> gNB

Proposal 7: Support following Xn-based handover scenarios for NR Femto.

-  gNB or any NR Femto node -> open access NR Femto node

-  gNB or any NR Femto node -> hybrid access NR Femto node

-  Hybrid access NR Femto node or closed access NR Femto node -> closed access NR Femto node

-  Any NR Femto node -> gNB

However, for the UE using CAG, it can be configured an Allowed CAG list and optional a CAG-only indication, which indicates whether the UE is only allowed to access 5GS via CAG cells. Then for the UE handover of NR Femto, the handover cannot be supported if the target cell is a non-CAG cell.

Proposal 8: For the UE handover of NR Femto, the handover cannot be supported if the target cell is a non-CAG cell.

In addition, as in LTE HeNB, a proximity indication was introduced for mobility of hybrid cell. However, only stage 2 impact is expected on this objective. Then, we suggest to not introduce the proximity indication for NR Femto in this release.

Proposal 9: Not introduce the proximity indication for NR Femto.

# Conclusion

This contribution aims to discuss architecture, access control and handover mechanisms for NR Femto. And following observations and proposals are concluded.

Proposal 1: The normative work for NR Femto on NG interface should be based on Option 2 captured in TR 38.799.

Proposal 2: One NR Femto node serves only one cell.

Proposal 3: A NR Femto node shall only connect to a single NR Femto GW at one time, and will not simultaneously connect to another NR Femto GW or AMF.

Proposal 4: Not support Xn interface between NR Femto GW and other nodes.

Proposal 5: NNSF function is located at the NR Femto GW.

Proposal 6: Agree the protocol stacks for NG-C/NG-U of NR Femto via the NR Femto GW.

Proposal 7: Support following Xn-based handover scenarios for NR Femto.

-  gNB or any NR Femto node -> open access NR Femto node

-  gNB or any NR Femto node -> hybrid access NR Femto node

-  Hybrid access NR Femto node or closed access NR Femto node -> closed access NR Femto node

-  Any NR Femto node -> gNB

Proposal 8: For the UE handover of NR Femto, the handover cannot be supported if the target cell is a non-CAG cell.

Proposal 9: Not introduce the proximity indication for NR Femto.

# Reference

- RP-242395. New WID on additional topological enhancements for NR.

- TR 38.799. Study on additional topological enhancements for NR (Release 19).