3GPP TSG-RAN WG3 Meeting #125bis	R3-245449

Hefei, China, 14th – 18th Octorber, 2024

Agenda Item:	12.3

Source: 	Lenovo

Title: 	(TP to 38.300) On NR Femto

Document for:	Discussion and decision

# Introduction

In this paper, text is proposed to capture the aspects related to NR Femto based on conclusions in another paper [1] and TR 38.799 [2].

# Reference

- R3-24xxx3. Architecture and access control for NR Femto. Lenovo.

- TR 38.799. Study on additional topological enhancements for NR (Release 19).

# TP to 38.300

## 4.x	NR Femto

### 4.x.1	Architecture

Figure 4.x.1-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto node to the 5GC.

Figure 4.x.1-1: NR Femto Architecture

NOTE:	The SeGW and the NR Femto Management System are out of RAN3 scope.

The NG-RAN architecture may deploy an NR Femto Gateway (NR Femto GW) to allow the NG interface between the NR Femto node and the 5GC to support a large number of NR Femto nodes in a scalable manner. The NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface.

The NG interface is defined as the interface:

-	Between the NR Femto GW and the 5GC;

-	Between the NR Femto node and the NR Femto GW;

-	Between the NR Femto node and the 5GC;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same regardless of whether the NR Femto node is connected to the 5GC via an NR Femto GW or not.

A NR Femto node shall only connect to a single NR Femto GW at one time, and will not simultaneously connect to another NR Femto GW or AMF. In addition, a NR Femto node serves only one cell. For the NR Femto GW, Xn interface is not supported between NR Femto node and other nodes.

### 4.x.2	Protocol Stacks

The NG-C and NG-U protocol stacks between NR Femto and 5GC(AMF/UPF) are relayed via the NR Femto GW in case with deployment of gateway, as shown in Figures 4.x.2-1 and 4.x.2-2.

Figure 4.x.2-1: NG-U interface for 5G Femto with GW

Figure 4.x.2-2: NG-C Interface for NR Femto with GW

### 4.x.3	Access Control

- With the existing CAG mechanism, the open, hybrid and closed access mode, can be supported as follows:

-	To support the open access mode: The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	To support the closed access mode: The NR Femto node activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

Xn-based HO involving NR Femto node is allowed as shown in Table 4.x.3-1. For the UE handover of NR Femto, the handover cannot be supported if the target cell is a non-CAG cell.

Table 4.x.3-1: Xn-based HO support

| Source | Target | Notes |
| --- | --- | --- |
| gNB or any NR Femto node | open access NR Femto node |  |
| gNB or any NR Femto node | hybrid access NR Femto node |  |
| Hybrid access NR Femto node or closed access NR Femto node | closed access NR Femto node | Only applies for same CAG ID and PLMN, and if the UE is a member of the CAG cell. |
| Any NR Femto node | gNB |  |