3GPP TSG-RAN WG3 Meeting #125bis	R3-245452

Hefei, China, 14th – 18th October 2024

Agenda item:	16.2

Source: 	Lenovo

Title: 	On RAN architecture of ambient IoT

Document for:	Discussion and Approval

1. Introduction

The ambient IoT was discussed at the RAN3#123bis meeting with the following agreements.

RAN3 considers both Topologies at the same time looking whether commonalities are applicable. [Topo1] AIoT RAN node: Corresponds to the basestation in Figure 4.2.1.1-1 in TR 38.848; A RAN node providing AIoT radio, and connecting with an AIoT-aware CN node via the XX interface. Details of the AIoT-aware CN node is subject to SA2. [Topo2] UE Reader: A UE (corresponds to the intermediate node in Figure 4.2.1.2-1 in TR 38.848); Providing AIoT radio, and connecting with a gNB (may be an AIoT enhanced gNB, corresponds to the basestation in Figure 4.2.1.2-1 in TR 38.848) via NR Uu interface. For Topology 1, RAN3 starts with AIoT RAN node being aggregated. AIoT Paging can be used to reach one or more devices for identified AIoT services (e.g., inventory, command). Use cases for locating an AIoT device: - Find an appropriate “reader” close to the A-IoT device; - Find where the A-IoT device is. Support locating the A-IoT device at “reader” granularity.

In RAN3#125 meeting, it is further agreed that:

System architecture figures agreed for Topology1 and Topology2.

In this contribution, we discuss the remaining issues of RAN architecture aspects of AIoT related to RAN3 impact.

2. Discussion

During the RAN plenary study, several connectivity topologies for AIoT networks and devices were defined [1]. In addition, RAN#102 meeting agreed the following two topologies for the study objective:

-	Topology 1: BS ↔ AIoT device

In topology 1, the AIoT device bidirectionally communicates with the BS directly. The communication between the BS and the AIoT device includes the AIoT data and/or signalling.

-	Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

In topology 2, the AIoT device communicates bidirectionally with an intermediate node between the device and the BS. In addition, the intermediate node is a UE which is capable of AIoT. The intermediate node transfers Ambient IoT data and/or signalling between the BS and the AIoT device.

Different topologies will introduce different RAN requirements. For better understanding, we will discuss the RAN architecture considerations for the topology 1 and topology 2 separately.

Topology 1: BS ↔ AIoT device

As described in the SID, there are two traffic types for the AIoT device to be studied: DT (device-terminated) and DO-DTT (device-originated – device-terminated triggered).

In topology 1, the AIoT RAN node acts as a reader to receive the service operation request from the AIoT-aware CN node, and/or to send the service operation result to the AIoT-aware CN node.

In one example, on receipt of the inventory request sent from the AIoT-aware CN node, the AIoT RAN node performs the inventory procedure towards the AIoT device, e.g., the AIoT RAN node sends a triggering message to the AIoT device to initiate the AIoT device ID report by the AIoT device. This is a typical DO-DTT traffic.

In another example, the AIoT RAN node receives a writing command request sent from the AIoT-aware CN node, and then forwards the writing command request to the AIoT device. The AIoT device does not need to provide a response to the AIoT-aware CN node via the AIoT RAN node. This is a typical DT traffic.

Therefore, it needs to introduce a protocol between the AIoT RAN node and the AIoT-aware CN node to support the following functionalities:

-	Perform inventory, and deliver inventory result (e.g., AIoT device ID) to the AIoT-aware CN node.

-	Deliver command request from the AIoT-aware CN node to the AIoT device.

-	Deliver command response received from the AIoT device to the AIoT-aware CN node.

Proposal 1: For topology 1, the protocol between the AIoT RAN node and the AIoT-aware CN node should support the following functionalities:

-	Perform inventory, and deliver inventory result (e.g., AIoT device ID) to the AIoT-aware CN node.

-	Deliver command request from the AIoT-aware CN node to the device.

-	Deliver command response received from the AIoT device to the AIoT-aware CN node.

In topology 1, we think that BS Reader is different from gNB (i.e. doesn’t need to implement the whole gNB functionality). If AIoTF is co-located with AMF, NGAP enhancement is enough (e.g. BS reader capability, AIoT command type, etc.).  If AIoTF is stand-alone, the following options are possible:

-	Opt. 1.1: container in NGAP for BS reader and AIoTF;

-	Opt. 1.2: enhanced NGAP between BS reader and AMF and new interface between AMF and AIoTF

-	Option 2: direct new interface btw. BS reader and AIoTF without AMF

Proposal 2: If AIoTF is co-located with AMF, NGAP enhancement is enough. If AIoTF is stand-alone, RAN3 studies the different options for the interface between reader and AIOTF.

Proposal 3: Capture options of protocol stacks for topology 1 in TR.

Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

In topology 2, the intermediate node behaves as a reader (UE reader) to read information from the AIoT device and write information to the AIoT device. In one example, on receipt of the inventory request sent from the AIoT-aware CN node, the UE reader performs the inventory procedure towards the AIoT device, e.g., the UE reader sends a triggering message to the AIoT device to initiate the AIoT device ID report by the AIoT device. In another example, the command information sent from the AIoT-aware CN node to the AIoT device is encapsulated in the NAS message sent from the AIoT-aware CN node to the UE reader.

Like the functionalities in the topology 1, the protocol between the UE reader and the AIoT-aware CN node supports the following functionalities:

-	Perform inventory, and deliver inventory result (e.g., AIoT device ID) to the AIoT-aware CN node.

-	Deliver command request from the AIoT-aware CN node to the AIoT device.

-	Deliver command response received from the AIoT device to the AIoT-aware CN node.

Proposal 4: For topology 2, the protocol between the UE reader and the AIoT-aware CN node should support the following functionalities:

-	Perform inventory, and deliver inventory result (e.g., AIoT device ID) to the AIoT-aware CN node.

-	Deliver command request from the AIoT-aware CN node to the device.

-	Deliver command response received from the AIoT device to the AIoT-aware CN node.

The service operation information between the AIoT device and the AIoT-aware CN node is transparent to the RAN node. UE Reader connects to AIoTF via:

-	Option 1: C-plane signaling exchange over UE N1 NAS, e.g. similar how legacy PCF/SMF communicates with UE.

-	Option 2: PDU Session connectivity via U-plane

Due to high amount of AIoT traffic btw. AIoTF and UE reader, using U-plane connectivity is considered as more efficient.

Proposal 5: For topology 2, UE Reader connects to AIoTF either by C-plane exchange over UE NAS or PDU session connectivity. Capture the corresponding protocol stacks in TR.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: For topology 1, the protocol between the AIoT RAN node and the AIoT-aware CN node should support the following functionalities:

-	Perform inventory, and deliver inventory result (e.g., AIoT device ID) to the AIoT-aware CN node.

-	Deliver command request from the AIoT-aware CN node to the device.

-	Deliver command response received from the AIoT device to the AIoT-aware CN node.

Proposal 2: If AIoTF is co-located with AMF, NGAP enhancement is enough. If AIoTF is stand-alone, RAN3 studies the different options for the interface between reader and AIOTF.

Proposal 3: Capture options of protocol stacks for topology 1 in TR.

Proposal 4: For topology 2, the protocol between the UE reader and the AIoT-aware CN node should support the following functionalities:

-	Perform inventory, and deliver inventory result (e.g., AIoT device ID) to the AIoT-aware CN node.

-	Deliver command request from the AIoT-aware CN node to the device.

-	Deliver command response received from the AIoT device to the AIoT-aware CN node.

Proposal 5: For topology 2, UE Reader connects to AIoTF either by C-plane exchange over UE NAS or PDU session connectivity. Capture the corresponding protocol stacks in TR.

4. Reference

- 3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN.