3GPP TSG-RAN WG3 Meeting #125bis	R3-245453

Hefei, China, 14th – 18th October 2024

Agenda item:	16.3

Source: 	Lenovo

Title: 	On basic procedures of ambient IoT

Document for:	Discussion and Approval

1. Introduction

The ambient IoT was discussed at the RAN3#124 meeting with the following agreements.

Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices. The Inventory Request from the AIoT CN to the AIOT RAN node, may include the following: (1) AIoT Device Identification (to find a single device, a group of devices, or all devices) Note 1: The definition of this identification is out of RAN3 scope. Editor’s Note: It is FFS whether AIOT RAN node needs to interpret/store/process it. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

In this contribution, we discuss the basic procedures for inventory, inventory and command, command only.

2. Discussion

As per RAN2 agreements, the overall procedures between A-IOT device and reader has been captured in TR 38.769:

The overall AS procedures can be formulated as:

-	Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond. See clause 6.3.3.

-	Step B: D2R data (device ID) transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure.

-	Step C1: Possible R2D data transmission (e.g., for sending the command).

-	Step C2: Possible D2R data transmission (e.g., the corresponding response to command).

As per RAN2 agreements:

-	For the detailed use case of “inventory-only”, it is supported by the procedure with Step A and Step B as baseline.

-	For the detailed use case of “inventory and command”, it is supported by the procedure with Step A, Step B, Step C1 and Step C2, as baseline.

-	For the detailed use case of “command-only”:

-	It can be also supported by the baseline procedure with Step A, Step B, Step C1 and Step C2.

-	In addition, another candidate to support this use case is following, whose feasibility still depends on the conclusion from [7] and [8]:

-	Step A’: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message including the command, indicating device(s) to process/respond the command.

-	Step C2: Possible D2R data transmission (e.g., the device ID or the corresponding response to command), via the A-IoT random access procedure or without using the A-IoT random access procedure.

Procedure for inventory-only

In RAN3#125 meeting, the basic procedure for inventory is agreed as:

Figure 1. Basic procedure for inventory only

Besides that, Information exchanged between AIoT CN and AIOT RAN node should have more information.

Firstly, the AIOT CN node (AIOTF or AMF with collocated AIOT function) sends an Inventory Request message to the AIOT RAN node to trigger the inventory procedure.

The inventory request message should include the following information:

-	AIoT Device identification (to find a single device, a group of devices, or all devices);

-	Scope of inventory request: our understanding is the scope of inventory request should be reader ID(s) that needs to send paging message to A-IOT devices.

Above information has been agreed in RAN3#124 meeting. There are additional information needs to be provided to AIOT RAN node for inventory procedure.

Firstly, there may be multiple inventory procedures which are performed in parallel. In this case, it needs to distinguish the inventor procedures in the interface between AIOT RAN node and AIOT CN node. It makes sense to introduce a inventory session or task ID to identify different inventory procedures.

Proposal 1: Introduce an inventory session or task ID to identify different inventory procedures in the interface between AIOT RAN NODE and AIOT CN node.

We also think the number of A-IOT Devices for which ID to be collected should also be provided in the inventory request message. According to RAN1’s agreement, Slotted ALOHA-based random-access procedure is used as baseline for inventory. In this case, the AIOT RAN node needs to know how many times of Query or QueryRep procedures should be performed towards A-IOT devices IDs. Following the Slotted ALOHA-based random access procedure in RFID, the random access procedure in 5G system could be:

-	DL triggering (Msg 0). A gNB sends DL signal (similar as Query or QueryRep in RFID) to trigger the A-IoT device(s) to perform access.

-	A-IoT message 1 transmission (Msg 1). Each A-IoT device joining this access round randomly selects one slot number j among N=fun.(Q) slots. If j=0, the A-IoT device performs UL transmission based on the scheduling information indicated by the Reader, including at least a (Random Number) RN16.

-	Reader Access Response (Msg 2). gNB sends ACK with the same RN16 to acknowledge the successful reception and scheduling information for next UL transmission.

-	A-IOT Device ID Reporting (Msg 3). Based on the Msg2, the A-IOT device sends corresponding parameters asked by paging for inventory, i.e., the A-IOT Device ID, as PC/XPC, EPC, packet CRC in RFID, this is referred to Msg3.

-	gNB trigger next round of random access procedure.

We think the number of A-IOT Devices for which ID to be collected should be provided for AIOT RAN Node deciding the number of time slots for A-IOT device ID reporting in air interface.

Proposal 2: The estimated number of A-IOT Devices for which ID to be collected is provided in the Inventory Request message so that the AIOT RAN NODE can decide the number of A-IOT device ID reporting procedure in air interface.

Considering that the AIoT device is with limited energy storage and the energy is provided through the harvesting of radio waves, light or any other power source that could be suitable, the AIoT device can’t be always on to receive the message from the AIoT RAN node. The AIoT RAN node should determine the inventory starting time or completing time towards the AIoT device. As discussed in RAN1, if the antenna is shared for RF energy harvester and receiver/transmitter, the AIoT RAN node can’t communicate with the AIoT device directly when the AIoT device is performing energy harvesting. Therefore, it is beneficial that the AIoT-aware CN node could provide some assistance information included in the Inventory Request message to the AIoT RAN node, which will be used by the AIoT RAN node to determine the inventory starting time or completing time towards the AIoT device. For example, based on the antenna type (e.g., shared or separate between the RF energy harvester and receiver/transmitter) provided by the AIoT-aware CN node, the AIoT RAN node determines the duration of providing RF for energy harvesting to the AIoT device or the waiting time for energy harvesting by the AIoT device.

Proposal 3: Assistance information (e.g., AIOT device type or category) is included in the inventory request message, to be used by the AIoT RAN node to determine the inventory starting time or completing time.

Considering the number of ambient IOT devices could be very large, it would be better to allow the AIOT RAN node to report a list of AIOT device IDs in a signal message. For example, the AIOT RAN node stores the A-IOT Device ID received from the A-IOT devices until all device IDs have been collected. The AIOT RAN node reports all A-IOT Device IDs in a single message to the AIOT CN node. According to proposal 3, the AIOT RAN node has already known the number of A-IOT Devices for which ID to be collected. This information can be used for AIOT RAN node deciding when to send the device IDs to the AIOT CN node.

Proposal 4: AIOT RAN Node should be able to send a list AIOT Device IDs in a single message to AIOT CN node.

Additionally, for the agreed information ‘targeted for one or more than one devices’, it’s not clear whether the ‘more than one devices’ are identified by a group ID or multiple specific device IDs, which may trigger different reader behaviours. For example, if the more than one devices are identified by a group ID, reader may trigger CBRA procedure and indicate shared resource for the devices. If the more than one devices are identified by multiple specific device IDs, reader may trigger CFRA procedure and indicate dedicated resource for each target device. For more clarity, the agreed information is suggested being updated as ‘targeted for one device or group of devices or multiple devices’, or ‘device ID type’.

Proposal 5: Whether ‘the more than one devices’ are identified by a group ID or multiple specific device IDs is useful to be visible to the reader from CN.

Procedure for inventory and command

The basic procedure for inventory and command is shown in the Figure 2. Firstly, the AIOT RAN node should be aware of whether the procedure is originated for inventory only or inventory and command. If a command is followed the inventory procedure, the AIOT RAN node should maintain the UE context for the AIOT device for the following data transmission for command. Secondly, the data transmission for command should be carried by UE associated signalling since it is AIOT device specific. Thirdly, the AIOT RAN node should know when the data transmission for command is complete so that the AIOT RAN node can release the corresponding UE context and triggers random access for next AIOT device.

Figure 2. Basic procedure for inventory and command

Proposal 6: Agree the basic procedure for inventory and command as shown in Figure 2.

Proposal 7: AIOT RAN node should be aware whether there is command procedure following inventory procedure.

Proposal 8: Data transmission for command should be carried by UE associated signalling between AIOT RAN node and AIOT CN node.

Proposal 9: AIOT RAN node should be aware when the data transmission for command is complete.

In last RAN2 meeting, it has been agreed that at least service type of AIoT (e.g., inventory, command), targeted for one or more than one devices, approximate of target devices (if available) are considered as useful to be visible to the reader from CN. And whether more information on command type (e.g., read/write/disable) is useful are FFS. From our point of view, command type is useful especially for resource scheduling. For example, if command type is ‘read’ and reader is visible of it, reader may include a large amount of D2R resource for the data to be read or reader may request D2R message size from device, when sending the command to the device. If the command type is ‘write’ or ‘disable’ and reader is visible of it, reader may include a small amount of D2R resource for ACK/NACK when sending the command to the device, and doesn’t need to request D2R message size from device. Hence, command type indication (e.g., read/write/disable) is preferred than just indicating ‘command’, without introducing more signalling overhead and with benefits for resource scheduling.

Proposal 10: Command type (e.g., read/write/disable) information is useful to be visible to the reader from CN.

Procedure for command only

According to RAN2 agreements, the baseline procedure for command only is the same as the procedure for inventory and command. The further enhancements is pending to discussion.

-	For the detailed use case of “command-only”:

-	It can be also supported by the baseline procedure with Step A, Step B, Step C1 and Step C2.

-	In addition, another candidate to support this use case is following, whose feasibility still depends on the conclusion from [7] and [8]:

-	Step A’: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message including the command, indicating device(s) to process/respond the command.

-	Step C2: Possible D2R data transmission (e.g., the device ID or the corresponding response to command), via the A-IoT random access procedure or without using the A-IoT random access procedure.

Proposal 11: The baseline procedure for command only is the same as the procedure for inventory and command. FFS on enhancements.

3. Conclusion

Based on the discussion in this paper, we have the following proposals.

Procedure for inventory-only

Proposal 1: Introduce an inventory session or task ID to identify different inventory procedures in the interface between AIOT RAN NODE and AIOT CN node.

Proposal 2: The estimated number of A-IOT Devices for which ID to be collected is provided in the Inventory Request message so that the AIOT RAN NODE can decide the number of A-IOT device ID reporting procedure in air interface.

Proposal 3: Assistance information (e.g., AIOT device type or category) is included in the inventory request message, to be used by the AIoT RAN node to determine the inventory starting time or completing time.

Proposal 4: AIOT RAN Node should be able to send a list AIOT Device IDs in a single message to AIOT CN node.

Proposal 5: Whether ‘the more than one devices’ are identified by a group ID or multiple specific device IDs is useful to be visible to the reader from CN.

Procedure for inventory and command

Proposal 6: Agree the basic procedure for inventory and command as shown in Figure 2.

Proposal 7: AIOT RAN node should be aware whether there is command procedure following inventory procedure.

Proposal 8: Data transmission for command should be carried by UE associated signalling between AIOT RAN node and AIOT CN node.

Proposal 9: AIOT RAN node should be aware when the data transmission for command is complete.

Proposal 10: Command type (e.g., read/write/disable) information is useful to be visible to the reader from CN.

Procedure for command only

Proposal 11: The baseline procedure for command only is the same as the procedure for inventory and command. FFS on enhancements.

4. Reference

- 3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN.