3GPP TSG-RAN WG3 #125bis	R3-245457

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	20.2

Source:	Lenovo

Title: 		AIML for gNB assisted positioning

Document for:	Discussion and Approval

- Introduction

In this paper, we discuss use case related to AIML for gNB assisted positioning. From RAN3 point of view, case 3a and case 3b are relevant to NRPPa protocol thus most relevant as also assumed by RAN2 in their previous discussion.

RP-242399 Revised WID for NR_AIML_air - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

RAN2 Agreements: 1 For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs 2 For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3.

- Discussion

  - Case 3a: AIML for NG-RAN node assisted positioning with gNB-side model

RAN1 agreements relevant to Case 3a: Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

Based on the RAN1 agreements above, we have identified the following aspects that would require RAN3 discussion and has possible specification impact on NRPPa:

- Training:

  - Where is gNB-sided model trained?

  - What is the required data for training?

  - The procedure of training data collection

- Inference:

  - What is the inference output?

- Monitoring:

  - Which entity performs the monitoring?

- Activation/Deactivation decision:

  - Which entity makes the decision to activate the gNB-sided model?

In the following we try to analyse each aspect.

    - Training

Where is gNB-sided model trained?

For the gNB-sided model, in the AI for RAN topic discussion, RAN3 used to assume an AIML model that located at gNB can be trained by gNB itself or by OAM. It seems to be a reasonable assumption to be inherited here as well.

For Case 3a, the gNB-sided model can be either trained by gNB or by OAM.

What is the required data for training?

To train the AIML model for case 3a, based on RAN1 agreement, there are two types of data needed

- Label (e.g., ground truth) and its relevant data (e.g., timestamp), which is generated by at least LMF

- Measurement (e.g., SRS measurement result) and its relevant data (e.g., timestamp), which is generated by TRP/gNB

For Case 3a, RAN3 assumes the following two types of training data is needed until RAN1’s further update

  - Label (e.g., ground truth) and its relevant data (e.g., timestamp), which is generated by at least LMF

  - Measurement (e.g., SRS measurement result) and its relevant data (e.g., timestamp), which is generated by TRP/gNB

The procedure of training data collection

For the case that OAM trains the gNB-sided model, the training data would be transmitted from gNB or LMF to OAM. There is no RAN3 impact

For the case that gNB trains the gNB-sided model, for the label and relevant data that is generated by the LMF, RAN3 is suggested to discuss possible procedure over NRPPa for gNB to collect necessary training data from LMF.

For Case 3a, when the gNB-sided model is trained by the gNB, RAN3 is suggested to discuss enhancements to NRPPa procedure for gNB to collect necessary training data from LMF.

    - Inference

What is the inference output?

According to the RAN1 agreements, at least LOS/NLOS indicator and/or timing information are supported for reporting. RAN3 can simply confirm.

For Case 3a, RAN3 assumes the gNB-sided model will output at least LOS/NLOS indicator and/or timing information until RAN1’s further update.

2.1.3	Monitoring

Which entity performs the monitoring?

Based on RAN1 discussion, there are two options to perform monitoring:

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

In our view, Option A and Option B are not mutual exclusive thus both can be supported. For example, when a UE is moving, the measured SRS by gNB may or may not be suitable to produce an accurate enough NLOS/LOS determination due to the applicability of the AI model, gNB may stop using the AI based method. In another example, LMF may be aware of the ground truth of the PRU/UE location or NLOS/LOS status, LMF can simply compare the inference output (NLOS/LOS indicator) or the estimated UE location based on the inference output with the ground truth to understand if the AIML model of the gNB is performing well.

For Case 3a, RAN3 is suggested to support both gNB based monitoring and LMF based monitoring.

2.1.4	Activation/Deactivation decision

Which entity makes the decision to activate the gNB-sided model?

Since case 3a is gNB-assisted positioning, wherein LMF estimates the UE location using the measurement/prediction result from gNB, we believe LMF should be the one that makes the final decision of activating/deactivating the gNB-sided model, that is LMF explicitly requests gNB to start or stop provide the NLOS/LOS indication using AI method or not.

For Case 3a, gNB will start or stop providing NLOS/LOS indication using gNB-sided AI model based on the request from LMF.

  - Case 3b: AIML for NG-RAN node assisted positioning with LMF-side model

RAN1 agreements relevant to Case 3b: Agreement For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

According to RAN1 progress so far, for case 3b, the only RAN3 impact would be any enhancement to support new measurement exchange between gNB and LMF. For example,

For sample-based measurement (if supported), LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

When gNB reports the channel measurement result to LMF, gNB may also report timing information and paired timing information and power information.

For Case 3b, based on RAN1 discussion, RAN3 supports exchanging the following information between gNB and LMF over NRPPa:

  - From LMF to gNB, if sample-based measurement is supported, LMF can signal parameter values of Nt, Nt', k to gNB

  - From gNB to LMF, together with the channel measurement result, gNB may also report timing information and/or paired timing information and power information.

- Conclusion

Based on the discussion above, we observe:

Based on the discussion above, we propose:

Proposal 1	For Case 3a, the gNB-sided model can be either trained by gNB or by OAM.

Proposal 2	For Case 3a, RAN3 assumes the following two types of training data is needed until RAN1’s further update

a.	Label (e.g., ground truth) and its relevant data (e.g., timestamp), which is generated by at least LMF

b.	Measurement (e.g., SRS measurement result) and its relevant data (e.g., timestamp), which is generated by TRP/gNB

Proposal 3	For Case 3a, when the gNB-sided model is trained by the gNB, RAN3 is suggested to discuss enhancements to NRPPa procedure for gNB to collect necessary training data from LMF.

Proposal 4	For Case 3a, RAN3 assumes the gNB-sided model will output at least LOS/NLOS indicator and/or timing information until RAN1’s further update.

Proposal 5	For Case 3a, RAN3 is suggested to support both gNB based monitoring and LMF based monitoring.

Proposal 6	For Case 3a, gNB will start or stop providing NLOS/LOS indication using gNB-sided AI model based on the request from LMF.

Proposal 7	For Case 3b, based on RAN1 discussion, RAN3 supports exchanging the following information between gNB and LMF over NRPPa:

a.	From LMF to gNB, if sample-based measurement is supported, LMF can signal parameter values of Nt, Nt', k to gNB

b.	From gNB to LMF, together with the channel measurement result, gNB may also report timing information and/or paired timing information and power information.