3GPP TSG-RAN WG3 #125-bis	R3-245473

Hefei, China, 14-18 October 2024

Agenda Item:	20.2

Source:	Ericsson

Title:	Discussion on RAN3 impacts to support AI/ML Positioning accuracy enhancements

Document for:	Discussion, Approval

# 1	Introduction

RAN3#125bis is the first meeting for RAN3 to discuss the AI/ML for Air Interface WI targeting support for positioning accuracy enhancements. The latest WID from RAN#105 [1] stipulates the following objectives that have RAN3 impacts:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

From the above, Case 3b with LMF-side model and Case 3a with NG-RAN-side model are the ones that have RAN3 impacts, which we will detail in the next section. On UE-sided models, RAN3 does not need to be involved in those, as they do not impact RAN3.

# 2	Discussion

## 2.1	Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

In Direct AI/ML positioning Case 3b, the LMF hosts the AI/ML model and performs inference to generate a predicted UE location based on input data that can be generated by UE/gNB and terminated at LMF. According to RAN1 agreements, the training data can be generated by UE/PRU/gNB/LMF.

We list below the relevant RAN1 agreements:

Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information.

Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point.

Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. For training data collection of AI/ML based positioning, the collected data sample can include the following components: - Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement - Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

| Case | Entities for training data generation |  |
| --- | --- | --- |
|  | Part A | Part B |
| Case 1 | PRU Non-PRU UE | - PRU - Non-PRU UE with estimated location - LMF |
| Case 2a |  |  |
| Case 2b |  |  |
| Case 3a | TRP/gNB | At least LMF |
| Case 3b |  | - PRU - Non-PRU UE with estimated location - LMF |

The above agreements can be summarized in the bullets and figure below, where we take the following considerations:

- For Case 3b, LMF gets training data from NG-RAN (i.e., the information denoted by RAN1 as Part A) and from LMF/UE/PRU (Part B). RAN3 does not need to look into Part B information.

- As the measurements needed as Part A in case 3b are very similar is not identical to measurement ssignalled via the existing measurement request procedures in NRPPA and F1AP, i.e., UL-SRS-RSRPP and UL-RTOA. Hence signalling support of Part A information can be done by re-using the existing Positioning procedures in NRPPa and F1AP for measurement reporting.

- Part A information consists of channel measurements with quality and time stamp information. The channel measurements are defined in time and power measurements. It is FFS if existing measurements (UL-RTOA, UL-SRS-RSRPP) defined in F1AP/NRPPa are enough to be re-used, or if new channel positioning measurements will be defined.

- The LMF (and gNB-CU) will indicate sampling parameters when requesting for the channel measurements: Nt is the window size over which to select samples, Nt' is the number of samples to report, k is the resolution of the window, i.e. the spacing between samples.

Fig. 1 Summary of Case 3b data collection (only signalling of part A is relevant to RAN3).

Observation 1: For Case 3b, both NRPPa and F1AP Measurements procedures will need to be impacted to introduce channel based measurement (FFS if new measurements) and sample parameters (Nt, Nt', k)

Proposal 1: it proposed to re-use the Measurement procedure defined in NRPPa and the Positioning Measurement procedure defined in F1AP to support requesting and reporting of the channel measurements to LMF for Case 3b.

Regarding model performance monitoring, the LMF will compare the inferred UE location using information from part B coming from UE/PRU/LMF, this will involve signalling outside of RAN3 scope.

For model inference in Case 3b, the information needed by the LMF is the same as, or a subset of, Part A, identified for model training tasks. Therefore, RAN3 can focus on providing Part A to the LMF, which will also address the model inference case

## 2.2	Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

In AI/ML assisted positioning Case 3a, the NG-RAN hosts the AI/ML model and performs inference to generate predicted positioning measurements based on input data that can be generated by the gNB/TRP. This case needs some discussion since the WID stipulates that the AI/ML model is at gNB, but in RAN3 we have the high layer split between the gNB-CU and gNB-DU. Hence it is not clear whether the AI/ML model for case 3a should be at gNB-CU or gNB-DU. While it should be possible to have the model hosted at both entities, this will create two sub-cases for case 3a to support, which will increase specification work and implementation complexity. Therefore, it is necessary to decide on one entity only that hosts the AI/ML in case 3a.

Observation 2: It is not clear from the WID objective whether the AI/ML model for case 3a should be at gNB-CU or gNB-DU. Only one entity should be agreed to host the AI/ML model for Case 3a.

In terms of capacity, the gNB-CU is a more centralised node that can access more easily to higher processing and storage capacity than the gNB-DU. Moreover, the gNB-CU can aggregate positioning measurements from multiple gNB-DUs, therefore gaining a much wider range of information (that could be used e.g. as model inputs) as well as gaining a much more complete view of the network. On the contrary, the gNB-DU is limited to its own cells, and can hence gather less positioning measurements compared to the gNB-CU. In addition, if the AI/ML model is placed at the gNB-DU, it will increase the cost for maintaining, training and updating the models across different gNB-DUs, which is not cost-effective.

Observation 3: The gNB-CU can collect data from multiple gNB-DUs, and has hence more information to infer SRS measurements from channel measurements coming from multiple sites; while gNB-DU will be limited to inference based on measurements from its own cells. Having the AI/ML at the gNB-DU is not cost-effect in terms of OPEX.

In terms of AI/ML training, the gNB-CU is a much better suited node to host training functions because it can, as already mentioned, collect training data from a lot more sites and cells than a gNB-DU, as well as can have access to much better processing and storage resources to run training processes.

Finally, we would like to point out that all of the AI/ML based use cases discussed by RAN3 so far have the AI/ML training and inference functions at the gNB-CU, hence it would be advantageous to maintain this functional support valid also for AI/ML for Positioning, use case 3a.

Proposal 3: For case 3a, the AI/ML model (inference and training functions) is at the gNB-CU.

Having proposed to address this first issue, we list below the relevant RAN1 agreements for Case 3a:

Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

From the RAN1 agreements above, we can make the endeavour to summarize below the RAN3 impacts into two parts: Model training and model inference.

### 2.1.1 Model Training for Case 3a:

Data Collection can happen before model training, where the NG-RAN (gNB-CU) receives ground truth labels from the LMF. In our view, for training data collection the LMF has to indicate first to NG-RAN that data (i.e., ground truth) is available, since in NRPPa a transaction can only be initiated by the server – see TS 38.305, and also because whether data can be collected for a specific UE is subject to user consent, which the LMF can check with CN functions hosting such information.

Observation 4: Data Collection for Case 3a can happen before the inference takes place. The LMF needs to indicate to NG-RAN that training data is available, so that the NG-RAN can request for Data collection.

To achieve this purpose, we can specify a class 2 procedure “Data Collection Information Notification”, that is initiated by the LMF indicating the availability of ground truths information. Then, the NG-RAN can trigger a procedure similar to what has been introduced in support for AI/ML over the Xn interface, namely a new class 1 procedure with DATA COLLECTION INFORMATION REQUEST message, with “start” indication, and indicating the training data it wishes and the periodicity at which it wishes to receive reporting of the training data. The LMF replies with a DATA COLLECTION INFORMATION RESPONSE message to acknowledge the request.

Afterwards, a second class2 procedure, with DATA COLLECTION INFORMATION UPDATE message, can be defined where the LMF provides the training data to the NG-RAN at the requested periodicity, and which includes the requested (and previously indicated available) ground truths, such as positioning measurements or UE location (if consent for this information was given). Similarly to the Xn solution, the NG-RAN can send a DATA COLLECTION INFORMATION REQUEST message with a “stop” flag to indicate the LMF to stop the reporting.

In case the LMF during a reporting of positioning information, can no longer provide it, example due to UE’s consent being revoked. The LMF can send a class 2 DATA COLLECTION INFORMATION FAILURE INDICATION message.

Fig 3, summarizes the procedures mentioned above. Note the if the model is hosted at the gNB-CU, we gain a very considerable signalling reduction over the F1.

Fig.3 Signalling to support Training Data collection for Case 3a

Proposal 4: RAN3 to discuss the procedures to support data collection from LMF to NG-RAN in case 3a.

### 2.1.2 Model Inference for Case 3a:

For model inference, the LMF requests the NG-RAN for predicted measurements. Existing NRPPa procedures cannot be reused for this purpose, as we will be mixing the legacy positioning procedures with the new AI/ML prediction functionality. In the work carried out by RAN3 on AI/ML so far, this has been carefully avoided by introducing new procedures whenever the data to be exchanged were the result of inference, rather than pure measurements. Therefore, a better approach is to design new AI/ML positioning procedures separate from the NRPPa procedures for requesting positioning measurements.

This would ensure that, if the AI/ML positioning framework develops, for example by introducing new information for predictions of measurements in the future, it is preferable for these AI/ML procedures to be kept separate so that they can evolve independently without impacting on existing measurement procedures. Additionally, by introducing new procedures, there will not be the problem of ensuring that legacy IEs are correctly interpreted for the new information added..

Observation 5: Considering future possible development of the AI/ML positioning framework, it is best to define new NRPPa procedures that can be used for prediction purpose.

Proposal 5: The existing NRPPa measurement procedures are not re-used for AI/ML positioning inference purposes.

Similarly as for training data, we can define a new set of NRPPa procedures, e.g., Data Collection Transfer procedure that is LMF initiated. The procedure starts with the LMF sending a new NRPPa message to the gNB-CU to request providing the inferred measurements. The LMF can indicate the requested periodicity and in which specific area (list of TRPs and cells IDs) channel measurements can be done. The gNB-CU will then request for channel measurements from the gNB-DU via legacy F1 Positioning Measurement procedures (i.e., request for input data for time and power channel measurements). This latter step is similar to case 3b described in section 2.1.

After receiving the channel measurements from the gNB-DU and the inference is performed, the gNB-CU sends a NRPPa response message to the LMF to acknowledge the request is successful (NOTE the response message could also be sent before the F1 signalling happens), and provides the predicted measurements (e.g., predicted UL-RTOA, predicted gNB Tx-Tx time difference) via a new NRPPa Update message to the LMF, at the requested periodicity, including a LOS/NLOS indicator and time stamp. Fig.4 summarizes the NRPPa - F1AP interactions:

Fig.4 Signalling to support Model Inference for Case 3a

Observation 6: Case 3a and case 3b share similar signalling over F1AP for channel measurement reporting.

In case of periodical reporting and if the gNB-CU cannot report the predicted measurements due to, e.g. model unavailability, or if more model training data is needed, or channel measurements are absent, then the gNB-CU can indicate so by sending a new Failure Indication message to the LMF with a new cause value.

Proposal 6: RAN3 to discuss defining new NRPPa Data Collection Positioning procedures (Request/Response/Update/failure/Failure indication) for the purpose of reporting predicted measurements and indication of prediction failure to LMF.

Alternatively to the signalling structure in Fig. 4 where the structure of “Xn-like Data Collection” is re-used , the new procedures could follow a similar structure as the “NRPPa Measurement Request/Response/failure/abort/Failure Indication/Report/update” messages.

A further point to discuss is that RAN1 leaves it to RAN3 to decide on the option for model performance monitoring. In our view, we can support Option A, since anyway:

- The LMF provides training data; and the gNB makes its own monitoring.

- The gNB provides the inferred measurements results to LMF and can request the LMF in the response message to perform monitoring metric calculation. Based on the feedback from LMF, which can be an indication of data collection being available as in step 2 of Fig. 3, the gNB can assess the accuracy of the prediction, re-train the model, etc. and performs monitoring metric calculation for its own model.

It is actually unclear how Option B would work. In Option B the LMF would perform monitoring for the model hosted and owned by the gNB. What would the LMF do with the results of the monitoring when the LMF does not know what the RAN model implementation is and how it can be improved? 
The AI/ML framework in RAN3 has always assumed that the node that hosts the model performs performance monitoring, therefore we should reuse this principle because the node that hosts the model knows the details of the model implementation and can use feedback to e.g. retrain or adapt its model.

For these reasons it is proposed to agree that gNB makes its owns monitoring metric calculation and that Option B in RAN1 agreements is not pursued.

Proposal 7: the NG-RAN performs monitoring metric calculation for its own model (Option A in RAN1 agreements). Inform RAN1 of RAN3’s decision.

Proposal 8: RAN3 to not pursue Option B for model performance monitoring.

## 2.3	Way forward:

A draft CR to NRPPa is provided in [2].

We proposed to send a LS to RAN1 on RAN3 agreements.

# 3	Conclusion

In this contribution, we have made the following observations and propose the following

Observation 1: For Case 3b, both NRPPa and F1AP Measurements procedures will need to be impacted to introduce channel based measurement (FFS if new measurements) and sample parameters (Nt, Nt', k)

Proposal 1: it proposed to re-use the Measurement procedure defined in NRPPa and the Positioning Measurement procedure defined in F1AP to support requesting and reporting of the channel measurements to LMF for Case 3b.

Observation 2: It is not clear from the WID objective whether the AI/ML model for case 3a should be at gNB-CU or gNB-DU. Only one entity should be agreed to host the AI/ML model for Case 3a.

Observation 3: The gNB-CU can collect data from multiple gNB-DUs, and has hence more information to infer SRS measurements from channel measurements coming from multiple sites; while gNB-DU will be limited to inference based on measurements from its own cells. Having the AI/ML at the gNB-DU is not cost-effect in terms of OPEX.

Proposal 3: For case 3a, the AI/ML model (inference and training functions) is at the gNB-CU.

Observation 4: Data Collection for Case 3a can happen before the inference takes place. The LMF needs to indicate to NG-RAN that training data is available, so that the NG-RAN can request for Data collection.

Proposal 4: RAN3 to discuss the procedures to support data collection from LMF to NG-RAN in case 3a.

Observation 5: Considering future possible development of the AI/ML positioning framework, it is best to define new NRPPa procedures that can be used for prediction purpose.

Proposal 5: The existing NRPPa measurement procedures are not re-used for AI/ML positioning inference purposes.

Observation 6: Case 3a and case 3b share similar signalling over F1AP for channel measurement reporting.

Proposal 6: RAN3 to discuss defining new NRPPa Positioning Data Collection procedures (Request/Response/Update/failure/Failure indication) for the purpose of reporting predicted measurements and indication of prediction failure.

Proposal 7: the NG-RAN performs monitoring metric calculation for its own model (Option A in RAN1 agreements). Inform RAN1 of RAN3’s decision.

Proposal 8: RAN3 to not pursue Option B for model performance monitoring.

# 4	References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R3-245473, Support of AI/ML Positioning, NRPPa BL CR (Ericsson)

# 5	Annex – LS to RAN1

3GPP TSG-RAN3 Meeting # 125bis 	R3-24xxxx

# Hefei, P. R. China,  14th – 18th October, 2024

# Title:	LS reply on RAN3 agreements on AI/ML positioning

# Response to:

# Release:	Release 19

# Work Item:	NR_AIML_air-Core

Source:	RAN3

To:	RAN1, RAN2

Cc:

Contact Person:

#### Name:	Yazid Lyazidi

#### Tel. Number:

#### E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN3 has started the discussions on support of Case3 a and Cse 3b for AI/ML positioning.

RAN3 has made the following agreements:

Case 3b, direct AI/ML positioning, LMF-sided model: - For Case 3b, both NRPPa and F1AP Measurements procedures will be impacted to support requesting and reporting of the channel measurements to LMF for Case 3b. - The sample parameters (Nt, Nt', k) are introduced in the NRPPa and F1AP measurement request messages. Case 3a, assisted AI/ML positioning, NG-RAN hosted model: - For case 3a, the AI/ML model (inference and training functions) is at the gNB-CU. - RAN3 will introduce new procedures Data Collection Information Initiation to support data collection from LMF to NG-RAN in case 3a. - RAN3 will introduce new NRPPa procedures for AI/ML positioning inference information transfer over NRPPa. FFS on naming and messages structure. - Both Case 3a and case 3b share similar signalling over F1AP for channel measurement reporting. - RAN3 has agreed that the NG-RAN performs monitoring metric calculation for its own model (Option A in RAN1 agreements). RAN3 will not pursue Option B for model performance monitoring.

2. Actions:

To RAN1, RAN2.

ACTION: 	RAN3 asks RAN1 and RAN2 to take the above into account.

3. Date of Next TSG RAN WG3 Meetings:

TSG-RAN3 Meeting #126        18th Nov – 22nd Nov, 2024			Orlando, US

TSG-RAN3 Meeting #127        17th Feb – 21st Feb, 2025			Athens, Greece