3GPP TSG-RAN WG3 Meeting #125bis	R3-245474

# Hefei, China, October 14 – 18, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0170 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of AI/ML Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support of AI/ML for positioning. Introduce support for Case 3a NG-RAN-side model and Case 3b LMF-side model |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Case 3b: - Introduce of sample parameters (Nt, Nt', k) in the MEASUREMENT REQUEST message For Case 3a: - Introduce new procedures for Data Collection Information Transfer: - DATA COLLECTION INFORMATION NOTIFICATION, from LMF. - DATA COLLECTION INFORMATION REQUEST, NG-RAN initiated, with RESPONSE and FAILURE messages. - DATA COLLECTION INFORMATION UPDATE, LMF-initiated. - DATA COLLECTION INFORMATION FAILURE INDICATION, LMF-initiated. FFS on messages structure for inferred measurement request, reporting, update and failure. FFS on structure and names on all messages and IEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of AI/ML positioning |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR TBD |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-	Measurement Preconfiguration Information Transfer. This function allows the LMF to request the NG-RAN node to preconfigure and activate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer. This function allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

-	Data Collection Information Transfer: This function allows the LMF to exchange positioning information with the NG-RAN node for the purpose of Data Collection.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Preconfiguration Information Transfer | Measurement Preconfiguration Measurement Activation |
| Area-specific SRS Information Transfer | SRS Information Reservation Notification |
| Data Collection Information Transfer | a) Data Collection Information Notification b) Data Collection Information Initiation b) Data Collection Information Update c) Data Collection Information Failure Indication |

# 8	NRPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| Data Collection Information Initiation | DATA COLLECTION INFORMATION REQUEST | DATA COLLECTION INFORMATION RESPONSE | DATA COLLECTION INFORMATION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Data Collection Information Notification | DATA COLLECTION INFORMATION NOTIFICATION |
| Data Collection Information Update | DATA COLLECTION INFORMATION UPDATE |
| Data Collection Information Failure Indication | DATA COLLECTION INFORMATION FAILURE INDICATION |

Next change

## 8.X	Data Collection Information Transfer

### 8.X.1	Data Collection Information Notification

#### 8.X.1.1	General

The Data Collection Information Notification procedure allows the LMF to indicate the NG-RAN node that positioning information for a list of UEs is available for data collection.

#### 8.X.1.2	Successful Operation

Figure 8.5.1.2.1: Data Collection Information Notification procedure. Successful operation.

The LMF initiates the procedure by sending a DATA COLLECTION INFORMATION NOTIFICATION message to the NG-RAN node. The Data Collection Information List IE in the DATA COLLECTION INFORMATION NOTIFICATION message contains the positioning information for a list of UEs that can be requested for data collection.

#### 8.X.1.3	Unsuccessful Operation

Not applicable.

#### 8.X.1.4	Abnormal Conditions

Void.

### 8.X.2	Data Collection Information Initiation (NG-RAN initiated)

#### 8.X.2.1	General

The Data Collection Information Initiation procedure allows the NG-RAN to request positioning information from the LMF for data collection.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2.1: Data Collection Information Exchange procedure NG-RAN initiated. Successful operation.

The NG-RAN initiates the procedure by sending a DATA COLLECTION INFORMATION REQUEST message to the LMF. If the Registration Request for Data Collection Information IE is set to "start", the LMF shall reply with a DATA COLLECTION INFORMATION RESPONSE message and shall initiate the Data Collection Information Update procedure including the list of requested positioning information indicated in the Requested Positioning Information List IE.

#### 8.X.2.3	Unsuccessful Operation

Figure 8.X.2.3.1: Data Collection Information Exchange procedure NG-RAN initiated. Unsuccessful operation.

If the LMF cannot provide any of the requested positioning information for any of the UEs in the Requested Positioning Information List IE indicated in the DATA COLLECTION INFORMATION REQUEST message, it shall respond with a DATA COLLECTION INFORMATION FAILURE message with an appropriate cause value.

#### 8.x.2.4	Abnormal Conditions

Void.

### 8.X.3	Data Collection Information Update (LMF initiated)

#### 8.X.3.1	General

The Data Collection Information Update procedure allows the LMF to provide positioning information to the NG-RAN for data collection.

#### 8.X.3.2	Successful Operation

Figure 8.X.3.2.1: Data Collection Information Update procedure, LMF initiated. Successful operation.

The LMF initiates the procedure by sending a DATA COLLECTION INFORMATION UPDATE message to the NG-RAN. The DATA COLLECTION INFORMATION UPDATE message contains the requested positioning information for data collection at NG-RAN.

#### 8.X.3.3	Unsuccessful Operation

Not applicable.

#### 8.X.3.4	Abnormal Conditions

Void.

### 8.X.4	Data Collection Information Failure Indication (LMF initiated)

#### 8.X.4.1	General

The Data Collection Information Failure Indication procedure allows the LMF to indicate that it cannot provide any data collection information to the NG-RAN node.

#### 8.X.4.2	Successful Operation

Figure 8.X.4.2.1: Data Collection Information Failure Indication procedure, LMF initiated. Successful operation.

The LMF initiates the procedure by sending a DATA COLLECTION INFORMATION FAILURE INDICATION message to the NG-RAN node.

#### 8.X.4.3	Unsuccessful Operation

Not applicable.

#### 8.X.4.4	Abnormal Conditions

Void.

Next change

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, measure the UL SRS resources from the UE within the indicated time window(s).

If the Data Sampling Parameters IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring sample-based measurement.

Next change

### 9.1.4	Messages for Measurement Information Transfer Procedures

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Data Sampling Parameters (FFS) | O |  |  |  |  |  |
| >>>Nt |  |  |  | Size of the time window over which measurements are collected. FFS on the granularity of the time information |  |  |
| >>>Nt’ |  |  |  | Number of measurement samples to be collected |  |  |
| >>>k |  |  |  | Time between different measurement samples. FFS on the granularity of the time information |  |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

Next change

### 9.1.X	Messages for Data Collection Information Transfer Procedures

#### 9.1.X.1	DATA COLLECTION INFORMATION NOTIFICATION

This message is used to indicate the NG-RAN node that positioning information is available for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Data Collection Information List |  | 1 |  |  | YES | reject |
| >Data Collection Information Item |  | 1 .. <maxnoDataCCollectionTypes> |  |  | EACH | reject |
| >>UE ID | M |  | INTEGER(1..N) |  | - |  |
| >>CHOICE Information Type | M |  |  |  |  |  |
| >>>UE location | M |  | ENUMERATED(true,…) |  | - |  |
| >>>Measurement type | M |  | ENUMERATED(ul-rtoa, gnb-rx-tx-td, all, …) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoDataCollectionTypes | Maximum no. Data Collection information types that can be requested and reported from the LMF. Value is 64. |

#### 9.1.X.2	DATA COLLECTION INFORMATION REQUEST

This message is initiated to request positioning information for data collection.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Registration Request for Data Collection Information | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| TRP Area List |  | 0..1 |  | Indicates the area of interest | YES | ignore |
| >TRP Area Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| Requested Positioning Information List |  | 1 |  |  | YES | reject |
| >Requested Positioning Information Item |  | 1 .. <maxnoDataCCollectionTypes> |  |  | EACH | reject |
| >>UE ID | O |  | INTEGER(1..N) |  | - |  |
| >>CHOICE Information Type | M |  |  |  |  |  |
| >>>UE location | M |  | ENUMERATED(true,…) |  | - |  |
| >>>Measurement type | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement: first bit: UL-RTOA Second bit: gNB Rx-Tx time difference. | - |  |
| >>Periodicity | O |  | ENUMERATED(160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoDataCollectionTypes | Maximum no. Data Collection information types that can be requested and reported from the LMF. Value is 64. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.X.3	DATA COLLECTION INFORMATION RESPONSE

This message is sent to confirm start reporting of the requested positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.X.3	DATA COLLECTION INFORMATION FAILURE

This message is sent to report failure of initiating data collection of positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.X.3	DATA COLLECTION INFORMATION UPDATE

This message is sent to report positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested Positioning Information List |  | 1 |  |  | YES | reject |
| >Requested Positioning Information Item |  | 1 .. <maxnoDataCCollectionTypes> |  |  | EACH | reject |
| >>UE ID | O |  | INTEGER(1..N) |  | - |  |
| >>CHOICE Data Collection Information Type | M |  |  |  | - |  |
| >>>UE location | M |  |  |  |  |  |
| >>>>Location Information | M |  | OCTET STRING | Location of the mobile TRP, Includes the locationEstimate IE as defined in TS 37.355 [14]. | - |  |
| >>>>Time Stamp | O |  | 9.2.42 | Location time stamp. | - |  |
| >>>Measurement | M |  |  |  |  |  |
| >>>>TRP Measurement List |  | 0..1 |  | Indicates the area of interest | YES | reject |
| >>>>>TRP Measurement Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>>>>>TRP ID | M |  | 9.2.24 |  | - |  |
| >>>>>>gNB Rx-Tx Time Difference | O |  | 9.2.40 |  | - |  |
| >>>>>>UL RTOA | O |  | 9.2.39 |  | - |  |
| >>>>>>Time Stamp | O |  | 9.2.42 |  | - |  |
| >>>>>>Measurement Quality | O |  | 9.2.43 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoDataCollectionTypes | Maximum no. Data Collection information types that can be requested and reported from the LMF. Value is 64. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.X.4	DATA COLLECTION INFORMATION FAILURE INDICATION

This message is sent to indicate that the previously requested positioning information can no longer be reported. for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |

Next change

ASN.1 to be added later