3GPP TSG-RAN3 #125-bis	R3-245513

14th October – 18th October 2024

Hefei, China

Agenda Item:	9.2

Source:	Ericsson, InterDigital, Nokia

Title:	Discussion about Stage 2 corrections in AI/ML for NG-RAN

Document for:	Discussion, Decision

# Introduction

In the previous meeting the issue of stage-2 corrections for AI/ML for NG-RAN was discussed and specifically the issue of model performance feedback. In this contribution we will further discuss the issue and look into past discussions and investigations of the topic and finally we will put forward our proposals.

# Discussion

In the previous meeting a discussion on the topic took place triggered by R3-244300. Eventually a revision was produced in R3-244781, where some examples of model performance feedback are provided. However, no consensus could be reached and finally the following was captured:

Opt1: Start from the following wording: The exchange of model-specific performance information over NG-RAN interfaces is out of scope.

Opt2: Remove the sentence in TS and minute the common understanding in Chair note

Opt3: Whether to consider OAM and RAN interface together?

In order to solve this issue in the best possible way, we went back to the discussions RAN3 had during Rel-17 when this topic was discussed heavily leading to agreements that eventually led in their turn to the text captured in TS 38.300. In the following we give a short overview of the discussions with the intention to bring some clarity and better understanding.

## 2.1	Model performance feedback in Rel-17

One of the point of discussion during RAN3-125 has been that RAN3 did not discuss model performance feedback in the past and that, as a consequence, a sentence in TS38.300 stating that model performance feedback is out of scope is ambiguous as RAN3 did not develop an understanding of what model performance feedback is. To clarify that RAN3 has discussed at length what model performance feedback is and what are the possible metrics that could represent it, we dug into RAN3´s history and provided the results below.

Going back to the contributions submitted during 2021 in Rel-17 we find a lot of detailed discussions about model performance feedback. As an example, we can mention R3-213196 that provided an overview of the most popular performance evaluation metrics for each task type and the official definition for each of the metric. In particular, classification metrics such as Accuracy ratio, Precision, Recall and F1 score were presented. Also, regression metrics such as Mean Absolute Error, Mean Square Error and Root Mean Square Error and metrics for online iterative optimization such as Dispersion and Risk were analysed.

Also, in R3-215330 examples of model performance were given. For instance: “In the example of traffic load prediction, the model performance could be evaluated by the accuracy or confidence interval by comparing the predicted traffic load with the actual measured traffic load in a certain time period. And in a reasonable implementation, the traffic load prediction will make use of the historical measured traffic load, which means the historical traffic load measurement will be provided to the model inference function as “Inference Data” and by nature model inference function can determine the model performance in a certain time period.

In another example of handover decision making, wherein reinforcement learning could be used. In this case, whether the last time handover decision is successful or unsuccessful can be provided to the model training function as part of “Training Data”.  There might be unnecessary to define a model performance feedback explicitly in this case since “Training Data” may serve for the same purpose.“

A last example is R3-215477 from where we cite the following excerpt:

“Another aspect that remains FFS is the Model Performance Feedback. Model Performance is an important aspect of ML Model workflow and relates to fluctuations of Model Performance over time due to changes in the environment after a ML Model is deployed. Those fluctuations may affect ML Model accuracy or suitability to a problem. To address Model Performance fluctuations (also known as ML Model Drift) continuous ML Model Monitoring may be needed. Through this monitoring, Model Performance is evaluated after an action is taken and a prediction is compared to real data (ground truth).

Model Performance Feedback can be provided by evaluating different metrics that depend on the exact ML model and ML Algorithm under consideration. Even though Model Performance Feedback is an important aspect in the ML Workflow, exact ML Models and Algorithms are not in the scope of the study item.”

It is worth noticing that the contributions quoted above are only some of the many contributions discussing model performance feedback, which was indeed a topic of discussion for one of the Agenda Items during the Rel17 work.

Based on all the contributions and subsequent discussions the following agreements were captured in the Chair notes as outcome of discussions in the Rel17 study:

From RAN3-114bis-e:

Performance monitoring of the trained and deployed RAN AI/ML in Model Inference shall be supported, Model Performance Feedback from Model Inference to Model Training shall be kept and FFS shall be removed.

Replace the definition of Model Performance Feedback as below:

Model Performance Feedback: It may be used for monitoring the performance of the AI/ML model.

Note: Details of the Model Performance Feedback process are out of RAN3 scope.

From RAN3-115-e:

Mark Model Performance Feedback in the Functional Framework figure with a dashed arrow.

Capture in the Chairman notes that the agreement taken during RAN3 #114bis-e is updated as follows:

Performance monitoring of the trained and deployed RAN AI/ML in Model Inference may shall be supported, Model Performance Feedback from Model Inference to Model Training shall be kept and FFS shall be removed.

It is confirmed that Inference Data comprises necessary information for Model Inference to calculate Model Performance Feedback in addition to Model Output. There is no need for further clarifications in the TR.

Update the TP for TR 37.817 as follows:

- Clarify in the TR the fact that Model Performance Feedback is optional by further stating the following: “Model Performance Feedback:  It may be used for monitoring the performance of the AI/ML model, when available.”

With all the above said, it is obvious that the topic of model performance feedback has been studied and analysed extensively and thoroughly in Rel-17.

- Observation 1   The topic of model performance feedback has been studied and analysed extensively and thoroughly in Rel-17.

It is also obvious that RAN3 has discussed specific metrics to represent Model Performance Feedback, such as those listed in R3-213196.

Observation 2   RAN3 has discussed metrics that could be used to represent model performance feedback.

Finally after thorough investigation RAN3 took an agreement not to standardise implementation specific model performance feedback.

- Observation 3   Based on the studies and analysis carried out, RAN3 took an agreement not to standardise implementation specific model performance feedback.

The current text in TS38.300 quotes the following:

“AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.”

This statement in TS 38.300 is too generic and not specific to the discussion RAN3 had, which was aimed at making out of scope of RAN3 the exchange of Model Performance Feedback metrics.

Therefore, it would be useful to provide some clarification to the text in TS38.300, to make it more specific to the work RAN3 is responsible. For clarity, it would also be beneficial to mention examples of Model Performance Feedback that RAN3 would consider out of scope for exchange over the RAN interfaces. 
To that end we suggest to amend the text in TS38.300 so to state that the exchange over RAN terminated interfaces of Model Performance Feedback is out of scope (namely limit the sentence to the work RAN3 is responsible for) and to include specific examples of the model performance feedback metrics, similar to the examples discussed in Rel17 and on the basis of which the decision not to specify model performance feedback was taken.

Provide clarification in TS 38.300 regarding the statement about model performance feedback by confining it to the work RAN3 is responsible for and by giving examples of model performance feedback metrics.

- In TS 28.105 we can read the following:

ML model training model performance indicators: performance indicators of the ML model itself, including but not limited to:

-	Accuracy indicator,

-	Precision indicator,

-	Recall indicator,

-	F1 score indicator,

-	MSE (Mean Squared Error) indicator, and

-	MAE (Mean Absolute Error) indicator,

-	RMSE (Root Mean Square Error) indicator.

- We note that the above model performance feedback indicators map very well with those discussed in RAN3, e.g. as part of R3-215330, and therefore reflect very well past discussions in RAN3.

- We suggest to provide reference to the above text in TS 28.105 in order to provide clarification and better understanding of statement in TS 38.300.

Provide reference to the above text in TS 28.105 in order to provide clarification and better understanding of statement in TS 38.300.

# Conclusion

In this contribution we discussed the model performance feedback and based on the discussion we made the following observations and proposals:

- Observation 1   The topic of model performance feedback has been studied and analysed extensively and thoroughly in Rel-17.

- Observation 2   RAN3 has discussed metrics that could be used to represent model performance feedback.

- Observation 3   Based on the studies and analysis carried out RAN3 took an agreement not to standardise implementation specific model performance feedback.

- Provide clarification in TS 38.300 regarding the statement about model performance feedback by confining it to the work RAN3 is responsible for and by giving examples of model performance feedback metrics.

Provide reference to the above text in TS 28.105 in order to provide clarification and better understanding of statement in TS 38.300.

A TP to TS 38.300 covering the above can be found in R3-245514.

RAN3 to agree the TP to TS 38.300 in R3-245514.