3GPP TSG-RAN WG3 Meeting #125-bis	R3-245519

Hefei , China, 14th – 18th Oct 2024

Title: 	Discussion on Support of regenerative payload for NR NTN

Source: 	CMCC

Agenda item: 	14.3

Document Type:	Discussion

- Introduction

In the 3GPP TSG RAN Meeting #103, the work plan for Non-Terrestrial Networks (NTN) for NR Phase 3 has been approved in RP-240775. One of the objectives is support of regenerative payload [RAN3, RAN2, RAN4]:

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

- Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

Based on it, in RAN3 #125 meeting, the following conclusions are captured:

NG Interface management Issue:

Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

Further discussions on benefits to introduce Option 3 NG Suspend/Resume for NG interface management are not precluded but there are oppositions of some companies to have 2 methods for NG interface management.

Inactive UE support

RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

Feeder link switch issue

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

2	Discussion

So based on the conclusions from RAN3#125-bis, in this contribution, we will focus on the discussion of NG interfaces management solution based on requirement inputs from satellite operators.

2.1 NG Removal Procedures Enhancement

It has been agreed in RAN3 #125 to introduce NG Removal for Regenerative Payload, to enhance the NG interface management, avoid too many SCTP shutdown messages for bunch of NG connections at the same time.

Since in NR NTN Regenerative Payload, gNB on board is moving and AMF remains the same, gNB knows exactly when all the UE have been handovered successfully and NG resource can be removed. So it is straightforward and safe to let gNB on board initiate the NG Removal procedures.

Though AMF may know the pre-configured gNB location through the moving orbit information, it is more direct to let gNB initiate this procedure when it is moving out of the AMF serving area. In some cases, if AMF initiated from AMF but the UE is not transferred to new gNB, gNB may have to terminate the NG Removal procedures.

Proposal 1: The NG Removal procedures shall be initiated from satellite (gNB on board) to AMF.

To remove all the NG interfaces between gNB and AMF at one time, the NG Removal procedures shall be non-UE-associated. Currently it is only used in NTN scenario, whether it can be used in somewhere else can be future discussed.

Proposal 2: The new introduce NG Removal procedures for NTN shall be non-UE associated.

However, in some scenarios it is also useful introduce UE-associated NG Removal Procedures. Since the satellite gNB usually covers a wide area, it may take several hours from the satellite moves into an AMF service area till it moves out. In this scenario, some UE on the boarder should first be changed to new gNB while others can still stay on the current gNB. So when parts of UE on the gNB on board handover to a new gNB successfully, the gNB on board can remove the NG connections between AMF for this part of UE at the same time.

Currently in TS38.413, NG reset procedure has been defined from gNB to AMF, to release the NG connections as following:

8.7.4.2.2	NG Reset initiated by the NG-RAN node

Figure 8.7.4.2.2-1: NG reset initiated by the NG-RAN node: successful operation

In the event of a failure at the NG-RAN node which has resulted in the loss of some or all transaction reference information, an NG RESET message shall be sent to the AMF.

At reception of the NG RESET message the AMF shall release all allocated resources on NG related to the UE association(s) indicated explicitly or implicitly in the NG RESET message and remove the NGAP ID for the indicated UE associations.

After the AMF has released all assigned NG resources and the UE NGAP IDs for all indicated UE associations which can be used for new UE-associated logical NG-connections over the NG interface, the AMF shall respond with the NG RESET ACKNOWLEDGE message.

If the NG RESET message contains the UE-associated Logical NG-connection List IE, then: The AMF shall use the AMF UE NGAP ID IE and/or the RAN UE NGAP ID IE to explicitly identify the UE association(s) to be reset.

9.2.6.11	NG RESET

This message is sent by both the NG-RAN node and the AMF to request that the NG interface, or parts of the NG interface, be reset.

Direction: NG-RAN node  AMF and AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| CHOICE Reset Type | M |  |  |  | YES | reject |
| >NG interface |  |  |  |  |  |  |
| >>Reset All | M |  | ENUMERATED (Reset all, …) |  | - |  |
| >Part of NG interface |  |  |  |  |  |  |
| >>UE-associated Logical NG-connection List | M |  | 9.3.3.25 |  | - |  |

9.3.3.25	UE-associated Logical NG-connection List

This IE contains a list of UE-associated logical NG-connections.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE-associated Logical NG-connection Item |  | 1..<maxnoofNGConnectionsToReset> |  |  |
| >AMF UE NGAP ID | O |  | 9.3.3.1 |  |
| >RAN UE NGAP ID | O |  | 9.3.3.2 |  |

As above, the message IE includes a list of UE-associated Logical NG-connection list, which means it can remove NG connections for a group of UE at one time. So the legacy NG reset procedures to remove the NG connections between AMF and gNB can be reused in NTN regenerative payload, to remove the NG connections step by step. Using the legacy NG reset procedures can manage these NG connections for different UEs more effectively and precisely, which reduces the signal interworking and improve the energy efficiency and satellite resource usage.

Proposal 3: The NG removal procedure shall be gNB-AMF pair level (non-UE-associated). However, it may be beneficial to introduce NG Removal procedure for a list of UEs in some scenarios.

Proposal 4: It is proposed to re-use the legacy NG reset procedures from gNB to AMF to remove a list of UE-associated logical NG-connections on the satellite (gNB on board).

Anyway, whether initiate non-UE associated NG Removal Procedure for all UEs at the same time, or initiate NG Removal Procedure for a list of UEs is up to the satellites’ implementation.

Proposal 5: It is up to satellite (gNB on board)’s implementation whether to initiate non-UE-associated NG Removal for all UEs at the same time or NG Removal Procedures for a list of UEs.

Meanwhile, when the gNB on board re-enters into the AMF serving area, it shall setup NG connection automatically. The legacy NG Setup procedure can be reused.

Scenario 1: UE transfer from Satellite1(gNB1 on board) to Satellite2(gNB2 on board) through Xn when it is leaving the AMF serving area, UE-associated NG Removal Procedure

Scenario 2: UE transfer from Satellite1(gNB1 on board) to Satellite2(gNB2 on board) through AMF when it is leaving the AMF serving area, UE-associated NG Removal Procedure

2.2 NG Suspend/Resume Procedures Enhancement

To recover the UE connections quickly, some companies suggest to introduce the NG Suspend and Resume Procedures.  When the satellite is moving out of the service area of AMF, it switches UEs over to the new gNB through Xn or through AMF. After UE Handover execution, the gNB can sends NG Suspend procedures to AMF and restore the UE context such as UE ID, PDU session ID and some others in the satellite (gNB on board), NG connections will be suspended. And when the satellite moves back to the previous AMF, it can quickly set up NG, RRC connections for the UE through NG Resume messages.

Proposal 6: It is beneficial to introduce NG Suspend/Resume procedures in some scenarios, to help the NG connection recover quickly.

The NG-RAN node ID, UE ID, PDU Session ID shall be carried in NG Resume message from NG-RAN node to AMF, to quickly recover the NG connection.

When AMF receive this NG Resume message, it shall check the UE ID and PDU session ID in its own UE context information, and know which PDU sessions are still active and shall be resumed.

Proposal 7: It is proposed to carry the NG-RAN node ID, UE ID, PDU Session ID in NG Resume message from NG-RAN node to AMF. Using these information, AMF can check with its own UE context information and know which PDU sessions shall be resumed.

Scenario 3: UE transfer from Satellite1(gNB1 on board) to Satellite2(gNB2 on board)  through Xn when it is leaving the AMF serving area, NG Suspend/Resume Procedures

Scenario 4: UE transfer from Satellite1(gNB1 on board) to Satellite2(gNB2 on board)  through AMF when it is leaving the AMF serving area, NG Suspend/Resume Procedures

However, some satellite companies think the storage and energy resource is very expensive and limited in satellite, store the so many UE context in satellite for such a long time cost too much.

Also some other companies think the UE won’t stay so long under the same coverage area for after the satellite moves back.  It is meaningless to store the UE context in satellite for such a long time.

So it is better to let the satellite decides whether it shall active the NG Suspend/Resume Procedures for quick recovery according to its own resource status and its moving trajectory.

Proposal 8：It is up to satellite (gNB on board) to make decision whether and when to activate NG Suspend/Resume Procedures.

2.3 NG Pre-Set Up Procedures Enhancement

In above procedures, to guarantee the continuity of UE service, the new satellite (gNB on board) needs to set up NG connection with AMF before the previous satellite (gNB on board) is leaving the AMF serving area.

Usually, if the ephemeris is provisioned, AMF know when to initiate the setup of NG connections with satellites in advance. If not, satellite can be triggered to setup NG with AMF in advance according to time and location condition which is provisioned, before the previous satellite leaves the AMF serving area.

The procedures for NG pre-setup we proposed can refer to the NTN time or location triggered CHO procedures via Xn. Time-based or location-based triggerment may be configured independently for NG pre-setup in NTN in at least hard satellite switch case where the service discontinuity gap time length is assumed to be zero or negligible.

Here the satellite (gNB on board) still keeps the NG connections with the previous AMF when it sets up the NG connections with the new AMF. The old NG connections will be released or deleted after UE HO, as procedures proposed in chapter 2.1 and 2.2.  Since at this moment, there is no UE context in the new satellite (gNB on board), so the pre-set up NG interfaces are gNB and AMF pair level.

Proposal 9: It is proposed to introduce the time and location-triggered NG pre-setup before the UE HO to the new satellite (gNB on board), to guarantee the UE service continuity.

3	Summary

In this contribution, we provide considerations on the NTN regenerative payload in RAN3, following observations and proposals are made:

Proposal 1: The NG Removal procedures shall be initiated from satellite (gNB on board) to AMF.

Proposal 2: The new introduce NG Removal procedures for NTN shall be non-UE associated.

Proposal 3: The NG removal procedure shall be gNB-AMF pair level (non-UE-associated). However, it may be beneficial to introduce NG Removal procedure for a list of UEs in some scenarios.

Proposal 4: It is proposed to re-use the legacy NG reset procedures from gNB to AMF to remove a list of UE-associated logical NG-connections on the satellite (gNB on board).

Proposal 5: It is up to satellite (gNB on board)’s implementation whether to initiate non-UE-associated NG Removal for all UEs at the same time or NG Removal Procedures for a list of UEs.

Proposal 6: It is beneficial to introduce NG Suspend/Resume procedures in some scenarios, to help the NG connection recover quickly.

Proposal 7: It is proposed to carry the NG-RAN node ID, UE ID, PDU Session ID in NG Resume message from NG-RAN node to AMF. Using these information, AMF can check with its own UE context information and know which PDU sessions shall be resumed.

Proposal 8：It is up to satellite (gNB on board) to make decision whether and when to activate NG Suspend/Resume Procedures.

Proposal 9: It is proposed to introduce the time and location-triggered NG pre-setup before the UE HO to the new satellite (gNB on board), to guarantee the UE service continuity.

4	Reference

[1] 	RP-240775, work plan for NTN

[2]       TS 38.300, NG-RAN stage2

[3]       TS 38.423, Xn application protocol (XnAP)

[4]       TS 38.413, NG application protocol (NGAP)

[5]       TR 38.821, Solutions for NR to support NTN